3GPP TSG-RAN WG2 Meeting #131	R2-2505877

Bangalore, India, August 25th – 29th, 2025

Agenda Item:	8.3.3

Source:	InterDigital Inc.

Title:	Network side model for AI/ML Mobility

Document for:	Discussion

# 1. Introduction

In RAN2-130, several agreements were made regarding the network sided model for AIML mobility [1]. In this contribution, we discuss some of the remaining issues.

# 2. Discussion

The following agreements were made in RAN2-130 regarding inference and performance monitoring for network sided model [1]:

Agreements (inference):

- RAN2 confirms that UE will not be informed about any network-sided functionality management decision (e.g., selection, (de)activation, switching, fallback, etc.) for AI mobility use case

- For performance monitoring of NW-side model, UE can provide the label data (i.e., actual measurement results) for gNB. The existing measurement/report configuration can be reused to configure UE to report the actual measurement. FFS on UE awareness and preference for NW sided model

Agreements (performance monitoring):

- RAN2 confirms that UE will not be informed about any network-sided functionality management decision (e.g., selection, (de)activation, switching, fallback, etc.) for AI mobility use case

- For performance monitoring of NW-side model, UE can provide the label data (i.e., actual measurement results) for gNB. The existing measurement/report configuration can be reused to configure UE to report the actual measurement. FFS on UE awareness and preference for NW sided model

As shown in the agreements above, the network needs the actual measurements from the UE (i.e., ground truth) to do the performance monitoring. One way to do so could be to employ the legacy periodic reporting. However, configuring the UE for frequent periodic measurement reporting for the purpose of performance monitoring may lead to high signalling overhead and it may be contradictory to some of the goals of AI/ML based mobility (e.g., UE power saving, reduction of measurement reports/signalling, etc.,).

Observation 1: Configuring periodic measurement reports from the UE, especially for the sake of performance monitoring of a network sided model, is highly inefficient.

Thus, a more optimal approach will be for the network to request the measurements on demand, whenever it wants some ground truth for doing the performance monitoring. Since it has already been agreed (for the case of inference input) that a single measurement report can contain signal levels from multiple time instances for a given cell, this can be extended to enable the network to request the UE to report measurements over a certain time duration (e.g., for the past x seconds, for the next seconds, etc.,). For example, the network could configure the UE to keep a (moving) window of measurement data logged and it can request it at any time to send the last logged window of data. Alternatively, the network can request the UE to log and report the measurements for a certain time duration window in the future.

Thus, we propose:

Proposal 1: The network can request the UE for an on-demand measurement report for a given time duration (past or future time durations).

Proposal 2: The network can configure the UE to log a moving window of measurements for a certain time duration.

Proposal 3: The UE responds to the on-demand measurement report request by sending a report that contains the RRM measurement results of the concerned cells for the requested time duration

- If the requested time duration is in the past, UE sends the latest window of logged measurements

- If the requested time duration is in the future, UE logs the measurements for the requested time duration and sends it.

Several agreements were also made for data collection for the network sided model [1], which were mostly aligned with the agreements for the BM case in the AI/ML PHY WI.

Whether it is to enable the inference, performance monitoring or data collection for the AI/ML mobility on the network side, the UE’s involvement is just the sending of (enhanced) measurement reports to the network. Thus, a UE that doesn’t have/support a UE sided model for AI/ML mobility may support the enhanced logging and/or measurement reporting to enable a network sided model for AI/ML mobility. Similarly, a UE with a model for RRM measurements or measurement event prediction may not support the enhanced logging and/or measurement reporting needed to enable an AI/ML mobility model on the network side.

Observation 2: To enable the inference/performance monitoring via a network sided model and data collection for the training of a network sided model, what is needed from the UE is enhanced measurement reporting (i.e., no AI/ML model/operation required at the UE).

Proposal 4: A UE may support enhanced measurement logging/reporting for enabling LCM of a network sided AIML model (e.g., inference, performance monitoring, data collection for training, etc.,) without having AI/ML support on its side (e.g., no AI/ML mobility model/capability) or vice versa.

# 3. Conclusion

In this contribution, we discussed several aspects of network sided model for AI/ML mobility and the following observations and proposals were made:

Observation 1: Configuring periodic measurement reports from the UE, especially for the sake of performance monitoring of a network sided model, is highly inefficient.

Observation 2: To enable the inference/performance monitoring via a network sided model and data collection for the training of a network sided model, what is needed from the UE is enhanced measurement reporting (i.e., no AI/ML model/operation required at the UE).

Proposal 1: The network can request the UE for an on-demand measurement report for a given time duration (past or future time durations).

Proposal 2: The network can configure the UE to log a moving window of measurements for a certain time duration.

Proposal 3: The UE responds to the on-demand measurement report request by sending a report that contains the RRM measurement results of the concerned cells for the requested time duration

- If the requested time duration is in the past, UE sends the latest window of logged measurements

- If the requested time duration is in the future, UE logs the measurements for the requested time duration and sends it.

Proposal 4: A UE may support enhanced measurement logging/reporting for enabling LCM of a network sided AIML model (e.g., inference, performance monitoring, data collection for training, etc.,) without having AI/ML support on its side (e.g., no AI/ML mobility model/capability) or vice versa.

# 4. References

[1]	RAN2 #130 Chairman Notes, Malta, May 2025.