3GPP TSG-RAN WG2 Meeting #131	R2-2505882

# Bengaluru, India, 25th – 29th August 2025

Agenda Item:	8.7.5

Source:	Ericsson

Title:	Remaining Open Issues on RLC Enhancements

Document for:	Discussion, Decision

# 1	Introduction

The following are the list of open issues as captured by the rapp relating to UE capability.

(UE capability-01): Whether a UE supporting multipleEntryDelayStatusReport-r19 shall also indicate support of delayStatusReport-r18.

(UE capability-04): Whether a UE supporting txRLC-StopReTxDiscardedSDU-r19 shall also indicate support of rxRLC-Discard-r19.

(UE capability-05): Whether maximum number of QoS flows subject to rate control should be defined as UE AS capability and the value range of the capability (if to be defined).

The following from the RLC rapp:

| Index | Issue description | Rapporteur suggestion |
| --- | --- | --- |
| RLC-10 | Whether/How to introduce any limitations or associations for the configuration of stopReTxObsoleteSDU and DL t-RxDiscard. - As raised by ZTE below | Issue Type: Not essential not important How to address it: based on companies’ contribution |
| RLC-11 | FFS how to handle the case “ if there are only SDUs buffered whose transmissions have been stopped due to discard indication from PDCP, there is no SDU to retransmit the poll with” - As raised by Nokia and Apple below RAN2 agreement in #130 meeting: Working assumption: (RLC-11) No need to address window stalling issue with polling retransmission (TBC next meeting) Assumption from companies is that this should not happen due to SR triggering from Rx side, but it needs to be checked whether this will always work | Issue Type: Not essential but important How to address it: based on companies’ contribution |
| RLC-14 | If stopReTxDiscardedSDU is configured, when indicated from upper layer to discard a particular RLC SDU, whether the transmitting side of an AM RLC entity shall discard the corresponding RLC SDU or RLC SDU segment(s) if the RLC SDU or the RLC SDU segment(s) thereof has been submitted to lower layers. As raised by NEC below | Issue Type: Not essential not important How to address it: based on companies’ contribution |

In this contribution, we discuss the open issues as listed above.

# 2	Discussion

## 2.1 	UE capability-04

(UE capability-04): Whether a UE supporting txRLC-StopReTxDiscardedSDU-r19 shall also indicate support of rxRLC-Discard-r19

From an operation standpoint, the Tx-side feature (stopReTxDiscardedSDU-r19) needs the Rx-side feature (t-RxDiscard) in the peer RLC entity to prevent window stalling. Considering two scenarios:

Scenario-1, UL: If the stopReTxDiscardedSDU-r19 is configured for the UE, the network RLC entity is required to support the Rx-side feature.

Scenario-2, DL: Whether the network wants to perform the stopReTxDiscardedSDU feature is up to implementation provided UE supports the Rx-side feature.

From the two scenarios above, if the UE supports only the Rx-side feature capability, it is up to network implementation to use the Tx-side feature. If the UE supports only the Tx-side feature, the network needs to implement the Rx-side feature, but this is again up to implementation. If a network does not support the Rx-side feature, it will not configure stopReTxDiscardedSDU at the UE. Then, relying on network to do the right thing, capabilities can be kept separate.

(UE capability-04) UE supporting txRLC-StopReTxDiscardedSDU-r19 need not indicate support for rxRLC-Discard-r19.

## 2.2 	RLC-10

Whether/How to introduce any limitations or associations for the configuration of stopReTxDiscardedSDU and DL t-RxDiscard.

Based on the description in Section 2.1, we believe there need not be any limitations or associations for the two configurations.

(RLC-10) No limitations or associations are needed for the configuration stopReTxDiscardedSDU and t-RxDiscard. It is left to network implementation to correctly configure the peer RLC entities.

## 2.3 	RLC-11

FFS how to handle the case “ if there are only SDUs buffered whose transmissions have been stopped due to discard indication from PDCP, there is no SDU to retransmit the poll with”

As discussed in the two sections above, from an operation standpoint, the the Tx-side feature (stopReTxDiscardedSDU) to work needs the peer RLC entity to implement the Rx-side feature (t-RxDiscard). Thus, as discussed in the last meeting with the outcome of a working assumption, we believe that the window stalling issue need not be addressed as the Rx side can always send the status report based on the expiry of t-RxDiscard.

(RLC-11) Confirm the working assumption that there is no need to address window stalling with polling retransmission.

# 3 	Conclusion

Based on the discussion in the previous sections, we have the following observations:

No table of contents entries found.

Based on the discussion in the previous sections, we propose the following:

Proposal 1	(UE capability-04) UE supporting txRLC-StopReTxDiscardedSDU-r19 need not indicate support for rxRLC-Discard-r19.

Proposal 2	(RLC-10) No limitations or associations are needed for the configuration stopReTxDiscardedSDU and t-RxDiscard. It is left to network implementation to correctly configure the peer RLC entities.

Proposal 3	(RLC-11) Confirm the working assumption that there is no need to address window stalling with polling retransmission.