3GPP TSG-RAN WG2 Meeting #131	R2-2505890

Bangalore, India, Aug 25th – Aug 29th, 2025

Agenda item:	8.3.2.3

Source:	Qualcomm Incorporated

Title:	Network-side model for RRM sub-cases 1 and 3

WID/SID:	FS_NR_AIML_Mob – Release 19

Document for:	Discussion and Decision

# Introduction

In the last RAN2 #130 meeting [1], substantial progress was achieved in the study of LCM aspects and specification impacts for AI/ML for mobility, and many agreements were made on Functionality management, scenarios for specification work for AI/ML for mobility, UE configuration and reporting to support a network-side model, Data collection for network-side model, and Performance monitoring. Based on the progress achieved in the SI so far, in the RAN #108 plenary meeting, a WID document [2] for the Rel-20 AI/ML for mobility WI was approved.

In this short contribution, we discuss support of some RRM sub-cases for a network-side RRM measurement prediction model.

# Support of RRM sub-cases 1 and 3 for a network-side RRM measurement prediction model

For network-side RRM measurement prediction, the legacy RRM measurement configuration and RRM measurement reporting framework can be used.   For L1-filtered beam-level RSRP reporting can be configured by setting co-efficient to zero.   FFS if there are specification impacts to support subcase 1 and 3 (if supported) and interference for cell level prediction.

In the last RAN2 #130 meeting [1], it was left FFS whether RRM sub-cases 1 and 3 can be supported for a network-side RRM measurement prediction model with the current RRC specification. Indeed,

Observation: Note 1 of section 5.5.3.2, clearly mentions that ki =0 disables layer 3 filtering:

NOTE 1:	If k is set to 0, no layer 3 filtering is applicable.

Effectively, if ki =0, a = 1/2(0/4)=1, then Fn = (1 – a)*Fn-1 + a*Mn  = (1 – 1)*Fn-1 + Mn = Mn  where Mn is the latest received measurement result from the physical layer.

#### 5.5.3.2 Layer 3 filtering The UE shall: 1> for each cell measurement quantity, each beam measurement quantity, each sidelink measurement quantity as needed in clause 5.8.10, for each CLI measurement quantity that the UE performs measurements according to 5.5.3.1, for each L2 U2N Relay UE measurement quantity according to 5.5.3.4, for evaluating the selected or detected NR sidelink U2N Relay UEs according to 5.8.15.3, for evaluating the SyncRef UE according to 5.8.5 and 5.8.6, for evaluating the NR sidelink U2U Relay/Remote UE threshold conditions according to 5.8.16.2 and 5.8.17.2, for evaluating the conditions for selection and reselection of NR sidelink U2U Relay UE according to 5.8.17.3, and for evaluating the detected NR sidelink U2U Relay UEs according to 5.8.17.4: 2> filter the measured result, before using for evaluation of reporting criteria, for measurement reporting, for U2N/U2U Relay (re)selection evaluation or for evaluating the SyncRef UE, by the following formula: Fn = (1 – a)*Fn-1 + a*Mn where Mn is the latest received measurement result from the physical layer; Fn is the updated filtered measurement result, that is used for evaluation of reporting criteria, for measurement reporting, for U2N/U2U Relay (re)selection evaluation or for evaluating the SyncRef UE; Fn-1 is the old filtered measurement result, where F0 is set to M1 when the first measurement result from the physical layer is received; and for MeasObjectNR, a = 1/2(ki/4), where ki is the filterCoefficient for the corresponding measurement quantity of the i:th QuantityConfigNR in quantityConfigNR-List, and i is indicated by quantityConfigIndex in MeasObjectNR; for other measurements, a = 1/2(k/4), where k is the filterCoefficient for the corresponding measurement quantity received by the quantityConfig; for UTRA-FDD, a = 1/2(k/4), where k is the filterCoefficient for the corresponding measurement quantity received by quantityConfigUTRA-FDD in the QuantityConfig; 2> adapt the filter such that the time characteristics of the filter are preserved at different input rates, observing that the filterCoefficient k assumes a sample rate equal to X ms; The value of X is equivalent to one intra-frequency L1 measurement period as defined in TS 38.133 [14] assuming non-DRX operation, and depends on frequency range. NOTE 1: If k is set to 0, no layer 3 filtering is applicable. NOTE 2: The filtering is performed in the same domain as used for evaluation of reporting criteria, for measurement reporting, for U2N/U2U Relay (re)selection evaluation or for evaluating the SyncRef UE, i.e., logarithmic filtering for logarithmic measurements. NOTE 3: The filter input rate is implementation dependent, to fulfil the performance requirements set in TS 38.133 [14]. For further details about the physical layer measurements, see TS 38.133 [14]. NOTE 4: For CLI-RSSI measurement, it is up to UE implementation whether to reset filtering upon BWP switch. NOTE 5: For SSB measurements when multiple altitude range-based ssb-ToMeasure are configured, it is up to UE implementation whether to reset filtering upon entering a different altitude range. NOTE 6: Upon satellite switch with resynchronization, it is up to UE implementation to reset filtering for the serving cell.

Proposal. Resolve the FFS by concluding that no additional specification changes are needed to support sub-cases 1 and 3 for the network-side RRM measurement prediction model.

# 3 Conclusions

Observation: Note 1 of section 5.5.3.2, clearly mentions that ki=0 disables layer 3 filtering:

NOTE 1:	If k is set to 0, no layer 3 filtering is applicable.

Proposal. Resolve the FFS by concluding that no additional specification changes are needed to support sub-cases 1 and 3 for the network-side RRM measurement prediction model.

# References

[1] Chair notes, RAN2 #130 meeting.

[2] RP-251864, New WI: Artificial Intelligence (AI)/Machine Learning (ML) for mobility in NR.