3GPP TSG-RAN WG2 Meeting #131	R2-2505891

Bengaluru, India, 25 - 29 August 2025

Agenda Item:	8.12.2

Source:	Huawei, HiSilicon

Title:	Remaining issues on Asymmetric DL sTRP/UL mTRP

Document for:	Discussion and Decision

# 1	Introduction

For Rel-19 MIMO, objectives relevant to asymmetric DL/UL TRP deployment are identified to have significant RAN2 impacts. Progress on 2TA study for this topic has been made in previous meetings. In the last meeting, RAN2 agreements were as follows:

Agreements

Introduce a new RRC parameter per BWP that explicitly enables the Rel-19 sDCI-mTRP 2TA, and clarify in FD of tag2 to include all cases where tag2 is configured that “it is optionally configured in a serving cell for mDCI mTRP 2TA if coresetPoolIndex for a BWP is configured with more than one value, and for sDCI mTRP 2TA if [the new parameter] is configured.”; 2TA operation is supported for Rel-19 single-DCI mTRP without the restriction that coresetPoolIndex needs to be configured with more than one value, and single-DCI mTRP 2TA is applied to both the scenarios that PL offset is configured and PL offset is not configured. Regarding Rel-19 sDCI mTRP 2TA operation for the scenario PL offset is not configured (UE is configured with SSB-MTC-additionalPCI), RAN2 assumes both n-TimingAdvanceoffset and n-TimingAdvanceOffset2 are configured unless RAN1 has different agreement. Rel-17/18 Unified TCI States A/D MAC CE is reused for asymmetric DL sTRP/UL mTRP deployment. No MAC spec impact is expect, can confirm in the post meeting email discussion RAN2 understands UE maintains the internal configuration for this element (including Need R) in case the parent element (element in elementsToAddList) is absent as legacy. RAN2 to confirm that the PL offset value stored in the UE, i.e. in the internal UE configuration is not updated based on the MAC CE for PL update. RAN2 to confirm that the PL offset value stored in the source gNB/anchor gNB, i.e. in the UE RRC AS configuration, is not updated based on the MAC CE for PL update. It is up to network implementation whether to 1) apply the stored RRC configured PL offset value to the UE in case the associated tci-State (i.e. the parent parameter in tci-StatesToAddList) is absent, or 2) release the stored RRC configured PL offset value in case PL update is absent and the associated tci-State (i.e. the parent parameter in tci-StatesToAddList) is present, or 3) configure a new RRC value to the UE for the associated tci-State during the RRC resume procedure. No specification change is needed. Need R is applied for PL offset.

According to the post email discussion “[Post130][219][MIMO_Ph5] MAC open issues for MIMO (Samsung)”[3], the following issue has been raised for the asymmetric DL sTRP/UL mTRP topic.

- Issue5: Is sDCI mTRP 2TA (in case of no PL offset) supported for legacy Rel-18 LTM cell switch with no MAC specification impact?

Meanwhile, there is also one remaining issue on this topic in the post email discussion “[Post130][219][MIMO_Ph5] MAC open issues for MIMO (Offino)”[4] as follows

- Other issue: Discuss whether to support Rel-19 sDCI mTRP 2TA with PL offset for Rel-18 LTM cell switch. If it is supported, how to indicate the up-to-date PL offset for LTM cell switch.

In this contribution, we present detailed analysis on RAN2 work on the above issues based on RAN1 and RAN2 agreements so far, and provide our considerations.

# 2	Discussion

## 2.1	Combination with LTM

Rel-18 mDCI mTRP 2TA is supported in LTM scenario, where two TAGs are configured for the candidate cell and one of them is indicated by the TCI State in the LTM cell switch command to enable RACH-less cell switch. For Rel-19 sDCI mTRP 2TA, we need to discuss whether and how to support LTM. From our understanding, for 2TA feature, when UE is not configured with PL offset (discussed in RAN1#119), the only difference between Rel-19 mTRP and Rel-18 mTRP is whether to use single or multiple DCI, which has no impact on the core mechanism of LTM cell switch (TCI state indication and TA configuration). Therefore, in such case, Rel-19 sDCI mTRP 2TA can be naturally applied for LTM in the same way as Rel-18 mDCI mTRP 2TA without any enhancement.

RAN1 agreement in RAN1#119[1]

- From the perspective of UE: if UE is configured with PL offset in joint/UL TCI state(s), UE does not expect to receive SSB from UL TRP(s), else, UE may expect to receive SSB from UL TRP(s).

Since all the companies answered yes to issue5 in the email discussion, we have the following observation:

Observation: sDCI mTRP 2TA (in case of no PL offset) is supported for legacy Rel-18 LTM cell switch with no MAC specification impact.

However, according to the previous RAN2 agreements in RAN2#130, when PL offset is not configured in joint/UL TCI state for asymmetric DL sTRP/UL mTRP, the UE should be configured with SSB-MTC-additionalPCI, which means DL-UL TRP and UL-only TRP belong to different cells, as shown in Figure 1. It should be noted that such inter-cell mTRP case is just one case for asymmetric DL sTRP/UL mTRP, the other common case is that UE is configured with a PL offset and both DL-UL TRP and UL-only TRP belong to the same cell. In such intra-cell mTRP case, the UL-only TRP does not send SSB and the PL offset objectively exists. In order to ensure a reasonable power of first PUSCH transmission, the discussion on how to apply the PL offset should not be ignored.

RAN2 agreement in RAN2#130[2]

- Regarding Rel-19 sDCI mTRP 2TA operation for the scenario PL offset is not configured (UE is configured with SSB-MTC-additionalPCI), RAN2 assumes both n-TimingAdvanceoffset and n-TimingAdvanceOffset2 are configured unless RAN1 has different agreement.

Figure 1. R19 sDCI mTRP 2TA (with/without a PL offset)

Observation: For asymmetric DL sTRP/UL mTRP in LTM scenario, the case of sDCI mTRP 2TA with a PL offset should be discussed.

In the post email discussion[3], the rapporteur thinks that without specification impact, the current Rel-18 LTM cannot support sDCI mTRP 2TA with PL offset. In the current RRC running CR, it is true that the PL offset is not associated with the CandidateTCI-State-r18/ CandidateTCI-UL-State-r18 configured in ltm-TCI-Info-r18 per LTM candidate, however, there is still one possible way for UE to apply the PL offset without any specification impact, which is to pre-configure PL offset in the target configuration (ltm-CandidateConfig-r18(containing RRCReconfiguraion)-> ServingCellConfig->UL-TCI-State), where UE can find the PL offset associated with the same RS as the TCI state indicated in LTM cell switch MAC CE. Through this way, the UE can apply a suitable power for the first PUSCH transmission without any specification enhancement.

LTM-Candidate-r18 ::=     SEQUENCE {

ltm-CandidateId-r18                            LTM-CandidateId-r18,

ltm-CandidatePCI-r18                           PhysCellId                                            OPTIONAL,    -- Need M

ltm-SSB-Config-r18                             LTM-SSB-Config-r18                                    OPTIONAL,    -- Need M

ltm-CandidateConfig-r18                        OCTET STRING (CONTAINING RRCReconfiguration)          OPTIONAL,    -- Need M

ltm-ConfigComplete-r18                         ENUMERATED {true}                                     OPTIONAL,    -- Need R

ltm-EarlyUL-SyncConfig-r18                     OCTET STRING (CONTAINING EarlyUL-SyncConfig-r18)      OPTIONAL,    -- Need R

ltm-EarlyUL-SyncConfigSUL-r18                  OCTET STRING (CONTAINING EarlyUL-SyncConfig-r18)      OPTIONAL,    -- Need R

ltm-TCI-Info-r18                               LTM-TCI-Info-r18                                      OPTIONAL,    -- Need M

ltm-NoResetID-r18                              INTEGER (1..maxNrofLTM-Configs-plus1-r18)             OPTIONAL,    -- Need M

ltm-UE-MeasuredTA-ID-r18                       INTEGER (1..maxNrofLTM-Configs-plus1-r18)             OPTIONAL,    -- Need M

...

}

Another straightforward way for UE to apply the PL offset is to directly add the PL offset field into the CandidateTCI-State-r18/ CandidateTCI-UL-State-r18 configured in ltm-TCI-Info-r18. Upon receiving the LTM cell switch MAC CE, UE can apply the PL offset configured in Candidate TCI State, which shares the same TCI State ID as indicated in LTM cell switch MAC CE.

CandidateTCI-UL-State-r18 ::=           SEQUENCE {

tci-UL-StateId-r18                      TCI-UL-StateId-r17,

referenceSignal-r18                     CHOICE {

ssb-Index                               SSB-Index,

csi-RS-Index                            NZP-CSI-RS-ResourceId

},

pathlossReferenceRS-Id-r18              PathlossReferenceRS-Id-r17                                   OPTIONAL,   -- Need R

tag-Id-ptr-r18                          ENUMERATED {n0,n1}                                           OPTIONAL,   -- Cond 2TA

ul-PowerControl-r18                     Uplink-powerControlId-r17                                    OPTIONAL,   -- Cond separateTCI

pathlossOffset-r19                  ENUMERATED {

dB-12, dB-8, dB-4, dB0, dB4, dB8,

dB12, dB16, dB20, dB24, dB28, dB32, dB36,

dB40, dB44, dB48, dB52, dB56, dB60}                 OPTIONAL    -- Need R

...

}

Considering how to pre-configure a suitable PL offset value in LTM configuration, we think it is totally up to the NW implementation.

Observation: sDCI mTRP 2TA with PL offset can also be supported for legacy Rel-18 LTM cell switch via the following two methods:

- Option1: PL offset can be pre-configured in the target configuration and further deduced by TCI State in LTM cell switch command.

- Option2: Add PL offset field into the Candidate TCI State configuration.

Since both methods would be checked by RAN1, we suggest discussing with RAN1 to select one possible solution. Otherwise, RAN2 may conclude that sDCI mTRP 2TA with PL offset is not supported in LTM.

Proposal 1: Discuss whether to ask RAN1 about the possible solutions above, or conclude that sDCI mTRP 2TA with PL offset is not supported in LTM.

# 3	Conclusion

From the analysis on RAN2 impacts for MIMO, we made the following proposals:

Observation: sDCI mTRP 2TA (in case of no PL offset) is supported for legacy Rel-18 LTM cell switch with no MAC specification impact.

Observation: For asymmetric DL sTRP/UL mTRP in LTM scenario, the case of sDCI mTRP 2TA with a PL offset should be discussed.

Observation: sDCI mTRP 2TA with PL offset can also be supported for legacy Rel-18 LTM cell switch via the following two methods:

- Option1: PL offset can be pre-configured in the target configuration and further deduced by TCI State in LTM cell switch command.

- Option2: Add PL offset field into the Candidate TCI State configuration.

Proposal 1: Discuss whether to ask RAN1 about the possible solutions above, or conclude that sDCI mTRP 2TA with PL offset is not supported in LTM.

# 4	Reference

- RAN1#119 Chair’s Notes

- RAN2#130 Chair’s Notes

- [Post130][219][MIMO_Ph5] MAC open issues for MIMO (Samsung)

- [Post130][219][MIMO_Ph5] MAC open issues for MIMO (Offino)