3GPP TSG-RAN WG2 Meeting #130	R2-2505892

Bengaluru, India 25 - 29 August 2025

Agenda Item:	8.12.3

Source:	Huawei, HiSilicon

Title:	UE-initiated/event-driven beam management

Document for:	Discussion and Decision

# 1	Introduction

For Rel-19 MIMO, according to the WID as shown below and recent working assumption made by RAN2, objectives relevant to UE-initiated/event-driven beam management (UEIBM) are identified to have significant RAN2 impacts in terms of RRC configuration and MAC procedure, which are outlined in the WID.

RP-242394, Revised Work Item: NR MIMO Phase 5 The detailed objectives are as follows: RAN1: - Specify enhancement to facilitate UE-initiated/event-driven beam management for reducing overhead and/or latency, assuming the unified TCI while leveraging (as much as possible) legacy CSI measurement and reporting configuration frameworks, targeting FR2 and sTRP with intra- and inter-cell beam management - UL signaling content(s) (and procedure(s) as required) for UE-initiated/event-driven beam reporting facilitating fast beam switching - UL signaling medium/container considering the UE-initiated/event-driven nature of the UL transmission, designed primarily for the purpose of beam reporting.

In this contribution, we present our considerations on the remaining issues of UEIBM based on discussion in RAN2 and RAN1 so far.

# 2	UE Initiated/Event-driven Beam Management

In order to reduce the periodic CSI reporting overhead and latency, event-based CSI reporting was agreed by RAN1 for Rel-19 MIMO WI i.e., the UE reports CSI only when the event is triggered. This includes specification of the event triggering determination procedure and of the reporting procedure after the event is triggered. Since RAN1 and RAN2 have reached an agreement that the entire event-triggering and event reporting procedures are performed in the PHY layer, we now provide our considerations on how these procedures impact the RRC specification and the impact of SCell(s) deactivation on UEIBM.

According to the recent RAN2 meeting, RAN2 has already reached some agreements regarding the impacts of DRX on the MAC layer, dormant BWPs and TAT of the pTAG expires. We now proceed to consider how the other impacts can be effectively modelled within the RRC&MAC specifications.

Agreement - In Mode A of UE-initiated CSI reporting, the active time of a DRX operation includes the time after a new UCI for UE-initiated beam reporting is sent on first PUCCH. - Confirm the following RAN2 understandings: - The CG type-1 PUSCH carrying the beam report of Mode-B does not carry MAC PDU (i.e. UL-SCH). - The DG PUSCH carrying the beam report of Mode-A carries MAC PDU (i.e. UL-SCH) as legacy. - FFS if any other MAC impact for UL skipping - The UE continues to perform CSI measurements for the UEIBM procedure when the active BWP is the dormant BWP. - If the BWP in an SCell is a dormant BWP, the UE should not report mode-A beam measurement results. The UE cannot perform mode-B beam reporting on this BWP. - When the TAT of the pTAG expires, UE releases PUCCH resource for mode-A/B UEI report and clears type-1 CG for mode-B UEI report. FFS for the case when the TAT expires on the sTAG.

## 2.1	RRC impacts

| Topic | Issue |
| --- | --- |
| UEI BMR | FFS whether UEI BM parameters should be moved to CSI-MeasConfig. |

According to the list of open issues in the email discussion, there will be a discussion on whether the RRC parameters of the UEIBM should be set in the CSI-ReportConfig, or if they should be moved to the CSI-MeasConfig.

Some companies may argue that including the RRC parameters of the UEIBM in the CSI-MeasConfig would result in a cleaner design, as many of the fields in the CSI-ReportConfig are not used for the UEIBM and would be redundant if the CSI-ReportConfig is used. However, all legacy CSI report configurations are currently included in the CSI-ReportConfig, such as periodic CSI reports, semi-persistent CSI reports, and aperiodic CSI reports. It would be more natural to include the event-triggered CSI report in the same RRC configuration, following the legacy design, in order to maintain consistency.

Moreover, the UEIBM topic is led by RAN1, and they have already agreed to add these RRC parameters into the CSI-ReportConfig. It would be preferable to align with RAN1’s guidance; otherwise, RAN2 should send a LS to inform about the change in the location of the RRC parameters. It is more efficient to maintain a simple approach that follows legacy behavior and the RRC list established by RAN1.

Proposal 1: RAN2 confirms that the UEIBM parameters should be in the CSI-ReportConfig.

## 2.2	MAC impacts

### 2.2.1	UE-initiated report triggered while the TAT is expired

In the email discussion [Post130][219][MIMO_Ph5] Running CR for 38.321 (Samsung), one of the issues considered with whether the UE initiates RA if a UE-initiated report is triggered while the TAT associated to the resource for reporting is expired.

The email discussion rapporteur is proposing to further discuss the following two options:

-	option 1: the UE does not initiate RACH;

-	option 2 (was option 3):	the UE initiates RACH and indicates the cause of RACH e.g., in msg3.

Currently, when the TA is not valid, the UE does not perform periodic/semi-persistent or aperiodic reporting.

If the UE would trigger RA when it does not have a valid TA while a periodic or semi-persistent reporting is triggered, the UE would rapidly trigger RA, so there would be little point for the network to let the TAT expire while such reporting is configured. If the network wants the UE to perform a report while the TAT is expired, the network can trigger a PDCCH order.

However, UE-initiated beam reported is significantly different, in that the reporting is not periodic, and may not be triggered for a significant time, depending on the configuration. From that perspective, if the UE would trigger a RA procedure when the TAT is expired and a UE-initiated report is triggered, the UE may still remain for a significant amount of time not triggering the RA procedure e.g., if the radio conditions are rather stable. Compared with periodic/semi-persistent or aperiodic reporting, the introduction of event-driven reporting aims to reduce overhead and/or latency. Therefore, the time between measurement and reporting should be minimized. If option-1 is adopted, the UE must wait for the completion of the RACH procedure before transmitting the first PUCCH (indication) for beam reporting. Conversely, if option-3 is adopted, the indication will be sent through Msg.3, significantly reducing latency.

Furthermore, the specification impact is straightforward. It only requires modifying the C-RNTI MAC CE to include a one-bit indication for event-driven reporting in Msg.3. This modification does not affect any RAN1 specs.

Observation 1: Depending on the UE-initiated beam report configuration, UE-initiated beam report may not be triggered for a significant amount of time, during which the network may want to let the TAT expire.

Some concerns were raised on specification impact if the UE would initiate a RA procedure when a UE-initiated CSI report is to be transmitted while the UE does not have a valid TA. In our understanding, the procedure could be e.g.:

step 1)	if the UE is required to transmit a UERI, as specified in TS 38.214 clause 5.2.1.4.1, while the corresponding PUCCH resource is not valid, the UE initiates a Random Access procedure;

step 2)	the UE performs the existing Random Access procedure, except that, upon reception of the RAR, the UE includes in msg3 an indication that UE-initiated CSI report is triggered;

step 3)	upon reception of this indication, the network can configure the necessary PUCCH resources in msg4;

step 4)	after the resources are configured, the UE can transmit the UERI.

This would be sufficient for the network to know that it should provide PUCCH resources for UERI, and only steps 1 and 2 would have specification impact, which could be only in TS 38.321 and minimal.

Observation 2: Triggering RACH and indicating in msg3 that the UE has UERI to transmit would be sufficient for the network to configure the necessary SR resource in msg4.

Observation 3: The only specification impact is one additional RACH trigger and one piece of information to include in msg3.

Proposal 2: The UE initiates RACH when UERI is to be transmitted but there is no associated PUCCH resource, and indicates the cause in msg3.

### 2.2.2	Deactivation of SCell(s) Impact on UEIBM

Agreements on other aspects - When the TAT of the pTAG expires, UE releases PUCCH resource for mode-A/B UEI report and clears type-1 CG for mode-B UEI report. FFS for the case when the TAT expires on the sTAG.

According to the agreement made in the last meeting, it has been supported that UE should release PUCCH resource for mode-A/B UEI report and clears type-1 CG for the mode-B UEI report. Despite the PUCCH resource of the first uplink transmission, however, we should also consider other cases in the MAC layer where UE may also need to consider to release the type-1 CG PUSCH resources of the second uplink transmission. For instance, we need to discuss how to handle the configured uplink grant of UEIBM report for SCell deactivation case.

Agreement On cross-CC beam report transmission procedure for UE-initiated/event-driven beam reporting, regarding Event-2, the following is supported - the first PUCCH and the second PUSCH associated for UE-initiated/event-driven beam reporting are from the same PUCCH group. - (working assumption) the first PUCCH and the second PUSCH associated for UE-initiated/event-driven beam reporting are from different PUCCH groups - Subject to separate UE capability

Agreement On cross-CC beam report transmission procedure for UE-initiated/event-driven beam reporting, regarding Event-2, the following working assumption is confirmed with the following modification. - The first PUCCH and the second PUSCH associated for UE-initiated/event-driven beam reporting are from different PUCCH groups - Subject to separate UE capability - Note: From RAN1 perspective, the above does NOT introduce any spec impact except for UE capability signalling.

Based on the RAN1 agreement, UE-initiated or event-driven beam reporting can also be extended to the cross-CC beam reporting scenario. In this context, a CSI reporting configuration for CC#A is associated with a measurement resource configuration for CC#B. This cross-CC RS measurement framework has been introduced to enable the reporting of measurement results for cells operating in FR2 via cells operating in lower frequency bands. Since the current NR standards already support intra-CC RS measurement, this framework can be reused for R19 UEIBM.

Observation 4: UEIBM supports cross-CC beam reporting scenarios, enabling a CSI reporting configuration in CC#A to be linked with a measurement resource configuration in CC#B.

In a CA scenario, when the UE is configured with one or more SCells, the network has the flexibility to activate or deactivate these SCells as needed. When an SCell is deactivated, the UE is no longer required to receive PDCCH or PDSCH on the SCell and cannot transmit on the associated uplink.

Observation 5: For legacy active, periodic, semi-persistent, and aperiodic CSI reporting, there is no need to transmit a measurement report for a deactivated SCell.

Consequently, in the UEIBM procedure, there is also unnecessary to transmit a beam report for a deactivated SCell. In the context of cross-CC RS measurement, if CC#B (configured with a measurement resource) is deactivated while CC#A (configured with a CSI reporting configuration) remains active, periodic, semi-persistent, and aperiodic CSI reporting are not performed. Further discussion is required on how the UEI beam reporting procedure should be applied in such scenarios.

TS 38.321-i40 clause 5.9 1> else if an SCell Activation/Deactivation MAC CE or an Enhanced SCell Activation/Deactivation MAC CE is received deactivating the SCell; or 1> if the sCellDeactivationTimer associated with the activated SCell expires; or 1> if the SCG associated with the activated SCell is deactivated: 2> deactivate the SCell according to the timing defined in TS 38.213 [6]; 2> stop the sCellDeactivationTimer associated with the SCell; 2> stop the bwp-InactivityTimer associated with the SCell; 2> deactivate any active BWP associated with the SCell; 2> clear any configured downlink assignment and any configured uplink grant Type 2 associated with the SCell respectively; 2> clear any PUSCH resource for semi-persistent CSI reporting associated with the SCell; 2> suspend any configured uplink grant Type 1 associated with the SCell; 2> flush all HARQ buffers associated with the SCell; 2> cancel, if any, triggered consistent LBT failure for the SCell.

In mode-A/mode-B of UEI beam reporting, the configured PUCCH resource for the first uplink transmission, which is the "PUCCHResource-r19" in the RRC spec, is dedicated for UEIB beam reporting. When the SCell CC#B is deactivated, the corresponding PUCCH resource for UEI beam reporting in PCell/PUCCH SCell CC#A should also be released, in order to be used for another uplink transmission to avoid resource waste and improve uplink resource utilization efficiency.

Proposal 3: In the cross-CC beam reporting transmission procedure of UEIBM, if the CC#B is deactivated, UE should release any PUCCH resources allocated for UEIBM of CC#B in CC#A, and the text proposal is provided in the Annex.

Besides, a configured CG PUSCH resource is also used for the second uplink transmission in mode-B of UEI beam reporting. When the reported cell is deactivated in a same-CC beam reporting scenario, the corresponding CG PUSCH resource is cleared, as specified in the spec as “suspend any configured uplink grant Type 1 associated with the SCell”. Since the CG resources will not be used when the SCell is deactivated, so it should be released as well.

In the cross-CC RS reporting scenario, if CC#B is deactivated, the corresponding CG PUSCH resource in CC#A allocated for UEI beam reporting should also be released to maintain consistency. The proposed change to the specification, as depicted in the text proposal, is also outlined in the Annex.

Proposal 4: For mode-B of UEIBM, in the cross-CC beam reporting transmission procedure, if CC#B is deactivated, the UE should release any PUSCH resources allocated for UEIBM of CC#B in CC#A. Additionally, the text proposal is also provided in the Annex.

# 3	Conclusion

In this paper, we have addressed the objective regarding some analysis on RAN2 impact for MIMO. There are the following observations and proposals, and text proposal is attached in Annex.

RRC impacts

Proposal 1: RAN2 confirms that the UEIBM parameters should be in the CSI-ReportConfig.

UE-initiated report which the TAT is expired

Observation 1: Depending on the UE-initiated beam report configuration, UE-initiated beam report may not be triggered for a significant amount of time, during which the network may want to let the TAT expire.

Observation 2: Triggering RACH and indicating in msg3 that the UE has UERI to transmit would be sufficient for the network to configure the necessary SR resource in msg4.

Observation 3: The only specification impact is one additional RACH trigger and one piece of information to include in msg3.

Proposal 2: The UE initiates RACH when UERI is to be transmitted but there is no associated PUCCH resource, and indicates the cause in msg3.

Deactivation of SCell(s) Impact on UEIBM

Observation 4: UEIBM supports cross-CC beam reporting scenarios, enabling a CSI reporting configuration in CC#A to be linked with a measurement resource configuration in CC#B.

Observation 5: For legacy active, periodic, semi-persistent, and aperiodic CSI reporting, there is no need to transmit a measurement report for a deactivated SCell.

Proposal 3: In the cross-CC beam reporting transmission procedure of UEIBM, if the CC#B is deactivated, UE should release any PUCCH resources allocated for UEIBM of CC#B in CC#A, and the text proposal is provided in the Annex.

Proposal 4: For mode-B of UEIBM, in the cross-CC beam reporting transmission procedure, if CC#B is deactivated, the UE should release any PUSCH resources allocated for UEIBM of CC#B in CC#A. Additionally, the text proposal is also provided in the Annex.

# 4	Reference

- RP-242394, WID revision: NR MIMO Phase 5;

- RAN1 #120bis chairman notes;

- R2-2502982, RAN2 #129bis Report from session on Rel-18 MIMO, Rel-19 MIMO, LPWUS, SBFD, NR Others;

- TS 3GPP 38.300 V18.5.0;

- TS 3GPP 38.321 V18.5.0.

# 5	Annex – TP

## 5.9	Activation/Deactivation of SCells

If the MAC entity is configured with one or more SCells, the network may activate and deactivate the configured SCells. Upon configuration of an SCell, the SCell is deactivated unless the parameter sCellState is set to activated for the SCell by upper layers.

The configured SCell(s) is activated and deactivated by:

-	receiving the SCell Activation/Deactivation MAC CE described in clause 6.1.3.10;

-	receiving the Enhanced SCell Activation/Deactivation MAC CE described in clause 6.1.3.55;

-	configuring sCellDeactivationTimer timer per configured SCell (except the SCell configured with PUCCH, if any): the associated SCell is deactivated upon its expiry;

-	configuring sCellState per configured SCell: if configured, the associated SCell is activated upon SCell configuration;

-	receiving scg-State: the SCells of SCG are deactivated.

The MAC entity shall for each configured SCell:

1>	if an SCell is configured with sCellState set to activated upon SCell configuration, or an SCell Activation/Deactivation MAC CE or an Enhanced SCell Activation/Deactivation MAC CE is received activating the SCell:

2>	if the SCell was deactivated prior to receiving this Enhanced SCell Activation/Deactivation MAC CE and a TRS is indicated for this SCell:

3>	indicate to lower layers the information regarding the TRS.

2>	if the SCell was deactivated prior to receiving this SCell Activation/Deactivation MAC CE or this Enhanced SCell Activation/Deactivation MAC CE; or

2>	if the SCell is configured with sCellState set to activated upon SCell configuration:

3>	if firstActiveDownlinkBWP-Id is not set to dormant BWP:

4>	activate the SCell according to the timing defined in TS 38.213 [6] for MAC CE activation and according to the timing defined in TS 38.133 [11] for direct SCell activation; i.e. apply normal SCell operation including:

5>	SRS transmissions on the SCell;

5>	CSI reporting for the SCell;

5>	PDCCH monitoring on the SCell;

5>	PDCCH monitoring for the SCell;

5>	PUCCH transmissions on the SCell, if configured.

3>	else (i.e. firstActiveDownlinkBWP-Id is set to dormant BWP):

4>	stop the bwp-InactivityTimer of this Serving Cell, if running.

3>	activate the DL BWP and UL BWP indicated by firstActiveDownlinkBWP-Id and firstActiveUplinkBWP-Id respectively.

2>	start or restart the sCellDeactivationTimer associated with the SCell according to the timing defined in TS 38.213 [6] for MAC CE activation and according to the timing defined in TS 38.133 [11] for direct SCell activation;

2>	if the active DL BWP is not the dormant BWP:

3>	(re-)initialize any suspended configured uplink grants of configured grant Type 1 associated with this SCell according to the stored configuration, if any, and to start in the symbol according to rules in clause 5.8.2;

3>	trigger PHR according to clause 5.4.6.

1>	else if an SCell Activation/Deactivation MAC CE or an Enhanced SCell Activation/Deactivation MAC CE is received deactivating the SCell; or

1>	if the sCellDeactivationTimer associated with the activated SCell expires; or

1>	if the SCG associated with the activated SCell is deactivated:

2>	deactivate the SCell according to the timing defined in TS 38.213 [6];

2>	stop the sCellDeactivationTimer associated with the SCell;

2>	stop the bwp-InactivityTimer associated with the SCell;

2>	deactivate any active BWP associated with the SCell;

2>	clear any configured downlink assignment and any configured uplink grant Type 2 associated with the SCell respectively;

2>	clear any PUSCH resource for semi-persistent CSI reporting associated with the SCell;

2>	clear any Type-1 CG PUSCH resource for mode-B UE Initiated CSI reporting associated with the SCell;

2>	clear any configured PUCCH resource for UE Initiated CSI reporting associated with the SCell;

2>	suspend any configured uplink grant Type 1 associated with the SCell;

2>	flush all HARQ buffers associated with the SCell;

2>	cancel, if any, triggered consistent LBT failure for the SCell.

1>	if PDCCH on the activated SCell indicates an uplink grant or downlink assignment; or

1>	if PDCCH on the Serving Cell scheduling the activated SCell indicates an uplink grant or a downlink assignment for the activated SCell; or

1>	if a MAC PDU is transmitted in a configured uplink grant and LBT failure indication is not received from lower layers; or

1>	if a MAC PDU is received in a configured downlink assignment:

2>	restart the sCellDeactivationTimer associated with the SCell.

1>	if the SCell is deactivated:

2>	not transmit SRS on the SCell;

2>	not report CSI for the SCell;

2>	not transmit on UL-SCH on the SCell;

2>	not transmit on RACH on the SCell;

2>	not monitor the PDCCH on the SCell;

2>	not monitor the PDCCH for the SCell;

2>	not transmit PUCCH on the SCell;

2>	if the SCell is configured as a scheduled cell in MC-DCI-SetOfCells and with the search space for DCI to schedule multiple cells (as specified in TS 38.213 [6]) of the same searchSpaceId as the serving cell in which MC-DCI-SetOfCells containing the SCell is configured:

3>	not monitor the PDCCH for scheduling multiple cells (as specified in TS 38.213 [6]) for the set of cells in MC-DCI-SetOfCells including the SCell.

When the measurement reporting for fast unknown SCell activation is configured by RRC, the MAC entity shall:

1>	if SCell Activation/Deactivation MAC CE or an Enhanced SCell Activation/Deactivation MAC CE is received activating the SCell(s):

2>	if SCell(s) was deactivated prior to receiving this SCell Activation/Deactivation MAC CE or this Enhanced SCell Activation/Deactivation MAC CE:

3>	indicate to upper layers SCell(s) activation indication.

HARQ feedback for the MAC PDU containing SCell Activation/Deactivation MAC CE or Enhanced SCell Activation/Deactivation MAC CE shall not be impacted by PCell, PSCell and PUCCH SCell interruptions due to SCell activation/deactivation in TS 38.133 [11].

When SCell is deactivated, the ongoing Random Access procedure on the SCell, if any, is aborted.