3GPP TSG-RAN WG2 Meeting #131	R2-2505893

Bengaluru, India, 25 - 29 August 2025

Agenda Item:	8.6.2

Source:	Huawei, HiSilicon

Title:	Inter-CU LTM

Document for: Discussion and Decision

# 1	Introduction

LTM (L1/L2 Triggered Mobility) was introduced in Rel-18 for the intra-CU scenario. In Rel-19 [1], the scope is extended to inter-CU to support low latency and low interruption cell switch in broader areas. In this contribution, we discuss some remaining issues on inter-CU LTM.

# 2	Discussion

## 2.1	RACH-less LTM with two TAGs (MAC-26)

In RAN2 #130 Post email discussion [111], there is an open issue related to early RACH towards to a candidate cell with two TAGs.

| Index | Issue description | Rapporteur suggestion |
| --- | --- | --- |
| MAC-26 | How to consider the TAG ID when determining whether to set the REAMBLE_POWER_RAMPING_COUNTER to 1 for an (C)LTM candidate cell which is configured with two TAGs in RACH-less (C)LTM_ Editor’s NOTE: In RACH-less (C)LTM, for an (C)LTM candidate cell which is configured with two TAGs, TAG ID is not considered when determining whether to set the REAMBLE_POWER_RAMPING_COUNTER to 1. Rapporteur realized the same issue exists for Rel-18 LTM, which was missing. | Issue Type: Essential How to address it: based on companies’ contribution |

In Rel-18 LTM design, the early RACH and the cell switch procedure to a target cell with two TAGs is shown in the figure below:

Step 1: in the preparation step, the candidate cell provides its TCI state and tag-Id-ptr mapping to the source cell, and further to the UE.

Step 2: the source cell triggers PDCCH order to the UE.

Step 3: the UE sends a preamble to the candidate cell. If the PDCCH order indicates preamble initial transmission, the UE resets the power ramping counter. If it indicates preamble re-transmission, the UE increments the power ramping counter by 1.

Step 4: the candidate cell transfers the TA value and the corresponding tag-Id-ptr to the source cell.

Step 5: the source cell determines whether it can trigger RACH-less cell switch. The source cell first selects a target TCI State ID according to the received measurement reports, and then it checks whether the tag-Id-ptr of the selected TCI state is the same as the tag-Id-ptr received in Step 4. If yes, it can trigger RACH-less cell switch by including TA value and the TCI State ID in the LTM cell switch command MAC CE.

Step 6: the UE determines the TA is for which TAG by finding the tag-Id-ptr mapped with the indicated TCI State.

Step 7: the UE applies the TA and completes the LTM cell switch procedure.

In the above procedure, the UE power ramping is directly controlled by the PDCCH order content. When the source cell sends a PDCCH order, it does not need to know which TRP of the candidate cell will receive the preamble and calculate the TA. The source cell just indicates preamble initial transmission for the first early TA acquisition attempt, if it fails, the source cell can indicate preamble retransmission, and the UE increases the power. This power ramping procedure does not need to distinguish the TAG ID. Therefore, there is no additional MAC impact for the early RACH procedure when the candidate cell is configured with two TAGs.

Proposal 1: (MAC-26) When the candidate cell is configured with two TAGs, there is no additional MAC impact for the early RACH procedure.

## 2.2	Master key stream re-use in LTM recovery

When inter-CU LTM cell switch fails, LTM recovery is supported when the selected cell has the same Rel-19 ID (ltm-NoSecurityChangeID-r19) like the target cell.

RAN2 #130 - If the selected cell has same R19 ID as the target cell after the inter-CU MCG LTM failure, UE performs LTM fast recovery to the selected cell with the security key update using the NCC in the LTM cell switch command and the selected cell’s PCI and frequency.

In the following, the source cell is cell1, and the target cell of an inter-CU cell switch is cell2. The UE re-establishes PDCP and derives a new key to protect the first UL message to the cell2. However, this cell switch fails, then the UE reverts back to cell1 configuration and selects cell2 again during the LTM recovery procedure. The UE derives the same key and accesses cell2 again, with the same PDCP SN i.e. the key stream is re-used, which violates security requirements.

A simple way to solve this problem is that the UE does not perform LTM recovery if the selected cell is the target cell of a failed LTM cell switch attempt and performs RRC re-establishment instead.

For intra-CU LTM cell switch failure and LTM recovery to the same cell, there is no key stream re-use problem because the UE does not revert the PDCP state variables, so there is no need to preclude recovery to the target cell.

TS 38.331 Running CR 3> if attemptLTM-Switch is configured and the T304 of the MCG expiry is caused by an LTM cell switch execution triggered by an indication from lower layers or fulfilment of LTM cell switch execution conditions, as described in 5.3.5.18.6: 4> revert back to the UE configuration used in the source Pcell except for the PDCP state variables for SRB(s) associated to the MCG;c

Proposal 2: If inter-CU LTM cell switch triggered by the enhanced LTM CSC MAC CE fails, LTM recovery is configured, and the selected cell is the same as the target cell, the UE performs RRC re-establishment.

## 2.3	SCG LTM cell switch with the MN involved

### 2.3.1	MCG reconfiguration with sync

According to the draft running CR for TS 38.331, the following will be executed at SCG LTM cell switch with MN-involvement (i.e., when the LTM configuration is from ltm-ConfigNRDC):

1>	if the LTM cell switch is triggered on the MCG; or

1>	if the LTM cell switch is triggered on the SCG and the LTM candidate configuration to be applied is configured via ltm-ConfigNRDC:

2>	release/clear all current dedicated and common radio configurations which have neither been received via SRB1 within mrdc-SecondaryCellGroup, nor via SRB3 except for the following:

-	the radio bearer configuration (configured via RadioBearerConfig)

-	the logicalChannelIdentity and logicalChannelIdentityExt of RLC bearers configured in RLC-BearerConfig and the associated RLC entities, their state variables, buffers, and timers, except for triggering the associated RLC entities to reset the variable RETX_COUNT its initial value, as specified in TS 38.322 [4];

-	the bh-LogicalChannelIdentity of BH RLC channels configured in BH-RLC-ChannelConfig and the associated RLC entities, their state variables, buffers, and timers, except for triggering the associated RLC entities to reset the variable RETX_COUNT its initial value, as specified in TS 38.322 [4];

-	the UE variables VarLTM-ServingCellNoResetID, VarLTM-ServingCellUE-MeasuredTA-ID, and VarLTM-ServingCellNoSecurityChange;

-	the ltm-Config and ltm-ConfigNRDC (if configured);

-	the MCG C-RNTI;

-	the AS security configurations associated with the master key;

-	the logged measurement configuration;

This means that the PCell configuration, including the common PCell configuration in ServingCellConfigCommon, is discarded, so it needs to be included in the RRCReconfiguration message, and the only place where the common PCell configuration can be included is reconfigurationWithSync in the masterCellGroup.

According to the draft running TS 38.331 CR, when this field is included, the UE will execute the reconfiguration with sync procedure for the MCG, which will reset MCG MAC, and since LTM is performed on the SCG but not on the MCG, the UE will perform random access to the target PCell, whether it is the same as the previous PCell or not.

If inter-CU SCG LTM cell switch can be RACH-less, it seems a bit ironic, that, even if the PCell would not be changed, data transmission on the MCG would be interrupted, MCG MAC would be reset and MCG RLC buffers serving MN-terminated DRBs would be flushed.

In addition, this contradicts with the following RAN2 agreement made at RAN2#127bis in Hefei:

Agreements on inter-CU LTM

- The Rel18 handling on failure is reused in R19 if the UE selects an intra-CU LTM candidate cell after intra-CU LTM failure; for other cases, e.g. inter-CU LTM failure, the failure handling is FFS (related to SA3’s inputs).

- For non-DC case, if the new Rel-19 IDs of the serving cell and the target cell have same values, the UE compares the ltm-ServingCellNoResetID and ltm-NoResetID and performs the corresponding L2 reset operation as defined in Rel-18.

- The SCPAC-similar security update configuration is introduced for inter-CU SCG LTM, i.e. similar to IEs sk-CounterConfiguration, servingSecurityCellSetId and securityCellSetId. The names of the new IEs are to be discussed in stage3.

- Regarding the candidate and reference configuration generation and signaling design, the following SCPAC-similar principles can be applied for inter-CU SCG LTM as baseline:

- The reference configuration for inter-CU SCG LTM at least include SCG part, FFS on MCG part.

- FFS: Network ensures that when UE combines the reference and candidate configuration for inter-CU SCG LTM, the configuration generated by UE must contain both MCG and SCG part configurations.

- The candidate configuration and reference configuration are modeled as an MN RRCReconfiguration message.

- Upon inter-CU SCG LTM, the UE performs reconfiguration with sync towards SCG, but the reconfiguration with sync in MCG is not allowed.

- The MN generates the MCG part of the reference configuration (if any), while the SN (source or candidate) generates the SCG part of the reference configuration.

- The MN is responsible for the reference configuration generation for SN initiated inter-CU SCG LTM. It can be up to the NW implementation whether to include the MCG part.

- The MN can request an SCG reference configuration from any of the involved SNs.

For subsequent CPAC, which is exactly the same situation, ServingCellConfigCommon of the PCell is not discarded, and it is assumed that reconfiguration with sync of the MCG is never used. Therefore, the same solution should be adopted..

Proposal 3: Correct the RRC procedure for SCG LTM cell switch, with the MN involved:

-	the candidate configuration cannot include reconfigurationWithSync in the masterCellGroup;

-	at SCG LTM cell switch, the UE does not release ServingCellConfigCommon of the PCell.

### 2.3.2	MN/MCG L2 behaviours

In the running RRC CR, for MN-terminated DRBs, the PDCP/RLC behaviours are as follows:

3>	else if LTM cell switch is triggered on the SCG and this DRB is using the master key:

4>	if this DRB is an AM DRB:

5>	after the end of this procedure, re-establish the corresponding RLC entity as specified in TS 38.322 [4], after applying the LTM configuration in ltm-CandidateConfig within the LTM-Candidate IE in ltm-Config or ltm-ConfigNRDC;

5>	after the end of this procedure, trigger the PDCP entity of this DRB to perform data recovery as specified in TS 38.323 [5], after applying the LTM configuration in ltm-CandidateConfig within LTM-Candidate IE in ltm-Config or ltm-ConfigNRDC;

Accordingly, it seems that:

-	any RLC bearer serving an MN-terminated DRB, MCG and/or SCG RLC bearer, is re-established;

-	PDCP data recovery is performed on all MN AM DRBs.

However, at RAN2#128 in Orlando, RAN2 had the following agreement:

Agreements on inter-CU LTM:

- Rel-19 Set ID is configured for a candidate configuration. For DRB:

- Inter-CU MCG LTM: When Rel-19 Set ID of candidate cell is different from serving cell,

> UE performs PDCP re-establishment to all radio bearers.

- Inter-CU SCG LTM: When Rel-19 Set ID of candidate cell is different from serving cell,

> For MN terminated bearer without change of termination point, UE does not perform PDCP re-establishment. PDCP recovery is needed if it is split bearer.

> For SN terminated bearer without change of termination point, UE performs PDCP re-establishment.

> If there is change of termination point for a radio bearer (the keyToUse in the RadioBearerConfig is different from the keyToUse in the current UE configuration), UE performs PDCP re-establishment.

So it was agreed that PDCP data recovery is not always needed for MN terminated bearers, it is only needed if the bearer had an SCG leg.

This is aligned with SCPAC with MN involvement, for which the procedure is as follows:

3>	else:

4>	if there is an associated SCG RLC bearer in the selected subsequent CPAC candidate configuration that is part of the current UE configuration:

5>	re-establish the SCG RLC entity as specified in TS 38.322 [4];

4>	if the RLC entity of the associated RLC bearer(s) is re-established; or

4>	if an associated RLC bearer is released in the selected subsequent CPAC candidate configuration:

5>	if the bearer is an AM DRB:

6>	trigger the PDCP entity of the bearer to perform PDCP data recovery as specified in TS 38.323 [5];

MCG RLC entities are never re-established, and PDCP data recovery is only performed if really needed i.e., if the DRB is served by an RLC entity that was re-established or released. So the SCPAC solution looks more efficient and we proposed to adopt it.

Proposal 4: Correct the RRC procedure for SCG LTM cell switch, with the MN involved:

-	MCG RLC bearers are not re-established;

-	DRBs using the master key before and after the SCG LTM cell switch are never re-established;

-	PDCP data recovery on DRBs using the master key before and after the SCG LTM cell switch is only performed if needed i.e, they are served by an RLC bearer that was re-established or released.

# 3	Conclusion

This contribution makes the following proposals:

Proposal 1: (MAC-26) When the candidate cell is configured with two TAGs, there is no additional MAC impact for the early RACH procedure.

Proposal 2: If inter-CU LTM cell switch triggered by the enhanced LTM CSC MAC CE fails, LTM recovery is configured, and the selected cell is the same as the target cell, the UE performs RRC re-establishment.

Proposal 3: Correct the RRC procedure for SCG LTM cell switch, with the MN involved:

-	the candidate configuration cannot include reconfigurationWithSync in the masterCellGroup;

-	at SCG LTM cell switch, the UE does not release ServingCellConfigCommon of the PCell.

Proposal 4: Correct the RRC procedure for SCG LTM cell switch, with the MN involved:

-	MCG RLC bearers are not re-established;

-	DRBs using the master key before and after the SCG LTM cell switch are never re-established;

-	PDCP data recovery on DRBs using the master key before and after the SCG LTM cell switch is only performed if needed i.e, they are served by an RLC bearer that was re-established or released.

# 4	Reference

- RP-250339, Revised Work Item: NR mobility enhancements Phase 4, Apple Inc, China Telecom.