3GPP TSG-RAN WG2 Meeting #131	R2-2505901

Bengaluru, India, 25 – 29 August 2025

Agenda item:	8.12.2

Source:	Nokia

Title:	Remaining issues on asymmetric DL sTRP/UL mTRP

WID/SID:	NR_MIMO_Ph5-Core - Release 19

Document for:	Discussion and Decision

# 1	Introduction

In this contribution we share our views on the open issues listed in [Post130][219][MIMO_Ph5] Running CR for 38.321 (Samsung) and ([Post130][218][MIMO_Ph5] Running CR for 38.331 (Ericsson) related to asymmetric DL sTRP / UL mTRP.

# 2	Discussion

## 2.1	MAC issues

### sDCI mTRP 2TA in LTM cell switch

For legacy Rel-18 LTM cell switch, PL offset is not supported for LTM candidate TCI states; therefore, only the no-PL-offset case of Rel-19 sDCI mTRP 2TA can be applied using the existing Rel-18 LTM cell switch procedure with no change. The with-PL-offset case of sDCI mTRP 2TA is not supported for legacy Rel-18 LTM cell switch, and RAN2 should not try to optimize for that case since it would require cross work-item coordination.

Proposal 1: The with-PL-offset case of sDCI mTRP 2TA is not supported for legacy Rel-18 LTM cell switch.

## 2.2	RRC issues

### singleDCI-MultiTRP-2TA parameter

During the offline discussion of the RRC CR, one open issue was identified for asymmetric DL sTRP / UL mTRP:

| - Topic | - Issue | - Remarks |
| --- | --- | --- |
| Asymmetric DL sTRP/UL mTRP | In the last meeting, we have agreed to introduce a new parameter to enable Rel-19 2TA configuraiton, currently named singleDCI-MultiTRP-2TA. According to RAN1 UE feature, Rel-19 2TA is introduced for both intra-cell 2TA with no mDCI mTRP and inter-cell 2TA with no mDCI mTRP. The new parameter should be applied to both and LS to RAN1 may be needed to inform this. | This could be discussed based on company contributions. |

The relevant details introduced to 38.331 (i.e. in the field descriptions for singleDCI-MultiTRP-2TA, prachAssociationDCI-1-0, and tag2) in relation to this parameter are likely sufficient, but what is needed is for RAN1 to capture the relevant details in their specs.

In our view, RAN2 can simply send an LS to inform RAN1 on the relevant agreements made at RAN2#130 as well as the details of the new parameter singleDCI-MultiTRP-2TA (including the parent IE) and the related field description updates; then RAN1 can decide what updates are needed to their specifications. RAN2 should anyway send an LS to inform RAN1 of the other changes made to the parameters implemented in 38.331 (see discussion in our other TDoc R2-2505902) so this could be done at the time.

Proposal 2: Send an LS to inform RAN1 about the parameter details of singleDCI-MultiTRP-2TA, including the relevant agreements from RAN2#130. This could be included in a more general LS that captures all of the other parameter changes RAN2 made while implementing the RAN1 parameters in 38.331.

# 3	Conclusion

Proposal 1: The with-PL-offset case of sDCI mTRP 2TA is not supported for legacy Rel-18 LTM cell switch.

Proposal 2: Send an LS to inform RAN1 about the parameter details of singleDCI-MultiTRP-2TA, including the relevant agreements from RAN2#130. This could be included in a more general LS that captures all of the other parameter changes RAN2 made while implementing the RAN1 parameters in 38.331.