3GPP TSG-RAN WG2 Meeting #131	R2-2505932

Bengaluru, India, Aug. 25th – 29th, 2025

Agenda Item:	8.13.3

Source:	NIST

Title:	Feasibility of Including RRC State Information through RRC Container in Discovery Messages for L2 Multi-hop U2N Relay

Document for: Discussion and Decision

# 1	Introduction

During RAN2 #130 meeting [1], the following working assumption was made, together with the request to check with SA2 colleagues:

Working assumption: The Relay UE includes an indication of whether it is RRC_CONNECTED in the discovery message RRC container. As a baseline, remote UE relay (re)selection behaviour based on this information is left to implementation, and it can be discussed in spec implementation/maintenance if some guidance is needed. No SA2 spec impact is expected. Companies are asked to confirm with SA2 colleagues if there is any concern with a candidate first relay UE sending the RRC container with its own information.

In this contribution, we study the applicability of the working assumption to multi-hop U2N relay with Model A discovery and potential SA2 impacts. Next, three options are listed and compared for the handling of RRC container by an intermediate relay and for the potentials to include first relay UE’s RRC state information. Both Model A and Model B are discussed, and a proposal is made.

# 2	Discussion

SA2’ agreements on multi-hop relays are captured in TS 23.304 [2], and stage 3 details are discussed by CT1 and captured in TS 24.554 [3] and other specifications. Although SA2’s multi-hop relay SI+WI focused on L3 solutions, it is stated at the beginning of clause 6.3.2.5.1 of [2] that “5G ProSe Multi-hop UE-to-Network Relay Discovery is applicable to both 5G ProSe Multi-hop Layer-3 and Layer-2 UE-to-Network relay discovery for public safety use and commercial services.”. Therefore, the discovery procedures outlined in clause 6.3.2.5 [2] are also applicable to RAN2 discussion on L2 U2N solution as the starting point.

According to SA2 agreements, one major difference between Model A and Model B multi-hop U2N solution is:

-	With Model A, the remote UE only picks its next hop, i.e., the first relay, and does not choose the entire path (second relay, third relay, etc.) to the last relay;

-	With Model B, the remote UE determines the entire path (first relay, second relay, etc.) to the last relay;

The above difference can be seen by comparing clauses 6.3.2.5.2 and 6.3.2.5.3 of [2]: in Model A (6.3.2.5.2), specific discovery entries are created in the Intermediate relays to store the parent relay information (including the optional RRC container) and not the end-to-end path information, while in Model B (6.3.2.5.3) the information is relayed end-to-end.

The above difference is also reflected in the content of Discovery messages defined by CT1 in clause 10.2.1 of [3]: there is Multi-hop path info IE in both Discovery Solicitation (Table 10.2.1.17) and Discovery Response (Table 10.2.1.18) for Model B, while no such IE in Discovery Announcement (Table 10.2.1.15) for Model A.

Consequently, with Model A, the remote UE does not know the User Info ID of the candidate second relay and the associated RRC state. Even if the remote UE has the knowledge of candidate second relay’s RRC state alone without User Info ID through other implementation (e.g., with Option 2 discussed later in this contribution), it still cannot choose the second relay based on that because it is up to the first relay to pick the second relay. If RAN2 decides to change the above expected behavior to accommodate L2 U2N solution, it will likely lead to a new clause or at least new texts in TS 23.304 to specify Model A discovery procedure for L2 multi-hop U2N separately. This is not ideal, because such changes will go against the main idea of the Model A solution and lead to yet another implementation option. It might also impact the discovery messages contents already defined by CT1.

Observation: With Model A, the remote UE cannot select the second relay or other intermediate relays (except the first relay) based on their RRC states. If this expected behavior is to be changed, there will be SA2 and CT1 impacts.

Now let’s address the question whether “any concern with a candidate first relay UE sending the RRC container with its own information.”

For Model A discovery procedure, it can be found under Step 3 of clause 6.3.2.5.2 [2] that the RRC Container may be part of the discovery entry, and therefore may be included in the Discovery Announcement message that the intermediate relay will send to the next hop in step 4a or 4b. However, although there are texts under Step 3 specifying how to set several parameters in the discovery entry, the handling of the content in RRC container is not specified yet and may be up to RAN2 to decide. The intermediate relay has at least three options for RRC container handling:

Option 1: No change to the RRC container received from the parent node, just forward.

Option 2: Add its own RRC state indication to the RRC container received.

Option 3: Replaces the RRC state indication in the RRC container received with its own RRC state indication.

Under Option 1, the remote UE receives only the original RRC container prepared by the last relay, and no RRC state info about the candidate first relay is included.

Under Option 2, the remote UE receives the original RRC container prepared by the last relay, plus the RRC states of all candidate intermediate relays along the path, including the candidate first relay. Although Option 2 provides the remote UE with the most information on RRC states of other candidate relays, the additional information cannot be fully utilized in remote UE’s path selection as pointed out in the Observation above.

Under Option 3, the remote UE receives the original RRC container prepared by the last relay, plus the RRC state of the candidate first relay.

For Model B U2N multi-hop relay discovery procedure outlined in clause 6.3.2.5.3 [2], although RRC container is not explicitly mentioned, it is part of the information passing from node to node based on the reference to clause 6.3.2.3.3 (and then to clause 5.8.3.1 eventually) in Step 6-7. The Intermediate Relay UE also has at least the three options above to handle the RRC container. Option 2 is more aligned with the main idea of the Model B solution.

Depending on which option RAN2 picks for Model A and Model B respectively, it may be necessary to inform SA2.

Proposal: RAN2 is kindly asked to discuss the three options for the intermediate relay to handle the RRC container in the discovery messages, taking into account the difference between Model A and Model B based solutions as pointed out in the Observation. Inform SA2 if needed

# 3	Conclusion

Observation: With Model A, the remote UE cannot select the second relay or other intermediate relays (except the first relay) based on their RRC states. If this expected behavior is to be changed, there will be SA2 and CT1 impacts.

Proposal: RAN2 is kindly asked to discuss the three options for the intermediate relay to handle the RRC container in the discovery messages, taking into account the difference between Model A and Model B based solutions as pointed out in the Observation. Inform SA2 if needed

# 4	Reference

- “Report of 3GPP TSG RAN WG2 meeting #130, St Julian's, Malta” (draft v3),

- TS 23.304 V19.4.0, “Proximity based Services (ProSe) in the 5G System (5GS)”

- TS 24.554 V19.2.1, “Proximity-services (ProSe) in 5G System (5GS) protocol aspects; Stage 3”