3GPP TSG-RAN WG2 Meeting #131	R2-2505942

Bangalore, India  Aug 25th – 29th , 2025

# Agenda item:	8.4.4

# Source:	CMCC

# Title:	Discussion on LP-WUS operation in CONNECTED mode

# Document for:	Discussion, Decision

- Introduction

Following are the open issues related to LP-WUS operation in RRC connected state based on [1]:

| Index | Issue description | Rapporteur suggestion |
| --- | --- | --- |
| RRC-3 | whether/how to support LP-WUS (including Option 1-1 and 1-2) and dual DRX group Editor’s NOTE: FFS on whether/how to support LP-WUS (including Option 1-1 and 1-2) and dual DRX group. | Issue Type: not essential but important How to address it: can be discussed based on companies’ contribution |
| RRC-5 | whether it is allowed to report an empty UAI on offset for LP-WUS monitoring for both option 1-1 and option 1-2 Editor’s NOTE: There is no conclusion on whether it is allowed to report an empty UAI on offset for LP-WUS monitoring for both option 1-1 and option 1-2. | Issue Type: not essential How to address it: can be discussed based on companies’ contribution |

In this contribution, we will provide our view on these open issues.

- Discussion

  - Dual DRX with Option 1-1/2

For the case that dual DRX with LP-WUS was discussed in both RAN1 and RAN2. There’s no consensus in RAN1 on dual DRX, while RAN2 make a working assumption to support the dual DRX group case:

RAN1# 120bis Conclusion [2] For the case when UE is configured with CA with dual DRX groups in RRC CONNECTED mode, at least for LP-WUS procedure Option 1-1, there is no consensus in RAN1 which of the following options to support for this case in Rel-19. - Alt1 - Basic capability - LP-WUS can be configured only on PCell - LP-WUS indication is applicable to all serving cells of all DRX groups. - Note: There is no impact to PDCCH monitoring behaviour related to SCell dormancy/activation/deactivation - Advanced capability - LP-WUS can be configured on PCell for primary DRX group and an activated SCell for secondary DRX group - LP-WUS indication is applicable to all serving cells in the DRX group where LP-WUS is monitored - Note: There is no impact to PDCCH monitoring behaviour related to SCell dormancy/activation/deactivation - This capability has prerequisite of the above basic capability - Alt2 - LP-WUS can be configured on PCell for primary DRX group and/or an activated SCell for secondary DRX group - LP-WUS indication is applicable to all serving cells in the DRX group where LP-WUS is monitored - Note: There is no impact to PDCCH monitoring behaviour related to SCell dormancy/activation/deactivation - This capability has candidate values for the LP-WUS configuration with {primary DRX group, secondary DRX, both}

RAN2#130 Agreements on RRC_CONNECTED [3] Working assumption: LP-WUS can be configured on the PCell with secondary DRX. LP-WUS with secondary DRX is supported with option 1-1 and 1-2, i.e. the UE monitors LP-WUS before the on-duration occasion or periodically outside ActiveTime. When LP-WUS is detected, then UE starts the drx-onDurationTimer (with option 1-1) or the lpwus-PDCCHMonitoringTimer (with option 1-2) in both DRX groups.

Based on the work assumption,  it can be seen that the only LP-WUS indication wakes up all cells of the two drx groups. If separate wake-up for different drx group is expected, two LP-WUS indication is needed, which may introduce extra limitation  and complexity to the UE, like supporting to receive two LP-WUS simultaneously, and we think this is a new case/UE capability, which is not discussed in the WI. Besides, currently, for the UE configured with dual DRX groups, SCells is indicated to be the secondary DRX group or not by the network. And the two DRX groups share the same DRX cycle, while each has its own drx on-duration timer and inactivity timer value. That’s no say, if there’s no “early indication”, the two drx group’s on-duration timers are started simultaneously. Therefore, it seems feasible to support the only on LP-WUS indication for both two DRX groups.

Observation 1: If separate LP-WUS indication is needed, UE need to support to receive two LP-WUS simultaneously.

Observation 2: In current dual-DRX group mechanism, the same DRX cycle is used by the two DRX groups, and on-duration timers of the two DRX groups start simultaneously.

Based on the discussion above, we propose RAN2 to confirm the working assumption.

Proposal 1: RAN2 confirms the working assumption on LP-WUS with secondary DRX group: LP-WUS can be configured on the PCell with secondary DRX. LP-WUS with secondary DRX is supported with option 1-1 and 1-2, i.e. the UE monitors LP-WUS before the on-duration occasion or periodically outside ActiveTime.  When LP-WUS is detected, then UE starts the drx-onDurationTimer (with option 1-1) or the lpwus-PDCCHMonitoringTimer (with option 1-2) in both DRX groups.

  - UAI for LP-WUS

RAN2 discussed UE reports preferred timer offset via UAI in the previous meetings:

RAN2#129bis meeting agreements: [4] If configured, the UE can signal a preferred time offset via UAI signalling. RAN2#130 meeting agreements: On open issues for TS38.331 RRM relaxation / offloading configuration is provided in SIB2. RAN2 assumes the design of UAI reporting for preferred time offset is same as the legacy, e.g. including the configuration, procedure, as well as prohibit timer, etc.

And the FFS is about whether it is allowed to report an empty UAI on offset for LP-WUS monitoring for both option 1-1 and option 1-2. In current UAI reporting, UE may not include a specific parameter if it has no preference. Following is the example about UE preferred DRX configuration [5]:

…… 1> if transmission of the UEAssistanceInformation message is initiated to provide drx-Preference of a cell group for power saving according to 5.7.4.2 or 5.3.5.3: 2> include drx-Preference in the UEAssistanceInformation message; 2> if the UE has a preference on DRX parameters for the cell group: 3> if the UE has a preference for the long DRX cycle: 4> include preferredDRX-LongCycle in the DRX-Preference IE and set it to the preferred value; 3> if the UE has a preference for the DRX inactivity timer: 4> include preferredDRX-InactivityTimer in the DRX-Preference IE and set it to the preferred value; 3> if the UE has a preference for the short DRX cycle: 4> include preferredDRX-ShortCycle in the DRX-Preference IE and set it to the preferred value; 3> if the UE has a preference for the short DRX timer: 4> include preferredDRX-ShortCycleTimer in the DRX-Preference IE and set it to the preferred value; 2> else (if the UE has no preference on DRX parameters for the cell group): 3> do not include preferredDRX-LongCycle, preferredDRX-InactivityTimer, preferredDRX-ShortCycle and preferredDRX-ShortCycleTimer in the DRX-Preference IE; ……

We think the similar way can be reused on UE preferred offset for LP-WUS monitoring.

Proposal 2: If the UE has no preference on offset for LP-WUS monitoring,  it does not include the related IE in the UAI.

  - Configuration and (de)activation of LP-WUS monitoring

In RAN1#116 meeting [6], gNB RRC signalling based LP-WUS enable/disable is agreed and whether additional options on top of it is for further study.

Agreement For RRC CONNECTED mode, from RAN1 perspective, - PDCCH monitoring triggered by LP-WUS is enabled/disabled by gNB RRC signaling - FFS whether to support UE assistance. - LP-WUS monitoring by UE is known to gNB. - FFS whether implicit/explicit indication from UE is necessary - In case LP-WUS monitoring is enabled, following options are further studied - Option 1: No additional indication/condition are introduced for activation/deactivation of LP-WUS monitoring - Option 2: Activation/deactivation of LP-WUS monitoring by gNB L1/L2 signaling with or without UE assistance. - Option 3: Activation/deactivation of LP-WUS monitoring based on condition(s), such as timer. - Option 4: Activation/deactivation of LP-WUS monitoring based on implicit indication/condition, e.g. UL transmission.

Considering the motivation of using LP-WUS in CONNECTED mode is to reduce the PDCCH monitoring power consumption while guarantee the latency performance, it is not necessary to adapt the on/off of LP-WUS monitoring frequently based on fast L1/L2 enable/disable signalling. Besides, UE assistance information can be introduced to assist gNB to enable/disable the LP-WUS monitoring. For example, when a UE is in lower battery status, UE can report to gNB that it prefers to activate the LP-WUS monitoring function. As to the condition based activation/deactivation mechanism, similar performance can be achieved by proper network configuration based on UE reported information, including UE assistant information and SR/BSR. Therefore, in our understanding, Option 3 and Option 4 can be achieved by network implementation.

Proposal 3: RRC signalling based approach is used to enable LP-WUS monitoring in RRC_CONNECTED.

Proposal 4: Once the UE receives LP-WUS monitoring configuration, it start LP-WUS monitoring based on the configuration.

Proposal 5: No additional indication/condition are introduced for activation/deactivation of LP-WUS monitoring. LP-WUS monitoring is only enabled(activated) or disable(deactivated) by RRC signalling.

Proposal 6: UE assistance information can be introduced to assist gNB to enable/disable the LP-WUS monitoring.

- Conclusions

In this contribution, we discuss the LP-WUS operation in RRC_CONNECTED mode, and following are the proposals and observations:

Proposals:

Proposal 1: RAN2 confirms the working assumption on LP-WUS with secondary DRX group: LP-WUS can be configured on the PCell with secondary DRX. LP-WUS with secondary DRX is supported with option 1-1 and 1-2, i.e. the UE monitors LP-WUS before the on-duration occasion or periodically outside ActiveTime.  When LP-WUS is detected, then UE starts the drx-onDurationTimer (with option 1-1) or the lpwus-PDCCHMonitoringTimer (with option 1-2) in both DRX groups.

Proposal 2: If the UE has no preference on offset for LP-WUS monitoring,  it does not include the related IE in the UAI.

Proposal 3: RRC signalling based approach is used to enable LP-WUS monitoring in RRC_CONNECTED.

Proposal 4: Once the UE receives LP-WUS monitoring configuration, it start LP-WUS monitoring based on the configuration.

Proposal 5: No additional indication/condition are introduced for activation/deactivation of LP-WUS monitoring. LP-WUS monitoring is only enabled(activated) or disable(deactivated) by RRC signalling.

Proposal 6: UE assistance information can be introduced to assist gNB to enable/disable the LP-WUS monitoring.

Observations:

Observation 1: If separate LP-WUS indication is needed, UE need to support to receive two LP-WUS simultaneously.

Observation 2: In current dual-DRX group mechanism, the same DRX cycle is used by the two DRX groups, and on-duration timers of the two DRX groups start simultaneously.

- References

- Discussion summary and list of RRC open issue for LP-WUS WUR, vivo;

- Report of 3GPP TSG RAN WG1 meeting #120bis;

- Report of 3GPP TSG RAN WG2 meeting #130;

- Report of 3GPP TSG RAN WG2 meeting #129bis;

- TS 38.331 Radio Resource Control (RRC) protocol specification (Release 18);

- Report of 3GPP TSG RAN WG1 #116.