# 3GPP TSG-RAN WG2 Meeting #131		                                                       R2-2505943

# Bangalore, India, 25th – 29th Aug. 2025

# Agenda item:	8.5.2

# Source:	CMCC

# Title:	Discussion on open issues of on-demand SSB for SCell

# Document for:	Discussion, Decision

- Introduction

Following are the open issues identified in the email discussion of RRC running CR[1]:

Open issue list: OD-SSB: Issue 1: Q9 Issue 2: CATT003 Issue 3: Fujitsu006 /HW001/HW002

In this paper, we further discuss open issues related to OD-SSB for SCell.

- Discussion

  - The value for maxNrofOD-SSB

Q9 is about the value for maxNrofOD-SSB:

IE CellGroupConfig Editor’s note: FFS value for maxNrofOD-SSB Q9: Suggest value for this parameter or indicate if another WG needs to be liaised?

Based on the running CR, maxNrofOD-SSB is the max number of OD-SSB configurations, while OD-SSB configuration is used to configure the OD-SSB activated by a MAC CE. Though there could be multiple SCells, and one SCell can be configured with multiple OD-SSB, every OD-SSB is associated with a measurement ID, while the maxNrofMeasId is 64, we think the maxNrofOD-SSB can be the same value.

Proposal 1: The value of maxNrofOD-SSB can be 64.

  - OD-SSB activation

Issue 2 is about OD-SSB SCell operation, especially OD-SSB activation:

| CATT003 | od-ssb-nrofBurst Indicates the number of OD-SSB bursts to be transmitted after OD-SSB is indicated.. Agreement (RAN1 #121) For a cell supporting on-demand SSB SCell operation, the following combinations are supported. - For OD-SSB transmission activation (OD-Tact) and OD-SSB transmission adaptation (OD-TA), - Case A1: RRC-based OD-Tact without N (i.e., od-ssb-nrofBurst) configured + MAC CE-based OD-TA; - Subject to UE capability - Case B1: MAC CE-based OD-Tact without N configured + MAC CE-based OD-TA; - Case B2: MAC CE-based OD-Tact with N configured + MAC CE-based OD-TA. - For OD-SSB transmission deactivation (OD-TD), - Case X1: RRC-based OD-Tact without N configured + MAC CE-based OD-TD; - Subject to UE capability - Case Y1: MAC CE-based OD-Tact or OD-TA without N configured + MAC CE-based OD-TD; - Case Y2: MAC CE-based OD-Tact or OD-TA with N configured + implicit OD-TD; - Case Y3: MAC CE-based OD-Tact or OD-TA with N configured + MAC CE-based OD-TD. - Conclusion: There is no RAN1 consensus to support RRC activation of OD-SSB transmission configuring od-ssb-nrofBurst. - Note: “Implicit OD-TD” above implies that the on-demand SSB is deactivated based on the value for od-ssb-nrofBurst according to NW indication. According to the conclusion of RAN1, od-ssb-nrofBurst can only be configured by RRC and activated by MAC CE, and RAN2 has not discussed this issue. The od-ssb-nrofBurst is configured in parallel with od-ssb-ActivationStatus in OD-SSB-Config, so od-ssb-nrofBurst should be able to be configured and activated through RRC? We think we need to make a clarification. | Open issue |
| --- | --- | --- |

Based on RAN1 conclusion, there’s no RAN1 consensus to support RRC activation of OD-SSB transmission configuring od-ssb-nrofBurst. In our opinion, RRC configured OD-SSB activation state and configuring od-ssb-nrofBurst are two ways to activate an OD-SSB configuration, and the following network/UE behaviour is the same during OD-SSB transmission period. What’s more, if both configured, the UE behaviour is not clear. Therefore, there’s no need to configure RRC activation state and od-ssb-nrofBurst simultaneously.

Proposal 2: RRC activation state and od-ssb-nrofBurst are not configured to the UE simultaneously.

  - ServingCellMO configuration

Issue 3 is about how to capture MO of OD-SSB in ServingCellConfig:

| Fujitsu 006 | – OD-SSB-Config RAN2 confirms the working consumption that when AO-SSB and OD-SSB have different center frequency, introduce a new servingCellMO in ServingCellConfig to indicate MO of OD-SSB, as agreement.; Field servingCellMO is introduced in IE OD-SSB-Config. In this way it is possible to keep the same field name an hence all the procedural text intact.. [Fujitsu] RAN2 agreed to introduce a new servingCellMO in ServingCellConfig in RAN2#130. Although the current CR may work, we think the original agreement is simpler for implementation, because it can keep the same message structure. We also think even if it is introcuded in ServingCellConfig, the current procedural text is not changed (similar to servingCellMO-r17). We would like to add it into open issue list and ask other company’s views. [Apple] We have same view as Fujisu. | Open issue. |
| --- | --- | --- |

Currently, there is a servingCellMO per OD-SSB-Config in the running CR, while the RAN2 agreements in RAN2#129bis is When A-SSB and OD-SSB have different center frequency, introduce a new servingCellMO in ServingCellConfig to indicate MO of OD-SSB [2]. And some companies comment that the current description does not fully align with the agreements.

The IE ServingCellMO is associated to a measurement ID to indicate the related measurement objective and reporting configuration. Though RAN2 agreements is to introduce a separate ServinCellMO for OD-SSB, considering multiple OD-SSB-Config is supported,  it should be a “ServingCellMO list” for OD-SSB, or at least a measurement ID list, if to completely follow the agreements. And for the OD-SSB activation/deactivation via MAC CE, what is indicated to the UE is the OD-SSB configuration ID. If a separate serving cell MO list or measurement ID list is used, it will increase UE complexity to associate the OD-SSB configuration ID and measurement ID. Therefore, we prefer the current way to configure ServingCellMO of OD-SSB in the CR, which is simple and clear.

Proposal 3: The current description way to configure ServingCell MO to indicate MO of OD-SSB is preferred, since it’s simple and clear.

- Conclusion

In this contribution, we discuss the OD-SSB related issues. Following are the proposals made in this contribution:

Proposals:

Proposal 1: The value of maxNrofOD-SSB can be 64.

Proposal 2: RRC activation state and od-ssb-nrofBurst are not configured to the UE simultaneously.

Proposal 3: The current capture way to configure ServingCell MO to indicate MO of OD-SSB is preferred, since it’s simple and clear.

- References

- Report of [POST130][107][NES]: Collection of comments to 38.331 CR for NES (Ericsson);

- Report of 3GPP TSG RAN WG2 meeting #129bis.