3GPP TSG RAN WG2 Meeting #131	R2-2505957

Bangalore, India, August 25th – 29th, 2025

Agenda Item:	8.8.4

Source:	CMCC

Title:	Remaining issues on broadcast service for NR NTN

Document for:	Discussion, Decision

Introduction

In last RAN2 meeting, there were some discussions on support of broadcast service and some agreements were achieved as follows [1]:

Agreements:

- Introduce mapping between a MBS broadcast session (TMGI) and ISA ID(s) extending MBS-SessionInfoList to avoid signaling overhead (duplication of TMGIs).

- Capture in a Note in Stage 2 that when using ISA(s) for MBS broadcast service reception or MBS service continuity, it is up to UE implementation how to determine if it is in the ISA(s) of MBS broadcast service or not.

3.	Do not introduce service area information specific for FSAI in SIB21.

4.	Do not introduce service area per neighbour cell in MCCH

5.	RAN2 assumes that the ISA can be valid for all the frequencies providing the MBS session

6.	The new SIBxx containing MBS ISA(s) can be cell or area specific, which is up to NW implementation (No specs change).

And based on RRC open issue list [2] provided by the rapporteur, there are two open issues are needed to further discuss:

FFS to align RAN2 specifications and use the Ellipsoid Point with uncertainty Circle from TS 37.355 to define the circle shape of the ISA in the new SIBXX. FFS on the terminology misalignment between ‘broadcast service area’ vs. ‘intended service area’ and the potential impact on the MII procedure.

Therefore, in this contribution, we would like to provide some further considerations on the open issues for broadcast service in NTN system.

Discussion

Whether to align RAN2 specifications and use the Ellipsoid Point with uncertainty Circle from TS 37.355

As mentioned in [2], for the circle description of ISA, SA4 has agreed to use PointUncertaintyCircle formats as defined in TS 29.572 [S4-251482]. While in RAN2, we has adopted a circular representation based on referenceLocation and distanceRadius directly specified in RRC.

Per the TS 29.572 the PointUncertaintyCircle seems to offer a larger area range due to that the maximum value of the Uncertainty is not defined. That is to say if using Ellipsoid-PointWithUncertaintyCircle (with 7bits uncertainty) to represent a circle area, it will narrow down the range of service area that can be represented, which also failed to achieve the alignment between specification. And the typical use of ellipsoid point with uncertainty circle is to indicate a point when its position is known only with a limited accuracy as in the TS 23.032. Further the current distanceRadius (16bits) could provide more 9bits information than the Ellipsoid-PointWithUncertaintyCircle. Therefore, it is not needed to use the Ellipsoid Point with uncertainty Circle from TS 37.355.

Proposal 1: It is not needed to use the Ellipsoid Point with uncertainty Circle from TS 37.355.

Misalignment between‘broadcast service area’ vs. ‘intended service area’

In last meeting, discussion on the term ‘broadcast service area’ vs. ‘intended service area’ was raised:

Observation 1: Behaviours with respect to the initiation of MII procedure upon entering or leaving the area of broadcast service has already been defined. Proposal 1: RAN2 to confirm that, with respect to the MBS service, the term ‘broadcast service area’ refers to the ‘intended service area’. It could be captured in Stage 2 specification. Come back in the next meeting

It may have some impact on the MBS Interest Indication initiation description. As current specification, the MBS interest indication procedure is to inform the network that the UE in RRC_CONNECTED is receiving or is interested to receive MBS broadcast service(s) and/or to inform the network about the priority of MBS broadcast versus unicast and multicast MRB reception. MBS Interest Indication can only be sent after AS security activation. And the trigger condition is described in section 5.9.4.2 of TS 38.331.

- 5.9.4.2 Initiation An MBS capable UE in RRC_CONNECTED may initiate the procedure in several cases including upon successful connection establishment/resume, upon entering or leaving the broadcast service area, upon MBS broadcast session start or stop, upon change of interest, upon change of priority between MBS broadcast reception and unicast/multicast reception, upon change to a PCell providing SIB21 (i.e. where the SIB1 scheduling information contains SIB21), upon receiving SIB20 of an SCell via dedicated signalling, upon handover, and upon RRC connection re-establishment, upon change to a PCell providing nonServingCellMII in SIB1, upon starting or stopping reception of MBS broadcast service on a non-serving cell, upon change of CFR information or subcarrier spacing for MBS broadcast reception on a non-serving cell. If the UE does not have the CFR information and subcarrier spacing for MBS broadcast reception on a non-serving cell at the time it sends the MBS Interest Indication, the UE sends another MBS Interest Indication after it has acquired this information from the non-serving cell.

Clearly the term ‘broadcast service area’ is used, per the current TS 23.247, the ‘Broadcast service area’ is the area within which data of one or multiple Broadcast MBS session(s) are sent. Different from the ‘broadcast service area’, based on the WID scope description, the ‘intended service area’ is expected to be smaller than the coverage of a Uu cell in some cases, which is why some enhancements are needed to notify the UE of the service area information. Then in the stage 2 running CR [3], current description is ‘Intended Service Area’, and the ISA is a geographical information that can cover an area larger than one NTN cell (i.e. serving and neighbour cells) or portions thereof. To avoid the misalignment, we could consider to make a clarification in stage 2 specification as in [4] or add intended service area in the MII initiation section (e.g. upon entering or leaving the broadcast service area or intended service area).

Observation 1: In the current MII initiation procedure, the term ‘broadcast service area’ is used.

Observation 2: The ‘Broadcast service area’ is the area within which data of one or multiple Broadcast MBS session(s) are sent, which is different from the ‘intended service area’.

Proposal 2: To avoid the misalignment, we could consider to make a clarification in stage 2 specification as in [4] (i.e. capture in stage 2 that the term ‘broadcast service area’ refers to the ‘intended service area’)or add intended service area in the MII initiation section (e.g. upon entering or leaving the broadcast service area or intended service area, ...).

Then for MII further enhancement, UE is able to determine whether in the intended service area based its own GNSS and service area information provided by network. Further, network may have no acknowledge of the UE location, i.e., UE is in which service area may not be clear at network side. That is whether initiate the MII procedure could be just up to UE itself. Therefore, further enhancement for MBS Interest Indication initiation is not needed based on proposal 2.

Proposal 3: If proposal 2 is supported, the existing MII initiation description (i.e. An MBS capable UE in RRC_CONNECTED may initiate the procedure in several cases including...) is sufficient, further enhancement is not needed.

Conclusion

Based on the discussions mentioned above, in this contribution we provide some discussions on MBS broadcast enhancements for NTN and have the following observations and proposals:

Proposal 1: It is not needed to use the Ellipsoid Point with uncertainty Circle from TS 37.355.

Observation 1: In the current MII initiation procedure, the term ‘broadcast service area’ is used.

Observation 2: The ‘Broadcast service area’ is the area within which data of one or multiple Broadcast MBS session(s) are sent, which is different from the ‘intended service area’.

Proposal 2: To avoid the misalignment, we could consider to make a clarification in stage 2 specification as in [4] (i.e. capture in stage 2 that the term ‘broadcast service area’ refers to the ‘intended service area’)or add intended service area in the MII initiation section (e.g. upon entering or leaving the broadcast service area or intended service area, ...).

Proposal 3: If proposal 2 is supported, the existing MII initiation description (i.e. An MBS capable UE in RRC_CONNECTED may initiate the procedure in several cases including...) is sufficient, further enhancement is not needed.

Reference

- R2-130 NR-NTN-IoT-NTN (Sergio)_ EOM

- (5841) R2-25xxxx [draft] Open_issue_list_RRC_NTN_v06_Xiaomi

- R2-250XXXX_was_4761 Stage 2 CR for NTN Ph3_v09_Rapp

- R2-2503560	Discussions on supporting broadcast service 	Fujitsu	discussion	Rel-19	NR_NTN_Ph3-Core