3GPP TSG RAN WG2 Meeting #131	R2-2505958

Bangalore, India, August 25th – 29th, 2025

Agenda Item:	8.9.3

Source:	CMCC

Title:	Discussion on remaining issues of uplink capacity enhancement for IoT-NTN

Document for:	Discussion, Decision

Introduction

- In last meeting, there were some discussions on uplink capacity enhancements for IoT-NTN and achieved some agreements[1]. And in this contribution, we would like to provide some further discussion on the remaining issues for CB-msg3-EDT.

Discussion

  - (MAC-2): CB-RNTI calculation

For CB-RNTI calculation, a working assumption is reached as following:

Working Assumption:

The formula for RNTI for mMsg4 monitoring is:

RNTI=X + Msg3_W_index modulo (Y) + Y*CE_level + 3*Y*carrier_id.

•	X is the starting RNTI for Msg4 reception, which can be defined by RAN2 e.g. X=2401 for eMTC or 4097 for NB-IoT,

•	Msg3_W_index is the index of Msg3 transmission window within a periodicity of 1024 SFNs and index 0 corresponds to the Msg3 transmission window starts at the SFN defined by IE startSFN-r19,

•	Y is ceil (Msg4_WS/Msg3_WP),

•	CE_level is the CE level, 0 <= CE_level < 3

•	carrier_id is the index of the UL carrier of the CB-Msg3 resources, anchor carrier has index 0,

0 <= carrier_id < 16

Can come back to check if the NW can also simply configure RNTI = X

Agreements – part 2:

1.	The value of X is 4097 for NB-IoT and 2401 for eMTC

2.	The value of Msg4_WS is the maximum Msg4 window size

3.	The value of Msg3_WP is the minimum Msg3 window periodicity

Per the MAC open issues discussion [2], most companies agree to confirm the CB-RNTI calculation WA, and no need have an alternative to configure RNTI = X due to the potential false alarm issue. And the following two issues needs further discussion:

- Whether to define Msg3_W_index to floor(start SFN_id of Tx window / minimum Tx window length or periodicity]

- Whether to define Y = 16.

Per the legacy RA-RNTI calculation, where floor(SFN_id/4) factor is used, and 4 is related to PRACH resource periodicity. Therefore we also could consider a similar solution to use floor(start SFN_id of Tx window / minimum Tx window periodicity] instead of the Msg3_W_index.

Then for Y, whether to define the Y = 16 to instead the current Y which is ceil (Msg4_WS/Msg3_WP) where Msg4_WS is the maximum Msg4 window size and Msg3_WP is the minimum Msg3 window periodicity. From our perspective, there seems no obvious gain to make such kind of modification. Therefore it is preferred to leave it as the current calculation formula.

Proposal 1: (MAC-2) Confirm the CB-RNTI calculation working assumption.

Proposal 2: (MAC-2) It is proposed to use floor(start SFN_id of Tx window / minimum Tx window periodicity] instead of the Msg3_W_index.

Proposal 3: (MAC-2) For Y, it is preferred to leave it as the current calculation formula.

  - (MAC-6): The power ramping parameters and how the power ramping is done

In RAN2 #129bis meeting, we sent an LS [3] including power ramping support to check with RAN1 and request the the L1 parameters, and in RAN1 #121 meeting, they discuss the RAN2 LS and feedback a reply LS [4]:

Reply to Q1: RAN1 has not evaluated the potential performance of power ramping for CB-msg3-EDT, and it is likely that there will not be sufficient time to evaluate this topic within the R19 timeframe - For open loop power control the following UL power control parameters can be reused for CB-msg3-EDT - p0-UE-NPUSCH-r16 and alpha-r16 for NB-IoT NTN

And in MAC open issue discussion[2], the majority companies agreed no to introduced the power ramping for CB-Msg3-EDT due to there is no R19 time in RAN1 to further evaluate this issue. In order to avoid the impact on RAN1, at this stage, it is more reasonable not to support the power ramping for CB-Msg3-EDT for R19.

Proposal 4: (MAC-6) Not to pursue power ramping for CB-Msg3-EDT in R19.

  - (MAC-9): Whether NW/UE processing time is needed when determine the Msg4 monitoring starts.

Most companies support not to introduce the processing time in [2]. However, the CB-Msg EDT procedure is also based on the shared PUSCH resource to perform the transmission, which is similar to the legacy transmission using PUR, which considers both the processing time and UE-eNB RTT to start or restart pur-ResponseWindowTimer in NTN network.

- 5.4.7.1 Transmission using PUR <<omitted>> After transmission using PUR, the MAC entity shall monitor PDCCH identified by PUR-RNTI in the PUR response window using timer pur-ResponseWindowTimer: - if PUR was transmitted in a non-terrestrial network and UE supports delaying the start of the pur-ResponseWindowTimer: - the MAC entity shall start pur-ResponseWindowTimer at the subframe that contains the end of the corresponding PUSCH transmission plus 4 subframes plus UE-eNB RTT. <<omitted>> While pur-ResponseWindowTimer is running, the MAC entity shall: - if the PDCCH transmission is addressed to the PUR-RNTI and contains an UL grant for a retransmission: - if PUR was transmitted in a non-terrestrial network and UE supports delaying the start of the pur-ResponseWindowTimer: - restart pur-ResponseWindowTimer at the last subframe of a PUSCH transmission corresponding to the retransmission indicated by the UL grant plus 4 subframes plus UE-eNB RTT. <<omitted>>

Therefore for CB-Msg4 monitoring, we also need to consider the NW/UE processing time as legacy PDDCH monitoring for transmission using PUR.

Proposal 5: (MAC-9) NW/UE processing time (e.g. 4 subframes) is needed to be considered for CB-Msg4 monitoring.

## (MAC-15): FFS on the details of the failure behaviour

Regarding the fallback mechanism, the following agreement is achieved:

4.	When max re-attempt number for current CE level has been reached, the UE does not move to the next CE level (FFS on the details of the failure behaviour)

Then in [2], all the companies support MAC notifying the RRC layer about the CB-Msg3-EDT failure. However, there is no clear consensus on what RRC should do for this failure. Considering that if the CB-Msg3-EDT is still failure when max re-attempt number for current CE level has been reached, then performing legacy EDT procedure may be also failure. And too many re-attempt would lead to an increase in transmission delay. At this moment, it is better for UE to directly initiate the legacy 4-step RA procedure based on network configuration.

Proposal 6: (MAC-15) It is proposed that when the CB-Msg3ResponseTimer expires and the maximum number of re-attempts has been reached, MAC notifies the RRC layer.

Proposal 7: (MAC-15) UE initiate the legacy 4-step RA procedure based on network configuration.

## (MAC-20): Should the CQI report in Msg3 in NB-IoT be supported in CB-Msg3-EDT

During the MAC open issue discussion , the CQI report issue is raised firstly and needs further discuss. From our perspective, CQI report is unnecessary and seems no significant gain for CB-Msg3 EDT procedure. Further, RAN4 check is needed, which may have no sufficient time to evaluate this. Therefore, suggest RAN2 not to consider the CQI report in R19.

Proposal 8: (MAC-20) suggest RAN2 not to consider the CQI report in R19.

## Other issue 1: DSA transmission window configuration

In RAN2 #129bis meeting, we agreed that the CB-msg3 EDT transmission window length and periodicity may be different and also could be the same. Then in case the length and periodicity are the same, some solutions could be discussed for signaling overhead reduction. As mentioned in the discussion, only one of the window length and window periodicity is needed at this moment. From our perspective, the DSA transmission window related configuration could be provided in specification optionally, and if window length or window periodicity is absent, it could implicitly indicated the two parameters are the same value, we could just provide some description in the field, further enhancement is not needed.

Proposal 9: If window length or window periodicity is absent, it could implicitly indicated the two parameters are the same value, we could just provide some description in the field, further enhancement is not needed.

## Other issue 2: OCC for CB-Msg3-EDT

In last meeting, we discuss OCC for CB-Msg3-EDT without convergence and postpone to this meeting.

Proposal 7 RAN2 do not work on OCC for CB-Msg3-EDT in release 19. - QC does not support the proposal and thinks that OCC could be supported without additional work for RAN2 (procedure wise, only the corresponding configuration is needed) - Ericsson doubts that OCC would work in this case. Nokia agrees - Toyota thinks that from RAN2 perspective there is no problem to support OCC We continue the discussion on OCC for CB-msg3-EDT in the next meeting.

Per the previous RAN1 reply LS [5], the feedback for OCC for CB-Msg3-EDT is as following:

Response: The OCC solution that RAN1 is working on may be applied to CB-Msg3 NPUSCH format 1 single tone at least for OCC length 2, for the subcarrier spacing(s) for which CB-Msg3 is supported, when the power imbalance is small. The design details for the OCC solution are ongoing.

Then in RAN2 #129 meeting, we reached related agreements:

1.	It is FFS if separate CB-msg3 resources would be needed for CB-msg-3 using OCC or if the same CB-msg3 resources could be used

2.	RAN2 assumes that one possibility to take power imbalance under control is to define RSRP ranges that need to be respected to transmit CB-msg3 using OCC

Considering that whether support OCC for CB-Msg3-EDT may need RAN1 further evaluation, while at this stage, there is no more time for RAN1 R19. Therefore, it is proposed not to support OCC for CB-Msg3-EDT in R19.

Proposal 10: It is proposed not to support OCC for CB-Msg3-EDT in R19.

## Other issue 3: issues in RAN1 reply LS

In RAN1 reply LS [4], the following issues are still need RAN2 discussion:

- RAN1 wonders whether RAN2 intends to support multi-PRB allocation or sub-PRB allocation or both Note: To be confirmed by RAN2 whether to support both singleTone and multitone, or singleTone only for HL parameter npusch-MCS-r16.

For whether support multi-PRB allocation and/or sub-PRB allocation, from our perspective, sub-PRB is beneficial for flexible resource allocation, especially when the data volume is small, the spectrum resources can be utilized more efficiently. And multi-PRB is more suitable for high data rate scenarios. Therefore, both multi-PRB allocation or sub-PRB allocation could be supported.

Then for whether to support both singleTone and multiTone, or singleTone only for HL parameter npusch-MCS-r16, based on the previous RAN1/4 reply LS [6][7], it is mentioned that the NB-IoT with 15kHz SCS may not satisfy the required timing accuracy for Msg3 transmission without Msg1/Msg2. However, in RAN2 #129bis meeting, the following agreement was achieved:

5.	We specify support for NB-IoT with 15kHz with no specific enhancements, leaving to NW implementation whether to implement this or not, accepting potential performance degradation.

Therefore, we could consider to support both singleTone and multiTone, and up to network implementation.

Proposal 11: It is proposed to support both multi-PRB allocation or sub-PRB allocation.

Proposal 12: It is proposed to support both singleTone and multiTone, and up to network implementation.

Conclusion

Based on the discussions mentioned above, in this contribution we provide some discussions on EDT enhancements for IoT-NTN and have the following observations and proposals:

Proposal 1: (MAC-2) Confirm the CB-RNTI calculation working assumption.

Proposal 2: (MAC-2) It is proposed to use floor(start SFN_id of Tx window / minimum Tx window periodicity] instead of the Msg3_W_index.

Proposal 3: (MAC-2) For Y, it is preferred to leave it as the current calculation formula.

Proposal 4: (MAC-6) Not to pursue power ramping for CB-Msg3-EDT in R19.

Proposal 5: (MAC-9) NW/UE processing time (e.g. 4 subframes) is needed to be considered for CB-Msg4 monitoring.

Proposal 6: (MAC-15) It is proposed that when the CB-Msg3ResponseTimer expires and the maximum number of re-attempts has been reached, MAC notifies the RRC layer.

Proposal 7: (MAC-15) UE initiate the legacy 4-step RA procedure based on network configuration.

Proposal 8: (MAC-20) suggest RAN2 not to consider the CQI report in R19.

Proposal 9: If window length or window periodicity is absent, it could implicitly indicated the two parameters are the same value, we could just provide some description in the field, further enhancement is not needed.

Proposal 10: It is proposed not to support OCC for CB-Msg3-EDT in R19.

Proposal 11: It is proposed to support both multi-PRB allocation or sub-PRB allocation.

Proposal 12: It is proposed to support both singleTone and multiTone, and up to network implementation.

Reference

- R2-130 NR-NTN-IoT-NTN (Sergio)_ EOM

- Draft_R2-250xxxx IoT NTN MAC Open issues_v15_Nokia

- R2-2503175 LS_CB_EDT

- R1-2504959-Reply LS to RAN2 on CB Msg3 EDT for IoT NTN Ph3

- R2-2500010_R1-2410895 Reply LS on OCC for CB-msg3 NPUSCH

- R2-2407920_R1-2407548 Reply LS to RAN2 on UL synchronization for contention based Msg3 transmission without Msg1/Msg2

- R2-2407931_R4-2414114 Reply LS to RAN2 on UL synchronization for contention based Msg3 transmission without Msg1/Msg2