# 3GPP TSG-WG2 Meeting #131                 	   R2-2505962

# Bengaluru, IN, 25th Aug 2025 - 29th Aug 2025

# Source: 			CMCC

# Title:  Discussion on Store and Forward remaining issues

# Agenda item:	8.9.2

# Document for: 	Discussion

Introduction

In the last RAN2#130 meeting, the following agreements [1] were approved as below:

Agreements:

1.	When the S&F operation indication is present, the agreed time information in SIB31 (i.e., t-ModeSwitching) indicates when the normal mode will start; otherwise, the agreed time information in SIB31 (i.e., t-ModeSwitching) indicates when the S&F mode will start.

2.	In a S&F network deployment which also exhibits discontinuous coverage, existing mechanisms to handle discontinuous coverage can be leveraged (e.g. satellite assistance information, UE not needing to perform idle mode tasks when the UE determines that is out of coverage, etc.). There is no need to modify existing discontinuous coverage features due to the addition of S&F Satellite operation. FFS if we clarify in discontinuous coverage procedure in idle mode that the UE also takes into account the information about NAS configured S&F monitoring list.

3.	Clarify in Stage 2 from Rel-18 (so in general, not only for S&F) that the values used in “satelliteId-r17” are expected to correspond to the values used in “satelliteId-r18” in other SIBs (e.g. the value used to identify the serving satellite in SIB33, SIB32 and SIB31 should be the same). Come back in the next meeting with an actual Rel-18 CR for this. FFS if in Rel-19 we need to say anything about the correspondence between the “satelliteId” IE and “S&F Monitoring List”.

Working Assumption:

1.	In the neighbour cell list we introduce an indication whether the cell operates in S&F mode or not (FFS if we also include the transition time). This WA can only be confirmed if we converge on the corresponding UE behaviour.

Based on the discussions and conclusions in the previous meetings, we continue discussing remaining issues on S&F operation.

Discussion

S&F Mode parameter in neighbour cell and UE Procedure changes

The first remaining issue is about whether to introduce additional assistance information for neighbour cells, e.g., the neighbour cell operation mode and/or mode transition time. The following WA has been made in the RAN2#meeting:

Working Assumption:

1.	In the neighbour cell list we introduce an indication whether the cell operates in S&F mode or not (FFS if we also include the transition time). This WA can only be confirmed if we converge on the corresponding UE behaviour.

This question was also mentioned in the remaining issues of TS36.304 [2]. In our opinion, both two assistance information (neighbour cell operation mode and mode transition time) are useful. For the operation mode, when there are multiple neighbour cells for UE to reselect and t-serviceStartNeigh/t-serviceStart of neighbour cells are almost same, introducing a mode operation indication can be useful for UE to reselect to a normal cell which can provide better services and data transmission. In our opinion, compared with S&F cells, normal cells shall have higher priority during the cell reselection procedure. Besides, for R19 UEs not supporting S&F operation, providing such an indication in neighbour cell list will help R19 UE not capable of S&F operation to ignore S&F mode cell and select to a normal cell it can camp on.

As for mode transition time, one potential benefit is that when the serving cell is in S&F mode, and the coming neighbour cell is in S&F mode but not in the satellite list provided by MME which means the neighbour cell does not have the UE context and has not recover the feeder link with the MME-ground yet. In this case, if the operation transition time e.g. t-serviceStartS&F smaller than t-service of the serving cell, that means UE can reselect to this normal cell before t-service expired (higher priority for normal cell). If the operation transition time e.g. t-serviceStartS&F larger than t-service, UE does not need to access the neighbour cell before t-service expiring unless the neighbour cell turns into normal mode. This has impacts on UE behaviour on measurement and cell reselection. Thus, we consider introducing operation mode transition time is useful.

Proposal 1 (TS 36.304 Open issue 2): It is proposed to introduce operation mode of the neighbour cells as the additional assistance information.

Proposal 2 (TS 36.304 Open issue 2): It is proposed to introduce mode transition time of the neighbour cells as the additional assistance information.

NAS Configured List for Discontinuous Coverage Operation

Another open issue in 36.304 is that whether to introduce NAS configured satellite ID list into RAN scope. In TS 23.401 [3], the following note has been added in SA2 spec:

NOTE 7:	How UE behaves when receiving the S&F Monitoring List is up to UE implementation. When a UE receives a S&F Monitoring List and the UE access a satellite that supports Store and Forward Satellite operation that is not on the S&F Monitoring List there is increased probability that it will not be able to complete the NAS procedure. The UE can continue to use the previously provided S&F Monitoring List, if the MME did not send one and the UE has previously been provided with one.

In our understanding, SA2 does not exclude the possibility that UE camps on a satellite not in the list but increasing the risk of uncompleted NAS procedure. Therefore, from the RAN side, we do not need to import NAS list and use it to exclude cell not in the list. There exists a possible scenario that: When UE makes cell selection, there is only a satellite not in the NAS configured list. However, through broadcasted system information, UE knows this satellite will soon switch from S&F mode to normal. In this way, UE can still wait until the cell switches to normal mode rather than bar this cell through NAS configured list directly. It is thus not recommended to use the NAS configured list as a rule for UE cell selection which may let UE miss some potential connection chances.

Furthermore, SA2 indicates that this list is information for NAS operation with no impact on AS. If the NAS list is introduced on the AS side, it would be necessary to issue a Liaison Statement (LS) to SA2 to inform them of the RAN side's conclusion and potential impacts on SA2 specifications. Considering limited time slot for R19, it is better not to make things complicated. In conclusion, the introduction of the NAS configured list is not recommended.

Proposal 3 (TS 36.304 Open issue 3): It is proposed to not introduce NAS configured list into RAN layer.

Paging Monitoring Relaxation for S&F mode

- In the remaining issues of TS36.304 [2], some companies show interests for paging monitoring relaxation in S&F operation. From our point of view, we agree this is beneficial in certain cases. For instance, when UE is within the satellite coverage area and camps on a cell in S&F operation and the network has already transmitted stored DL data, during the time information of the S&F operation mode, the network will not send new downlink data to the UE since feederlink does not recover. Thus the UE can have a relaxation on paging monitoring when UE goes back to idle state after receiving all stored data from the satellite.

- As for how the network realizes there is no stored DL data and tell UE about the information, the simplest way is to carry one bit indication in the RRCConnectionRelease message. However, in RAN2#129bis meeting, such agreement was made as “We don’t introduce any new release cause related to S&F operation”. However, the situation now is different from that time. We used to reject introducing a releasing reason due to the satellite switching to S&F mode. Now the situation is that when the satellite is in S&F mode and all stored DL data have been sent out, the network can send a bit indication in the RRC release message to tell UE. Thus, if we agree on relaxing paging monitoring when there is no stored DL data, the interaction between UE and network needs to be further studied (e.g. introducing a bit indication in RRC release message).

- As for how to relax paging monitoring, there are several ways to implement, e.g. 1) extend DRX cycle, 2) extend eDRX cycle TeDRX or 3) relax nB value.

- For 1) extend DRX cycle T, we can set long dormancy cycle for UE to sleep. However, extending DRX cycle mainly has two drawbacks: a)There are three kinds of DRX cycle in the current spec for NB-IoT: default, configured by NAS and configured by eNB. If the eNB or upper layer provides a DRX cycle, the current formula is to select a minimum value among different DRX cycles as : T = min (default DRX value, max (UE specific DRX value, minimum UE specific DRX value broadcast in system information)). Therefore, extending DRX cycle for S&F reason have some impacts to current DRX cycle selecting mechanism. b) For extending DRX cycle, it is needed to find a proper value to let UE wake up and do idle state task before t-service expired or t-ModeSwitching expired. It will make things complicated.

- For 2) extend eDRX cycle TeDRX, the longer sleep cycle can be configured than DRX cycle. But there is an issue that TeDRX is configured by the upper layers, RAN2 has not discussed whether eDRX cycle can be updated when the satellite is in S&F mode. Before we work on this solution, it’s better to ask CT1 firstly.

- For 3) Relax nB value, it is the simplest method since no need to modify DRX/eDRX cycle but can reduce numbers of PF/PO. To have better power saving performance, nB value can be configured to the minimum value T/T=1. That means no matter how long DRX cycle T is configured, UE only needs to monitor a PO in a PF of a DRX cycle. The summary of Pros and Cons of three methods are given in the table below.

Figure 1 Pros and cons analysis of three methods of paging monitoring relaxation

| - Paging monitoring relaxation methods | - Pros | - Cons |
| --- | --- | --- |
| - Extend DRX cycle T | - Long dormancy cycle | - Impacts to current NB-IoT DRX cycle selecting mechanism. - The updated DRX cycle needs to be closed to t-service or t-ModeSwitching time point which makes things complicated. |
| - Extend eDRX cycle TeDRX | - Longer dormancy cycle than DRX cycle | - It is not clear whether eDRX cycle can be updated when the satellite is in S&F mode. |
| - Relax nB value | - Simplest. No need to modify DRX cycle but can reduce numbers of PF/PO. | - / |

- Based on the above analysis, in our opinion, the most simplest way is to relax nB value to T/T=1. eNB could provide updated parameters to UE through system information. We propose corresponding TP for TS 36.304 in Annex A

- Proposal 4 (TS 36.304 Open issue 4): It is proposed to support paging monitoring relaxation for cell operating in S&F mode without any stored DL data.

- Proposal 5 (TS 36.304 Open issue 4): RAN2 consider how to indicate the completion of satellite stored DL data. (e.g. Reconsider adding a bit indication in RRCConnectionRelease message)

- Proposal 6 (TS 36.304 Open issue 4): RAN2 adopts the TP in Annex A as the baseline for S&F paging monitoring relaxation, if Proposal 4 is accepted.

UE capability

- In the remaining issues of TS36.306 [4], Question 4 is for whether to introduce a new AS capability for S&F capable UE given SA2 has already defined NAS level capability. In our opinion, As UE capability is not needed since the eNB can get S&F related information from MME. That means the core network can tell the eNB whether the UE supports S&F mode operation or not. Hence, it is not necessary for RAN layer to define a new UE capability for UE supporting S&F mode operation.

- Proposal 7 (TS 36.306 Question 4): Do not introduce a new AS capability for UEs supporting S&F mode operation.

System information change when UE in RRC_Connected state

For the possible changes of time information from S&F to normal mode, there was an agreement that a change to the (S&F-> normal mode) time indication shall not result in system information change notification. As for the changes of S&F operation indication in SIB1, we think similar rules can be applied at least for S&F capable UE in RRC connected mode in the operation mode switching procedure. NB-IoT NTN UE can deduce the mode switching based on agreed time information or T-service and eMTC NTN UE can get the updated SIB from dedicated signalling. Otherwise, if S&F operation indication changes result in system information change notification which may let network release UE to idle state to monitor paging message, UE in RRC connected mode has to go back to idle mode to receive SIB1 and then enter RRC connected mode again which will greatly increase the power consumption. Besides, this kind of operation also have a potential risk that UE can not go back to RRC connected mode after receiving system information change notification due to the heavy load of network (if cell turns into normal mode, many legacy UE or R19 S&F not capable UE will try to camp on the cell simultaneously). We should avoid this kind of corner case from happening. Thus, we propose that the changes of S&F operation indication during mode switching will not lead to UE in RRC connected mode go back to idle to monitor paging which may include system information change notification.

Proposal 8: Before mode switching, eMTC NTN UE in RRC connected mode can update changed S&F operation indication through dedicated signaling, rather than going back to idle mode and receiving paging message.

Proposal 9: Before mode switching from normal to S&F, NB-IoT NTN UE in RRC connected mode can deduce the time of operation mode switching by using operation time information based on implementation, the network does not need to release UE to idle state for the reason of changing the update system information.

Conclusions

- S&F Mode parameter in neighbour cell and UE Procedure changes

Proposal 1 (TS 36.304 Open issue 2): It is proposed to introduce operation mode of the neighbour cells as the additional assistance information.

Proposal 2 (TS 36.304 Open issue 2): It is proposed to introduce mode transition time of the neighbour cells as the additional assistance information.

- NAS Configured List for DC Operation

Proposal 3 (TS 36.304 Open issue 3): It is proposed to not introduce NAS configured list into RAN layer.

- Paging Monitoring Relaxation for S&F mode

- Proposal 4 (TS 36.304 Open issue 4): It is proposed to support paging monitoring relaxation for cell operating in S&F mode without any stored DL data.

- Proposal 5 (TS 36.304 Open issue 4): RAN2 consider how to indicate the completion of satellite stored DL data. (e.g. Reconsider adding a bit indication in RRCConnectionRelease message)

- Proposal 6 (TS 36.304 Open issue 4): RAN2 adopts the TP in Annex A as the baseline for S&F paging monitoring relaxation, if Proposal 4 is accepted.

- UE capability

- Proposal 7 (TS 36.306 Question 4): Do not introduce a new AS capability for UEs supporting S&F mode operation.

- System information change when UE in RRC_Connected state

Proposal 8: Before mode switching, eMTC NTN UE in RRC connected mode can update changed S&F operation indication through dedicated signaling, rather than going back to idle mode and receiving paging message.

Proposal 9: Before mode switching from normal to S&F, NB-IoT NTN UE in RRC connected mode can deduce the time of operation mode switching by using operation time information based on implementation, the network does not need to release UE to idle state for the reason of changing the update system information.

References

- 3GPP Chair_Notes, R2-130 NR-NTN-IoT-NTN (Sergio) EOM

- R2-2505872 Rapporteur Summary TS36.304 Open Issues

- 3GPP TS 23.401: "General Packet Radio Service (GPRS) enhancements for Evolved Universal Terrestrial Radio Access Network (E-UTRAN) access" (V19.3.0)

- R2-2505542 Open issues on Rel-19 IoT NTN UE capabilities

- Annex A: [TS 36.304] TP for Paging monitoring relaxation in DRX

  - 7.1	Discontinuous Reception for paging

The UE may use Discontinuous Reception (DRX) in idle mode in order to reduce power consumption. One Paging Occasion (PO) is a subframe where there may be P-RNTI transmitted on PDCCH or MPDCCH or, for NB-IoT on NPDCCH addressing the paging message. In P-RNTI transmitted on MPDCCH case, PO refers to the starting subframe of MPDCCH repetitions. In case of P-RNTI transmitted on NPDCCH, PO refers to the starting subframe of NPDCCH repetitions unless subframe determined by PO is not a valid NB-IoT downlink subframe then the first valid NB-IoT downlink subframe after PO is the starting subframe of the NPDCCH repetitions. The paging message is same for both RAN initiated paging and CN initiated paging.

The UE initiates RRC Connection Resume procedure upon receiving RAN paging. If the UE receives a CN initiated paging in RRC_INACTIVE state, the UE moves to RRC_IDLE and informs NAS.

One Paging Frame (PF) is one Radio Frame, which may contain one or multiple Paging Occasion(s). When DRX is used the UE needs only to monitor one PO per DRX cycle.

One Paging Narrowband (PNB) is one narrowband, on which the UE performs the paging message reception.

PF, PO, and PNB are determined by following formulae:

PF is given by following equation:

SFN mod T= (T div N)*(UE_ID mod N)

Index i_s pointing to PO from subframe pattern defined in 7.2 will be derived from following calculation:

i_s = floor(UE_ID/N) mod Ns

If P-RNTI is monitored on MPDCCH, the PNB is determined by the following equation:

PNB = floor(UE_ID/(N*Ns)) mod Nn

If P-RNTI is monitored on NPDCCH and the UE supports paging on a non-anchor carrier, and if paging configuration for non-anchor carrier is provided in system information, then the paging carrier is determined by the paging carrier with smallest index n (0 ≤ n ≤ Nn-1) fulfilling the following equation:

floor(UE_ID/(N*Ns)) mod W < W(0) + W(1) + … + W(n)

System Information DRX parameters stored in the UE shall be updated locally in the UE whenever the DRX parameter values are changed in SI. If the UE has no IMSI, for instance when making an emergency call without USIM, the UE shall use as default identity UE_ID = 0 in the PF, i_s, and PNB formulas above. If the UE has no 5G-S-TMSI, for instance when the UE has not yet registered onto the network, the UE shall use as default identity UE_ID = 0 in the PF and i_s formulas above.

The following Parameters are used for the calculation of the PF, i_s, PNB, wg, and the NB-IoT paging carrier:

-	T: DRX cycle of the UE.

In RRC_IDLE state:

-	Except for NB-IoT: If a UE specific extended DRX value of 512 radio frames is configured by upper layers according to 7.3, T =512. Otherwise, T is determined by the shortest of the UE specific DRX value, if allocated by upper layers, and a default DRX value broadcast in system information. If UE specific DRX is not configured by upper layers, the default value is applied.

In RRC_INACTIVE state, if extended DRX is not configured by upper layers as defined in 7.3:

-	T is determined by the shortest of the RAN paging cycle, if configured, the UE specific paging cycle, if allocated by upper layers, and the default paging cycle.

In RRC_INACTIVE state if extended DRX is configured by upper layers according to 7.3:

-	If a UE specific extended DRX value of 512 radio frames is configured, T is determined by the shortest of the RAN paging cycle, if configured, and 512 radio frames.

-	If a UE specific extended DRX value other than 512 radio frames is configured:

-	During the PTW, T is determined by the shortest of the RAN paging cycle, if configured, the UE specific paging cycle, if allocated by upper layers, and the default paging cycle. Outside the PTW, T is determined by the RAN paging cycle, if configured.

In RRC_INACTIVE state, if the UE supports inactiveStatePO-Determination and the network broadcasts ranPagingInIdlePO with value "true", the UE uses the T value applicable for RRC_IDLE state for the determination of i_s. Otherwise, the UE uses the T value applicable for RRC_INACTIVE state.

In RRC_INACTIVE state, a BL UE or a UE in enhanced coverage uses the T value applicable for RRC_IDLE state for the determination of PNB and i_s.

For NB-IoT: If UE specific DRX value is allocated by upper layers and minimum UE specific DRX value is broadcast in system information, T = min (default DRX value, max (UE specific DRX value, minimum UE specific DRX value broadcast in system information)). If UE specific DRX is not configured by upper layers or if the minimum UE specific DRX value is not broadcast in system information, the default DRX value is applied.

-	nB: 4T, 2T, T, T/2, T/4, T/8, T/16, T/32, T/64, T/128, and T/256, and for NB-IoT also T/512, and T/1024.

For IoT NTN: If a UE supporting R19 Store-and-Forward (S&F) operation specific the completion of stored downlink data of the serving satellite operating in S&F mode, nB value T/T=1 is applied. If the serving satellite is not related to mode switch, the applied nB value is valid till t-service expired. Otherwise, the applied nB value is valid till t-ModeSwitching expired.

NOTE: How the UE specific the completion of stored downlink data is FFS.

N: min(T,nB)

-	Ns: max(1,nB/T)