3GPP TSG-RAN WG2 Meeting #131	R2-2505964

Bangalore, India, Aug 25th – 29th, 2025

Agenda Item:	8.3.2.1

Source:	CMCC

Title:	Discussion on RRM measurement prediction with UE sided model

WID/SID:				FS_NR_AIML_Mob

Document for:	Discussion

# 1 Introduction

In last meeting RAN2#130 [1], RAN2 discussed LCM framework for UE sided model of AI Mobility and achieved the following agreements.

Agreements - Use the beam management agreements as the baseline, including the following aspects (details can be updated after further progress is made in AI/ML PHY BM use case) • Consider Option A and Option B like scheme (if/when AI/ML PHY makes further progress) • Upon receiving a full inference configuration, the UE sends the initial applicability report in RRCReconfigurationComplete. UAI can be sent to update applicability. • Upon receiving one or more full inference configuration(s) via RRCReconfiguration message, UE shall maintain all the full inference configuration(s) no matter the full inference configuration is applicable or inapplicable until the network releases it explicitly. • Support the explicit reporting of applicability/inapplicability in the initial report and subsequent reporting when the applicability changes. • Together with inapplicability reporting, UE further indicates a simple cause value of inapplicability (FFS pending AI/ML PHY progress). • No prohibit timer is introduced. - Associated ID should be optionally configurable for training and inference (i.e. it may not mandatorily required for all training/inference configurations). FFS for WI phase further details (what absence means, which use case, whether it is per cell or multiple cells/frequency, terminology).

Agreements - The following potential “inference parameters” can be considered for inference/measurement configuration, which provides the necessary information for UE to examine the applicability, it may contain • PW length (for temporal case A) • Skipping pattern (e.g. SSB config that indicates SSBs transmitted) (optional). MRRT (for temporal case B ) not discussed in study item, but if RAN4 says otherwise can be considered in WI • Measured and predicted beam pattern (optional). MRRS (for spatial domain prediction) not discussed in study item, but if RAN4 says otherwise can be considered in WI • Measured and predicted frequency (For inter-freq domain prediction) - Model related choices (i.e., cluster-based vs. cell-based, L1 filtering, RRM sub-use cases, OW length, direct vs. indirect) can be up to UE implementation unless a requirement is identified to specify them.

Agreements 1 For UE-sided model, NW-sided monitoring, the performance monitor procedure contains - NW sends monitoring configuration and inference configuration to UE. - UE reports measurements and inference output based on the corresponding configurations to NW. - NW performs monitoring and makes decisions, which can be fully NW-implemented. 2 For UE-sided model, UE-sided monitoring, the performance monitor procedure as baselined contains - NW sends monitoring configuration to UE - UE measures monitoring data and generates monitoring results by comparing inference output and monitoring data. - (NW-decision): UE reports the monitoring result to NW, and NW makes the management decision. - Can further consider UE sided monitoring and (UE-decision) for some use cases: UE makes the management decision based on network configuration and sends the decision to NW. FFS which use cases. - For RRM measurement prediction - RSRP differences can be used as the performance metric for monitoring.

This contribution will focus on applicability procedure, inference and reporting configuration for UE sided model for RRM measurement prediction.

# 2 Discussion

2.1 Applicability procedure

For AI/ML for NR air interface, the legacy UE capability framework serves as the baseline to report UE’s supported AI/ML-enabled Feature/FG. RRCReconfiguration message is used to carry the full inference configuration and/or inference related info which are used for UE’s decision on applicability/inapplicability of functionality. And the initial applicability/inapplicability reporting is carried in RRCReconfigurationComplete message. When there are changes for applicability/inapplicability of functionality, the subsequent applicability/inapplicability reporting could be carried in UAI.

The legacy UE capability framework could also be reused as the baseline for AI Mobility. Therefore, for RRM measurement prediction, this information is indicated in UE AS capability in RRC (e.g., UECapabilityEnquiry/ UECapabilityInformation).

According to the agreements achieved in RAN2#129bis [2], the spec impact of the related AI Mobility cases/subcases will be addressed first, and functionality granularity will be discussed later. Therefore, the further discussions on UE capability details (e.g., functionality granularity, content, structure of the related UE capabilities, etc.) could be carried during the WI phase.

Start by discussing spec/ran2 impact of the agreed cases/subcases and then discuss functionality granularity

Proposal 1: UE capability framework (i.e. UECapabilityEnquiry/UECapabilityInformation) could be used to report UE’s supported AI/ML-enabled Feature and/or supported functionalities for RRM measurement prediction with UE-sided model.

Proposal 2: The details on UE capability (e.g., functionality granularity, content, structure of the related UE capabilities, etc.) could be decided during the WI phase.

2.2 Inference

Condition when to perform the inference

During last meeting, RAN2 discussed the following potential “inference parameters” considered for inference/measurement configuration, which provides the necessary information for UE to examine the applicability, it may contain:

  - PW length (for temporal case A)

  - Skipping pattern (e.g. SSB config that indicates SSBs transmitted) (optional).  MRRT (for temporal case B ) not discussed in study item, but if RAN4 says otherwise can be considered in WI

  - Measured and predicted beam pattern (optional).   MRRS (for spatial domain prediction) not discussed in study item, but if RAN4 says otherwise can be considered in WI

  - Measured and predicted frequency (For inter-freq domain prediction)

For RRM measurement prediction with UE sided model, besides the above potential inference parameters that are used for UE to examine the applicability, we think RAN2 should further consider whether some condition information or event triggers are needed from the network to UE to activate/deactivate UE sided model or trigger UE to start/stop performing inference.

From our point of view, it is not reasonable for UE to activates the functionality directly and starts performing inference if the inference configuration is applicable. For example, the inference could not provide additional gain for the UE in cell centre and it is not friendly for UE complexity and power saving. On the other hand, when the UE approaches cell edge with a high probability of handover, the functionality should be activated and start performing inference quickly to provide the assistance information for handover decision. But if the inference is not able to provide the desired benefits or not needed any more, UE should deactivate the functionality promptly to reduce the complexity.

Observation 1: It is not reasonable for UE to activates the functionality directly and starts performing inference if the inference configuration is applicable.

Observation 2: The inference could not provide additional gain for the UE in cell centre and it is not friendly for UE complexity and power saving.

Observation 3: When the UE approaches cell edge with a high probability of handover, the functionality should be activated and start performing inference quickly to provide the assistance information for handover decision.

Observation 4: If the inference is not able to provide the desired benefits or not needed any more, UE should deactivate the functionality promptly to reduce the complexity.

Therefore, we think condition information/event triggers are needed for AI Mob since it could make UE sided model start/stop performing inference more quickly based on UE status, that is beneficial to improve HO performance or reduce UE complexity.

Observation 5: Condition information/event triggers could make UE sided model start/stop performing inference more quickly based on UE status, that is beneficial to improve HO performance or reduce UE complexity.

Proposal 3: RAN2 should consider to support condition information/event triggers provided from the network to UE to activate/deactivate UE sided model or trigger UE to start/stop performing inference.

We think the following potential condition information could be considered as the part of inference configuration or inference related information, which are provided from the network to UE and used for event triggered UE sided model activation or event triggered inference for RRM measurement prediction (e.g., Temporal domain case A) with UE sided model. For example,

  - The threshold of actual RRM measurement results, e.g., RSRP, it could be used to examine whether UE is near the cell edge and has high probability of handover;

  - The threshold of UE speed; When UE’s mobility is high, more unintended events e.g., handover failure, radio link failure etc. may occur. In this case, UE sided model inference could provide assistant information to network to avoid the unintended events.

Proposal 4: The following potential condition information could be considered as the part of inference configuration or inference related information, which are provided from the network to UE and used for event triggered UE sided model activation or event triggered inference for RRM measurement prediction (e.g., Temporal domain case A) with UE sided model. For example,

  - The threshold of actual RRM measurement results

  - The threshold of UE speed

Furthermore, we think some special scenarios might be regarded as UE sided model no longer being able to provide any additional benefits or not needed any more, and UE could stop performing inference directly. For example,

  - RLF or handover failure occurs during UE sided model activated, this means that the inference does not achieve the expected handover performance improvement.

  - The RRCRelease message could be used as implicit indication to deactivate UE sided model or release the full inference configuration(s).

Observation 6: RLF or handover failure occurs during UE sided model activated mean that the inference does not achieve the expected handover performance improvement.

Observation 7: The RRCRelease message could be used as implicit indication to deactivate UE sided model or release the full inference configuration(s).

Proposal 5: Some special scenarios might be regarded as UE sided model no longer being able to provide any additional benefits or not needed any more, and UE could stop performing inference directly. For example,

  - RLF or handover failure occurs during UE sided model activated

  - The RRCRelease message received

2.3 Reporting configuration

For RRM measurement prediction with UE sided model, the content of reporting from UE to NW could be considered as following:

For Temporal domain case A, UE could report the predicted cell level results/L3 filtered beam level results at multiple time instances with PW length in one measurement report.

For Temporal domain case B, UE could report the latest real or predicted cell level result(s)/L3 filtered beam level result(s) in one measurement report.

For Frequency domain prediction, UE could report predicted cell level result(s)/L3 filtered beam level result(s) of the predicted frequency in one measurement report.

For Spatial domain prediction, UE could report real or predicted L3 filtered beam level results as in the legacy.

Proposal 6: For RRM measurement prediction with UE sided model, the reporting content of RRM measurement prediction with UE sided model from UE to NW could be considered as following:

  - For Temporal domain case A, UE could report the predicted cell level results/L3 filtered beam level results at multiple time instances with PW length in one measurement report.

  - For Temporal domain case B, UE could report the latest real or predicted cell level result(s)/L3 filtered beam level result(s) in one measurement report.

  - For Frequency domain prediction, UE could report predicted cell level result(s)/L3 filtered beam level result(s) of the predicted frequency in one measurement report.

  - For Spatial domain prediction, UE could report real or predicted L3 filtered beam level results as in the legacy.

Based on the above analysis, we think the following spec impacts should be considered:

  - To report RRM measurement results at multiple time instances in one measurement report UE sided model.

  - If it is necessary to distinguish the predicted measurement results from the actual measurement results, new separate IE or indicator for the predicted measurement results may be introduced in the MeasurementReport message.

Proposal 7: For RRM measurement prediction with UE sided model, the following spec impacts should be considered:

  - To report RRM measurement results at multiple time instances in one measurement report UE sided model.

  - If it is necessary to distinguish the predicted measurement results from the actual measurement results, new separate IE or indicator for the predicted measurement results may be introduced in the MeasurementReport message.

# 3 Conclusion

In this paper, we provide our consideration on applicability procedure, inference and reporting configuration for RRM measurement prediction with UE-sided model. We have the following observations and proposals:

Applicability procedure

Proposal 1: UE capability framework (i.e. UECapabilityEnquiry/UECapabilityInformation) could be used to report UE’s supported AI/ML-enabled Feature and/or supported functionalities for RRM measurement prediction with UE-sided model.

Proposal 2: The details on UE capability (e.g., functionality granularity, content, structure of the related UE capabilities, etc.) could be decided during the WI phase.

Condition when to perform the inference

Observation 1: It is not reasonable for UE to activates the functionality directly and starts performing inference if the inference configuration is applicable.

Observation 2: The inference could not provide additional gain for the UE in cell centre and it is not friendly for UE complexity and power saving.

Observation 3: When the UE approaches cell edge with a high probability of handover, the functionality should be activated and start performing inference quickly to provide the assistance information for handover decision.

Observation 4: If the inference is not able to provide the desired benefits or not needed any more, UE should deactivate the functionality promptly to reduce the complexity.

Observation 5: Condition information/event triggers could make UE sided model start/stop performing inference more quickly based on UE status, that is beneficial to improve HO performance or reduce UE complexity

Proposal 3: RAN2 should consider to support condition information/event triggers provided from the network to UE to activate/deactivate UE sided model or trigger UE to start/stop performing inference.

Proposal 4: The following potential condition information could be considered as the part of inference configuration or inference related information, which are provided from the network to UE and used for event triggered UE sided model activation or event triggered inference for RRM measurement prediction (e.g., Temporal domain case A) with UE sided model. For example,

  - The threshold of actual RRM measurement results

  - The threshold of UE speed

Observation 6: RLF or handover failure occurs during UE sided model activated mean that the inference does not achieve the expected handover performance improvement.

Observation 7: The RRCRelease message could be used as implicit indication to deactivate UE sided model or release the full inference configuration(s).

Proposal 5: Some special scenarios might be regarded as UE sided model no longer being able to provide any additional benefits or not needed any more, and UE could stop performing inference directly. For example,

  - RLF or handover failure occurs during UE sided model activated

  - The RRCRelease message received

Reporting configuration

Proposal 6: For RRM measurement prediction with UE sided model, the reporting content of RRM measurement prediction with UE sided model from UE to NW could be considered as following:

  - For Temporal domain case A, UE could report the predicted cell level results/L3 filtered beam level results at multiple time instances with PW length in one measurement report.

  - For Temporal domain case B, UE could report the latest real or predicted cell level result(s)/L3 filtered beam level result(s) in one measurement report.

  - For Frequency domain prediction, UE could report predicted cell level result(s)/L3 filtered beam level result(s) of the predicted frequency in one measurement report.

  - For Spatial domain prediction, UE could report real or predicted L3 filtered beam level results as in the legacy.

Proposal 7: For RRM measurement prediction with UE sided model, the following spec impacts should be considered:

  - To report RRM measurement results at multiple time instances in one measurement report UE sided model.

  - If it is necessary to distinguish the predicted measurement results from the actual measurement results, new separate IE or indicator for the predicted measurement results may be introduced in the MeasurementReport message.

# Reference

- Draft_RAN2_130_Meeting_Report_v3, May 2025.

- Draft_RAN2_129bis_Meeting_Report_v2, April 2025.