3GPP TSG-RAN WG2 Meeting #131	R2-2505966

Bangalore, India Aug 25th – 29th , 2025

Agenda Item:	8.6.3

Source:	CMCC

Title:	Remaining issues of L1 event triggered measurement reporting

Document for:	Discussion

Introduction

At the RAN2#130 meeting we reached the following agreements [1]:

RAN2 #129bis: Agreements on L1 event triggered MR - UE deactivates SP CSI-RS resource of candidate cells (other than the target cell) after cell switch. FFS on the target cell. - Instead of candidate cell id and SP CSI-RS resource set id, LTM-CSI-ResourceConfigId is included into SP CSI-RS activation/deactivation MAC CE. - Confirm working assumption, i.e. four codepoints to represent one of {type1, type2, type3, type4}, as an agreement. For detailed description of type 1, it can be handled as part of running CR preparation. - (MAC-6) For the truncated MR MAC CE, the UE determines the beam to be included in the MAC CE, based on the priority: Type 1 beam > Type 2 beam > Type 3 beam > Type 4 beam. If the UE cannot include all of the beams with the same priority, the beams are selected in the (truncated) MR MAC CE based on measured RS quality. Among all selected beams, the UE puts the best quality beam as the reference beam as the first one included in the (truncated) MR MAC CE. - Working assumption: If network configures to include the current serving beam in MR, the UE always includes the current serving beam in the truncated MR MAC CE. In this case, minimum size of truncated MR MAC CE is {at least one triggered beam + the current serving beam}. If grant is not enough for that, the UE does not assemble a truncated MR MAC CE. - Same priority between regular MR MAC CE and truncated MR MAC CE. It is up to UE implementation how to handle/include if more than one triggering events are pending and UL grant is not sufficient for all regular L1 MR MAC CEs. - No prohibit timer is applied to L1 event-triggered MR.

Discussion

MAC-25: Handling of the activated SP CSI-RS for target cell after LTM cell switch

At the RAN2 130 meeting, discussions were held regarding when to deactivate the SP CSI-RS resource of candidate cells, but there was no consensus on how to handle target cell. Some companies believe that the UE should continue to measure the SP CSI-RS resource of target cell to obtain CSI after cell switch, but we believe this may not be a technical issue. First, it's important to clarify that RAN1 introduced SP CSI-RS for two purposes, as summarized in the meeting conclusions below:

RAN1 118bis Agreement： Working Assumption In addition to periodic CSI-RS, semi-persistent CSI-RS is supported for candidate cell L1-RSRP measurement for gNB scheduled reporting from RAN1 perspective - Send an LS to RAN3 (CC RAN2) to ask for the feasibility of specifying the signalling for coordination between serving cell and candidate cell(s) on the transmission of semi-persistent CSI-RS(s) and any other potential issues (e.g. RAN3 workload). Support of semi-persistent CSI-RS is subject to UE capability. Comeback for final LS. RAN1 120bis Agreement： For candidate cell CSI acquisition - In addition to periodic CSI-RS resource, semi-persistent CSI-RS resource is supported - Support of semi-persistent CSI-RS resource is subject to separate UE capability. - MAC CE is used to activate/deactivate the semi-persistent CSI-RS resource similarly to the legacy mechanism for a serving cell which will be specified in RAN2 Send an LS to RAN2 and RAN3 to inform this agreement.

Due to their different purposes, the two types of semi-persistent CSI-RS resources are configured differently. The most significant difference is that the CSI-RS for Beam Management, used to measure L1-RSRP, is a single-port resource, while the CSI-RS for candidate cell CSI acquisition, based on the conclusions of the RAN1 120bis meeting, is a resource with a maximum of 128 ports:

RAN1 118 Agreement For gNB scheduled reporting and event triggered reporting - At least CSI-RS for beam management is supported for L1-RSRP measurement for candidate cell. - FFS: CSI-RS for mobility. RAN1 119 Agreement No consensus to support CSI-RS for mobility for L1 measurement in Rel-19 LTM. RAN1 120bis Agreement For the number of CSI-RS ports of CSI-RS resource(s) associated with a CSI report configuration for a candidate cell for LTM CSI acquisition. - Up to 128 ports is supported. - The supported max number of CSI-RS ports is up to separate UE capability.

Observation 1: CSI-RS for beam management L1-RSRP measurement is a single-port resource, while CSI-RS for candidate cell CSI acquisition is a resource with up to 128 ports.

Therefore, it is clear that the SP CSI-RS used for L1-RSRP measurement and the SP CSI-RS used for CSI acquisition are completely independent in terms of measurement resources and reporting configuration.

Observation 2: The SP CSI-RS used for L1-RSRP measurement and CSI acquisition are completely independent in terms of measurement resources and reporting configuration.

Therefore, the deactivation of the SP CSI-RS resource of the target cell should be discussed separately based on different purposes. From our perspective, the SP CSI-RS used for L1-RSRP measurement should be deactivated after the cell switch, because continuing to perform RRM measurements under the source cell configuration is neither reasonable nor meaningful. Measurements after cell switch should be based on the target cell configuration.

Proposal 1: The SP CSI-RS used for L1-RSRP measurement should be deactivated after the cell switch.

The RAN1#120 meeting clarified that the UE can measure CSI before CSC, depending on the UE capabilities, but CSI reporting should be performed after handover.

Therefore, for the SP CSI-RS used for CSI acquisition, the timing of deactivation should depend on whether the UE has completed the CSI measurement of the target cell. After completion, it should be deactivated, regardless of whether this time point is before or after the cell switch.

Proposal 2: For the SP CSI-RS used for CSI acquisition, the timing of deactivation should depend on whether the UE has completed the CSI measurement of the target cell. After completion, it should be deactivated, regardless of whether this time point is before or after the cell switch.

MAC-8: The coexistence between event triggered report and mTRP

In the email discussion at [POST130][117], there were still some disagreements on two issues:

Question 1: How the UE selects the serving beam to be included in the MR MAC CE when mTRP is configured in the serving cell? Please select from the options listed below:

- UE should use the best beam of the two “current beams” for event evaluation

- It is up to the UE which of the two beams it uses for L1 event evaluation

- Network indicates which beam the UE shall use

- Specify to use one of the two TCI states (e.g., just use the first indicated TCI state)

- Other

For option (a), the best beam at a given moment may not be meaningful due to the rapid variation in L1-RSRP. Furthermore, consider a scenario where beam1 has the best L1-RSRP at time T1, and the UE triggers the evaluation of event LTM2/3/5. Later, at time T2, before the TTT is met, beam2 becomes the best beam. How should the UE handle this evaluation event? Therefore, we believe option (a) is not a good solution and may lead to additional technical discussions.

For (b), this may cause different UEs to have completely different reporting behaviors under the same channel conditions, which is very detrimental to network management.

(c) and (d) offer flexibility in network control and are therefore preferred. Furthermore, since (c) introduces additional signaling overhead, (d) is based on a simple, deterministic, hard-coded rule, and the network still maintains control flexibility through the configuration order of TCI states, it is the optimal option in our opinion.

Proposal 3: Option (d) is supported because it provides a simple, deterministic rule while still allowing network control flexibility through the configuration order of TCI states.

Question 2: Do you agree to indicate in MR MAC CE when mTRP is used which TCI state is associated with the reported RSRP value for the serving/current beam?

Because we support Option (d) in Question 1, the network knows which TRP the UE has selected for its serving beam without requiring additional indication. Furthermore, even for Option (a) or Option (b), event-triggered L1 measurement reporting is used for LTM cell switch or early sync. The network does not need to know the TRP of the current serving beam when making LTM decisions and selecting candidate cells/beams, so TRP information is meaningless.

Proposal 4: No need to indicate TRP information in MR MAC CE.

Conclusions

In this contribution, we analyse for measurement event evaluation, following are the observations and proposals.

Observations:

Observation 1: CSI-RS for beam management L1-RSRP measurement is a single-port resource, while CSI-RS for candidate cell CSI acquisition is a resource with up to 128 ports.

Observation 2: The SP CSI-RS used for L1-RSRP measurement and CSI acquisition are completely independent in terms of measurement resources and reporting configuration.

Proposals:

Proposal 1: The SP CSI-RS used for L1-RSRP measurement should be deactivated after the cell switch.

Proposal 2: For the SP CSI-RS used for CSI acquisition, the timing of deactivation should depend on whether the UE has completed the CSI measurement of the target cell. After completion, it should be deactivated, regardless of whether this time point is before or after the cell switch.

Proposal 3: Option (d) is supported because it provides a simple, deterministic rule while still allowing network control flexibility through the configuration order of TCI states.

Proposal 4: No need to indicate TRP information in MR MAC CE.

References

- R2-2504671, Report from session on R16/17/18 V2X/SL, R18/19 MOB and R19 NES.