3GPP TSG-RAN WG2 Meeting #131			         R2-2505977

Bangalore, India  Aug 25th – 29th , 2025

Agenda Item:	8.4.3

Source: 	LG Electronics Inc.

Title:         	Remaining issues on measurement offloading and relaxation

Document for: 	Discussion and Decision

Introduction

In this contribution we discuss the official/unofficial open issues on the RRM measurement offloading and relaxation.

Discussion

Official open issues

[Open issue RRC-15/38304-2] FFS (if needed) on enhancements based on R16 criteria (e.g., based on the LR measurements) for the case when MR serving cell measurement results are not available.

Since the Rel-16 relaxed measurement criteria, i.e. for UE with low mobility and UE not at cell edge, are based on Srxlev. UE supporting both Rel-16 RRM relaxation and Rel-19 RRM offloading will not be able to derive Srxlev during RRM offloading, and therefore will not be able to start or stop the Rel-16 relaxed measurement.

There are two possible solutions to address this issue. The first is to disallow Rel-16 relaxed measurement during RRM offloading. If the network configures Rel-19 RRM relaxation together, then Rel-19 UEs would not need to perform Rel-16 RRM relaxation. The second approach is to introduce a LR threshold that allows UEs to determine whether to perform Rel-16 relaxed measurement based on the LR measurements. However, it does not seem desirable that network configures separate configuration to enable Rel-16 RRM relaxation for Rel-19 UEs, although it supports Rel-19 RRM relaxation also. Therefore, we propose not to consider the scenario where network configures RRM offloading and Rel-16 RRM relaxation without Rel-19 RRM relaxation, and if Rel-19 RRM relaxation and Rel-16 RRM relaxation are configured together, Rel-19 UEs performs Rel-19 RRM relaxation only.

Proposal 1	No enhancements of Rel-16 relaxed measurement criteria for UEs supporting the measurement offloading to LR.

Proposal 2	Do not consider the scenario where the measurement offloading to LR and Rel-16 measurement relaxation are configured together without Rel-19 measurement relaxation.

Proposal 3	If Rel-19 measurement relaxation and Rel-16 measurement relaxation are configured together, Rel-19 UEs performs Rel-19 measurement relaxation only.

[Open issue RRC-7/38304-3] FFS on exit condition for serving cell RRM relaxation

While the UE is performing LP-WUS monitoring or RRM offloading, it does not have MR measurement results. Therefore, an exit condition based on LR measurement results is necessary. This is why we have defined separate exit conditions for LP-WUS monitoring and RRM offloading, in addition to the entry condition.

However, since the UE can still obtain MR measurement results while performing RRM relaxation, it can determine whether to discontinue RRM relaxation based on both MR and LR measurement results, just as it does for the entry condition. As in R16 RRM relaxation, the UE can stop RRM relaxation if the entry condition is no longer met without separate exit condition. Therefore, we propose following:

Proposal 4	No separate exit condition other than ‘not fulfilling the entry condition’ for Rel-19 RRM relaxation.

[Open issue RRC-10/38304-6] FFS on low mobility criteria

Sone companies have expressed concerns regarding the radio switching of RRM offloading/relaxation status for UEs in a high mobility state. However, this issue can be mitigated to some extent by configuring separate thresholds for entry condition and exit condition. I.e. higher threshold in entry condition. Furthermore, even if the ping-pong happens, it should not be problematic, as it only results in UE changing the receiver used for serving cell measurement or a change in the measurement requirements. Therefore, the low mobility criteria do not need to be considered in Rel-19 RRM relaxation/offloading.

Proposal 5	Do not consider the Low mobility criteria for Rel-19 RRM offloading/relaxation.

Unofficial open issues

[HW (003)] UE behavior when the exit condition for serving cell measurement offloading is fulfilled

In current running CR, the UE behaviour when the exit condition for serving cell measurement offloading is fulfilled is specified as follow:

LP-WUS UE is not required to perform serving cell measurement offloading according to requirements specified in TS 38.133 [8] if the exit condition for serving cell measurement offloading in clause 5.2.4.x.4 is fulfilled.

This means UE can still perform serving cell measurement offloading (i.e., serving cell measurement fully offloaded to LR and no serving cell measurement via MR is required), though the exit condition for serving cell measurement offloading is fulfilled. If the exit condition for serving cell measurement offloading is fulfilled, the UE should stop serving cell measurement offloading as suggested by HW, so that the UE can resume the serving cell measurement via MR.

Proposal 6	Change CR as follow: LP-WUS UE shall stop performing serving cell measurement offloading according to requirements specified in TS 38.133 [8] if the exit condition for serving cell measurement offloading in clause 5.2.4.x.4 is fulfilled.

Whether to support LR measurement-based uplink carrier selection for RA/SDT (NUL or SUL)

Upon initiation of random access of SDT, UE selects uplink carrier between NUL and SUL based on the measurement results of the serving cell, i.e. the RSRP of the downlink pathloss reference. The uplink carrier should be selected regardless of whether the serving cell quality is good or poor, so UE should be able to select the uplink carrier even when the serving cell is measured by LR only. Since the transmit power of the SSB and that of the LP-SS may differ, it is not appropriate to use the existing threshold intended for comparing the SSB measurement results, i.e. rsrp-ThresholdSSB-SUL, for comparing the LR measurement. Accordingly, a separate threshold, e.g. rsrp-ThresholdLP-SS-SUL, is needed to enable the UE to select the uplink carrier based on the LR measurements.

Alternatively, if RRM offloading is stepped and the serving cell measurement using MR is started when the random access or SDT is initiated, the UE will be able to select the uplink carrier based on the MR measurements. However, the UL carrier selection may be delayed due to the time it takes to reach a state where the MR measurement is possible and the time required to acquire a stable MR measurement result for the serving cell. Therefore, we propose to discuss which option is preferred and choose one of them.

Proposal 7	For uplink carrier selection between NUL and SUL, choose one of the following two options:

- Option 1. Introduce a LR threshold, e.g. rsrp-ThresholdLP-SS-SUL, that allows UE to select the uplink carrier for random access or SDT based on LR measurements.

- Option 2. UE stops RRM offloading and starts serving cell measurement using MR upon initiation of the random access or SDT.

Whether to support LR measurement-based SDT condition

For a UE configured with SDT, the decision to perform SDT is made by comparing the SSB based measurements, i.e. RSRP of the downlink pathloss reference, and the SSB thresholds, i.e. sdt-RSRP-Threshold, cg-SDT-RSRP-ThresholdSSB, mt-SDT-RSRP-Threshold. If UE starts to perform the serving cell measurement using MR upon initiation of SDT as in Option 2 in Proposal 2, LR thresholds would not be necessary. However, if we go with Option 1 in Proposal 2, we would similarly need to introduce LR thresholds for SDT.

Proposal 8	For initiation of SDT, choose one of the following two options:

- Option 1. Introduce LR thresholds, e.g. sdt-LR-RSRP-Threshold, cg-SDT-LR-RSRP-ThresholdLP-SS, and mt-SDT-LR-RSRP-Threshold, that allows UE to determine whether to initiate SDT based on the LR measurements.

- Option 2. UE stops RRM offloading and starts serving cell measurement using MR upon initiation of SDT.

Whether to support LR measurement-based conditions for resuming RRC connection for multicast reception

Network can configure a threshold, i.e. ThresholdMBS, for UEs receiving a multicast session in RRC_INACTIVE, and if the serving cell quality becomes below that the threshold, the UE transition to RRC_CONNECTED via RRC resume. Since this threshold is also designed for comparison with SSB measurements, it is not appropriate for use while performing the RRM offloading to LR.

However, considering that the purpose of the RRM offloading to LR is to prevent UE’s MR from waking up solely for serving cell measurement, it is difficult to expect power saving through the RRM offloading to LR for UEs receiving MBS session using MR. In fact, the LR measurement might consume even more power. Therefore, we propose selecting one of the following two options.

Proposal 9	For UEs supporting Rel-18 multicast reception in RRC_INACTIVE and the RRM offloading, choose one of the following two options:

- Option 1. Introduce LR threshold, e.g. ThresholdMBS-LR, that allows UEs receiving multicast in RRC_INACTIVE to determine whether to perform RRC resume based on LR measurements.

- Option 2. UEs receiving MBS session in RRC_IDLE/INACTVEI do not perform the RRM offloading.

Whether to support LR measurement-based Early Measurement Reporting

The legacy RRM relaxation is not allowed for a frequency if EMR is configured for that frequency.

TS 38.304 The above relaxed measurements and no measurement are not applicable for frequencies that are included in VarMeasIdleConfig, if configured and for which the UE supports dual connectivity or carrier aggregation between those frequencies and the frequency of the current serving cell.

Similar to the legacy relaxed measurement, the LR measurement is also not appropriate for EMR purpose. Only normal measurement using MR should be reported for EMR.

Proposal 10	If EMR is configured for the serving frequency, the UE does not perform the RRM offloading to LR.

Proposal 11	UE does not perform Rel-19 relaxed measurement for frequencies for which EMR is configured.

Conclusion

Official open issues:

Proposal 1	[RRC-15/38304-2] No enhancements of Rel-16 relaxed measurement criteria for UEs supporting the measurement offloading to LR.

Proposal 2	[RRC-15/38304-2] Do not consider the scenario where the measurement offloading to LR and Rel-16 measurement relaxation are configured together without Rel-19 measurement relaxation.

Proposal 3	[RRC-15/38304-2] If Rel-19 measurement relaxation and Rel-16 measurement relaxation are configured together, Rel-19 UEs performs Rel-19 measurement relaxation only.

Proposal 4	[RRC-7/38304-3] No separate exit condition other than ‘not fulfilling the entry condition’ for Rel-19 RRM relaxation.

Proposal 5	[RRC-10/38304-6] Do not consider the Low mobility criteria for Rel-19 RRM offloading/relaxation.

Unofficial open issues:

Proposal 6	[HW003] Change CR as follow: LP-WUS UE shall stop performing serving cell measurement offloading according to requirements specified in TS 38.133 [8] if the exit condition for serving cell measurement offloading in clause 5.2.4.x.4 is fulfilled.

Proposal 7	For uplink carrier selection between NUL and SUL, choose one of the following two options:

- Option 1. Introduce a LR threshold, e.g. rsrp-ThresholdLP-SS-SUL, that allows UE to select the uplink carrier for random access or SDT based on LR measurements.

- Option 2. UE stops RRM offloading and starts serving cell measurement using MR upon initiation of the random access or SDT.

Proposal 8	For initiation of SDT, choose one of the following two options:

- Option 1. Introduce LR thresholds, e.g. sdt-LR-RSRP-Threshold, cg-SDT-LR-RSRP-ThresholdLP-SS, and mt-SDT-LR-RSRP-Threshold, that allows UE to determine whether to initiate SDT based on the LR measurements.

- Option 2. UE stops RRM offloading and starts serving cell measurement using MR upon initiation of SDT.

Proposal 9	For UEs supporting Rel-18 multicast reception in RRC_INACTIVE and the RRM offloading, choose one of the following two options:

- Option 1. Introduce LR threshold, e.g. ThresholdMBS-LR, that allows UEs receiving multicast in RRC_INACTIVE to determine whether to perform RRC resume based on LR measurements.

- Option 2. UEs receiving MBS session in RRC_IDLE/INACTVEI do not perform the RRM offloading.

Proposal 10	If EMR is configured for the serving frequency, the UE does not perform the RRM offloading to LR.

Proposal 11	UE does not perform Rel-19 relaxed measurement for frequencies for which EMR is configured.