3GPP TSG-RAN WG2 Meeting #131	R2-2505982

Bengaluru, India, Aug. 25th– 29th, 2025

Agenda Item:	8.11.2

Source:	vivo

Title:	Remaining issues for RACH in SBFD

Document for:	Discussion and Decision

1	Introduction

In the post RAN2#130 email discussion on running CR of TS 38.321, some open issues are raised in [1], and we will discuss these issues in this contribution.

2	Discussion

2.1	SBFD RO location derivation in CFRA

In the post RAN2#130 email discussion on running CR of TS 38.321 [1], the following scenario on SBFD RO location derivation in CFRA is raised,

- CBRA does not provide any SBFD RO, but gNB can still schedule SBFD RO to UE in CFRA.

That is, SBFD RO can only be used in CFRA in such case, however, this restriction is not appropriate due to it leads to that SBFD cannot be used for idle UEs, i.e. SBFD cannot be used within most of the time. In our view, when SBFD is configured, network shall provide SBFD RACH configuration at least for CBRA. Regarding the scenario mentioned above, it can be left to network implementation to ensure that this strange scenario is avoided.

Proposal 1: RAN2 confirm that Network can indicate RO type as SBFD RO for CFRA, and it is left to network implementation whether to configure SBFD RO for CBRA.

2.2	Parameter initialization

In the post RAN2#130 email discussion on running CR of TS 38.321 and open issue discussion, there is one open issue that whether parameter initialization is needed when the RAR reception is not successful and the RACH procedure is not completed. In our view, parameter initialization when the RAR reception is not successful is needed for the next preamble attempt for the following cases,

- Case 1: Switch to another RACH configuration in the case of RACH configuration Option 2

- Case 2: Switch to next higher Msg1 repetition number

For Case 1, there are two separate RACH configurations(i.e. one legacy RACH configuration and one additional RACH configuration) to support random access operation for SBFD-aware UEs, and the configuration may be different. When the number of preamble (re)transmission attempts reaches preambleTransMaxRO-Type, RO type switching is triggered and followed by the selection of the set of Random Access resources of the other RO type. If the RO type is switched from non-SBFD RO to SBFD RO, and the RACH configurations are also adjusted, i.e. legacy RACH configuration switch to additional RACH configuration, the parameter initialization is needed since different configurations maybe configured. In such case, UE should perform parameter initialization for all of the parameters in additional RACH configuration as specified in TS 38.321.

Proposal 2: Parameter initialization is performed for all of the parameters in RACH-ConfigCommon after RO type switching in the case of RACH configuration Option 2, as specified in MAC spec.

For Case 2, the following agreement was achieved at RAN2#130.

=> The UE is allowed to switch to an RO type that is configured with the same Msg1 repetition number. FFS on higher Msg1 repetition number, if the same is not available.

The FFS was discussed during the post RAN2#130 email discussion, and most company think that the UE is allowed to switch to the next higher Msg1 repetition number if the same not available. Similar case has been specified in the coverage enhancement topic as shown below.

2> if the Random Access procedure is not completed: 3> if the Random Access Preamble is transmitted with repetitions and neither contention-free Random Access Resources nor Random Access resources for SI request have been provided for this Random Access procedure: 4> if PREAMBLE_TRANSMISSION_COUNTER = [preambleTransMax-Msg1-Repetition] + 1; or 4> if PREAMBLE_TRANSMISSION_COUNTER = 2 × [preambleTransMax-Msg1-Repetition] + 1: 5> if set of Random Access resources configured with the same prach-ConfigurationIndex and associated with a higher Msg1 repetition number with the same feature or feature combination as the current set of Random Access resources is available: 6> select the set of Random Access resources associated with the next higher Msg1 repetition number with the same feature or feature combination for this Random Access procedure; 6> initialize startPreambleForThisPartition, numberOfPreamblesPerSSB-ForThisPartition, ssb-SharedRO-MaskIndex and numberOfRA-PreamblesGroupA parameters for the Random Access procedure according to the values configured by RRC for the selected set of Random Access resources.

Similarly, when UE selects the set of Random Access resources associated with the next higher Msg1 repetition number when RO type switching, the parameters startPreambleForThisPartition, numberOfPreamblesPerSSB-ForThisPartition, ssb-SharedRO-MaskIndex and numberOfRA-PreamblesGroupA should also be initialized.

Proposal 3: Initialize startPreambleForThisPartition, numberOfPreamblesPerSSB-ForThisPartition, numberOfRA-PreamblesGroupA and msg1-RepetitionTimeOffsetROGroup parameters when UE is switched to an RO type with the next higher Msg1 repetition number.

3	Conclusion

Based on the discussion, we have the following proposals,

Proposal 1: RAN2 confirm that Network can indicate RO type as SBFD RO for CFRA, and it is left to network implementation whether to configure SBFD RO for CBRA..

Proposal 2: Parameter initialization is performed for all of the parameters in RACH-ConfigCommon after RO type switching in the case of RACH configuration Option 2, as specified in MAC spec.

Proposal 3: Initialize startPreambleForThisPartition, numberOfPreamblesPerSSB-ForThisPartition, numberOfRA-PreamblesGroupA and msg1-RepetitionTimeOffsetROGroup parameters when UE is switched to an RO type with the next higher Msg1 repetition number.

4	Reference

[1] R2-2505560, Summary of the SBFD open issues in MAC, Samsung

5  Annex TP for Proposal 2 and Proposal 3

### 5.1.4	Random Access Response reception

Once the Random Access Preamble is transmitted and regardless of the possible occurrence of a measurement gap, the MAC entity shall:

<irrelevant text omitted>

2>	if the Random Access procedure is not completed:

3> if preambleTransMaxRO-Type is applied, and neither contention-free Random Access Resources nor Random Access resources for SI request have been provided for this Random Access procedure, and PREAMBLE_TRANSMISSION_COUNTER = preambleTransMaxRO-Type + 1:

4> if the RO_TYPE is set to 2nd-RO, and set of Random Access resources associated with the same feature or feature combination, and with the same Msg1 repetition number (if the Random Access Preamble is transmitted with repetitions), as the current set of Random Access resources, is available for the first PRACH occasions as defined in TS 38.213 [6]:

5> set the RO_TYPE to 1st-RO;

5> select the set of Random Access resources for this Random Access procedure.

4> else if the RO_TYPE is set to 1st-RO, and set of Random Access resources associated with the same feature or feature combination, and with the next higher Msg1 repetition number (if the Random Access Preamble is transmitted with repetitions), as the current set of Random Access resources, is available for the second PRACH occasions as defined in TS 38.213 [6]:

5> set the RO_TYPE to 2nd-RO;

5> select the set of Random Access resources for this Random Access procedure.

6>	initialize startPreambleForThisPartition, numberOfPreamblesPerSSB-ForThisPartition, numberOfRA-PreamblesGroupA and msg1-RepetitionTimeOffsetROGroup parameters for the Random Access procedure according to the values configured by RRC for the selected set of Random Access resources.

4> else if the RO_TYPE is set to 1st-RO, and set of Random Access resources associated with the same feature or feature combination, and with the same Msg1 repetition number (if the Random Access Preamble is transmitted with repetitions) or the next higher Msg1 repetition number (if the Random Access Preamble is transmitted with repetitions and the same Msg1 repetition number is not available), as the current set of Random Access resources, is available for the second PRACH occasions as defined in TS 38.213 [6]:

5> set the RO_TYPE to 2nd-RO;

5> select the set of Random Access resources for this Random Access procedure.

6>	initialize the parameters for the Random Access procedure as specified in clause 5.1.1 according to the values configured by RRC for the selected set of Random Access resources.

3>	if the Random Access Preamble is transmitted with repetitions and neither contention-free Random Access Resources nor Random Access resources for SI request have been provided for this Random Access procedure:

4>	if PREAMBLE_TRANSMISSION_COUNTER = [preambleTransMax-Msg1-Repetition] + 1; or

4>	if PREAMBLE_TRANSMISSION_COUNTER = 2 × [preambleTransMax-Msg1-Repetition] + 1:

5>	if set of Random Access resources configured with the same prach-ConfigurationIndex and associated with a higher Msg1 repetition number with the same feature or feature combination as the current set of Random Access resources is available:

6>	select the set of Random Access resources associated with the next higher Msg1 repetition number with the same feature or feature combination for this Random Access procedure;

6>	initialize startPreambleForThisPartition, numberOfPreamblesPerSSB-ForThisPartition, numberOfRA-PreamblesGroupA and msg1-RepetitionTimeOffsetROGroup parameters for the Random Access procedure according to the values configured by RRC for the selected set of Random Access resources.

<irrelevant text omitted>