3GPP TSG-RAN WG2 Meeting #131	R2-2506000

Bengaluru, India, August 25th – 29rd, 2025

Source:        ZTE Corporation

Title:           Consideration on Supporting 7M Channel Bandwidth

Agenda item:   8.20.1

Document for:  Discussion and Decision

- Introduction

In the last meeting, a Reply LS was received on the 7M channel bandwidth [1], in this paper, we share our views on this issue.

ACTION: RAN4 respectfully asks that RAN 2 take the following into consideration: It is RAN4’s view that 7 MHz will never be the minimum channel bandwidth for a given NR band, but it could be the maximum channel bandwidth for a given NR band, and for carrier aggregation 7 MHz could be the maximum or minimum channel bandwidth that a UE supports for a band in a band combination. In summary, signaling to indicate both minimum and maximum channel bandwidth for a band in a band combination is needed for 7 MHz to reflect these possibilities.

- Discussion

In this chapter, we first discuss the release version for this feature, then discuss whether it has impact on the channelBWs-DL/UL.

2.1 Release Version for the 7M Feature

In the LS, RAN4 has preference from R15 and it’s also acceptable to have it in R18/19.

Also, in response to some offline requests for feedback on which release this signalling should apply to, RAN4 would like to clarify that we would like 7 MHz to be release independent from Rel-15 (up to RAN2 as to which version of the spec it appears with early implementation allowed), but it is acceptable if the signalling for 7 MHz as a minimum or maximum channel bandwidth for a band in a band combination is introduced in Rel-18 or Rel-19.

In the LS, it also clarifies that “7 MHz will never be the minimum channel bandwidth for a given NR band, but it could be the maximum channel bandwidth for a given NR band, and for carrier aggregation 7 MHz could be the maximum or minimum channel bandwidth that a UE supports for a band in a band combination”, thus the 7M should be added for the featureSetsDownlinkPerCC-vxy/ featureSetsUplinkPerCC-vxy. Now the R19 running CR is undergoing, and new featureSetsDownlinkPerCC-v1900/ featureSetsUplinkPerCC-v1900 has been added in the running CR.

#Downlink FeatureSetDownlinkPerCC-v1840 ::= SEQUENCE { /*********omit the detail******************/ } FeatureSetDownlinkPerCC-v1900 ::= SEQUENCE { -- R1 67-1: 32 DL HARQ processes for TN in FR1 and FR2-1 support32-DL-HARQ-ProcessTN-r19 ENUMERATED {supported} OPTIONAL } #Uplink FeatureSetUplinkPerCC-v1850 ::= SEQUENCE { /*********omit the detail******************/ } FeatureSetUplinkPerCC-v1900 ::= SEQUENCE { -- R1 67-1a: 32 UL HARQ processes for TN in FR1 and FR2-1 support32-UL-HARQ-ProcessTN-r19 ENUMERATED {supported} OPTIONAL }

If we intends to implement the signaling from the earlier version than the R18/R19, it means that the signaling need to be added similar to the featureSets-v17d0, which is added under the UE-NR-Capability-vxy immediately to avoid NBC issue.

/*****************omit the unrelated part**********************************/ UE-NR-Capability-v17d0 ::= SEQUENCE { featureSets-v17d0 FeatureSets-v17d0 OPTIONAL, nonCriticalExtension SEQUENCE {} OPTIONAL } FeatureSets ::= SEQUENCE { /*****************omit the unrelated part**********************************/ [[ featureSetsDownlink-v1860 SEQUENCE (SIZE (1..maxDownlinkFeatureSets)) OF FeatureSetDownlink-v1860 OPTIONAL ]], [[ featureSetsDownlink-v1900 SEQUENCE (SIZE (1..maxDownlinkFeatureSets)) OF FeatureSetDownlink-v1900 OPTIONAL, featureSetsDownlinkPerCC-v1900 SEQUENCE (SIZE (1..maxPerCC-FeatureSets)) OF FeatureSetDownlinkPerCC-v1900 OPTIONAL, featureSetsUplink-v1900 SEQUENCE (SIZE (1..maxUplinkFeatureSets)) OF FeatureSetUplink-v1900 OPTIONAL, featureSetsUplinkPerCC-v1900 SEQUENCE (SIZE (1..maxPerCC-FeatureSets)) OF FeatureSetUplinkPerCC-v1900 OPTIONAL ]] } /*****************omit the unrelated part**********************************/ FeatureSets-v17d0 ::= SEQUENCE { featureSetsDownlink-v17d0 SEQUENCE (SIZE (1..maxDownlinkFeatureSets)) OF FeatureSetDownlink-v17d0 OPTIONAL }

Considering the above signaling complexity issue, it’s better to implement it from the R18/19. Then if companies prefer to have early implementation, similar to the introduction of supportedMinBandwidthDL-r17 in [2], it can be early implemented from the R15.

Proposal 1: From the ASN.1 signaling complexity perspective, the ASN.1 signaling should be designed from R18/R19 and allow early implementation from the R15.

2.2 Impact on the channelBWs-DL/UL

Another issue is whether there is a need to include 7M for the channelBWs-DL/UL, for example, the supported maximum channel bandwidth is larger than 7M and the mimumbandwidth is lower than 7M, then according to the current bandwidth determine rule, it needs to be further indicated whether 7M is supported in the channelBWs-DL/UL, e.g. with the existing reserved bits.

Proposal 2: RAN2 to confirm that the 7M should also be included in the channelBWs-DL/UL (e.g. with the existing reserved bits).

- Conclusion and proposals

With the above analysis, we have the following proposals:

Proposal 1: From the ASN.1 signaling complexity perspective, the ASN.1 signaling should be designed from R18/R19 and allow early implementation from the R15.

Proposal 2: RAN2 to confirm that the 7M should also be included in the channelBWs-DL/UL (e.g. with the existing reserved bits).

- Reference

- R4-2508088  Reply LS to RAN2 on Signalling for 7 MHz Channel Bandwidth    Tmobile

- R2-2203898  Introduction of BCS4 and BCS5	Xiaomi