3GPP TSG-RAN WG2 Meeting #131	 R2-2506007

Bangalore, India, August 25-29, 2025

Agenda Item: 8.3.3

Source:      NEC

Title:         Discussion on AIML mobility for Network Sided Model Performance

Document for: Discussion and Decision

# 1 Introduction

In RAN2#130 meeting, the following agreements were made with respect to performance monitoring of network side model for AI/ML mobility use case.

Agreements

- RAN2 confirms that UE will not be informed about any network-sided functionality management decision (e.g., selection, (de)activation, switching, fallback, etc.) for AI mobility use case

- For performance monitoring of NW-side model, UE can provide the label data (i.e., actual measurement results) for gNB. The existing measurement/report configuration can be reused to configure UE to report the actual measurement

- FFS on UE awareness and preference for NW sided model

RAN2 chair indicates that network sided model sub agenda should discuss including (sub)use cases to be supported, assistance information from the UE (e.g., measurements for inference or performance monitoring), and data collection. Thus, in this contribution, we highlight some understanding of the UE awareness for network side model performance monitoring (FFS from RAN2#130). We also highlight some additional UAI that can improvise the performance of AI/ML mobility use case.

# 2 Discussion

## 2.1 UE awareness for Network sided model

In AI/ML-based mobility procedures, especially those involving network-side model performance monitoring (e.g., RRM measurement prediction or event prediction), feedback from the UE plays a critical role in enabling effective evaluation of model accuracy and behaviour. The gNB may rely on data or ground truth measurement reports from the UE to assess how well the model performs under real-world conditions. However, such performance monitoring support can impose additional overhead on the UE in terms of battery usage, computational effort, or conflicting task priorities. Therefore, it is essential that the UE is made aware when it is being asked to assist in performance monitoring. This can be achieved by configuring the UE with an explicit purpose indicator in the RRC signalling to mark the intention for performance monitoring. Furthermore, the UE may be allowed to express its preference or capability to participate, based on real-time constraints such as low battery or high-priority tasks. Such preference or limitation can be communicated through the UAI, enabling the gNB to take an informed decision on whether to proceed with the configuration or avoid unnecessary burdening of the UE.

Proposal-1: RAN2 may define that in the context of performance monitoring of network-side AI/ML models, the gNB should indicate the monitoring purpose in the configuration, and the UE may signal its awareness and preference (including non-suitability) using UAI.

## 2.2 Reporting of Inference Inputs

In AI/ML-based mobility procedures, UAI can enable the gNB to make informed and dynamic adjustments to data collection and reporting configurations based on real-time UE conditions. The need for UAI arises due to varying UE constraints such as low battery, limited memory, buffer threshold exceedance, increased computational load, and handover or RLF scenarios, which may hinder continuous data logging or inference. UAI allows the UE to signal data availability, the reason for stopping or modifying logging, and mismatches in configuration (e.g., frequency or model version). This ensures efficient resource usage and prevents unnecessary burden on the UE during critical conditions.

Proposal-2: RAN2 may specify some additional information through the UAI to indicate constraints affecting logging/inference, enabling the gNB to adapt data collection configurations accordingly.

# 3 Conclusions

We have the following proposals for the network sided model for performance monitoring.

Proposal-1: RAN2 may define that in the context of performance monitoring of network-side AI/ML models, the gNB should indicate the monitoring purpose in the configuration, and the UE may signal its awareness and preference (including non-suitability) using UAI.

Proposal-2: RAN2 may specify some additional information through the UAI to indicate constraints affecting logging/inference, enabling the gNB to adapt data collection configurations accordingly.