3GPP TSG-RAN WG2 Meeting #131	R2-2506033

Bangalore, India, Aug 25th – 29th, 2025	Revision of R2-2504497

Agenda Item:	8.9.2

Source:			ASUSTeK

Title:	Discussion on usage of time information for S&F

Document for: 	Discussion and Decision

- Introduction

For time information for Store & Forward operation, following agreements are made in previous meetings.

In RAN2 #129bis meeting:

Agreements:

1.	For UEs supporting S&F, the UE AS indicates the information on transition time (if any) from current "S&F operation mode" to "normal mode" (if received) to the upper layers. Whether/how it is used by the upper layers is up to CT1.

2.	We introduce an indication in system information for the normal mode to S&F mode transition, at least for NAS use. FFS on the details (e.g. whether we can link this to other existing information). The information on transition time for the normal mode to S&F mode transition is sent from AS to NAS

3.	We don’t introduce any new release cause related to S&F operation

In RAN2 #130 meeting:

Agreements:

1.	When the S&F operation indication is present, the agreed time information in SIB31 (i.e., t-ModeSwitching) indicates when the normal mode will start; otherwise, the agreed time information in SIB31 (i.e., t-ModeSwitching) indicates when the S&F mode will start.

2.	In a S&F network deployment which also exhibits discontinuous coverage, existing mechanisms to handle discontinuous coverage can be leveraged (e.g. satellite assistance information, UE not needing to perform idle mode tasks when the UE determines that is out of coverage, etc.). There is no need to modify existing discontinuous coverage features due to the addition of S&F Satellite operation. FFS if we clarify in discontinuous coverage procedure in idle mode that the UE also takes into account the information about NAS configured S&F monitoring list.

3.	Clarify in Stage 2 from Rel-18 (so in general, not only for S&F) that the values used in “satelliteId-r17” are expected to correspond to the values used in “satelliteId-r18” in other SIBs (e.g. the value used to identify the serving satellite in SIB33, SIB32 and SIB31 should be the same). Come back in the next meeting with an actual Rel-18 CR for this. FFS if in Rel-19 we need to say anything about the correspondence between the “satelliteId” IE and “S&F Monitoring List”.

Working Assumption:

1.	In the neighbour cell list we introduce an indication whether the cell operates in S&F mode or not (FFS if we also include the transition time). This WA can only be confirmed if we converge on the corresponding UE behaviour.

In this contribution, we would like to further discuss the usage of the time information for S&F mode.

- Discussion

The Store and Forward (S&F) operation in IoT NTN is intended to provide some level of communication service for UEs under satellite coverage with intermittent/temporary satellite connectivity, e.g., for delay-tolerant communication service. Based on the current agreements and running RRC CR [1], the UE could receive sf-OperationMode via SIB1 and t-ModeSwitching via SIB31. If sf-OperationMode is present in SIB1, t-ModeSwitching in SIB31 indicates the time information on when an NTN cell is going to switch from the S&F Satellite operation mode to the normal mode. Otherwise, t-ModeSwitching in SIB31 indicates the time information on when an NTN cell is going to switch from the normal mode to the S&F Satellite operation mode. And the UE supporting S&F would forward the t-ModeSwitching to upper layers.

The UE could camp on or connect to a cell supporting S&F while the NTN cell is operating in the normal mode. The UE would receive sf-OperationMode in SIB1 and receive t-ModeSwitching in SIB31 indicating when the cell is going to switch to the S&F Satellite operation mode. The UE may perform data transmission in the cell when it is in normal mode. However, if the UE is not supporting S&F, the UE could not continue its AS operation after the cell switches to S&F Satellite operation mode. For example, the UE may not be able to continue the data transmission when the cell is operating in S&F Satellite operation mode.

Observation 1: The UE not supporting S&F could not continue its AS operation after the NTN cell switches to S&F Satellite operation mode.

Therefore, the UE not supporting S&F should handle the case when t-ModeSwitching indicating transition time on normal mode to S&F Satellite operation mode. For example, the UE performs measurements of neighbouring cells (which are or would be in normal mode after t-ModeSwitching) before t-ModeSwitching, and the UE considers radio link failure upon t-ModeSwitching. The UE could know the operation mode for neighbouring cells if the working assumption made in the last meeting could be agreed. Then the UE could select a new cell operating in normal mode without other NW signalling. It would be beneficial for radio link and UL transmissions handling.

Proposal 1: When t-ModeSwitching indicates transition time on normal mode to S&F mode, for a UE not supporting S&F, the UE performs measurements of neighbouring cells before t-ModeSwitching and considers radio link failure upon t-ModeSwitching.

- Conclusion

We have the following observation and proposal for the S&F transition time information:

Observation 1: The UE not supporting S&F could not continue its AS operation after the NTN cell switches to S&F Satellite operation mode.

Proposal 1: When t-ModeSwitching indicates transition time on normal mode to S&F mode, for a UE not supporting S&F, the UE performs measurements of neighbouring cells before t-ModeSwitching and considers radio link failure upon t-ModeSwitching.

- Reference

- R2-2505246, “RRC Running CR for IoT NTN.”