3GPP TSG-RAN WG2 Meeting #131	R2-2506043

Bangalore, India Aug 25th – 29th, 2025

Source:	Qualcomm Incorporated

Title:	Open issues on relay discovery and (re)selection

Agenda Item:	8.13.2

Document for:	Discussion and Decision

# Introduction & Background

In the WID [1], multi-hop U2N relay discovery and Relay (re)selection needs to discuss. This contribution discusses the related aspects.

- Specify mechanisms to support up to two additional hops relays on top of Rel-17 U2N relay. The work starts with one additional hop relay (i.e., remote UE -> first relay UE -> last relay UE -> gNB) until RAN#107 and further check will be made in RAN#107 if it can be easily extended to two additional hops relays (i.e., remote UE -> first relay UE -> second relay UE -> last relay UE -> gNB). A necessary criterion for the specified mechanisms is easy extensibility to support two additional hop relays for the work done until RAN#107 and to be forward compatible for future extensions for additional relays.

  - Relay discovery and (re)selection [RAN2]

  - Signalling support for relay UEs and remote UE authorization if SA2 concludes it is needed [RAN3]

  - Impact on SRAP and QoS handling for multi-hop [RAN2]

  - Control plane procedures [RAN2, RAN3]

# Discussion

During RRC running CR email discussion, there are some open issues left for further discussion.

Editor’s Note: The cause value/ indication Type is FFS. Intermediate Relay UE can set the indication type e.g as relayUE-RelayReselection to reflect its own action.

Open issue RRC-5: NotificationMessageSidelink message handling

Open Issue RRC-12: FFS Cause Value / Indication Type for Notifications from Intermediate Relay UE

These issues are related, so this contribution discusses them together.

When a last Relay UE performs HO and recieves reconfigurationWithSync, the last relay UE will sends NotificationMessageSidelink message to the child UE(s). In Rel-17 single-hop U2N relay, the U2N Relay UE set the cause value to relayUE-HO, then Remote UE can perform relay reselection or cell selection based on the cause value. In multi-hop U2N relay, the existing cause value can be reused to indicate the intermediate Relay UE. When the intermediate Relay UE receives the NotificationMessageSidelink message with cause value relayUE-HO, the intermediate Relay UE performs relay reselection.

- When last Relay UE recieves reconfigurationWithSync, the last relay UE will sends NotificationMessageSidelink message to the child UE(s) with the cause value relayUE-HO.

- When an intermediate Relay UE receives NotificationMessageSidelink with relayUE-HO, the intermediate Relay UE can perform relay reselection.

RAN2 agreed that the notification message is used for the L2 child UE in case of upstream relay (re)selection triggered by AS layer failure cases (Uu/PC5 link failure/release, using legacy cause value) where the upper layer does not trigger link release towards the child.

The notification message is used for the L2 child UE in case of upstream relay (re)selection triggered by AS layer failure cases (Uu/PC5 link failure/release, using legacy cause value) where the upper layer does not trigger link release towards the child.  For reselection triggered by upper layers we rely on the upper-layer notification/release mechanism already defined.

When an intermediate relay UE transmits the NotificationMessageSidelink message due to relay reselection triggered by AS layer or receiving NotificationMessageSidelink from the parent UE, set the indicationType as RelayReselctionduetoAS.

- When an intermediate relay UE transmits the NotificationMessageSidelink message due to relay reselection triggered by AS layer or receiving from the parent UE, set the indicationType in NotificationMessageSidelink to RelayReselctionduetoAS.

From the last relay UE perspective, RLF or cell reselection is AS event and transparent to upper layer, then the last relay UE can notify the child UE.

When the intermediate relay UE receives notification from the parent relay UE, the intermediate relay UE could perform relay reselection acting as a Remote UE.

- When the last relay UE performs, RLF, cell reselection, the last relay UE should notify the child UE using or receiving NotificationMessageSidelink from the parent UE with existing cause value.

- When the intermediate relay UE receives notification from the parent relay UE, the intermediate relay UE could perform relay reselection acting as a Remote UE.

- Conclusion

- When last Relay UE recieves reconfigurationWithSync, the last relay UE will sends NotificationMessageSidelink message to the child UE(s) with the cause value relayUE-HO.

- When an intermediate Relay UE receives NotificationMessageSidelink with relayUE-HO, the intermediate Relay UE can perform relay reselection.

- When an intermediate relay UE transmits the NotificationMessageSidelink message due to Relay reselection triggered by AS layer, set the indicationType in NotificationMessageSidelink to RelayReselctionduetoAS.

- When the last relay UE performs HO, RLF, cell reselection, the last relay UE should notify the child UE.

- When the intermediate relay UE receives notification from the parent relay UE, the intermediate relay UE could perform relay reselection acting as a Remote UE.

# Reference

[1]. RP-241609    New WID on NR sidelink multi-hop relay    LG Electronics, InterDigital, FirstNet