3GPP TSG-RAN WG2 Meeting #131		R2-2506044

Bangalore, India  Aug 25th – 29th , 2025

Source:	Qualcomm Incorporated

Title:	Ensure L2 Multi-hop U2N relay operation conditions

Agenda Item:	8.13.3

Document for:	Discussion and Decision

# Introduction & Background

In the WID [1], multi-hop U2N relay was approved as Rel-19 item, and this contributions discuss the contron plane issues.

- Specify mechanisms to support up to two additional hops relays on top of Rel-17 U2N relay. The work starts with one additional hop relay (i.e., remote UE -> first relay UE -> last relay UE -> gNB) until RAN#107 and further check will be made in RAN#107 if it can be easily extended to two additional hops relays (i.e., remote UE -> first relay UE -> second relay UE -> last relay UE -> gNB). A necessary criterion for the specified mechanisms is easy extensibility to support two additional hop relays for the work done until RAN#107 and to be forward compatible for future extensions for additional relays.

  - Relay discovery and (re)selection [RAN2]

  - Signalling support for relay UEs and remote UE authorization if SA2 concludes it is needed [RAN3]

  - Impact on SRAP and QoS handling for multi-hop [RAN2]

  - Control plane procedures [RAN2, RAN3]

# Discussion

RAN2 discussed L2 multi-hop U2N relay under some conditions, i.e. in a limited scenarios. These conditions are:

- All the UEs access to the last relay UE’s serving cell.

- The intermediate Relay UE cannot support different indirect paths to the gNB with same/different last/U2N/parent intermediate Relay UE(s) based on different PC5 unicast links.

These conditions can be shown as follows as examples.

Currently, there is no way to ensure L2 multi-hop U2N relay only happens under these conditions.

Taking the following as example, if the intermediate Relay UE already established red relay path for Remote UE(s), then the green relay path should not be established. How to avoid green relay path is the issue to be addressed.

Figure 1

Taking the above Figure 1 as example, if the intermediate Relay UE already established red relay path for Remote UE(s), then the green relay path should not be established. How to avoid green relay path is the issue to be addressed.

Figure 2

Another example is Figure 2, if the a UE has Uu connection with the different cell or same cell with the last Relay UE cell, the intermediate Relay UE established RRC connection with on green path for its own service towards same or different cell of the last relay UE, the intermediate Relay UE cannot serve the Remote UE on the red path.

Observation : Currently, there is no ways to ensure L2 multi-hop Relay operates under the following agreed conditions:

- Condition 1: The intermediate Relay UE cannot support different indirect paths to the gNB with same/different last/U2N/parent intermediate Relay UE(s) based on different PC5 unicast links.

- Condition  2: All the UEs access to the last relay UE’s serving cell.

For condition 1, If the intermediate Relay UE has established PC5 connection for one RSC associated with parent UE and transmitted in discovery message, the intermediate Relay UE does not transmit another RSC in discovery message. With this option, when the Remote UEs select relay path, the Remote UEs only considers the RSC for which the PC5 link is already established on red relay path.

- If the intermediate Relay UE has established PC5 connection for one RSC associated with one last Relay UE, the intermediate Relay UE does not transmit other RSC(s) in any discovery message.

For condition 2, when the intermediate Relay UE receives relay establishment request from Remote UE, there are two options:

Option 1: The intermediate Relay UE performs path switch from direct path to indirect path when receiving relay request from the Remote UE/child UE. This option has no limitation on relay service application. But there will be spec impact that a new path switching trigger needs to be introduced.

Option 2: The Relay UE does not forward discovery message received from a relay UE, that means the UE cannot be an intermediate Relay UE. This option will have limitation on the relay UE application.

It is proposed to discuss these two options.

- RAN2 discusses the two options for the case whether a UE has established connection via direct path can be act as an intermediate Relay UE.

Option 1: The intermediate Relay UE performs path switch from direct path to indirect path when receiving relay request from the Remote UE/child UE.

Option 2: The Relay UE does not forward discovery message received from a relay UE, that means the UE cannot be an intermediate Relay UE.

- Sends LS to SA2 to on the above two proposals.

# Conclusion

Observation: Currently, there is no way to ensure L2 multi-hop Relay operates under the following agreed conditions:

- Condition 1: The intermediate Relay UE cannot support different indirect paths to the gNB with same/different last/U2N/parent intermediate Relay UE(s) based on different PC5 unicast links.

- Condition  2: All the UEs access to the last relay UE’s serving cell.

- If the intermediate Relay UE has established PC5 connection for one RSC associated with one last Relay UE, the intermediate Relay UE does not transmit other RSC(s) in any discovery message.

- RAN2 discusses the two options for the case whether a UE has established connection via direct path can be act as an intermediate Relay UE.

Option 1: The intermediate Relay UE performs path switch from direct path to indirect path when receiving relay request from the Remote UE/child UE.

Option 2: The Relay UE does not forward discovery message received from a relay UE, that means the UE cannot be an intermediate Relay UE.

- Sends LS to SA2 to on the above two proposals.

# Reference

[1]. RP-241609    New WID on NR sidelink multi-hop relay    LG Electronics, InterDigital, FirstNet