3GPP TSG-RAN WG2 Meeting #131	R2-2506045

Bangalore, India Aug 25th – 29th, 2025

Source:	Qualcomm Incorporated

Title:	Service continuity discussion

Agenda Item:	8.13.4

Document for:	Discussion and Decision

# Introduction & Background

This contribution discusses service continuity in multi-hop U2N.

# Discussion

## Open Issue RRC-14: Clarification on Reporting of Target Relay UEs in RRC_CONNECTED State for Scenarios C and D

According to current RAN2 discussion, the Remote UE reports L2 ID of the first Relay UE, the network can identify whether the first Relay UE is in CONNECTED state based on received L2 ID of the first relay UE. When the first Relay UE is in CONNECTED state, all the other Relay UEs will be in CONNECTED state.

- When the Remote UE report L2 ID of the first Relay UE, the network can identify whether the first Relay UE is in CONNECTED state. When the first Relay UE is in CONNECTED state, all the other Relay UEs will be in CONNECTED state.

## Intermediate Relay UE performs path switching

In RAN2 #129 meeting, RAN2 agreed that for multi-hop i2i, first evaluates adjacent upstream relay link and candidate relay link, where the sidelink relay measurement report shall include at least serving intermediate/ last relay UE's source L2 ID, serving cell ID and sidelink measurement quantity result.  FFS if intermediate relay UE reports.

Agreements:

For multi-hop i2d, Remote UE evaluates both relay link (for the link between remote UE and serving first relay UE) and Uu link, where the sidelink relay measurement report shall include at least serving first relay UE's source L2 ID, serving cell ID and sidelink measurement quantity result.

For multi-hop i2d, first relay UE evaluates adjacent upstream relay link and Uu link, where the sidelink relay measurement report shall include at least measured relay UE's source L2 ID, serving cell ID and sidelink measurement quantity result.  FFS if intermediate relay UE reports.

For multi-hop i2i, the remote UE reports serving first relay UE and candidate single-hop relay UE(s), including at least a source L2 ID, serving cell ID, and a sidelink measurement quantity information.

For multi-hop i2i, first evaluates adjacent upstream relay link and candidate relay link, where the sidelink relay measurement report shall include at least serving intermediate/ last relay UE's source L2 ID, serving cell ID and sidelink measurement quantity result.  FFS if intermediate relay UE reports.

If intermediate Relay UE sends measurement report to the gNB, then intermediate Relay UE could perform path switching. There could be cases that the intermediate relay UE performs path switching, and the following figures show the scenarios where service continuity may or may not be supported from Remote UE perspective.

Case 1: Intermediate relay UE performs path switching to direct path from multi-hop U2N path (example that path switching from multi-hop indirect path to single-hop indirect/direct path from Remote UE perspective)

Case 2: Intermediate relay UE performs path switching to multiple-hop U2N path from direct path

(example that hop number of the target path is larger than the source path from Remote UE perspective)

Case 3: Intermediate relay UE performs path switching to single-hop U2N path from multi-hop U2N path

(example that the Remote UE service continuity scenario is out of WID scope)

Similar with existing single-hop U2N relay, when the relay UE performs HO, the relay UE notifies the remote UE, and Remote UE goes back to IDLE. Here, in multi-hop relay, the similar principle can be reused, i.e. the intermediate Relay UE can send measurement report to the gNB for path switching, and notifies the Remote UE. When the Remote UE receives path switching notification from the relay UE, the Remote UE should go back to IDLE.

- The intermediate Relay UE can report measurement report UE to gNB for path switching to direct path or indirect path acting as a Remote.

- When the intermediate Relay UE performs path switching to direct path or indirect path, the intermediate Relay UE informs the Remote UE via the child intermediate relay UE with release cause 'RRC connection failure'.

- When the Remote UE receives notification from the relay UE, the Remote UE should go back to IDLE.

- Conclusion

- When the Remote UE report L2 ID of the first Relay UE, the network can identify whether the first Relay UE is in CONNECTED state. When the first Relay UE is in CONNECTED state, all the other Relay UEs will be in CONNECTED state.

Intermediate Relay UE performs path switching

- The intermediate Relay UE can report measurement report UE to gNB for path switching to direct path or indirect path acting as a Remote.

- When the intermediate Relay UE performs path switching to direct path or indirect path, the intermediate Relay UE informs the Remote UE via the child intermediate relay UE with release cause 'RRC connection failure'.

- When the Remote UE receives notification from the relay UE, the Remote UE should go back to IDLE.

# Reference

[1]. RP-241609    New WID on NR sidelink multi-hop relay    LG Electronics, InterDigital, FirstNet