3GPP TSG-RAN WG2 Meeting #131	R2-2506064

Bangalore, India, August 25th – 29th, 2025

Agenda Item:   8.5.4

Source:	HONOR

Title:	Discussion on adaptation of common signal/channel transmissions

Document for:	Discussion and decision

1. Introduction

In this contribution, we will discuss the remaining issues and potential solutions on paging adaptation and RACH adaptation in the current running CR of TS 38.300.

- Discussion

2.1 Paging adaptation

According to the last paragraph of section 9.2.5 in the running CR for TS 38.300 [1], paging adaptation is described as:

Paging adaptation for network energy saving for UEs in CM_IDLE and RRC_INACTIVE: in order to increase gNB sleeping time, the value of N and Ns are extended to increase the number of POs per PF with sparser PFs.

However, in the first paragraph of section 9.2.5 in the running CR for TS 38.300 [1], paging adaptation is described as:

Paging allows the network to reach UEs in RRC_IDLE and in RRC_INACTIVE state through Paging messages, and to notify UEs in RRC_IDLE, RRC_INACTIVE and RRC_CONNECTED state of system information change (see clause 7.3.3) and ETWS/CMAS indications (see clause 16.4) through Short Messages.

The UE will receive a paging in RRC_IDLE or RRC_INACTIVE state.

In the section 7.1 of TS 38.304 [2], the description of the discontinuous reception for paging is:

The UE may use Discontinuous Reception (DRX) in RRC_IDLE and RRC_INACTIVE state in order to reduce power consumption.

The UE in CM_CONNECTED mode will be released the RRC connection after a configured duration of inactivity, and turn to RRC_IDLE state. Then, the gNB will release N2 and N3 interface connections, thereby transitioning the UE to CM_IDLE mode. The CM_IDLE mode emphasizes the connection between the UE and the CN. In this agenda, paging adaptation is introduced to achieve network energy saving by extending the value of N and Ns, which is closely related to the enhancements on the RAN side.

Hence, we support to modify the description of UE state from "UE in CM_IDLE" to "UE in RRC_IDLE" in the last paragraph of section 9.2.5 in the running CR for TS 38.300.

Proposal 1: Support to modify the description of UE state from "UE in CM_IDLE" to "UE in RRC_IDLE" in clause 9.2.5 of 38.300 running CR in the last paragraph of section 9.2.5 in the running CR for TS 38.300.

2.2	RACH adaptation

According to the agreement of the meetings:

RAN1#120 Agreement For adaption of PRACH in time-domain, for a connected mode UE, support a 1-bit field in DCI 1_0 with C-RNTI used to trigger PRACH (i.e. PDCCH order) to indicate whether the additional PRACH resource(s) is available for the triggered PRACH. • FFS: UE behaviour (e.g. applicable resources for PRACH mask index) when it is indicated of additional PRACH resource(s) • FFS: Details on how to reuse existing bit for the 1-bit indication Agreement For DCI-based adaptation for additional PRACH resources, DCI 1_0 with P-RNTI indicates the availability information for additional PRACH resource from a reference point and for a validity time duration • FFS: Validity time duration for availability is configured by higher layer signaling or predefined • FFS: Location of the reference point defined in the specification • FFS: Value/granularity of the validity time duration. • FFS: Whether DCI can be used to explicitly deactivate the additional PRACH resources RAN1#120bis Agreement For adaptation of PRACH in time-domain, at least for 4-step RACH, at least for DCI 1_0 with P-RNTI, • Support configuration of the additional PRACH resources within [the same] RACH-ConfigCommon in SIB1 used to configure the legacy PRACH resources o the legacy PRACH resources used for ‘additional RO validation before the SSB-RO mapping’ are configured in the RACH-ConfigCommon Agreement For DCI-based adaptation for additional PRACH resources, the following 1-bit field is used for adaptation indication in DCI format 1_0 with P-RNTI - Use one bit from the Bits 5-8 within the Short Message field (from upper layers) - Send LS to RAN2 to confirm the use of this bit. Above applies for cell that transmits the DCI for connected UEs and IDLE/INACTIVE mode UEs Agreement For DCI-based adaptation for additional PRACH resources, for the availability information of additional PRACH resources indicated by DCI 1_0 with P-RNTI - the validity duration is configured via higher layer signalling. RAN1#121 Agreement ‘PRACH resource indicator’ field is present in DCI 1_0 with C-RNTI for PDCCH order when the configuration of the additional RACH resources is provided in SIB1(i.e. addl-RACH-Config-Adaptation). • Above applies for PCell. RAN2#130 Agreement 14. RAN2 confirms the newly introduced 1-bit indication in DCI 1_0 for C-RNTI, i.e., PDCCH order, applies to both CBRA and CFRA.

According to the agreements of the meetings, there is only one method to indicate the availability of additional PRACH resources: via DCI.

In the second paragraph of section 15.4.2.x3 in the running CR for TS 38.300 [1], the description of the SSB adaptation is:

SSB adaptation is indicated via DCI.

To ensure alignment with SSB adaptation description, we recommend adding the method description to include clear specifications for indicating the availability of additional PRACH resources. That is, we support to modify the description of PRACH adaptation in the last paragraph from " Additional RACH resources are configured together with the common RACH resources in the same set of RACH resources, and the network can indicate whether the additional RACH resources are available as specified in section 8.1 of TS 38.213. " to " Additional RACH resources are configured together with the common RACH resources in the same set of RACH resources, and the network can indicate via DCI whether the additional RACH resources are available as specified in section 8.1 of TS 38.213." in the section 15.4.2.x3 of the running CR for TS 38.300.

Proposal 2: Support to add the method description and modify it to "Additional RACH resources are configured together with the common RACH resources in the same set of RACH resources, and the network can indicate via DCI whether the additional RACH resources are available as specified in section 8.1 of TS 38.213.".

- Conclusions

In this contribution, we made the following proposals:

Proposals

- Paging adaptation in time-domain

Proposal 1: Support to modify the description of UE state from "UE in CM_IDLE" to "UE in RRC_IDLE" in clause 9.2.5 of 38.300 running CR in the last paragraph of section 9.2.5 in the running CR for TS 38.300.

- RACH adaptation in time-domain

Proposal 2: Support to add the method description and modify it to "Additional RACH resources are configured together with the common RACH resources in the same set of RACH resources, and the network can indicate via DCI whether the additional RACH resources are available as specified in section 8.1 of TS 38.213.".

- References

- Email discussion - R2-25xxxxx 38300-i50 NES enh CR v05 rm.docx.

- TS 38.304, User Equipment (UE) procedures in Idle mode and RRC Inactive state.