3GPP RAN WG2 Meeting #131 	R2-2506066

Bangalore, India, August 25th – 29th, 2025

Agenda Item:	8.6.4

Source:	HONOR

Title: 	Discussion conditional LTM

Document for:	Discussion and decision

1	Introduction

During the after-meeting email discussion for the MAC open issues, issues related to CLTM are identified as to be discussed by company contributions:

| Index | Issue description | Rapporteur suggestion |
| --- | --- | --- |
| MAC-14 | FFS for the case when CLTM candidate TAT timer is running, an (Enhanced) LTM Cell switch Command MAC CE is received Editor’s NOTE: FFS for the case when CLTM candidate TAT timer is running, an (Enhanced) LTM Cell switch Command MAC CE is received. [Rapp]: New added based on Anil’s comments. | Issue Type: Not essential but important How to address it: based on companies’ contribution |

In this contribution, we discuss the general concept of the conditional LTM like execution trigger events and the early UL synchronization. Then we propose some solutions/way-forwards accordingly.

- Discussion

2.1 MAC-14: CLTM TA upon LTM Cell switch Command MAC CE

With the early UL synchronization designed in Rel-18, UE could perform the RACH-less LTM execution on the triggered candidate. To avoid signalling and power overhead resulted from frequent TA pre-acquisition, RAN2 agreed that TA could be maintained in the UE after the CLTM execution, and the corresponding early TA timer would be kept running to monitor the validity. With this solution, the UE could perform the RACH-less CLTM upon execution condition met. Since the coexistence of CLTM and NW-triggered LTM, even the NW didn’t carry the TA within the LTM Cell switch Command MAC CE (CSC), the UE could perform RACH-less execution by utilizing the corresponding TA if the TA of this candidate is still valid.

Proposal 1: (MAC-14) UE could perform RACH-less execution upon the reception of the (Enhanced) LTM Cell switch Command MAC CE when CLTM candidate TAT timer is running,

2.2 Other Issues

During previous discussion, RAN2 already confirmed that the event triggered L1 measurement is designed for LTM early synchronization. This enables that the early synchronization could be obtained earlier than the cell switch by performing synchronization upon the satisfaction of an easily satisfied condition and executing the cell switch upon a stricter condition. The TA of a candidate cell is sent from the serving cell before the execution of the CLTM could enable the RACH-less LTM. However, the L1 measurement is performed by UE and sent to the NW by MR to provide the information to trigger the TA acquisition for the candidate cell with one/several good enough beams which may cause long latency. To increase the possibility of the RACH less, it is beneficial if the UE detects the TA is not valid and directly report the explicit information to the NW.

Proposal 2: UE could request the TA acquisition to the NW when it detects the TA of one candidate is invalid/needed.

3	Conclusions

In this contribution, we discussed the conditional LTM for Rel-19 mobility WI. Based on the discussion, the following observation and proposals are concluded:

Proposal 1: (MAC-14) UE could perform RACH-less execution upon the reception of the (Enhanced) LTM Cell switch Command MAC CE when CLTM candidate TAT timer is running,

Proposal 2: UE could request the TA acquisition to the NW when it detects the TA of one candidate is invalid/needed.

4	References