3GPP TSG-RAN WG2#131	     					       R2-2506072

Bengaluru, India, August 25 – 29, 2025

Agenda item:	8.1.3

Source: 	Kyocera

Title: 	Remaining Issues on NW side data collection

Document for:	Discussion

Introduction

In this contribution, the remaining issues regarding NW side data collection, RRC-21, RRC-25, RRC-26 and RRC-27 are discussed.

Discussion

Open issue RRC-21: Timestamp in logged data

At RAN2#130, the following agreements were made for RRC-21 [1].

RAN2#130 Agreements Data is collected on per data logging configuration basis and UE indicates data logging configuration ID. An indication of the “gap” is needed. “Gap” is time interval larger than the configured logging periodicity. FFS if timestamp and relative time stamp for each group is needed per “group”.

In RAN2#130, it was agreed to have the indication of “gap”, while there were diverse opinions on whether the detailed temporal information (i.e., timestamp) was needed for certain use cases like AI/ML model training, considering signalling overhead and UE complexity.

To resolve the FFS and accommodate the different views expressed, a flexible mechanism with two mutually exclusive options would be considered, which allows the network to configure one of the options based on its requirements and specific use case.

Option 1: Gap Indicator

- Function: A 1-bit flag that simply indicates the presence of a gap before the sample.

- Implementation: The Gap Indicator is prepended to the first logged sample after the gap.

- Advantage: It fulfills the RAN2#130 agreement with minimal overhead, while it can address the concerns about complexity and overhead. The network can be aware of the data discontinuity (i.e., the gap).

Option 2: Relative Timestamp

- Function: A relative timestamp indicates the elapsed time (i.e., the duration of the gap) between the last sample before the gap and the first sample after the gap.

- Implementation: The relative timestamp is attached in place of the Gap Indicator.

- Advantage: This provides valuable information for more advanced use cases, such as temporal domain analysis, where the length of the discontinuity is a critical piece of data. It addresses the needs of companies who have highlighted the importance of knowing the gap duration.

By making the relative timestamp optional and configurable, the network can optimize the report contents depending on whether the relative timestamp is deemed necessary.

Based on the discussion above, the following proposal is made:

[RRC-21] To indicate the gap in logged data, the following options should be configurable by the network: 
Option 1: Gap Indicator (e.g., a 1-bit flag) to indicate the presence of a gap
Option 2: Relative timestamp to indicate the time period of the gap.

Open issue RRC-25: Dynamic activation/deactivation of data collection configurations for logging

The key point of this open issue is as follows:

- RRC-25 (Dynamic Activation/Deactivation): While the network can configure data collection to start immediately upon configuration, it remains for further study (FFS) whether dynamic activation/deactivation should be supported afterwards.

The dynamic activation and deactivation mechanism is considered useful for the network load management. For example, the network may detect sudden traffic demands (i.e., highly loaded) and want to suspend the AI/ML data collection campaign for a moment.

The static control with RRCReconfiguration can be used to de-configure the UEs from the measurements and logging, but it causes lots of signalling overhead which is not acceptable especially under the network congestion. In addition, RRCReconfiguration may cause a critical delay for scheduler operation, so the ability of dynamic control for data collection is necessary:

- NW Resource and Load Management: These are the most critical operational criteria for the network. Data collection can be resource-intensive for the gNB. It may be initiated during periods of normal load. However, in case of a sudden traffic surge or unexpected congestion, the gNB may need to preempt or pause the data collection in real-time to protect user QoS.

Given the light-weight and low latency controllability for the network load management, the dynamic activation/deactivation of data collection by a lower-layer signalling (e.g., DCI) is necessary.

Based on the discussion above, the following proposal is made:

[RRC-25] For the real-time load management by the network, the dynamic activation/deactivation of configured data collection should be supported with a lower-layer signalling (e.g., DCI).

Open issue RRC-26: Multiplexing legacy logged data and AI/ML logged data in new SRB

In RAN2#129bis, it was agreed as follows [2]:

RAN2#129bis Agreements New SRB can be configured for NW-side data collection (with lower priority)

The agreement led to the open issue over which SRB the UEInformationResponse message should be sent by the UE, particularly in case the message contain both new AI/ML logged data (for which the new SRB is intended) and the legacy logged data (e.g., MDT reports for which SRB2 is used), i.e., the multiplexing.

The multiplexing would make the UE select the SRB to use, based on the content of the UEInformationResponse message. For instance, a simple rule would be considered that the new SRB is used whenever AI/ML data is included.  However, this raises another issue that legacy logged data may be sent over the new SRB with a lower priority than SRB2, if the two types of logged data are multiplexed, which may introduce other challenges, e.g., negative impacts on legacy procedures.

Design Principle of new SRB

While a content-based selection mechanism between SRB1 and SRB2 already exists, the new SRB is intended for the AIML log transfer as very low priority and very large volume data, which is a different characteristic from legacy logged data. Therefore, it should be preferable to protect the legacy RRC message transfer from the AIML logged data transfer, to avoid any impact on legacy procedures.

Consideration on UE Implementation

From a UE implementation perspective, it is important to consider that the AS layer memory for logging is limited. The fact that the UE would send the Logged MDT report and the AI/ML logged data in the same message means both legacy logging and AIML logging would be handled simultaneously, which could impact on the UE AS memory resources and processing capabilities.

Simple Approach: No Multiplexing in a Single Message

If multiplexing legacy logged data and AI/ML logged data in a single UEInformationResponse message is prohibited, the aforementioned challenges are avoided. This approach has a clear and simple consequence:

- Legacy logged Data (e.g., MDT): is reported via UEInformationResponse message is sent on SRB2, as legacy.

- AI/ML logged Data: is reported via UEInformationResponse message is sent on the new SRB, according to the RAN2 agreement.

In addition, this simple approach also aligns well with the likely backend network architecture, where the consumers of legacy MDT data and AI/ML data would be separated network entities. A clean separation on the air interface simplifies the gNB's role and maintains a more efficient end-to-end data flow.

Based on the discussion above, the following proposals are made:

[RRC-26] Legacy logged data and AI/ML logged data should not be multiplexed in one message.

[RRC-26] In case both legacy logged data and AI/ML logged data need to be reported, the UE should transmit the legacy data on SRB2 and the AI/ML data on the new SRB, in response to their respective UEInformationRequest.

Open issue RRC-27: How to set logging periodicity

The open issue RRC-27 concerns the issue on how to set the logging periodicity for the NW side data collection feature, which is used for network optimization and AI/ML automation. The logging periodicity critically impacts data collection efficiency, data quality and UE power consumption.

Currently, the two main approaches have been proposed, the simple approach by aligning the logging periodicity with RS transmission periodicity and the flexible approach by configuring a specific logging periodicity.

It is considered as essential that the logging periodicity is flexible and configurable by the network. The simple approach (i.e., aligning the logging periodicity with the RS transmission periodicity) has drawbacks as follows:

- Efficient Data Collection: For advanced network optimization based on AI/ML, the network may require data at various granularities. For example, a long logging periodicity may be suitable for collecting ground truth data for AI/ML model training (e.g., for long-term trend or model drift analysis), while a short periodicity may be useful for instantaneous issue analysis. A flexible configuration allows the network to collect the most appropriate data depending on purposes, avoiding the inefficiency of collecting unnecessary data or failing to collect necessary data.

- UE Power Saving: RS transmission periodicity is often configured to be short to ensure link stability. If the UE is forced to log data at this short interval even when the network only needs long-term statistics, it leads to unnecessary processing and battery drain on the UE. Allowing the network to configure a longer logging periodicity significantly contributes to UE power saving.

Therefore, the logging periodicity should be separated from RS transmission periodicity and configured explicitly.

Based on the discussion above, the following proposal is made:

[RRC-27] The logging periodicity, independent of RS transmission periodicity, for NW side data collection should always be explicitly configured by the network.

Conclusion

This contribution discusses the issues in terms of NW side data collection. RAN2 is kindly asked to take into account the proposals below:

Proposal 1	[RRC-21] To indicate the gap in logged data, the following options should be configurable by the network:  
Option 1: Gap Indicator (e.g., a 1-bit flag) to indicate the presence of a gap 
Option 2: Relative timestamp to indicate the time period of the gap.

Proposal 2	[RRC-25] For the real-time load management by the network, the dynamic activation/deactivation of configured data collection should be supported with a lower-layer signalling (e.g., DCI).

Proposal 3	[RRC-26] Legacy logged data and AI/ML logged data should not be multiplexed in one message.

Proposal 4	[RRC-26] In case both legacy logged data and AI/ML logged data need to be reported, the UE should transmit the legacy data on SRB2 and the AI/ML data on the new SRB, in response to their respective UEInformationRequest.

Proposal 5	[RRC-27] The logging periodicity, independent of RS transmission periodicity, for NW side data collection should always be explicitly configured by the network.

References

- “RAN2 Chair’s Notes”, (InterDigital), RAN2#130

- “RAN2 Chair’s Notes”, (InterDigital), RAN2#129bis