3GPP TSG-RAN WG2 Meeting #131							R2-2506082

Bengaluru, India, 25 - 29 Aug, 2025

Title: 	Discussion on MRO for mobility

Source: 	Huawei, HiSilicon

Agenda item:	8.10.2

Document for:	Discussion/Decision

- Introduction

In the incoming LS [1], there is an action from RAN3, so this paper is to provide our view on it.

RAN3 therefore asks RAN2 to consider if the description of previousPSCellId and timeSCGFailure in SCGFailureInformation needs to be updated to cover the S-CPAC scenario.

In addition, RRC open issues have been discussed in [2], and some of open issues may need more discussions. This paper also provides some analysis for them.

- Discussion

- 2.1	Discussion on the LS [1]

In the running CR, the procedural text has been updated based on the RAN3 LS [1], e.g.

3>	if the failure occurred during a subsequent CPC:

4>	set the previousPSCellId to the physical cell identity and carrier frequency of the source PSCell associated to the last execution of RRCReconfiguration message including reconfigurationWithSync for the SCG, if available;

3>	else:

4>	set the previousPSCellId to the physical cell identity and carrier frequency of the source PSCell associated to the last received RRCReconfiguration message including reconfigurationWithSync for the SCG, if available;

However, the related field descriptions have not be updated. We suggest to update the field description of previousPSCellId, e.g. it also indicates the cell that is the source PSCell associated to the last execution of PSCell change for a subsequent CPC. For the IE timeSCGFailure, the legacy definition has already covered the subsequent CPC, so there is no need for more changes.

SCGFailureInformation field descriptions

measResultFreqList The field contains available results of measurements on NR frequencies the UE is configured to measure by measConfig.

measResultSCG-Failure The field contains the MeasResultSCG-Failure IE which includes available results of measurements on NR frequencies the UE is configured to measure by the NR SCG RRCReconfiguration message.

previousPSCellId This field indicates the physical cell id and carrier frequency of the cell that is the source PSCell of the last PSCell change. In case of PSCell addition failure, this field is absent.

failedPSCellId This field indicates the physical cell id and carrier frequency of the cell in which SCG failure is detected or the target PSCell of the failed PSCell change or failed PSCell addition.

timeSCGFailure This field is used to indicate the time elapsed since the last execution of RRCReconfiguration with reconfigurationWithSync for the SCG until the SCG failure. Actual value = field value * 100ms. The maximum value 1023 means 102.3s or longer.

Proposal 1: It is proposed to add a clarification to the field description of previousPSCellId, i.e. it also indicates the cell that is the source PSCell associated to the last execution of RRCReconfiguration message including reconfigurationWithSync for the SCG.

- 2.2	Open issue RRC-1: Avoiding duplicated measurement reports in SHR and SPR

In [2], the rapporteur's summary is listed as below:

Rapporteur summary: Among different options provided by different companies we some companies prefer option B (e.g., adding correlation indication inside the SHR and SPR). However, the detail of the correlation indication is not described. One drawback of the correlation inside the reports might be that the network does not know the correlation before fetching both reports and if one report is replaced by a new report (e.g., UE logs a new SHR (which is not correlated with the SPR)) the corrletaion indication of in the SPR should be removed. This creates additional complexity in the procedural text that should be taken into account when designing the solution. During some offline some companies proposed some other solutions like adding PCell C-RNTI (and time since generating SPR, equivalent to timeSinceSHR) in the SPR report and given that SHR logs same information correlation of SHR and SPR becomes feasible. Overall, given that different companies have different view on the way forward, companies are invited to submit contribution including a text proposal to visualize their preferred solution to the meeting RAN2#131. [Open issue RRC-1] For correlation of SHR and SHR generated for a CHO with Candidate SCG, RAN2 discuss and agree to one of the following options - Correlation indication in RRC_Complete message (a flag beside availability indication) - Correlation indication inside the SHR and SPR - Logging C-RNTI of PCell and timeSinceSPR in SPR - The duplicated information in correlated SHR and SPR is sufficient. no further information is needed for correlation.

Firslty, we support Option B, i.e. Adding a correlation indication in the SHR and SPR reports. We observe that some companies also preferred Option B since such correlation information will be useful for NW problem identification and analysis.

Secondly, for solutions, we slightly prefer c, i.e. Logging C-RNTI of PCell and timeSinceSPR in SPR. The possible changes could be:

- c-RNTI for PCell: this field indicates the C-RNTI assigned by the target PCell of the handover for which the successful PSCell change/addition report was generated

- timeSinceSPR: this field is used to indicate the itme elapsed since the execution of the last PSCell change/addition. Value in seconds. The maximum value 172800 means 172800s or longer

Proposal 2: (Open issue RRC-1) For correlation of SHR and SHR generated for a CHO with Candidate SCG, it is proposed RAN2 to agree on introducing Logging C-RNTI of PCell and timeSinceSPR in SPR, and details are:

- c-RNTI for PCell: this field indicates the C-RNTI assigned by the target PCell of the handover for which the successful PSCell change/addition report was generated

- timeSinceSPR: this field is used to indicate the itme elapsed since the execution of the last PSCell change/addition. Value in seconds. The maximum value 172800 means 172800s or longer

- Conclusion

In this paper, we are to address one RAN3 LS [1] and open issue RRC-1 [2]. It is proposed:

Proposal 1: It is proposed to add a clarification to the field description of previousPSCellId, i.e. it also indicates the cell that is the source PSCell associated to the last execution of RRCReconfiguration message including reconfigurationWithSync for the SCG.

Proposal 2: (Open issue RRC-1) For correlation of SHR and SHR generated for a CHO with Candidate SCG, it is proposed RAN2 to agree on introducing Logging C-RNTI of PCell and timeSinceSPR in SPR, and details are:

- c-RNTI for PCell: this field indicates the C-RNTI assigned by the target PCell of the handover for which the successful PSCell change/addition report was generated

- timeSinceSPR: this field is used to indicate the itme elapsed since the execution of the last PSCell change/addition. Value in seconds. The maximum value 172800 means 172800s or longer

- References

R3-253886, LS on support of MRO for S-CPAC, Source: RAN3, To: RAN2

[Post130][605][SONMDT] Running NR RRC CR (Ericsson)

- Annex

–	SCGFailureInformation

The SCGFailureInformation message is used to provide information regarding NR SCG failures detected by the UE.

Signalling radio bearer: SRB1

RLC-SAP: AM

Logical channel: DCCH

Direction: UE to Network

SCGFailureInformation message

-- ASN1START

-- TAG-SCGFAILUREINFORMATION-START

SCGFailureInformation ::=                SEQUENCE {

criticalExtensions                       CHOICE {

scgFailureInformation                    SCGFailureInformation-IEs,

criticalExtensionsFuture                 SEQUENCE {}

}

}

SCGFailureInformation-IEs ::=            SEQUENCE {

failureReportSCG                         FailureReportSCG                    OPTIONAL,

nonCriticalExtension                     SCGFailureInformation-v1590-IEs     OPTIONAL

}

SCGFailureInformation-v1590-IEs ::=      SEQUENCE {

lateNonCriticalExtension                 OCTET STRING                        OPTIONAL,

nonCriticalExtension                     SEQUENCE {}                         OPTIONAL

}

FailureReportSCG ::=                     SEQUENCE {

failureType                              ENUMERATED {

t310-Expiry, randomAccessProblem,

rlc-MaxNumRetx,

synchReconfigFailureSCG, scg-ReconfigFailure,

srb3-IntegrityFailure, other-r16, spare1},

measResultFreqList                       MeasResultFreqList                                    OPTIONAL,

measResultSCG-Failure                    OCTET STRING (CONTAINING MeasResultSCG-Failure)       OPTIONAL,

...,

[[

locationInfo-r16                         LocationInfo-r16                                      OPTIONAL,

failureType-v1610                        ENUMERATED {scg-lbtFailure-r16, beamFailureRecoveryFailure-r16,

t312-Expiry-r16, bh-RLF-r16, beamFailure-r17, spare3, spare2, spare1} OPTIONAL

]],

[[

previousPSCellId-r17                 SEQUENCE {

physCellId-r17                       PhysCellId,

carrierFreq-r17                      ARFCN-ValueNR

}                                                             OPTIONAL,

failedPSCellId-r17                   SEQUENCE {

physCellId-r17                       PhysCellId,

carrierFreq-r17                      ARFCN-ValueNR

}                                                            OPTIONAL,

timeSCGFailure-r17                   INTEGER (0..1023)        OPTIONAL,

perRAInfoList-r17                    PerRAInfoList-r16          OPTIONAL

]],

[[

perRAInfoList-v17b0                  PerRAInfoList-v1660      OPTIONAL

]],

[[

perRAInfoList-v1840                  PerRAInfoList-v1800      OPTIONAL

]]

}

MeasResultFreqList ::=                   SEQUENCE (SIZE (1..maxFreq)) OF MeasResult2NR

-- TAG-SCGFAILUREINFORMATION-STOP

-- ASN1STOP

SCGFailureInformation field descriptions

measResultFreqList The field contains available results of measurements on NR frequencies the UE is configured to measure by measConfig.

measResultSCG-Failure The field contains the MeasResultSCG-Failure IE which includes available results of measurements on NR frequencies the UE is configured to measure by the NR SCG RRCReconfiguration message.

previousPSCellId This field indicates the physical cell id and carrier frequency of the cell that is the source PSCell of the last PSCell change. In case of PSCell addition failure, this field is absent.

failedPSCellId This field indicates the physical cell id and carrier frequency of the cell in which SCG failure is detected or the target PSCell of the failed PSCell change or failed PSCell addition.

timeSCGFailure This field is used to indicate the time elapsed since the last execution of RRCReconfiguration with reconfigurationWithSync for the SCG until the SCG failure. Actual value = field value * 100ms. The maximum value 1023 means 102.3s or longer.