3GPP TSG-RAN WG2 Meeting #131	R2-2506089

Bengaluru, India, 19 – 23 August 2025

Agenda item:	8.5.2

Source:	Nokia, Nokia Shanghai Bell

Title:	On-demand SSB SCell operation

WID/SID:	Netw_Energy_NR - Release 19

Document for:	Discussion and Decision

# 1	Introduction

In this contribution, we will discuss the aspects related to the on-demand SSB SCell operation which is one of the objectives of the R19-network energy savings work item

# 2	On-demand SSB SCell Operation

## 2.1	On-demand SSB configuration

In this section, we would like to discuss specifically od-ssb-nrOfBurst parameter within the on-demand SSB configuration.

Based on the RAN1 #121 agreements, different combinations for the OD-SSB activation and OD-SSB adaptation are supported. RRC based activation of an on-demand SSB configuration is supported both during the initial configuration and even at later phase. Further RRC-based on-demand SSB activation is only supported when the od-ssb-config does not include od-ssb-nrofBurst parameter.

Agreement (RAN2) - RRC based OD-SSB transmission indication is used to indicate at least the initial activation/deactivation state of OD-SSB configuration. FFS on reconfiguration. - No need to restrict the OD-SSB activation/deactivation state indication in RRC to initial configuration. No special specification effort is required. - Don’t introduce further new MAC CE that combines SCell activation/deactivation and OD-SSB indication for scenario 2A. - NW should be able to send OD-SSB indication for multiple SCells simultaneously by a MAC CE. Agreement (RAN1#121) For a cell supporting on-demand SSB SCell operation, the following combinations are supported. - For OD-SSB transmission activation (OD-Tact) and OD-SSB transmission adaptation (OD-TA), - Case A1: RRC-based OD-Tact without N (i.e., od-ssb-nrofBurst) configured + MAC CE-based OD-TA; - Subject to UE capability - Case B1: MAC CE-based OD-Tact without N configured + MAC CE-based OD-TA; - Case B2: MAC CE-based OD-Tact with N configured + MAC CE-based OD-TA. - For OD-SSB transmission deactivation (OD-TD), - Case X1: RRC-based OD-Tact without N configured + MAC CE-based OD-TD; - Subject to UE capability - Case Y1: MAC CE-based OD-Tact or OD-TA without N configured + MAC CE-based OD-TD; - Case Y2: MAC CE-based OD-Tact or OD-TA with N configured + implicit OD-TD; - Case Y3: MAC CE-based OD-Tact or OD-TA with N configured + MAC CE-based OD-TD. - Conclusion: There is no RAN1 consensus to support RRC activation of OD-SSB transmission configuring od-ssb-nrofBurst. - Note: “Implicit OD-TD” above implies that the on-demand SSB is deactivated based on the value for od-ssb-nrofBurst according to NW indication.

The current description of the OD-SSB-Config IE indicates that the IE is used to configure OD-SSB activated by MAC-CE. This does not accurately reflect the latest agreements from RAN2 and RAN1.

Proposal 1 : RAN2 to enhance description of the OD-SSB-Config IE to indicate that OD-SSB-Config may be activated by MAC CE signaling or, if the od-ssb-nrofBurst parameter is absent, via RRC signaling.

Further based on below RAN1 agreement, the od-ssb-nrofBurst is defined as one out of eight possible values, where the range is different for FR1 and FR2.

Agreement For a cell supporting on-demand SSB SCell operation, for configuring od-ssb-nrofBurst of which the value range is {N2 integer values}, - N2= 8 - Note: This is updated from the previous RAN1 agreement. - The following values for od-ssb-nrofBurst are taken as the starting point and to be confirmed in RAN1#122 - For FR1, the value range of od-ssb-nrofBurst is {5, 10, 15, 20, 25, 30, 40, 50}. - For FR2, the value range of od-ssb-nrofBurst is {25, 30, 40, 50, 75, 100, 150, 200}.

At present the IE is defined as an INTEGER and needs to be changed to capture RAN1 agreements.

Further, the od-ssb-ActivationStatus parameter, there are discussion on how to use the parameter when configured with od-ssb-nrofBurst. As per our understanding, OD-SSB-Configuration that include od-ssb-nrOfBurst cannot be activated by RRC. Hence the existing IE definition may fit fine or another option

could be define enumeration with deactivated and activated status.

Proposal 2 : RAN2 to enhance the IE definition for od-ssb-nrOfBurst and od-ssb-ActivationStatus as indicated in the Annexure.

## 2.2	On-demand SSB indication via MAC-CE

The only open issue left is maximum number of OD-SSB configurations to be supported per SCell. From our point of view, maximum number of 16 OD-SSB configuration with 4 bits as currently captured in the MAC running CR should be more than  enough.

Proposal 3: support a maximum number of 16 OD-SSB configurations per SCell.

## 2.3	On-demand SSB and SMTC

For case#2, when always-on SSB and on-demand SSB are transmitted on the same frequency, the UE measures ssbFrequency in SMTC1 as per the legacy behaviour. When on-demand SSB is activated via MAC-CE, the SMTC associated with the on-demand SSB configuration is used for the measurement.

For case#1, if the MeasObject is configured as per the legacy behaviour, it should be ensured that RAN2 agreement of not measuring non activated SSBs is achieved in the signaling as well. Upon activation of on-demand SSB via MAC-CE, the new SMTC may be applied.

This requires to align the SMTC window with the on-demand SSB transmission.

Figure 4: on-demand SSB and SMTC

Agreements on OD-SSB 5. In L3 measurement in OD-SSB case 1, if MAC-CE/RRC based activation / deactivation: - The UE starts L3 measurement towards the activated OD-SSB based on configured servingCellMO after reception of the activation. - The UE stops L3 measurements after it determines the OD-SSB is deactivated implicitly or explicitly. - Spec impact can be further discussed in running CR preparation. - When the AO-SSB and OD-SSB have the same frequency, they share the same servingCellMO. - The UE applies the OD-SSB specific SMTC when the OD-SSB is activated and SCell is activated. This decision does not impact RAN4 discussion whether both OD-SSB and AO-SSB can be measured. - RAN2 confirms the working consumption that when AO-SSB and OD-SSB have different center frequency, introduce a new servingCellMO in ServingCellConfig to indicate MO of OD-SSB, as agreement. - RAN2 understands multiple OD-SSBs with the different frequencies for a given SCell is not supported. - When OD-SSB is activated, UE uses servingCellMO-OD to measure serving cell; when OD-SSB is deactivated, UE uses servingCellMO-AO (i.e., legacy servingCellMO) to measure serving cell. - RAN2 will leave whether to introduce OD-SSB MAC CE triggered MR or not to RAN4. Agreement Regarding the relation in terms of time location between the always-on SSB and on-demand SSB, - For the case when the center frequency locations of always-on SSB and on-demand SSB are same, - Alt Time-C: RAN1 specification has no restriction with regards to overlapping - From RAN1 perspective, - Alt Time-C1: The case that, during OD-SSB transmission, the union of AO-SSB transmission and OD-SSB transmission has a periodic time domain pattern is supported (the interval between SSB bursts is even and supported in legacy specification). - Alt Time-C2: The case that, during OD-SSB transmission, the union of AO-SSB transmission and OD-SSB transmission has a non-periodic time domain pattern is supported. - It is up to RAN4 to define requirements, if any, corresponding to both or either of Alt Time-C1 or Alt Time-C2 - At least the following is supported: PBCH payload for the same SSB index (other than SFN index, half frame index) is the same for AO-SSB and OD-SSB - FFS: Whether half frame index is the same or different for AO-SSB and OD-SSB - For the case when the center frequency locations of always-on SSB and on-demand SSB are different, - Alt Time-C: RAN1 specification has no restriction with regards to overlapping - UE assumes that frequency resources of always-on SSB are not overlapped with those of on-demand SSB in frequency domain. - AO-SSB and OD-SSB are located in the same BWP - FFS: PBCH payload for the same SSB index (other than SFN index, half frame index) should be the same for AO-SSB and OD-SSB

The definition of od-smtc-r19 parameter in the current draft CR is as shown below.

servingCellMO-r19                  MeasObjectId                OPTIONAL, -- Cond InterFreq

od-smtc-r19                        SSB-MTC                     OPTIONAL, -- Cond IntraFreq

...

}

| Conditional Presence | Explanation |
| --- | --- |
| ODssbOnly | The field is optionally present, Need R, for serving cell that does not have SSB. It is absent otherwise. |
| InterFreq | This field is mandatory present if od-ssb-absoluteFrequency indicates different frequency than absoluteFrequencySSB of the serving cell. It is absent otherwise. |
| IntraFreq | This field is mandatory present if od-ssb-absoluteFrequency indicates same frequency as absoluteFrequencySSB of the serving cell. It is absent otherwise. |

This implies that the smtc is only necessary when the on-demand SSB (OD-SSB) and the always-on SSB (AO-SSB) share the same frequency. However, as per the RAN1 agreement, there are no restriction on the time location between AO-SSB and OD-SSB. i.e OD-SSB may be transmitted in different time slots than the AO-SSB. Hence for OD-SSB measurements UE require SMTC configuration regardless of whether the OD-SSB and AO-SSB frequencies are the same or different frequency. To ensure UE measurements, SMTC configuration may need to  be provided per od-ssb-config .

We propose modifying the description of the od-smtc-r19 field to remove any frequency-dependent conditions. The description should clearly state that this field is mandatory for OD-SSB measurement when OD-SSB is activated, irrespective of the frequency relationship between OD-SSB and AO-SSB.

Proposal 4: RAN2 to modify the od-ssb-smtc-r19 description to clearly state that this field is mandatory for OD-SSB measurement when OD-SSB is activated, irrespective of the frequency relationship between OD-SSB and AO-SSB.

## 2.4	Asn.1 structure for OD-SSB (and SSB adaptation)

In the current draft CR, the ASN.1 fields are totally separate for OD-SSB and SSB adaptation, but some fields are the same (e.g. half frame index, sfn-offset, smtc). We propose to consider whether it makes sense to have an IE that has common fields in order to avoid possible errors/deviations between features that are not reflected in the other feature.

Proposal 5: Have a common IE sharing same fields used by both OD-SSB and SSB adaptation features (not in the TP).

Also, it does not seem necessary to repeat od-ssb/ssb adaptation names in the fields as the IE itself already implies the feature it is associated to.

Proposal 6: Simplify naming of fields to remove prefix of “od-ssb” within OD-SSB-Config

For the od-ssb-periodicity default value of 5ms is given but it is not clear where this comes from. If there is not clear reason why this value is chosen then we propose to must field presence as mandatory. Additionally with the default value the conditional presence of the field does not make much sense thus:

Proposal 7: Remove default value for periodicity and change the presence to mandatory

# 3	Conclusion

Proposal 1 : RAN2 to enhance description of the OD-SSB-Config IE to indicate that OD-SSB-Config may be activated by MAC CE signaling or, if the od-ssb-nrofBurst parameter is absent, via RRC signaling.

Proposal 2 : RAN2 to enhance the IE definition for od-ssb-nrOfBurst and od-ssb-ActivationStatus as indicated in the Annexure.

Proposal 3: support a maximum number of 16 OD-SSB configurations per SCell.

Proposal 4: RAN2 to modify the od-ssb-smtc-r19 description to clearly state that this field is mandatory for OD-SSB measurement when OD-SSB is activated, irrespective of the frequency relationship between OD-SSB and AO-SSB.

Proposal 5: Have a common IE sharing same fields used by both OD-SSB and SSB adaptation features (not in the TP).

Proposal 6: Simplify naming of fields to remove prefix of “od-ssb” within OD-SSB-Config

Proposal 7: Remove default value for periodicity and change the presence to mandatory

# Annex A: TP for SSB

#### –	OD-SSB-Config

The IE OD-SSB-Config is used to configure the OD-SSB activated by a MAC CE or RRC see TS 38.321 [3], clause XXX

OD-SSB-Config information element

-- ASN1START

-- TAG-OD-SSB-CONFIG-START

OD-SSB-Config-r19 ::= SEQUENCE {

configId-r19                    OD-SSB-ConfigId-r19,

activationStatus-r19            ENUMERATED {activated,deactivated},

periodicity-r19                 ENUMERATED { ms5, ms10, ms20, ms40, ms80, ms160, spare2, spare1 },

sfn-Offset-r19                   INTEGER (0..15)                                                         OPTIONAL, -- Cond ODssbAOssb

halfFrameIndex-r19              ENUMERATED {zero, one}                                                  OPTIONAL, -- Cond ODssbAOssb

absoluteFrequency-r19           ARFCN-ValueNR                                                           OPTIONAL, -- Cond ODssbAOssb

PositionsInBurst-r19            CHOICE {

shortBitmap                            BIT STRING (SIZE (4)),

mediumBitmap                           BIT STRING (SIZE (8)),

longBitmap                             BIT STRING (SIZE (64))

}                                                                                                              OPTIONAL, -- Cond ODssbAOssb

subcarrierSpacing-r19            SubcarrierSpacing                                                       OPTIONAL, -- Cond ODssbOnly

pbch-BlockPower-r19              INTEGER (-60..50)                                                       OPTIONAL, -- Cond ODssbOnly

nrofBurst-r19                   CHOICE {

fr1Range	ENUMERATED { n5, n10, n15, n20, n25, n30, n40, n50 },

fr2Range	ENUMERATED { n25, n30, n40, n50, n75, n100, n150, n200 }

}                                                                    OPTIONAL, -- Need R

servingCellMO-r19                      MeasObjectId                                                            OPTIONAL, -- Cond InterFreq

smtc1-r19                            SSB-MTC,

...

}

OD-SSB-ConfigId-r19   ::= INTEGER (0.. maxNrofOD-SSB-1-r19)

-- TAG-OD-SSB-CONFIG-STOP

-- ASN1STOP

OD-SSB-Config field descriptions

smtc1 Primary measurement timing configuration (see clause 5.5.2.10) to be used instead of smtc1 configured in servingCellMO in IE servingCellConfig when this OD-SSB is activated and the serving cell is activated

absoluteFrequency Indicates the frequency of the OD-SSB when the frequency is different from absoluteFrequencySSB configured in IE FrequencyInfoDL for this serving cell. Additional restrictions as described in subclause 4.4. of TS 38.213.

Indicates the activation status of this OD-SSB pattern upon configuration. Only one OD-SSB pattern can be activated at one point of time.

halfFrameIndex Indicates whether OD-SSB is in the first half or the second half of the frame. If the field is absent, the UE applies the value 0.

sfn-Offset Indicates SFN offset from the SFN which satisfies (SFN index *10) modulo (OD-SSB periodicity) = 0. The network configures this field according to the field od-ssb-Periodicity such that the indicated system frame does not exceed the OD-SSB periodicity. If the field is absent, the UE applies the value 0.

nrofBurst Indicates the number of OD-SSB bursts to be transmitted after OD-SSB is activated. The field may be only present in case activationStatus is deactivated. Value n5

periodicity The SSB periodicity in ms. (see TS 38.213 [13], clause 4.1).

positionsInBurst Indicates the time domain positions of the transmitted SS-blocks for OD-SSB in a half frame with SS/PBCH blocks as defined in TS 38.213 [13], clause 4.1. If absent, od-ssb-PositionsInBurst is the same as ssb-PositionsInBurst provided in ServingCellConfigCommon.

pbch-BlockPower Indicates average EPRE of the resources elements that carry secondary synchronization signals in dBm that the NW used for OD-SSB transmission, see TS 38.213 [13], clause 7.

subcarrierSpacing Indicates subcarrier spacing of OD-SSB, for Case #1, i.e., no always-on SSB on this serving cell. Only the following values are applicable depending on the used frequency: FR1: 15 or 30 kHz FR2-1/FR2-NTN: 120 or 240 kHz FR2-2: 120, 480, or 960 kHz

servingCellMO measObjectId of the MeasObjectNR in MeasConfig which is associated to the serving cell when this OD-SSB is activated instead of servingCellMO in IE ServingCellConfig.

| Conditional Presence | Explanation |
| --- | --- |
| ODssbOnly | The field is optionally present, Need R, when absoluteFrequencySSB of the serving cell is absent. It is absent otherwise. |
| ODssbAOssb | The field is mandatory present, Need R, when absoluteFrequencySSB of the serving cell is absent. It is optionally present otherwise. |
| InterFreq | This field is mandatory present if od-ssb-absoluteFrequency indicates different frequency than absoluteFrequencySSB of the serving cell. It is absent otherwise. FFS SSB-less SCell |

OD-SSB-Config field descriptions

od-smtc Primary measurement timing configuration (see clause 5.5.2.10) to be used instead of smtc1 configured in servingCellMO in IE servingCellConfig when this OD-SSB is activated.

od-ssb-absoluteFrequency Indicates the frequency of the OD-SSB when the frequency is different from absoluteFrequencySSB configured in IE FrequencyInfoDL for this serving cell. Additional restrictions as described in subclause 4.4. of TS38.213.

od-ssb-ActivationStatus Indicates the activation status of this OD-SSB pattern upon configuration. Only one of the OD-SSB-Config may be active at one point in time.

od-ssb-halfFrameIndex Indicates whether OD-SSB is in the first half or the second half of the frame. If the field is absent, the UE applies the value 0.

od-ssb-sfn-Offset Indicates SFN offset from the SFN which satisfies (SFN index *10) modulo (OD-SSB periodicity) = 0. The network configures this field according to the field od-ssb-Periodicity such that the indicated system frame does not exceed the OD-SSB periodicity. If the field is absent, the UE applies the value 0.

od-ssb-nrofBurst Indicates the number of OD-SSB bursts to be transmitted after OD-SSB is indicated.

od-ssb-Periodicity The SSB periodicity in ms. If the field is absent, the UE applies the value ms5. (see TS 38.213 [13], clause 4.1).

od-ssb-PositionsInBurst Indicates the time domain positions of the transmitted SS-blocks for OD-SSB in a half frame with SS/PBCH blocks as defined in TS 38.213 [13], clause 4.1. For Case #2 (i.e., Always-on SSB is periodically transmitted on the cell), if absoluteFrequencySSB is not configured in IE FrequencyInfoDL, the field is present. Otherwise it may be absent. if absent, od-ssb-PositionsInBurst is the the time domain positions are same as ssb-PositionsInBurst provided in ServingCellConfigCommon.

od-ss-PBCH-BlockPower Indicates average EPRE of the resources elements that carry secondary synchronization signals in dBm that the NW used for OD-SSB transmission, see TS 38.213 [13], clause 7, for Case #1, i.e., no always-on SSB on this serving cell.

od-ssbSubcarrierSpacing Indicates subcarrier spacing of OD-SSB. if absoluteFrequencySSB is not configured in IE FrequencyInfoDL, the field is present., for Case #1, i.e., no always-on SSB on this serving cell. Only the following values are applicable depending on the used frequency: FR1: 15 or 30 kHz FR2-1/FR2-NTN: 120 or 240 kHz FR2-2: 120, 480, or 960 kHz

servingCellMO measObjectId of the MeasObjectNR in MeasConfig which is associated to the serving cell when this OD-SSB is activated activated instead of servingCellMO in IE ServingCellConfig.

| Conditional Presence | Explanation |
| --- | --- |
| ODssbOnly | The field is optionally present, Need R, for serving cell where absoluteFrequencySSB is not configured in IE FrequencyInfoDL. It is absent otherwise. |
| InterFreq | This field is mandatory present if od-ssb-absoluteFrequency indicates different frequency than absoluteFrequencySSB of the serving cell. It is absent otherwise. |
| IntraFreq | This field is mandatory present if od-ssb-absoluteFrequency indicates same frequency as absoluteFrequencySSB of the serving cell. It is absent otherwise. |