3GPP TSG-RAN WG2 #131 	R2-250XXXX

Bangalore, India, August 25th– 29th, 2025

Agenda Item:	8.2.2

Source:	Ericsson

Title:	Discussion on DL messages for Ambient IoT UEs

Document for:	Discussion/Decision

# 1	Introduction

In RAN#107, a new WI has been approved for Ambient IoT devices [1]. The following RAN2 related objectives are capture in the WID:

  - Specify the necessary functions and procedures for an Ambient IoT compact protocol stack and lightweight signalling procedure to enable DO-DTT and DT data transmission:

    - A-IoT Paging, including subsequent paging for the same service. Support the options that a paging message contains one identifier, and that a paging message contains no identifier. Temporary identifier is not supported, unless required by SA WGs.

Note: RAN2 aims to design a paging message format such that multiple identifiers can be contained in one paging message, for forward compatibility purposes.

    - A-IoT Random access, including re-access for failure handling. Contention-based and contention-free cases are supported. For the contention-based random access, only Solution 1 (3-step only) is included (unless RAN2 decides to use Solution 3 (unified solution) by RAN2#129).

    - A-IoT data transmission, including data (re-)transmission for failure handling. Segmentation is supported at least in D2R.

    - Only MAC layer is included

In this contribution we continue to discuss the open issues on downlink messages for A-IoT devices based on the scope highlighted above considering the agreements so far.

# 2	Discussion

## 2.1 How to generate the transaction ID (Issue 1-2)

In RAN2#127, it was agreed to support a mechanism for the reader to trigger subsequent A-IoT paging messages that are associated with the same service request from the CN. In RAN2#127bis, it was agreed that A-IoT paging message can include information to avoid duplicate response from the device to a reader. In RAN2#129, it was agreed that the “transaction ID” can be generated by reader based on CN correlation ID.  But it is FFS how reader will generate “transaction ID” and the size of transaction ID”.

We think that there should be one to one mapping between transaction ID and correlation ID, otherwise one may observe the following:

- Same CN correlation ID maps to different transaction IDs, i.e., subsequent A-IoT paging messages associated with the same service request will have different transaction IDs. This may lead to that the device sends duplicate responses to subsequent paging unnecessarily.

- Different CN correlation IDs map to the same transaction ID. In this case the device will regard the later received A-IoT paging message as subsequent paging message and ignore if it has successfully responded earlier.

RAN2 assumes there is one to one mapping between CN correlation ID and transaction ID.

Since RAN2 has made an assumption that readers are coordinated, so they do not interleave each other’s paging procedures, a transaction ID can be generated by a reader following the principle above.

Transaction ID generation is up to network implementation.

Transaction ID may wrap around if the size is small. An A-IoT device may miss an A-IoT paging message due to bad link quality or since it is out of energy. In such case, it is possible that a device receives a paging message with a transaction ID same as the last one, assumes that it is a subsequent paging message, and ignores if it has responded successfully before. This can be mitigated using a transaction ID with at least 3 bits in size assuming that it is very unlikely that a device misses 7 consecutive A-IoT paging messages. The probably for such case is 0.00128% when 20% error probability is assumed per A-IoT paging message. Since multiple reader scenarios are to be considered in Rel-20, we think 3 bits may not be future proof enough.

The size for transaction ID is 6 bits.

Send an LS to SA2/RAN3 (cc: CT4) to inform those WGs about the assumptions in RAN2 and the size of transaction ID.

## 2.2 Paging message content for CFRA (Issue 1-5)

There was a discussion on whether transaction ID can be optional for CFRA [4]. In our view, determining whether or not to respond can be based on the paging identifier (in the upper layer message) rather than the transaction ID. RAN2 agreed the following in the last meeting: “Introduce an explicit 1-bit indication to indicate whether it is CFRA or CBRA per paging message.” Therefore, one can assume that same message type is used for CBRA and CFRA, since RAN2 has agreed to introduce a flag which explicitly indicates whether the message is CFRA or CBRA. Considering that the paging message always includes the paging identifier for CFRA, a transaction ID is not needed.

For CFRA, transaction ID is not included in the paging message.

## 2.3 Security parameter in paging message (Issue 1-7)

SA3 has concluded the discussion on providing a security parameter in the A-IoT paging message (see the text from TS33.369, clause 5.2.2 below). Note that this parameter RANDAIOT_n is included in the A-IoT paging message as a layer 2 information element rather than NAS information.

2. AIOTF shall send inventory request message including RANDAIOT_n to NG-RAN.

Editor’s Note: The inclusion of RANDAIOT_n in Paging Request and the size of RANDAIOT_n needs RAN confirmation.

3. NG-RAN shall send the paging request message including RANDAIOT_n to the AIoT device.

SA3 sent an LS from the May meeting (S3-252392) and asked if it is possible to include a 128-bit security parameter in the A-IoT paging message. Considering the maximum possible TB size, i.e., 1000 bits, and the content of the A-IoT paging message given in the table below this seems to be possible.

A-IoT paging message contains the 128-bit security parameter.

Table 1: Content of A-IoT paging message

| A-IoT paging field | Field length (bits) | Mandatory or optional | Comment |
| --- | --- | --- | --- |
| R2D message type | 3 bits | Mandatory |  |
| Random access type (RT) | 1 bit | Mandatory |  |
| Paging ID (PI) presence indicator | 1 bit | Optional |  |
| Length of Paging ID | 8 bits | Optional | Pending SA2 progress, not present in case no PID is indicated |
| Paging ID (PID) | ? | Optional | Not present in case no PID is indicated |
| Transaction ID | 6 bits | Optional | Not present in case of CFRA |
| Number of access occasions | 4 bits | Optional | Not present in case of CFRA |
| Security parameter | 128 bits? | Mandatory | Exact field size pending SA3 progress |
| D2R scheduling information for paging round | Variable | Optional |  |
| Padding bits | Variable | Optional | For byte alignment of R2D |

In the LS, SA3 also mentioned that the overall size of the security parameter may eventually be larger in case additional security parameters are needed, i.e., this is pending SA3 progress, and asked whether there is a RAN resource related limitation/restriction to include such security parameters.

From RAN2 standpoint the limitation would be the maximum possible TB size. Please see the related RAN1 agreement below:

“From RAN1 perspective, a maximum TB size of around 1000 bits in PHY for R2D and D2R directions can be supported.

•	How large TB that can be transported at a given time depends on target coverage/data rate, energy consumption/device availability, etc.”

Note that RAN2 has agreed that R2D segmentation is not supported in Rel-19. Additional security parameters should also not be a problem as long as the total message size does not exceed the maximum possible TB size.

Send an LS to SA3 (cc: SA2) to inform those WGs that it is feasible to include a 128bit random number and additional security parameters in the A-IoT paging message provided that the overall paging message size is less than or equal to the maximum TB size of 1000 bits.

## 2.4 Paging identifier visibility in the MAC layer (Issue 1-6)

In RAN2#129, the following agreements have been made:

- The “one identifier” in the paging message includes both the case of “one single device identifier” and “one group identifier”/” filtering criteria”, while the exact format of latter is supposed to be designed by SA2.”

- The current assumption is that the paging identifier is transparent to the A-IoT MAC Layer and carried by upper layer. FFS if there is really a need for visibility in the MAC layer”

For visibility in MAC, we think that RAN2 should clarify what is intended by “visible” here. If the discussion is on whether it should be possible for the reader to, for example, mask/regenerate the identifier provided by the CN, we prefer that this should not be possible. In that sense, we do not support MAC layer sub-group paging solutions. We assume that temporary ID is intended for paging a single device, hence there is no need for sub-grouping.

Besides, some companies argued that the paging identifier needs to be visible to reader’s MAC layer at least in CFRA for the reader to associate AS ID with upcoming command request [5]. To make this work CN needs to explicitly include the paging identifier in NGAP signalling together with the command request. This is not a good design practice as paging identifier is primarily used between the CN and the device, not between the CN and the reader. A NGAP device ID is more suitable for such association. Note that RAN3#127bis already agreed to introduce NGAP device ID and RAN2 can use this ID to associate the AS ID with the upcoming command request at reader.

RAN3 agreed to introduce NGAP device ID which can be used to associate AS ID with upcoming command request at reader.

The paging identifier should be transparent to the reader, i.e., it should not be possible for the reader to, e.g., mask/regenerate the identifier provided by the CN

## 2.5 Forward Compatibility (Issue 4-5)

In RAN2#130, the following agreements have been made:

- To ensure forward compatibility for paging with multiple identifiers, introduce at least one R field.   FFS if more than one R bit is required.

- Rel-19 devices would ignore the content of future release instead of ignoring the whole paging message.

Regarding the FFS on R field size, it is enough to introduce one R bit indicating whether there is one or multiple paging identifiers.

One R bit is enough to ensure forward compatibility for paging with multiple identifiers.

Moreover, use cases for applications such as sensors are to be introduced in Rel-20 [4]. If same message type is used for paging Rel-19 and Rel-20 devices, it would be beneficial to introduce some means to differentiate devices with respect to release so that it would be possible, for example, to address the following scenarios:

- a service request intended for only Rel-19 devices

- a service request intended for only Rel-20 devices

- a service request intended for all devices, i.e., Rel-19 + Rel-20 devices

In principle it would be beneficial to have the same message type for Rel-19 and Rel-20 devices for the same procedure. The differences can then be handled with optional bits in the message format. The alternative is reserving code points for message types for Rel-20 devices, but that would not make it possible to trigger a procedure that addresses both Rel-19 and Rel-20 devices at once. We may also end up with duplicating message types for Rel-19 and Rel-20 devices unnecessarily. This can be achieved with a bit in the paging message.

It would be beneficial to be able to differentiate addressing devices with respect to their release.

Introduce a bit in the paging message to indicate whether Rel-19 devices are addressed.

This also applies to the access trigger messages especially considering that multiple reader scenario is to be considered in Rel-20.

Introduce a bit in the access trigger message to indicate whether Rel-19 devices are addressed.

## 2.6	Value range for configuration of access occasions (Issue 4-4)

In RAN2#130, the following agreements have been made:

- Access occasion: a time-frequency resource for device(s) to transmit Msg1 (i.e., the Random ID message) during a CBRA.

- For number of access occasions introduce exponential way, 4 bits, value range FFS.

The value range for the number of access occasions is 20 to 215.

# 3	Conclusion

In this contribution we discussed downlink/paging messages for A-IoT devices. Based on the discussion in the previous sections we made the following observations:

Observation 1	RAN3 agreed to introduce NGAP device ID which can be used to associate AS ID with upcoming command request at reader.

Observation 2	It would be beneficial to be able to differentiate addressing devices with respect to their release.

Based on the discussion in the previous section, we propose the following:

Proposal 1	RAN2 assumes there is one to one mapping between CN correlation ID and transaction ID.

Proposal 2	Transaction ID generation is up to network implementation.

Proposal 3	The size for transaction ID is 6 bits.

Proposal 4	Send an LS to SA2/RAN3 (cc: CT4) to inform those WGs about the assumptions in RAN2 and the size of transaction ID.

Proposal 5	For CFRA, transaction ID is not included in the paging message.

Proposal 6	A-IoT paging message contains the 128-bit security parameter.

Proposal 7	Send an LS to SA3 (cc: SA2) to inform those WGs that it is feasible to include a 128bit random number and additional security parameters in the A-IoT paging message provided that the overall paging message size is less than or equal to the maximum TB size of 1000 bits.

Proposal 8	The paging identifier should be transparent to the reader, i.e., it should not be possible for the reader to, e.g., mask/regenerate the identifier provided by the CN

Proposal 9	One R bit is enough to ensure forward compatibility for paging with multiple identifiers.

Proposal 10	Introduce a bit in the paging message to indicate whether Rel-19 devices are addressed.

Proposal 11	Introduce a bit in the access trigger message to indicate whether Rel-19 devices are addressed.

Proposal 12	The value range for the number of access occasions is 20 to 215.

# 4      References

RP-250796, “New Work Item: Solutions for Ambient IoT (Internet of Things) in NR”, Huawei, 3GPP TSG RAN Meeting #107, Incheon, South Korea, March, 2025.

TR 22.840, V0.4.0, “Study on Ambient power-enabled Internet of Things (Release 19)”

[POST129bis][017][AIoT] 38.391 Running CR, Huawei, RAN2#129bis, Wuhan, China, 7th – 11th April, 2025.

RP-251855, “Moderator's summary for Enhancements for solutions for Ambient IoT (Internet of Things) in NR”, Huawei, 3GPP TSG RAN Meeting #108, Prague, Czech Republic, June, 2025.

R1-2504915, “LS on Ambient IoT R2D control information”, RAN WG1, 3GPP TSG RAN1#121, St Julian’s, Malta, May, 2025.

C4-252460, “Definition of AIoT Device Permanent Identifier”, CT4#129, Bratislava, Slovakia, May, 2025.

TS 23.369,  V1.0.0, “Architecture support for Ambient power-enabled Internet of Things; Stage 2 (Release 19)”