3GPP TSG-RAN WG2 Meeting #131	R2-2506114

Bengaluru, India, August 25th – 29th, 2025

Agenda Item:	8. 19.1

Source:	KDDI Corporation

Title:	Discussion on MobilityStateParameters in RRC_CONNECTED

Document for:	Discussion and Decision

1	Introduction

In high-speed mobility scenarios, such as on trains or in vehicles on a highway, Radio Link Failures (RLF) occur more frequently due to handover (HO) delays. Consequently, RLF leads to service interruptions and significantly degrades the user's Quality of Experience (QoE).

To mitigate this issue, LTE includes a standardized feature in TS36.331 5.5.6.2 that dynamically adjusts timeToTrigger by speed dependant scaling parameters based on the User Equipment's (UE) mobility state. These parameters can be configured for a specific UE via an RRCConnectionReconfiguration message.[1]

However, the current 5G NR specification does not include a comparable mechanism.[2] Therefore, this contribution proposes the introduction of a mobility state-dependent parameter configuration mechanism into NR, which has been proven effective in LTE.

2	Discussion

2.1	Benefit of Mobility State dependent parameter in RRC Connected

In LTE, the standardized feature to change timeToTrigger according to the mobility state is highly effective for improving QoE for mobile users. timeToTrigger-SF is used for this mechanism.

We conducted a trial in a live network environment leveraging this functionality. The trial was conducted on cells covering a railway line with a maximum speed of 100 km/h (total 741 cells). We compared two KPIs before and after applying timeToTrigger-SF. Figure 1 shows RLF Rate comparison. We defined RLF Rate as follows:

RLF Rate =  
(Total number of Too Late HO + Total number of Too Early HO + Total number of HO to Wrong Cell) / Total number of successful RRC Connection Establishments

Definitions of Too Late HO, Too Early HO and HO to Wrong Cell are referenced in TS36.300 section 22.4.2[3] as follows:

Definition of successful RRC Connection Establishments is referenced in TS36.331 section 5.3.3.1 as follows:

Figure 2 shows the Bad RTT Rate measured by our proprietary UE application, defined as RTT(Round-Trip-Time) values exceeding a threshold. Both Figure1 and Figure2 show improvements in the KPIs.

[Figure 1. RLF Rate Comparison Before and After Applying timeToTrigger-SF]

- [Figure 2. Bad RTT Rate Comparison Before and After Applying timeToTrigger-SF]

- Observation 1:  Optimizing mobility parameters, such as timeToTrigger, based on the mobilityState is effective in improving KPIs.

In NR, on the other hand, there is no mechanism for the UE to change parameters depending on the mobility state in RRC_CONNECTED.

2.2	Issues with the delivery of mobilityStateParameters

In LTE, mobilityStateParameters can be delivered via both System Information Block Type 3 (SIB3) and the RRCConnectionReconfiguration message.

In NR, on the other hand, mobilityStateParameters are only broadcast in System Information Block Type 2 (SIB2). This creates a problem in certain scenarios. When a UE in RRC_CONNECTED state hands over from a source cell to a target cell, the UE does not receive mobilityStateParameters. Consequently, the mobilityStateParameters configured in target cell are not applied until the UE transitions to RRC_IDLE.

Consequently, as illustrated in Figure 3, in a scenario where a UE hands in to a cell at low speed and then accelerates, the UE will not apply the parameters suitable for high-speed mobility, leading to handover delays and an increased risk of RLF.

[Figure 3. Parameter Application Timing upon HO in NR]

Observation 2:　The current mechanism for delivering mobilityStateParameters in NR is insufficient, especially for UEs moving between cells in RRC_CONNECTED state.

By 2.1 and 2.2 discussion, we propose the following:

Proposal 1:  RAN2 should discuss whether to introduce the same functionality as LTE in NR, which changes the values of mobility-related parameters (e.g., timeToTrigger) based on the UE's mobility state and delivers mobilityStateParameters in RRCReconfiguration.

Proposal 2:　RAN2 should discuss the release from which to introduce the functionality.

2.3	Issues with mobility state evaluation method

Mobility state is determined based on the number of cell changes within a certain period. However, this method may not be sufficient. An example is shown in Figure 4. The top scenario illustrates a situation with a wide cell radius, while the bottom scenario shows a smaller cell radius. Although the UE moves at the same speed in both scenarios, it is classified as being in a normal mobility state in the top scenario due to fewer cell changes, whereas in the bottom scenario, it is classified as being in a high mobility state because of more frequent cell changes. There may be a better solution for determining the mobility state.

[Figure 4. Comparison between Wide Cell Radius Situation and Small Cell Radius Situation]

Proposal 3:　RAN2 should discuss additional candidate solutions for NR Mobility State evaluation method.

3 Conclusion

Based on the discussion in the previous sections we propose the following:

- Observation 1:  Optimizing mobility parameters, such as timeToTrigger, based on the mobilityState is effective in improving KPIs.

Observation 2:　The current mechanism for delivering mobilityStateParameters in NR is insufficient, especially for UEs moving between cells in RRC_CONNECTED state.

Based on the discussion in the previous sections we propose the following:

Proposal 1:  RAN2 should discuss whether to introduce the same functionality as LTE in NR, which changes the values of mobility-related parameters (e.g., timeToTrigger) based on the UE's mobility state and delivers mobilityStateParameters in RRCReconfiguration.

Proposal 2:　RAN2 should discuss the release from which to introduce the functionality.

Proposal 3:　RAN2 should discuss additional candidate solutions for NR Mobility State evaluation method.

References

3GPP TS 36.331 V18.6.0 (2025-06): "Evolved Universal Terrestrial Radio Access (E-UTRA); Radio Resource Control (RRC); Protocol specification".

3GPP TS 38.331 V18.6.0 (2025-06): "NR; Radio Resource Control (RRC); Protocol specification".

3GPP TS 36.300 V18.5.0 (2025-07): “LTE; Evolved Universal Terrestrial Radio Access (E-UTRA) and Evolved Universal Terrestrial Radio Access Network (E-UTRAN); Overall description; Stage 2”.