3GPP TSG-RAN WG2 Meeting #131	R2-2506115

Bangalore, India, Aug 25-29, 2025

Source:	III

Title:	Discussion on XR DSR enhancements

Agenda item:	8.7.4

Document for:	Discussion

- Introduction

Topics relative to XR-specific capacity issues was raised. In this contribution, “The issues of non-delay critical data in the DSR” are addressed.

- Discussion

To support enhancement for scheduling, the following issues need to be further considered:

- The issues of non-delay critical data in the DSR.

  - The issues of non-delay critical data in the DSR

To manage network delays effectively, DSR can be configured with multiple thresholds. Each threshold corresponds to a specific delay level and triggers different network responses based on the severity of the delay. Non-delay critical data may also be included in the delay severity classification.

In XR applications, managing DSR effectively is essential for optimal performance. When the gNB receives a report of non-delay critical data in a DSR, it relies on the remaining time to determine when this data may transition to delay-critical status, without prioritizing immediate resource allocation for that data. When the gNB identifies non-delay critical data, it does not allocate resources to the UE right away. Instead, it monitors the remaining time to assess if and when the data will change status. Only when the gNB determines that the data has become delay-critical does it begin to consider resource allocation for that UE. The key issues related to DSR containing non-delay critical data include:

- Delay in Resource Allocation: The time taken for the gNB to recognize the transition from non-delay critical to delay-critical can lead to significant delays in resource allocation, potentially impacting performance.

- Lack of Feedback on Data Transmission: Since the UE does not inform the gNB about which specific data has been transmitted, the gNB cannot verify whether the data in the DSR has been sent. This lack of feedback creates ambiguity in the system.

- Inconsistent Data Requirements: The delay in resource allocation and the absence of feedback can result in a misalignment between the resource allocation of the gNB and the actual data transmission status from the UE. This inconsistency can lead to incorrect resource configurations, ultimately degrading service quality.

The process of managing non-delay critical data in DSR highlights significant synchronization challenges between the UE and gNB in XR. The inability of the gNB to ascertain the transmission status of data can lead to resource misallocation and performance degradation. Addressing these synchronization issues is critical for optimizing DSR functionality and improving overall network efficiency.

Observation 1: If the DSR includes non-delay critical data, managing the DSR in XR will lead to resource utilization synchronization challenges between the UE and gNB.
Observation 2: For DSRs that include non-delay critical data in XR, a real-time feedback mechanism between the UE and gNB is required to enhance resource allocation efficiency and synchronization.
Proposal 1: When the DSR includes non-delay critical data, the real-time feedback mechanism should be discussed.

- Conclusion

The related proposals and observations from above discussions are below:

Observation 1: If the DSR includes non-delay critical data, managing the DSR in XR will lead to resource utilization synchronization challenges between the UE and gNB.
Observation 2: For DSRs that include non-delay critical data in XR, a real-time feedback mechanism between the UE and gNB is required to enhance resource allocation efficiency and synchronization.
Proposal 1: When the DSR includes non-delay critical data, the real-time feedback mechanism should be discussed.

References

[1] 	Report of 3GPP TSG RAN WG2 #130