3GPP TSG RAN WG2#131   	R2-2506116

Bangalore, India  Aug 25th – 29th , 2025

Agenda Item:	8.2.2

Source:	III

Title:	Discussion on Ambient-IoT paging

Document for:	Discussion

Introduction

In the previous RAN2#130 meeting, some agreements were achieved in the following:

Agreements < Agreements on parallel service request > - Rel-19 devices are not expected to receive parallel service request for overlapping reader scenario based on network implementation. Capture this in stage 2 specification. - The Rel-19 device always responds to the new service indicated by the received paging message applicable for that device. Capture this in stage 3 specification. - Send LS to RAN3 to notify them of agreements 1 and 2 - Parallel service request for overlapping reader scenario can be addressed in Rel-20 < For CFRA > - For CFRA, as a baseline the fields related to the transaction ID, indication of paging ID present/absent and number of access occasions are absent. FFS on the need for the transaction ID for command case. - For CFRA, the device always responds to paging regardless of transaction ID (if we put a transaction ID) (i.e. as long as it is addressed to the corresponding device). - To ensure forward compatibility for paging with multiple identifiers, introduce at least one R field. FFS if more than one R bit is required. - Rel-19 devices would ignore the content of future release instead of ignoring the whole paging message. - Issue (1-4) For number of access occasions introduce exponential way, 4 bits, value range FFS

Based on RAN2 progress, this contribution discusses different service requests from different readers, transaction ID and device ID.

Discussion

Different service requests from different readers

In RAN2#130 discussion, option B was selected. This means that a Rel-19 Ambient IoT device always respond to the new service indicated by received paging message, even if another procedure is still ongoing. Although this approach avoids the deadlock concerns associated with Option A, it introduces potential issues in multi-reader overlapping coverage scenarios.

- Decoding failures due to interference – When a device is ongoing procedure triggered by one reader, the R2D transmission of this reader may overlap in time with R2D transmission of an uncoordinated reader. The device will be unable to correctly receive or decode the R2D message.

- Service is interrupted due to different group paging – A device in overlapping area may be paged by group A in one reader and may be paged by group B in another reader. When the device is in ongoing procedure since the reader paging devices in group A, the device may receive another paging message from another reader (paging group B). Due to option B is adopted, the device will always respond to the new service request, even the device in current reader is unexpected to be interrupted by new service request.

Observation 1: Separate group paging from different readers may cause unexpected service interrupting due to option B. 

To overcome above issues, coordination of R2D message transmissions of multi-readers is necessary. 
In R2-2503480, the interleaving procedure for overlapping multiple readers was proposed. The mechanism to configure time window of different slot for different readers may solve these issues.

Transaction ID

In TR 38.769[1] describe, the reader transmits multiple (subsequent) A-IoT paging messages that are associated with the same service request from the CN. The duplicated response should be avoided for the same service request. In RAN2#128 discussion, session ID/transaction ID between CN and reader (i.e., based on the candidate solutions of SA2) can be used the ID to avoid duplicated response for same service request.

In previous RAN2#128 meetings, as following:

Agreements on paging - An ID in paging message is beneficial to avoid duplicated response for same service request. FFS whether ID is generated by the reader or CN. FFS on the ID size

In RAN2#129 agreement, the transaction ID can be generated by reader based on CN

Agreements - The “transaction ID” can be generated by reader based on CN correlation ID. FFS how reader will generate “transaction ID”. FFS the size of transaction ID - 1 bit solution is excluded. FFS the size. Aim to have a reasonable size. RAN2 acknowledges that multi-reader scenario may exist but we will not specify something specific for this purpose. We can rely on transaction ID and implementation to handle it.

When the CN correlation ID was clarified by SA2, the reader can generate a transaction ID based on the CN correlation ID.
We provides three options for further discussion.

Option 1: Use X bits (e.g. LSB) of the CN Correlation ID to generate the transaction ID (but make sure X is large enough to avoid collisions),

Option 2: A mapping mechanism based on CN correlation IDs,

Option 3: The reader selects a transaction ID that has not been used recently to avoid ID duplication.

Observation 2: When the CN correlation ID was clarified by SA2, the reader can generate a transaction ID based on the CN correlation ID. We provides three options for further discussion.

Option 1: Use X bits (e.g. LSB),
Option 2: A mapping mechanism,

Option 3: The reader selects a transaction ID that has not been used recently to avoid ID duplication.

Ambient IoT Device ID

In the RAN2#129, agreement about paging ID:

Agreements on paging ID - The “one identifier” in the paging message includes both the case of “one single device identifier” and “one group identifier”/”filtering criteria”, while the exact format of latter is supposed to be designed by SA2.

The Ambient IoT Device ID was defined by TR 23.700-13 [2], it includes multiple identifiers.

Based on the above consideration, following components are considered necessary to compose the Ambient IoT Device ID, which is defined by 3GPP: - Home Network Identifier: an identifier used to identify the home MNO; - Owner Identifier: an identifier used to identify a 3rd party who sends service requests to trigger 5GC to perform Ambient IoT service operation; NOTE 1: The Owner Identifier is allocated by the home MNO corresponding to the Home Network Identifier. Editor’s Note: The use of the Owner Identifier is FFS and may depend on the solution for Inventory, etc. - Instance Identifier: an identifier used to identify a specific Ambient IoT device owned by the 3rd party. NOTE 2: The Instance Identifier is allocated by the home MNO which may coordinate with the 3rd party. Figure 6.2.1-2: Structure of Ambient IoT Device ID and optional 3rd Party-defined Identifier

In TS 23.003[3], the SUPI is a globally unique 5G subscription permanent identifier allocated to each subscriber in the 5G System. It is feasible to choose IMSI as SUPI type for the Ambient-IoT Device ID.

- 2.2A Subscription Permanent Identifier (SUPI) The SUPI is a globally unique 5G Subscription Permanent Identifier allocated to each subscriber in the 5G System. It is defined in clause 5.9.2 of 3GPP TS 23.501 [119]. The SUPI is defined as: - a SUPI type: in this release of the specification, it may indicate an IMSI, a Network Specific Identifier (NSI), a Global Line Identifier (GLI) or a Global Cable Identifier (GCI); and - dependent on the value of the SUPI type: - an IMSI as defined in clause 2.1; - a Network Specific Identifier (NSI), taking the form of a Network Access Identifier (NAI) as defined in clause 28.7.2; - a Global Cable Identifier (GCI) taking the form of a NAI as defined in clause 28.15.2; - a Global Line Identifier (GLI) taking the form of an NAI as defined in clause 28.16.2. NOTE: Depending on the protocol used to convey the SUPI, the SUPI type can take different formats.

Base on above, the single device ID in paging message can be based on Ambient IoT Device ID /IMSI for uniqueness of device ID in single/multiple readers. This should be further discussed in RAN2.

Observation 3: The single device ID in paging message can be based on Ambient IoT Device ID /IMSI for uniqueness in single/multiple readers.

- Conclusion

We have the following observation:
Observation 1: Separate group paging from different readers may cause unexpected service interrupting due to option B.

Observation 2: When the CN correlation ID was clarified by SA2, the reader can generate a transaction ID based on the CN correlation ID. We provides three options for further discussion.

Option 1: Use X bits (e.g. LSB),
Option 2: A mapping mechanism,

Option 3: The reader selects a transaction ID that has not been used recently to avoid ID duplication.

Observation 3: The single device ID in paging message can be based on IMSI / Instance Identifier for uniqueness in single/multiple readers.

- Reference:

[1] TR 38.769, “Study on solutions for ambient IoT (Internet of Things)”.

[2] TR 23.700-13,” Study on Architecture support of Ambient power-enabled Internet of Things”.

[3] TS 23.003,” Numbering, addressing and identification;”.