3GPP TSG RAN WG2 #131	R2-2506123

Bangalore, India, August 25th – 29th, 2025

Agenda Item:	8.2.4

Source:	Fraunhofer HHI, Fraunhofer IIS

Title:	Remaining issues on A-IoT data transmission

Document for:	Discussion

# Introduction

In the last RAN#106 plenary, a new work item on solutions for Ambient-IoT (Internet of Things) in NR was approved [1]. Ambient-IoT is a new 3GPP IoT technology that relies on ultra-low complexity devices with ultra-low power consumption for very low-end IoT applications. In the last RAN2#130 meeting, the following agreements were made regarding A-IoT data transmission and other general aspects agenda-item[2]:

RAN2#130

Agreements for data transmission: - R2D message scheduling non-first segment (re)transmission does not include upper layer command. - For the first segment and unsegmented packet (re)transmission, the “offset” indicator in R2D is not present. - This implies that the R2D message will either have command or offset (but not both). FFS whether we define two message types or one message type with optional fields. - The device is expected to send a MAC response to the reader in the D2R occasion. The MAC response contains the NAS message if available at the D2R occasion. If there is no NAS message available to transmit at the D2R occasion then the response contains MAC with 0 SDU and padding as needed. - Send LS to CT1 to inform the agreement 1 to CT1 and explain that we have an issue with delayed NAS write success response. RAN2 would prefer that this is handled by CT1 (and give the example of sending NAS response upon successful reception of write command). Ask if this can be handled by CT1. - A mandatory length field directly indicates the length of D2R data MAC SDU to support varying lengths of D2R data. The size of length field is 7-bit in bytes. - The offset indication for transmission/retransmission of the segments after the first segment of a D2R message is 7-bit length in bytes. Segmented SDUs are also byte aligned. - FFS D2R message type. Current running CR will capture no message type, but we can revisit this next meeting and also consider if any other bits are needed for the MAC header - The length field inside MAC for SDU is not needed for R2D messages, assuming R2D MAC padding is not needed. FFS can come back if padding is needed depending on granularity of TBS (only if needed) - For CBRA, to avoid AS ID being occupied for unnecessary time and to keep alignment between reader and device on AS ID release, device can release AS ID upon receiving paging message with different transaction ID, no matter the paging message is for it or not. FFS for CFRA - FFS for need for release message

In alignment with the agreements made in the last RAN2 meeting, our contribution further elaborates and discusses potential areas for improvement.

# Discussion

## AS ID Release

In the last meeting, two potential approaches were discussed for AS ID release in the context of Ambient IoT (A-IoT):

- release triggered implicitly by the reception of a paging message, and

- release triggered by an explicit signaling indication.

During the post-meeting email discussion on remaining A-IoT MAC open issues [3], many views were leaning towards the paging-based approach for AS ID release, primarily due to its lightweight nature and reuse of existing signaling. However, if the interval between two consecutive paging messages is too long, the reader and the device would need to keep the AS context for an extended period. This prolonged retention can lead to inefficient memory usage in devices and unnecessary reservation of identifiers in the network. Besides, in scenarios where paging messages may be lost, delayed, or misinterpreted; the release process becomes ambiguous.

In this context, our perspective is to adopt an explicit indication mechanism for AS ID release, as this allows the system to maintain clarity over session boundaries. Such signaling minimizes the risk of asynchronous state transitions between device and reader, which may be aggravated in case of considering mobility, where devices may leave the coverage area of their serving reader, causing the AS ID to remain active in the previous reader while the device enters a new reader’s coverage. Therefore, resulting in a mismatch in context.

Observation 1: Paging-based AS ID release, while lightweight, may be unreliable leading to inconsistent AS ID state across the network.

One practical implementation would be the introduction of a dedicated signal to explicitly mark the end of the current round. Such signaling can be broadcasted to all devices within the coverage area, ensuring that both active and passive listeners receive a clear and synchronized instruction to release the corresponding AS IDs. This approach can help align state transitions across multiple readers without relying solely on unicast or transaction-specific messages. Alternatively, a given AS ID could be associated with a transaction-specific lifetime, after which it is automatically released if no further activity is detected. This timer could act as a mechanism against stale AS IDs in cases where explicit release signaling is missed, ensuring network-wide consistency.

Proposal 1. Introduce an explicit AS ID release mechanism applicable to both CBRA and CFRA.

## D2R message content

### D2R message type

Following the email discussion [3], the issue of whether to support a D2R message type was discussed. From our perspective, while the introduction of such a message type may not be critical for Rel-19, it could become highly beneficial in Rel-20 and for future extendibility, particularly to support device-originated autonomous (DO-A) traffic.

If no dedicated D2R message type is introduced, the reader would need to infer which message type is expected in the D2R direction based solely on context. This approach is highly prone to errors, as the same message structure would need to serve both network-initiated and device-initiated cases. Additionally, we cannot exclude the case where different types of devices, e.g., active and passive, may be deployed within the same scenario, as the likelihood of such coexistence is high.

Proposal 2. Add a D2R type field to clearly identify autonomous device transmissions, supporting future DO-A use cases and forward compatibility beyond Rel-19.

### “More data indication” values for 0 SDU

Regarding the use of the MDI indication to signal additional data, from our perspective a 0-size SDU is sufficient to inform the reader that the device has not yet prepared its response, eliminating the need to set the MDI to any value. In this approach, the MAC layer relies on its internal knowledge of pending traffic to schedule uplink resources appropriately.

While letting the reader know that the device has data but not yet in the buffer may be beneficial, this information is inherently uncertain. The reader does not know how much time the device will need to prepare the data, and allocating resources prematurely could lead to wasted radio resources or inefficient scheduling. Therefore, reusing an explicit MDI bit under such conditions could be misleading and may not improve overall system performance.

Proposal 3. The MDI field is used only for segmentation.

# Conclusion

In this contribution we discussed aspects on the A-IoT data transmission. Based on the discussion in the previous sections we made the following observations and proposals:

Observation 1: Paging-based AS ID release, while lightweight, may be unreliable leading to inconsistent AS ID state across the network.

Proposal 1. Introduce an explicit AS ID release mechanism applicable to both CBRA and CFRA.

Proposal 2. Add a D2R type field to clearly identify autonomous device transmissions, supporting future DO-A use cases and forward compatibility beyond Rel-19.

Proposal 3. The MDI field is used only for segmentation.

# References

- 3GPP RP-243326, “New Work Item: Solutions for Ambient IoT (Internet of Things) in NR”, 3GPP TSG-RAN #106, Madrid, Spain, Dec 9th-12th, 2024.

- 3GPP “Draft Report of 3GPP TSG RAN WG2 meeting #130”, 3GPP TSG RAN WG2 #130, St. Julian’s, Malta, May 19th-23rd, 2025

- [POST130][027][AIoT] MAC Running CR (Huawei).