3GPP TSG-RAN2 Meeting #131	R2-2506126

Bangalore, India  Aug 25th – 29th , 2025

Source: 			ZTE Corporation

Title: 	Discussion on functionality management for RRM measurement prediction

Agenda item:	8.3.2.1

Document for: 	Discussion and Decision

- Introduction

In the last meeting, RAN2 has discussed functionality management for RRM measurement prediction, and reached some agreements. In this contribution, we’d like to share our view on the applicability procedure, inference configuration/reporting and performance monitoring for UE sided model for RRM measurement prediction.

- Discussion

2.1 Applicability configuration and reporting

In the last meeting, we have agreed to consider option A and Option B like scheme, as follows. In this sub-section, we’d like to discuss the applicability configuration and reporting for option A and option B, separately.

RAN2 130: - Use the beam management agreements as the baseline, including the following aspects (details can be updated after further progress is made in AI/ML PHY BM use case) • Consider Option A and Option B like scheme (if/when AI/ML PHY makes further progress) • Upon receiving a full inference configuration, the UE sends the initial applicability report in RRCReconfigurationComplete. UAI can be sent to update applicability. • Upon receiving one or more full inference configuration(s) via RRCReconfiguration message, UE shall maintain all the full inference configuration(s) no matter the full inference configuration is applicable or inapplicable until the network releases it explicitly. • Support the explicit reporting of applicability/inapplicability in the initial report and subsequent reporting when the applicability changes. • Together with inapplicability reporting, UE further indicates a simple cause value of inapplicability (FFS pending AI/ML PHY progress).

For Option A-_full inference configuration

Similar to RRM measurement prediction, we think the existing RRM framework can be reused to provide full inference configuration to the UE for measurement event prediction. In the current spec, the MeasObjectNR is used to indicate the frequency that the UE should perform measurement on. Similarly, we think the frequency indicated by MeasObjectNR is the frequency that the UE should perform prediction on.

Proposal 1	 For Option A, the existing RRM framework is reused to provide the full inference configuration to the UE. The frequency indicated by MeasObjectNR is the frequency that UE should perform prediction on.

Upon receiving a list of full inference configuration, the UE can check whether the corresponding inference configuration is applicable or not per measId, and sends the applicability report to the network via RRCReconfigurationComplete or UAI. And the applicability report shall include one or more measId that are applicable and one or more measId that are not applicable. If one measId is not applicable, applicability report shall also include the corresponding cause value.

Proposal 2	Upon receiving a list of full inference configuration, the UE determines the applicability per measId and sends applicability report to the network via RRCReconfigurationComplete or UAI.

Proposal 3	For Option A, the applicability report includes the following information:

- One or more measIDs that are applicable;

- One or more measIDs that are inapplicable and corresponding cause value;

And, in the RAN2 130, RAN2 already agreed some inference parameters for RRM measurement prediction, as follows:

- The following potential “inference parameters” can be considered for inference/measurement configuration, which provides the necessary information for UE to examine the applicability, it may contain • PW length (for temporal case A) • Skipping pattern (e.g. SSB config that indicates SSBs transmitted) (optional). MRRT (for temporal case B ) not discussed in study item, but if RAN4 says otherwise can be considered in WI • Measured and predicted beam pattern (optional). MRRS (for spatial domain prediction) not discussed in study item, but if RAN4 says otherwise can be considered in WI • Measured and predicted frequency (For inter-freq domain prediction)

While, it is still undetermined how to provide these inference configurations to the UE.

Observation 2:  In RAN2#130, RAN2 already agreed some inference parameters for RRM prediction, however, it is undetermined how to provide those configurations to UE.

In the current RRC specification, we have the following restriction:

#clause 5.5.2.1 of TS 38.331 The network applies the procedure as follows: //skip un-related part - to ensure that, in the measConfig associated with a CG: - for all SSB based measurements there is at most one measurement object with the same ssbFrequency;

With the restriction above, if inference parameters are put into MeasObjectNR, in order to applicability check, the network may provide multiple different inference parameters for a given frequency. Taking temporal domain case A as an example, for one frequency, each PW needs one MeasObjectNR, this breaks the rule that for a given SSB frequency, at most one MO can be configured for SSB based measurements. In the other hand, for performance monitoring, the network needs to configure the UE to perform actual measurement, in this case, one MeasObjectNR without any inference related parameters, which also break the rules.

Observation 3: In current RRC spec, there is restriction that for a given SSB frequency, at most one MO can be configured for SSB-based measurement. If inference parameters are put in MeasObject, we need to break the rule in order to provide different inference parameters for a given frequency.

Based on the analysis above, we think the agreed inference parameters above (e.g. PW length, skipping pattern, measured frequency) shall be defined in ReportConfigNR.

Proposal 4	For Option A, the inference parameters (e.g. PW length, skipping pattern, measured frequency) are defined in ReportConfigNR.

In the existing RRM measurement configuration, the UE always derive L3 cell results and included L3 cell results in the measurement report. While, for L3 beam results, the network can indicate the UE whether to derive L3 beam results and whether to include beam results in measurement report via reportQuantityRS-Indexes and maxNrofRS-IndexesToReport.

Observation 4: In reportConfigNR, reportQuantityRS-Indexes and maxNrofRS-IndexesToReport are used to indicate the UE whether to derive L3 beam results and whether to include beam results in measurement report.

In our understanding, the same inference configuration can be used to indicate L3 cell level prediction or L3 beam level prediction or both. For example, the network needs to provide prediction window length for both L3 cell level prediction and L3 beam level prediction. And, the existing IE reportQuantityRS-Indexes and maxNrofRS-IndexesToReport can be used to indicate whether the UE shall perform L3 beam level prediction in addition to L3 cell level prediction, following the same way as existing measurement configuration.

Proposal 5	The same inference configuration can be used to indicate L3 cell level prediction or L3 beam level prediction or both, e.g. the existing reportQuantityRS-Indexes and maxNrofRS-IndexesToReport IEs can be reused to indicate whether the UE shall perform L3 beam level prediction in addition to L3 cell level prediction.

For Option B-_partial inference configuration

For option B in AI-phy, the network can provide multiple sets of inference related parameters for applicability check via OtherConfig. Upon receiving multiple sets of inference related parameters for applicability check, the UE decides the applicability per set and reports the applicability to the network via RRCReconfigurationComplete and UAI. And in order to identity an inference parameter set, each set of inference parameters is associated with one set Id. Similarly, AI-mob can follow the same way.

Proposal 6	For Option B, the network provides multiple sets of inference parameters to the UE via otherConfig. Each set of inference parameters is associated with one set ID.

Proposal 7	Upon receiving the multiple sets of inference parameters, the UE determines the applicability per set, and sends applicability report to the network via RRCReconfigurationComplete or UAI.

Proposal 8	For Option B, the applicability report includes the following information:

- One or more set IDs that are applicable;

- One or more set IDs that are inapplicable and corresponding cause value;

Similar to option A, a set of inference parameters for option B can include the inference parameters above, e.g. the PW length, skipping pattern and measured frequency. Besides, some measurement configuration may also impact the applicability, for example, some AI models may only be able to work in the certain frequency(s), in this case, the network needs to provide predicted frequency to the UE; the AI model with temporal domain needs a certain measurement interval, the UE needs to check whether the UE can obtain the corresponding measurement interval with indicated SMTC; and if sub use 2 (cell->cell) and sub use 3 (beam->cell) is used, the beam consolidation configurations can also impact applicability check.

Observation 5: Similar to Option A, a set of inference parameters for Option B can include e.g. PW length, skipping pattern, measured frequency. In addition, other configuration may also impact the applicability check, such as: predicted frequency、SMTC、beam consolidation configurations (e.g. absThreshSS-BlocksConsolidation, nrofSSB-BlocksToAverage).

Since it is agreed to use existing RRM measurement configuration for inference configuration and the measObjectNR already supports to provide the configurations above, we think MeasObjectNR can be included in a set of inference parameters for option B. In this way, multiple set of inference parameters for the same prediction target can share one measurement configuration, which is simpler and reduces signaling overhead.

Observation 6: For simplicity and signalling reduction, the each set of inference parameters can link to one measObjectID.

Based on the analysis above, we think each set of inference parameters include one set Id, one measObjectId and the prediction related parameters, e.g. PW length, skipping pattern and measured frequency.

Proposal 9	For Option B, each set of inference parameters include:

- Set ID;

- MeasObjectID;

- Prediction related parameters, e.g. PW length, skipping pattern, measured frequency.

2.2	When to perform model inference

In the previous meeting, we have agreed that the UE applies the inference configuration for the applicable RRM measurement prediction upon receiving configuration via RRC. But it is still FFS that when to perform inference.

For AI-mobility, we have two study goals:

- Study goal 1: To reduce measurement efforts;

- Study goal 2: To improve the handover performance.

For the 1st study goal, the UE shall perform inference as early as possible, in order to obtain the gain that measurement reduction and measurement gap reduction as early as possible. For temporal domain case B, upon the receiving applicability reporting, the network can stop transmitting the SSB at the time instance for prediction, in order to save network energy and reduce UE measurement efforts. For frequency domain prediction, upon the receiving applicability reporting, the gap configuration can be reduced or even avoided. If the UE performs inference earlier, the throughput gain can be obtained earlier. Therefore, we think for temporal domain case B and frequency domain prediction, the UE performs model inference immediately after receiving the full inference configuration.

Observation 7: For the 1st study goal, earlier UE inference enables earlier realization of measurement reduction and throughput improvements.

Proposal 10 For temporal domain Case B and frequency domain prediction, the UE performs model inference immediately after receiving the full inference configuration.

For temporal domain Case A, the typical use case is to predict when the handover will happen. If the UE performs inference when the UE is near cell edge (i.e. when the channel quality of the serving cell is good), the prediction results can not be used by the network to make a handover decision, and increase GPU/CPU consumption. So, in order to avoid unnecessary prediction, it is fine to start the inference when the UE is near cell edge.

Observation 8: For temporal domain Case A, the typical use case is to predict when the handover will happen. So, to reduce the GPU/CPU consumption, it is fine to start the inference when the UE is near cell edge.

In order to make the UE performs inference when the UE is near cell edge, the following solutions can be considered:

Solution 1: The network sends inference configuration when the quality of the serving cell is bad, and the UE perform inference immediately after receiving inference configurations.

Solution 2-1: The network sends inference configuration in advance, and the UE performs inference when a pre-configured condition is met (e.g. s-Measure or A3 event).

Solution 2-2: The network sends inference configuration in advance, and the UE determines when to perform inference.

Option 1 is most simple and straightforward. For both option 2-1 and option 2-2, the network sends inference configuration in advance, and the UE will not perform inference immediately. The difference is that for option 2-1, the network can control when to perform inference, and for option 2-2, it is up to UE implementation to determine when to perform inference. In our understanding, with the control of when to perform inference, the network can have more flexibility on how to use the prediction results. For example, if the UE performs inference when the entering condition is met based on the actual measurement, the network may have more trust on prediction results. So, we think RAN2 can discuss and select among solution 1 or solution 2-1.

Observation 9: For temporal domain Case A, it is more flexible if the network can control when the UE starts model inference.

Proposal 11: For temporal domain prediction case A, RAN2 to discuss and select one of the following solutions:

•	Solution 1: Model inference is performed upon the reception of the full inference configuration;

•	Solution 2: Model inference is performed when a pre-configured condition is met (e.g. s-Measure or A3 event).

2.3 Inference report

For pure RRM measurement prediction, i.e, the prediction results are not used to evaluate whether a measurement event occurs at a (future) time instance, the network can set the reportType to periodical in ReportConfigNR.

Observation 10: For pure RRM measurement prediction, the network can set the reportType to periodical in ReportConfigNR.

In the existing RRM measurement framework, the network can control when and what to report via reportConfigNR, e.g. reporting periodicity (reportInterval), the number of measurement report (reportAmount), the max number of non-serving cells to include in the measurement report (maxReportCells), the max number of beam results per cell to include in the measurement report (reportQuantityRS-indexes) and so on. Upon receiving these configurations, the UE reports the corresponding measurement results to the network via MeasurementReport message. AI-mobility can follow the same way, i.e. the UE follows the configuration in reportConfigNR for inference results reporting, and report inference results via MeasurementReport message.

Proposal 12	For AI-Mobility, to reuse MeasurementReport message for inference results reporting. and the UE follows the configuration in reportConfigNR for inference results reporting (e.g. reportInterval, reportAmount, maxReportCells, reportQuantityRS-Indexes, etc).

For temporal domain prediction case B, depending on the time instance that measurement report is initiated, the type of available measurement results (e.g. historical measurement results from prediction, measurement results of future time instance) are different. For example, if the measurement report is initiated at t1, the latest historical measurement result is from prediction, and there is no future measurement result; if the measurement report is initiated at t3, the latest historical measurement is from actual measurement and the future measurement results are available.

Figure: An example for temporal domain case B

Regarding the content of the measurement report, the simplest way is to always report the latest historical measurement results, regardless that the latest historical measurement result is from prediction or actual measurement.

Proposal 13	For temporal domain Case B, the inference report includes the latest measurement results (regardless of actual results or predicted results).

The motivation of the temporal domain prediction case A is to improve handover performance, it is helpful if the network can know the measurement results of future time instances, which makes it possible for the network to take measures in advance for the upcoming channel quality change (i.e. in proactive way). Specifically, we can report the prediction results in the prediction window, similar to AI-phy beam management. Besides, we think the latest actual measurement result is also helpful, since: (1) as a network vender, before deciding to move the UE to another cell, it is helpful that the network can know the actual measurement results. (2) considering the baseline prediction quantity is RSRP, the UE may be unable to predict future RSRQ/SINR value. With latest historical measurement result in the measurement report, the network can know the channel interference information.

Proposal 14	For temporal domain Case A, the inference report includes the latest actual measurement results and multiple predicted results within the PW.

For frequency domain prediction, the output is measurement results at the frequency to be predicted for every time instance. The UE reports the latest measurement results to the network, no enhancement is needed in the existing measurement report.

Proposal 15	For frequency domain prediction, the inference report includes the latest measurement results of the predicted cell. (No change to the existing measurement report)

Besides, we think there is no need to differentiate actual measurement results and predicted results, since both of them are latest results. For frequency domain prediction, since the UE does not perform actual measurement on the predicted frequency, the network can know the received measurement results are from prediction. For temporal domain case A, since the UE reports both prediction results and latest actual results, the network shall know which result is from actual measurement and which results are from prediction.

Observation 11: For temporal domain Case B, there is no need to differentiate actual measurement results and predicted measurement results; But for temporal domain Case A and frequency domain prediction, the network should be aware whether the reported results are actual results or predicted results.

2.4 Performance monitoring

In the last meeting, we have agreed that RSRP differences can be used as the performance metric for monitoring, as follows:

- For RRM measurement prediction - RSRP differences can be used as the performance metric for monitoring.

It is still unclear how to calculate it. In this part, we’d like to share our view on it.

In the AI-phy beam management BM-Case 2, RAN1 has reached the following agreements:

For BM-Case 2, at least support to report one beam prediction accuracy for one configured time instance, configured by one CSI-ReportConfig for monitoring, - only one resource set is configured in the CSI-ReportConfig - the one configured time instance (i.e. f-th time instance of the time instance in one inference report) for metric calculation is configured in the CSI-ReportConfig for monitoring - the performance metric of the f-th time instance is calculated based on N latest transmission occasion(s) of monitoring resource, no later than CSI reference resource corresponding to the CSI report for monitoring - The measurement result of nth (n = 1,..,N) latest transmission occasion of the CSI-RS/SSB resources for monitoring is linked with the f-th time instance for prediction, where the f-th time instance has the minimal slot offset to the nth transmission occasion of the CSI-RS/SSB resources for monitoring. - Wherein, the corresponding inference reports, and the transmission occasions of the CSI-RS/SSB resources for monitoring, are no later than the CSI reference resource corresponding to the CSI report for monitoring - Predefines a threshold X =64, where the minimal slot offset k is no larger than X; otherwise, the transmission occasion for monitoring has no linked time instance.

In the AI-phy beam management Case 2, the different prediction point within the prediction window are monitored separately. The same principle can also apply on the AI-mob.

Observation 12: In the AI-phy BM-Case 2, the monitoring metric are calculated for different predicted points within PW, separately.

And, as observed in the simulation, the prediction distance is shorter, and the prediction performance is better. If L3 RSRP difference is calculated by averaging prediction errors of all the predicted points within PW, the bad prediction performance of latter part of the PW may be averaged by the front part. The information that the network can obtain from the monitoring report is limited.

Observation 13: For temporal domain prediction, averaging the prediction errors of all results within the PW to calculate the 'RSRP difference' may lead to a loss of information, because the bad prediction performance of latter part are averaged.

Proposal 16: For temporal domain case A and case B, the monitoring metric is the RSRP difference for each prediction result within PW.

In addition, in the existing RRM measurement framework, for cell results, the UE reports the cell results of serving cell and the best neighbor cells up to maxReportCells to the network; for beam results, the UE reports best beam results up to maxNrofRS-IndexesToReport per reported cell to the network. For performance monitoring, even if the prediction results on the cells with low quality is large, considering the UE will not report them to the network, they will not impact system performance. So, in our understanding, the UE does not need to monitor the prediction performance of all the cells on one frequency, which increases unnecessary workload. The UE can monitor the prediction performance of serving cell and top-K neighbor cells. Similarly, for beam level prediction, the UE can monitor the prediction performance of top-M beam for each monitored cell.

Observation 14: The UE reports cell results and beam results with top quality to the network, the prediction error on the cells/beams not included in the measurement report will not impact system performance.

Proposal 17: For monitoring on cell level prediction, to monitor the prediction performance of serving cell and top-K neighbor cell.

Proposal 18: For monitoring on beam level prediction, to monitor the prediction performance of top-M beam for each monitored cell.

- Conclusion and proposals

Observation 1: For applicability reporting, RAN2 agreed that Option A and Option B like schemes agreed in AI-Phy will be considered for AI-Mobility.

Observation 2:  In RAN2#130, RAN2 already agreed some inference parameters for RRM prediction, however, it is undetermined how to provide those configurations to UE.

Observation 3: In current RRC spec, there is restriction that for a given SSB frequency, at most one MO can be configured for SSB-based measurement. If inference parameters are put in MeasObject, we need to break the rule in order to provide different inference parameters for a given frequency.

Observation 4: In reportConfigNR, reportQuantityRS-Indexes and maxNrofRS-IndexesToReport are used to indicate the UE whether to derive L3 beam results and whether to include beam results in measurement report.

Observation 5: Similar to Option A, a set of inference parameters for Option B can include e.g. PW length, skipping pattern, measured frequency. In addition, other configuration may also impact the applicability check, such as: predicted frequency、SMTC、beam consolidation configurations (e.g. absThreshSS-BlocksConsolidation, nrofSSB-BlocksToAverage).

Observation 6: For simplicity and signalling reduction, the each set of inference parameters can link to one measObjectID.

Observation 7: For the 1st study goal, earlier UE inference enables earlier realization of measurement reduction and throughput improvements.

Observation 8: For temporal domain Case A, the typical use case is to predict when the handover will happen. So, to reduce the GPU/CPU consumption, it is fine to start the inference when the UE is near cell edge.

Observation 9: For temporal domain Case A, it is more flexible if the network can control when the UE starts model inference.

Observation 10: For pure RRM measurement prediction, the network can set the reportType to periodical in ReportConfigNR.

Observation 11: For temporal domain Case B, there is no need to differentiate actual measurement results and predicted measurement results; But for temporal domain Case A and frequency domain prediction, the network should be aware whether the reported results are actual results or predicted results.

Observation 12: In the AI-phy BM-Case 2, the monitoring metric are calculated for different predicted points within PW, separately.

Observation 13: For temporal domain prediction, averaging the prediction errors of all results within the PW to calculate the 'RSRP difference' may lead to a loss of information, because the bad prediction performance of latter part are averaged.

Observation 14: The UE reports cell results and beam results with top quality to the network, the prediction error on the cells/beams not included in the measurement report will not impact system performance.

For applicability reporting:

Proposal 1	 For Option A, the existing RRM framework is reused to provide the full inference configuration to the UE. The frequency indicated by MeasObjectNR is the frequency that UE should perform prediction on.

Proposal 2	Upon receiving a list of full inference configuration, the UE determines the applicability per measId and sends applicability report to the network via RRCReconfigurationComplete or UAI.

Proposal 3	For Option A, the applicability report includes the following information:

- One or more measIDs that are applicable;

- One or more measIDs that are inapplicable and corresponding cause value;

Proposal 4	For Option A, the inference parameters (e.g. PW length, skipping pattern, measured frequency) are defined in ReportConfigNR.

Proposal 5	The same inference configuration can be used to indicate L3 cell level prediction or L3 beam level prediction or both, e.g. the existing reportQuantityRS-Indexes and maxNrofRS-IndexesToReport IEs can be reused to indicate whether the UE shall perform L3 beam level prediction in addition to L3 cell level prediction.

Proposal 6	For Option B, the network provides multiple sets of inference parameters to the UE via otherConfig. Each set of inference parameters is associated with one set ID.

Proposal 7	Upon receiving the multiple sets of inference parameters, the UE determines the applicability per set, and sends applicability report to the network via RRCReconfigurationComplete or UAI.

Proposal 8	For Option B, the applicability report includes the following information:

- One or more set IDs that are applicable;

- One or more set IDs that are inapplicable and corresponding cause value;

Proposal 9	For Option B, each set of inference parameters include:

- Set ID;

- MeasObjectID;

- Prediction related parameters, e.g. PW length, skipping pattern, measured frequency.

For when to perform model inference

Proposal 10 For temporal domain Case B and frequency domain prediction, the UE performs model inference immediately after receiving the full inference configuration.

Proposal 11: For temporal domain prediction case A, RAN2 to discuss and select one of the following solutions:

•	Solution 1: Model inference is performed upon the reception of the full inference configuration;

•	Solution 2: Model inference is performed when a pre-configured condition is met (e.g. s-Measure or A3 event).

For inference report

Proposal 12	For AI-Mobility, to reuse MeasurementReport message for inference results reporting. and the UE follows the configuration in reportConfigNR for inference results reporting (e.g. reportInterval, reportAmount, maxReportCells, reportQuantityRS-Indexes, etc).

Proposal 13	For temporal domain Case B, the inference report includes the latest measurement results (regardless of actual results or predicted results).

Proposal 14	For temporal domain Case A, the inference report includes the latest actual measurement results and multiple predicted results within the PW.

Proposal 15	For frequency domain prediction, the inference report includes the latest measurement results of the predicted cell. (No change to the existing measurement report)

For performance monitoring

Proposal 16: For temporal domain case A and case B, the monitoring metric is the RSRP difference for each prediction result within PW.

Proposal 17: For monitoring on cell level prediction, to monitor the prediction performance of serving cell and top-K neighbor cell.

Proposal 18: For monitoring on beam level prediction, to monitor the prediction performance of top-M beam for each monitored cell.

- Reference

- Report of RAN2 129 meeting.