3GPP TSG-RAN2 Meeting #131	R2-2506128

Bangalore, India  Aug 25th – 29th , 2025

Source: 			ZTE Corporation

Title: 	Discussion on data collection for UE side model

Agenda item:	8.3.2.3

Document for: 	Discussion and Decision

- Introduction

In the RAN2 129bis meeting, we have agreed that the data collection for model training for beam management can be the baseline for AI-mobility. In this contribution, we’d like to share our view on the data collection procedure and configuration for RRM measurement prediction and measurement event prediction.

- Discussion

2.1 General procedure for data collection

In the RAN2 129bis meeting, we have agreed that the general LTM framework (including data collection) for beam management can be considered as baseline for AI mobility, which includes:

Figure 1: General procedure for AI mobility.

Step 1: The network provides the UE a list of candidate UE data collection configuration via otherConfig.

Step 2: The UE can response one or more of UE’s preferred configuration from the list via UAI;

Step 3: After receiving UE’s preferred data collection configuration, the UE configure final data collection configuration.

Step 4: Upon receiving data collection configuration, the UE performs measurement and logs the data.

Step 5: The UE can send/stop indication of data collection to the network via UAI.

We think the same procedures are also applicable for AI-mobility.

Proposal 1 	For data collection in UE side model, the network can provide UE a list of candidate UE data collection configuration via otherConfig, and UE can response to the network with one or more of UE’s preferred configuration from the list via UAI, same as in AI-Phy.

Proposal 2 	After receiving UE’s preferred data collection configuration, the network configures final data collection configuration by using the existing RRM framework.

For step 5, although AI-phy has agreed that the UE can request start or stop data collection via UAI, it is still under discussion that such indication is a per UE indication for AI-BM or a per configuration indication. In our understanding, we shall postpone the discussion on UE initiated start/stop indication in AI-mobility until AI-phy makes the corresponding conclusion. Then, we can follow the conclusion: if per-UE indication for AI-BM is agreed, we can define a per UE indication for AI-Mob; if per configuration indication is agreed, we can define a per configuration indication.

Observation 1: In AI-Phy, although it is agreed that the UE can send the start/stop indication of data collection to the network via UAI, the granularity of such indication is still under RAN2 discussion.

Proposal 3:	Postpone the discussion on UE initiated start/stop indication in AI-Mobility, follow the conclusion made for AI-Phy, for instance:

- If per-UE indication is agreed for AI-BM, then define separate per-UE indication for AI-Mobility;

- If per-configuration indication is agreed for AI-BM, then define per-configuration indication also for AI-Mobility.

2.2 Data collection configuration for RRM measurement prediction

For data collection for temporal domain case A and case B, the L3 measurement results with the same measurement interval (or similar measurement interval) are required. After obtaining these results, it can be up to UE implementation to determine how to train the model (e.g. PW length, OW length, skipping pattern…). So, for training data collection, we think there is no need to differentiate temporal domain case A and case B. The same candidate data collection configuration can be applied to both.

Observation 2: For data collection for model training, we don’t see the need to differentiate temporal domain case A and case B. The same candidate data collection configuration can be applied to both.

For data collection of temporal domain case A and case B, besides the measured frequency, the UE also needs to know the transmission periodicity of the RS, which can be used to determine whether the UE can obtain the measurement results with required measurement interval.

Observation 3:	For temporal domain case A and case B, in addition to the frequency info, the UE may also need to know the transmission periodicity of the RS.

In AI-mob, the existing RRM measurement configuration can be used as baseline for data collection configuration. Since the IE measObject already supports to configure frequency info and RS transmission periodicity, we think MeasObjectId can be included in the candidate data collection configuration to provide the info above. In addition, an ID is needed to identity a candidate data collection configuration.

Proposal 4: 	For temporal domain case A and case B, each candidate data collection configuration includes following information:

- candidate configuration ID;

- MeasObjectID

For frequency domain prediction, the measurement results at predicted frequency are predicted using the measurement results at measured frequency. So, for model training of frequency domain prediction, the measurement results at two different frequencies are needed. Considering the UE side model only supports frequency domain prediction at co-located scenario, and the UE is unaware of network deployment, the network needs to indicate possible inputs and output frequencies. Similarly, the frequency info can be provided via MeasObjectID.

Observation 4: For frequency domain prediction, only collocated scenarios is supported for UE side model, and only the network knows the deployment situation, so, it is necessary to indicate UE about the possible inputs and outputs frequencies.

Proposal 5 	For frequency domain prediction, each candidate data collection configuration includes following information:

- candidate configuration ID;

- MeasObjectID for model inputs;

- MeasObjectID for model output

For final data collection configuration, since the motivation of the measurement is training data collection, the UE is not expected to report the measurement results to the network. To achieve this, an explicit indication is introduced in the CSI-ReportConfig, as follows:

#RAN1 121: - For data collection for UE-sided model, in CSI-report configuration, reportQuantity is set to “none-BM-r19”

The similar solution can be applied to AI-mob. And in the existing RRM measurement framework, reportConfig is used to indicate how to report measurement results, we can introduce an explicit indication in ReportConfigNR to indicate the associated MeasObejctNR is for UE side data collection. Once receiving this indication, the UE does not need to send measurement report for this ReportConfigNR.

Observation 5: 	In AI-Phy, for UE side data collection, the UE is not expected to send the report to the network. To achieve this, an explicit indication is introduced in CSI-ReportConfig.

Proposal 6 	Same as in AI-Phy, to introduce explicit indication in ReportConfigNR, used to indicate whether the associated measObjectNR is configured for UE side data collection. Once received, the UE does not need to send MR for this ReportConfigNR.

For frequency domain prediction, the network needs to indicate which frequency can be used for model input. While in the existing RRM framework, one ReportConfigNR is only allowed to associate one MeasObjectNR. To solve this issue, we think a separate measObjectId can be introduced in the ReportConfigNR in order to indicate the frequency for model inputs.

Observation 6  The existing measurement framework only allows one ReportConfigNR to be associated with one MeasObjectNR, but for inter-frequency prediction, the network needs to indicate which frequency can be considered for model inputs (e.g. based on deployment situation).

Proposal 7	Regarding the UE data collection configuration for frequency domain prediction, to introduce a separate measObjectID in ReportConfigNR, used to indicate frequency for model inputs.

2.3 Data collection configuration for measurement event prediction

For indirect measurement event prediction

In indirect measurement event prediction, the RRM measurement is performed first to obtain prediction results, and then predicted results and (optional) actual measurement results are used to derive whether the measurement event at one time instance occurs, without further AI/ML involvement. No separate AI model runs for direct measurement event prediction. So, there is no need to train a separate AI model for indirect event prediction.

Observation 7 For indirect measurement event prediction, the UE performs event evaluation based on the predicted RRM results, there is no separate AI model runs for event prediction. So, there is no need to train a separate AI model for indirect event prediction.

Proposal 8 	No need to define separate (candidate) data collection configuration for indirect measurement event prediction.

For direct measurement event prediction

In direct measurement event prediction, the UE directly predicts the probability of event occurrence within a time window. The different event configuration (e.g. event type, TTT, Ax offset) may require different AI models.

So, for data collection of direct measurement event prediction, the network needs to provide the event reporting related configuration to the UE. Based on the existing RRM measurement framework, the candidate configuration can include one measId: the measObjectNR included in the measId can indicate frequency info and RS periodicity, and reportConfig included in the measId can indicate event related configuration.

Observation 8 For direct measurement event prediction, the UE directly predicts the event results, so, different event configuration (e.g. event type, TTT) may require different AI models.

Observation 9 For candidate data collection configuration for direct measurement event prediction, the network needs to provide the event reporting related configuration to the UE.

Proposal 9 	For direct measurement event prediction, each candidate data collection configuration includes following information:

- Candidate configuration ID;

- MeasID (i.e. the associated reportConfigNR includes reportType set to eventTriggered)

Similar to data collection configuration for RRM measurement prediction, the final data collection configuration can reuse existing RRM measurement framework, and include an explicit indication in the reportConfigNR to indicate the UE does not need to send the measurement report to the network.

Proposal 10: 	As proposed in P2 and P6, for direct measurement event prediction, the final data collection configuration is provided by reusing the existing RRM framework and including the indication in ReportConfigNR, so that the UE does not report the MR to the network.

- Conclusion and proposals

Observation 1: In AI-Phy, although it is agreed that the UE can send the start/stop indication of data collection to the network via UAI, the granularity of such indication is still under RAN2 discussion.

Observation 2: For data collection for model training, we don’t see the need to differentiate temporal domain case A and case B. The same candidate data collection configuration can be applied to both.

Observation 3:	For temporal domain case A and case B, in addition to the frequency info, the UE may also need to know the transmission periodicity of the RS.

Observation 4: For frequency domain prediction, only collocated scenarios is supported for UE side model, and only the network knows the deployment situation, so, it is necessary to indicate UE about the possible inputs and outputs frequencies.

Observation 5: 	In AI-Phy, for UE side data collection, the UE is not expected to send the report to the network. To achieve this, an explicit indication is introduced in CSI-ReportConfig.

Observation 6  The existing measurement framework only allows one ReportConfigNR to be associated with one MeasObjectNR, but for inter-frequency prediction, the network needs to indicate which frequency can be considered for model inputs (e.g. based on deployment situation).

Observation 7 For indirect measurement event prediction, the UE performs event evaluation based on the predicted RRM results, there is no separate AI model runs for event prediction. So, there is no need to train a separate AI model for event prediction.

Observation 8 For direct measurement event prediction, the UE directly predicts the event results, so, different event configuration (e.g. event type, TTT) may require different AI models.

Observation 9 For candidate data collection configuration for direct measurement event prediction, the network needs to provide the event reporting related configuration to the UE.

Proposal 1 	For data collection in UE side model, the network can provide UE a list of candidate UE data collection configuration via otherConfig, and UE can response to the network with one or more of UE’s preferred configuration from the list via UAI, same as in AI-Phy.

Proposal 2 	After receiving UE’s preferred data collection configuration, the network configures final data collection configuration by using the existing RRM framework.

Proposal 3:	Postpone the discussion on UE initiated start/stop indication in AI-Mobility, follow the conclusion made for AI-Phy, for instance:

- If per-UE indication is agreed for AI-BM, then define separate per-UE indication for AI-Mobility;

- If per-configuration indication is agreed for AI-BM, then define per-configuration indication also for AI-Mobility.

Proposal 4: 	For temporal domain case A and case B, each candidate data collection configuration includes following information:

- candidate configuration ID;

- MeasObjectID.

Proposal 5 	For frequency domain prediction, each candidate data collection configuration includes following information:

- candidate configuration ID;

- MeasObjectID for model inputs;

- MeasObjectID for model output.

Proposal 6 	Same as in AI-Phy, to introduce explicit indication in ReportConfigNR, used to indicate whether the associated measObjectNR is configured for UE side data collection. Once received, the UE does not need to send MR for this ReportConfigNR.

Proposal 7	Regarding the UE data collection configuration for frequency domain prediction, to introduce a separate measObjectID in ReportConfigNR, used to indicate frequency for model inputs.

Proposal 8 	No need to define separate (candidate) data collection configuration for indirect measurement event prediction.

Proposal 9 	For direct measurement event prediction, each candidate data collection configuration includes following information:

- Candidate configuration ID;

- MeasID (i.e. the associated reportConfigNR includes reportType set to eventTriggered)

Proposal 10: 	As proposed in P2 and P6, for direct measurement event prediction, the final data collection configuration is provided by reusing the existing RRM framework and including the indication in reportConfigNR, so that the UE does not report the MR to the network.

- Reference

- Report of RAN2 129bis meeting.