3GPP TSG-RAN WG2 Meeting #131	R2-2506136

Bengaluru, India, 25 – 29 August 2025

Agenda item:	8.3.2.3

Source:	Nokia

Title:	Data collection for UE sided model

WID/SID:	FS_NR_AIML_Mob – Release 19

Document for:	Discussion and Decision

# 1	Introduction

The following proposals were noted in RAN2#130 for AI mobility:

R2-2503593 Discussion on data collection for AIML mobility CATT, Turkcell discussion Rel-19 FS_NR_AIML_Mob Proposal 8: The following agreements on UE requests data collection for UE-side data collection in AI/ML PHY are also applied to RRM measurement prediction. - The UE can request measurement configuration for data collection. The request can contain one or more of the following: - An indication on start/stop of data collection - Preferred configuration from a list of candidate configurations provided by NW. It is up to network what it configures at the end. - Introduce UAI message for UE request of data collection measurement configuration. And it is up to UE implementation when to send the request. Proposal 9: The following agreements on data collection configuration for UE-side data collection in AI/ML PHY are also applied to RRM measurement prediction. - The network can provide or release the data collection configuration (at any point in time), with or without UE request. - The following methods for network control of the initiation and configuration for data collection: - The network can decide when to start/stop the data collection and send configuration. - The network can configure whether UE is allowed to initiate request for data collection (e.g. start/stop indication). - Data collection related configuration(s) and associated ID(s)(if needed) can be included in training data collection configuration. Proposal 10: For RRM measurement prediction, legacy L3 RRM measurement framework is used to provide UE-side data collection configuration from network to UE. Noted R2-2504380 Discussion on data collection for AI Mob CMCC discussion Rel-19 FS_NR_AIML_Mob Proposal 16: RAN2 takes Table 2 as starting point for model training data collection for measurement event prediction. Noted Table 2. Data content for model training for measurement event prediction LCM purpose; Prediction option; Model type; Data content. Model training; Indirect prediction; UE-sided model; - Same as RRM measurement prediction. Direct prediction; UE-sided model; - Temporal L3 cell level measurement results of serving cell and/or neighbor cells - Measurement event configuration (e.g. offset, TTT) - Occurrence time of measurement event.

Based on the agreements reached in RAN2#130 regarding UE sided data collection, this contribution primarily addresses the overall configuration necessary to enable data logging and reporting on the UE along with the inter-operability issues. It then outlines the specific content of the logged data that can be configured from the NW for the AIML mobility use cases.

# 2	Discussion

## 2.1	General aspects

Both configuration and transfer of UE-side data collection have been discussed at length throughout the AI PHY SI and WI. In RAN2#130, for AI Mobility we agreed the following:

For UE sided data collection – use AI/ML PHY request/configuration framework as baseline. FFS enhancements/or differences

The relevant procedures for AIM PHY that would extend to mobility would be the ones defined for the BM use-case. As discussed in [1], using the CSI measurement configuration for collecting mobility data would suffer from the limitations associated with the need for collecting data from more than one serving cell. Extending the framework to collect mobility data would also incur larger specification effort. On the other hand, re-using the RSs utilized for RRM measurements and mobility purposes configured through MeasConfig would mitigate such drawbacks associated with collecting data from multiple cells pertaining to multiple gNBs.

Proposal 1: For UE-side data collection, enhance the MeasConfig IEs for AI mobility use cases.

In case of AI PHY BM, the emphasis is mainly on intra-gNB configuration. However, in case of mobility, another important aspect to consider when collecting data for UE-side models relates to the potential inter-operability issues that arise in different deployments. We think it is important to identify and address such issues as part of the WI.

Proposal 2: During WI phase, identify and study potential inter-operability issues that arise when collecting data for UE-side models.

If allowed by the network, an idle UE may potentially want to initiate an RRC connection for data collection reasons by an RRC setup message and request a specific measurement configuration, for example to collect measurement and event data as it crosses a cell border, without risking a degradation in the user plane traffic, for example if additional measurement gaps are required. The RRC setup message can indicate the specific purpose of data collection or exploration and refer to a measurement configuration the UE is requesting for its experiment. If the network is able to accommodate the request, it may configure the UE with the requested configuration.

Observation 1: RRC connection for data collection should enable measurement and event data collection also when otherwise not connected and without risking a degradation in the data traffic, for example if additional measurement gaps are required.

Proposal 3: RAN2 to consider an RRC connection set up specifically for measurement and event data collection.

## 2.2	Data content for UE sided model

In 3GPP Rel.19 SI for AI mobility, all identified use cases—such as RRM measurement prediction across temporal, frequency, and spatial domains, alongside both direct and indirect measurement event prediction—apply to the UE-side model. Therefore, the content of the data collected for model training should be configured and tailored on a per use case basis by the network. Additionally, the UE should be configured to collect both the model input features and the output labels. The specific data requirements may also vary by use case.

Observation 2: The discussion on data content for training UE sided models should be tailored to each use case by specifying relevant features, labels, and other important data.

Furthermore, to enhance the analysis of data content for UE sided model and identify specific gaps and areas for investigation, we have summarized the following options for each sub-use case for RRM measurement prediction.

Table 2.2-1: RRM measurement prediction use case analysis for data content of UE sided model (cell level prediction)

| Sub-use case | Temporal domain | Frequency domain | Spatial domain |
| --- | --- | --- | --- |
| 1 | Feature: Time-series of L1 filtered beam level measurements Label: Time-series of L1 filtered beam level measurements | Feature: L1 filtered beam level measurements of Frequency 1 Label: L1 filtered beam level measurements of Frequency 2 | Feature: Set B of L1 filtered beam level measurements Label: Set A of L1 filtered beam level measurements |
| 2 | Feature: Time-series of L3 filtered cell level measurements Label: Time-series of L3 filtered cell level measurements | Feature: L3 filtered cell level measurements of Frequency 1 Label: L3 filtered cell level measurements of Frequency 2 | N/A |
| 3 | Feature: Time-series of L1 filtered beam level measurements Label: Time-series of L3 filtered cell level measurements | Feature: L1 filtered beam level measurements of Frequency 1 Label: L3 filtered cell level measurements of Frequency 2 | Feature: Set B of L1 filtered beam level measurements Label: Time-series of L3 filtered cell level measurements |

Note: Frequency 1 denotes the carrier frequency at which measurements are taken for inference input, whereas the model output corresponds to measurements taken at Frequency 2.

Observation 3: For each RRM measurement prediction use case for cell-level prediction, RRM measurements should be used as features and labels in the UE sided data collection.

Proposal 4: RAN2 to consider Table 2.2-1 as a starting point for configuring the UE-side data collection content for cell-level RRM measurement predictions.

Table 2.2-2: RRM measurement prediction use case analysis for data content of UE sided model (beam level prediction)

| Sub-use case | Temporal domain | Frequency domain | Spatial domain |
| --- | --- | --- | --- |
| 4 | Feature: Time-series of L1 filtered beam level measurements Label: Time-series of L1 filtered beam level measurements | Feature: L1 filtered beam level measurements of Frequency 1 Label: L1 filtered beam level measurements of Frequency 2 | Feature: Set B of L1 filtered beam level measurements Label: Set A of L1 filtered beam level measurements |
| 5 | Feature: Time-series of L3 filtered beam level measurements Label: Time-series of L3 filtered beam level measurements | Feature: L3 filtered beam level measurements of Frequency 1 Label: L3 filtered beam level measurements of Frequency 2 | N/A |
| 6 | Feature: Time-series of L1 filtered beam level measurements Label: Time-series of L3 filtered beam level measurements | Feature: L1 filtered beam level measurements of Frequency 1 Label: L3 filtered beam level measurements of Frequency 2 | Feature: Set B of L1 filtered beam level measurements Label: Set A of L3 filtered beam level measurements |

Note: Frequency 1 denotes the carrier frequency at which measurements are taken for inference input, whereas the model output corresponds to measurements taken at Frequency 2.

Observation 4: For each RRM measurement prediction use case for beam-level prediction, RRM measurements should be used as features and labels in the UE sided data collection.

Proposal 5: RAN2 to consider Table 2.2-2 as a starting point for configuring the UE-side data collection content for beam-level RRM measurement predictions.

Furthermore, we present the following table to specify the data content required for different measurement event prediction sub-use cases. Note that direct and indirect predictions are modelled differently. For direct prediction, a classification model is used to determine the event status as true or false. In contrast, for indirect events, a time-series regression approach predicts future RRM measurements—similar to the temporal domain RRM measurement prediction—which are then processed by a post-processing unit to generate the predicted event status.

Table 2.2-3: Measurement event prediction use case analysis for data content of UE sided model

| Use case | Sub-use case | Data content |
| --- | --- | --- |
| Measurement event prediction | Indirect prediction | Feature: Time-series of L3 filtered cell level measurements Label: Time-series of L3 filtered cell level measurements Other data: Measurement event configuration (e.g. offset, TTT) and occurrence time of measurement event |
|  | Direct prediction | Feature: Time-series of L3 filtered cell level measurements Label: Measurement event flag Other data: Measurement event configuration (e.g. offset, TTT) and occurrence time of measurement event |

Based on the preceding discussion and the summary in Table 2.2-3, we offer the following observations and proposals regarding measurement event predictions.

Observation 5: Direct prediction may use data content similar to temporal domain cell-level RRM measurement prediction, whereas indirect prediction requires distinct labelling data. Additionally, essential information, such as event occurrence time, should be incorporated.

Proposal 6: Consider Table 2.2-3 as a starting point for configuring the UE-side data collection content for measurement event prediction.

# 3	Conclusion

This document has made the following observations:

Observation 1: RRC connection for data collection should enable measurement and event data collection also when otherwise not connected and without risking a degradation in the data traffic, for example if additional measurement gaps are required.

Observation 2: The discussion on data content for training UE sided models should be tailored to each use case by specifying relevant features, labels, and other important data.

Observation 3: For each RRM measurement prediction use case for cell-level prediction, RRM measurements should be used as features and labels in the UE sided data collection.

Observation 4: For each RRM measurement prediction use case for beam-level prediction, RRM measurements should be used as features and labels in the UE sided data collection.

Observation 5: Direct prediction may use data content similar to temporal domain cell-level RRM measurement prediction, whereas indirect prediction requires distinct labelling data. Additionally, essential information, such as event occurrence time, should be incorporated.

And proposed the following:

Proposal 1: For UE-side data collection, enhance the MeasConfig IEs for AI mobility use cases.

Proposal 2: During WI phase, identify and study potential inter-operability issues that arise when collecting data for UE-side models.

Proposal 3: RAN2 to consider an RRC connection set up specifically for measurement and event data collection.

Proposal 4: RAN2 to consider Table 2.2-1 as a starting point for configuring the UE-side data collection content for cell-level RRM measurement predictions.

Proposal 5: RAN2 to consider Table 2.2-2 as a starting point for configuring the UE-side data collection content for beam-level RRM measurement predictions.

Proposal 6: Consider Table 2.2-3 as a starting point for configuring the UE-side data collection content for measurement event prediction.

# 4	Reference

[1] R2-2504286, Considerations on data collection, Nokia, 3GPP TSG-RAN WG2 Meeting #130, St Julian’s, Malta, 19 – 23 May 2025.