3GPP TSG RAN WG2 #131	                                                                                 R2-2506167

Bengaluru, India, Aug 25th – 29th, 2025

Source:	CEWiT

Title:	Discussion on A-IoT Random Access

Agenda Item:	8.2.3

Document for:	Discussion and Decision

- Introduction

In RAN2 #130 [1] meeting, the following agreements were made on random access aspects related to A-IoT.

Agreements 1. For Msg1 resource selection procedure capture as guidance the countdown behaviour in the MAC specification (use TP in R2-2503952). Capture a NOTE that other implementation are allowed. X, Y will be signalled by paging message 2. The start of the first set of MSG1 resources is indicated by Paging message directly instead of the new R2D trigger messages. R2D trigger message is not sent in CFRA procedure. Come back if RAN1/4 sees any issues. Send LS to RAN1/RAN4 3. FFS R2D byte alignment dependent on TBS size discussion Agreements on RA 1. Exclude the option of MSG2 transmission and any retransmission of MSG2 happens within a predefined time window (based on timer) 2. A device expecting MSG2 assumes CBRA failure if its MSG2 is not received before a boundary, where the boundary can be further down selected between option B and C below. A device receiving MSG2 within this boundary transmits MSG3. The device does not process MSG2 (re)transmission received after the boundary. - Option B – the boundary is the reception of either the next R2D trigger message or the subsequent paging message - Option C – the boundary is the reception of either the kth R2D trigger message or the subsequent paging message (K is FFS) - Option A (the boundary being the subsequent paging only) is excluded. For option C, further discuss in terms of complexity at the device vs reader flexibility. 3. Including frequency index along with RN16 in MSG2 to reduce collisions of MSG1 between different devices is feasible. FFS Discuss further whether to include it. Agreements on NACK reception: 1. After MSG3 transmission, upon receiving NACK with its AS ID before subsequent paging or command addressed to this device from the reader, device determines it will perform re-access. FFS how to specify. Agreements on RN16/AS ID maintenance: 1. Confirm a device is not expected to maintain both AS ID and RN16. After msg2 reception, RN16 becomes AS ID, if new AS ID was not assigned by reader. This implies that the reader cannot change AS ID and RN16 pair across message 2 retransmission. How to capture device behavior is FFS

In this contribution, we discuss our views on remaining open issues related to random access procedure aspects for A-IoT.

- Random access

2.1 	Msg2 monitoring

In the email discussion [POST130][027][AIoT] MAC Running CR (Huawei), A-IoT MAC CR Rapporteur listed open issues for Random access and for another topic as well.

| Issue 2-4: CBRA failure detection | - Further down selection between option B and C for msg2 monitor window in CBRA. - 1 Exclude the option of MSG2 transmission and any retransmission of MSG2 happens within a predefined time window (based on timer) - 2 A device expecting MSG2 assumes CBRA failure if its MSG2 is not received before a boundary, where the boundary can be further down selected between option B and C below. A device receiving MSG2 within this boundary transmits MSG3. The device does not process MSG2 (re)transmission received after the boundary. - Option B – the boundary is the reception of either the next R2D trigger message or the subsequent paging message - Option C – the boundary is the reception of either the kth R2D trigger message or the subsequent paging message (K is FFS) - Option A (the boundary being the subsequent paging only) is excluded. - For option C, further discuss in terms of complexity at the device vs reader flexibility. | To be discussed by company’s contributions |
| --- | --- | --- |

RAN2#130 [1] meeting reached a common understanding that the Msg2 monitoring interval for CBRA must be bounded and must not spill over past the subsequent paging message. In that spirit, a reception boundary needs to be fixed such that a device awaiting Msg2 treats the attempt as failed if Msg2 has not been received by that point and disregards any Msg2 (re)transmission that arrives afterwards. Two alternatives were discussed for defining this boundary i.e. either at the next R2D trigger or the subsequent paging message (Option B), or at the k-th R2D trigger or the subsequent paging message (Option C, with k FFS).

We support Option B. Under this rule, for each CBRA attempt the device listens for Msg2 only until whichever occurs first, the next R2D trigger from the serving reader within the same paging frame, or the subsequent paging message. Once that boundary is reached, any later Msg2 (re)transmission for the ongoing attempt is ignored; if Msg2 has not been received by then, the device declares CBRA failure and proceeds with the specified backoff/reattempt behavior. This delivers a short, predictable end to the attempt, cuts unnecessary time delay. Option B is also simpler to implementation and interoperability. Devices do not need to maintain or recover a trigger counter, eliminating state tracking and corner cases from missed triggers. By contrast, Option C lengthens and makes the monitoring window variable and obliges devices to count triggers up to k, increasing complexity, energy use, and sensitivity to missed events. Allowing multiple triggers before the boundary also expands the time during which delayed Msg2 can retrigger Msg3.

Proposal 1: Support option B i.e. the device does not process MSG2 (re)transmission received after the subsequent R2D trigger message or the subsequent paging message.

2.2 Random ID differentiation in Msg2

According to [POST130][027][AIoT] MAC Running CR (Huawei), following issue is listed regarding the frequency index indication in Msg2 for random ID differentiation to reduce the collision.

| Issue 2-5: random ID differentiation in Msg2 | Whether to include frequency index along with RN16 in MSG2 to reduce collisions of MSG1 between different devices. - In previous meetings, RAN2 discussed whether Msg2 need to include more information on top of the random ID to avoid random ID collision, but there was no consensus. - RAN2#130 agreement: Including frequency index along with RN16 in MSG2 to reduce collisions of MSG1 between different devices is feasible. FFS Discuss further whether to include it. | To be discussed by company contributions |
| --- | --- | --- |

At the last meeting [1], it was noted that it is technically possible to include in Msg2 the frequency index used for Msg1, so that two devices that picked the same 16-bit random ID but used different frequency resources can be distinguished. This would require Msg2 to carry, for each echoed RN16, the corresponding frequency index. However, because the random ID is 16 bits, the probability of two devices choosing the same value during an access occasion is already low. Given the low collision risk, adding a frequency index to Msg2 would increase payload size and device power consumption without sufficient benefit, therefore, we do not recommend introducing a frequency index in Msg2.

Proposal 2: Frequency index indication is not included in Msg 2 along with 16-bit Random ID to reduce collisions of Msg1 between different devices.

- Conclusion

In this contribution, we provide our views on random access for A-IoT. The proposals are summarized as follows.

Proposal 1: Support option B i.e. the device does not process MSG2 (re)transmission received after the subsequent R2D trigger message or the subsequent paging message.

Proposal 2: Frequency index indication is not included in Msg 2 along with 16-bit Random ID to reduce collisions of Msg1 between different devices.

# References

[1]   RAN2#130 Chair Notes.