3GPP TSG RAN WG2#131bis	R2-2506717

Prague, Czech Republic, 13th - 17th October 2025

3GPP TSG RAN WG1 #122			R1-2506553

Bengaluru, India, 25th-29th August, 2025

Title:	Reply LS on on CB Msg3 EDT for IoT NTN Ph3

Response to:	R1-2504959/ R2-2504776

Release:	Release 19

Work Item:	IoT_NTN_Ph3

Source:	RAN1

To:	RAN2

Contact Person:

Name:	Gilles Charbit

E-mail Address:	gilles.charbit@mediatek.com

# 1	Overall description

RAN1 would like to thank RAN2 for their LS on L1 parameters for CB-msg3-EDT. RAN1 has considered the questions Q1 and Q2 on CB ms3 EDT.

Further, RAN1 did not provide response to questions Q5 and Q8 for RAN2 LS on CB-msg3-EDT in LS R1-2503613 (R2-2503175) in the previous RAN1 reply LS (R1-2504905).

RAN1 made the following replies:

## Question Q1

RAN2 ask question Q1 on HARQ feedback for MSG4

Q1: RAN2 ask RAN1 whether there is any concern from RAN1 on the agreements regarding HARQ feedback for MSG4

- RAN2#130 Agreements regarding HARQ feedback for MSG4 (i.e., CB-msg3 response): - The HARQ feedback resource information can be included in the CB-Msg4 together with contention resolution ID which identity the specific UE. RAN2 could revisit this proposal if RAN1 has some concern. - Whether to send the HARQ feedback for CB-Msg4 can be controlled by NW. UE does not send HARQ NACK. - For NB-IoT, the SubCarrierSpacing of the HARQ feedback for CB-Msg4 is same as the CB-Msg3. - Reuse the existing format of HARQ ACK allocation signalling in the DCI. There is 2-bit HARQ ACK resource for eMTC and 4-bit HARQ ACK resource for NB-IoT. Reuse the meaning of DCI field in R1 SPEC. - Introduce a new CB-Msg3 Response (CBR) MAC sub-header in CB-Msg4. It has 1bit E for sub-header/payload indication, 2 bits T for sub-header type, 1bit T2 for HARQ ACK resource present, 1 bit T3 for TAC present, 1 bit T4 for C-RNTI present and 2bit R for reservation

Reply to Q1:

RAN1 reply to Q1:

- RAN1 sees no issue with the RAN2 agreement that HARQ feedback resource information is included in the CB-Msg4

- RAN1 also notes that using 2 bits for “HARQ ACK resource offset” in eMTC allows multiplexing only up to 4 users in the same response.

## Question Q2

RAN2 ask question Q2 regarding TAC in MSG4

Q2: RAN2 ask RAN1 whether there is any concern from RAN1 and RAN4 on the agreements regarding TAC in MSG4

- Agreements regarding TAC in MSG4 (i.e., CB-msg3 response): - The TAC is optionally used in the CB-Msg3 response. - RAN2 assumes that NTA=0 for initial CB-msg3 transmission. - RAN2 assumes the length of the TAC field is 6 bits (we can revisit this if there is major R1 impact on TA calculation)

Reply to Q2:

RAN1 reply to Q2:

- RAN1 has no issue on TAC in MSG4 (i.e., CB-msg3 response).

## Question Q5

Q5: For eMTC, RAN2 agrees to have PUCCH configuration for shared resource configuration and decides to reuse the parameters from PUR PUCCH configuration (in IE PUR-PUCCH-Config-r16, as below) as baseline. RAN2 would like to check with RAN1 on how to define the detailed PUCCH configuration.

RAN2 question on Q5

PUR-PUCCH-Config-r16 ::=			SEQUENCE {

n1PUCCH-AN-r16						INTEGER (0..2047)			OPTIONAL,	-- Need ON

pucch-NumRepetitionCE-Format1-r16	ENUMERATED {n1, n2, n4, n8}	OPTIONAL	-- Need ON

}

Reply to Q5:

RAN1 reply to Q5:

- RAN1 confirm that for CB-Msg3 EDT for eMTC, n1PUCCH-AN and pucch-NumRepetitionCE-Format1 can be reused.

## Question Q8

Q8: For both eMTC and NB-IoT, any other L1 parameters are needed for CB-Msg3-EDT procedure in additional to previous discussion?

In addition, RAN2 agrees that:

Introduce a new RNTI (i.e. CB-RNTI) for CB-Msg4 monitoring and CB-Msg3 scrambling. We include this agreement in the LS to RAN1

RAN2 would like to inform RAN1 on the new RNTI for CB-Msg4 monitoring and CB-Msg3 scrambling. RAN2 has not yet made agreements on how to determine the RNTI, but RAN2 assumes there may be some potential RAN1 specification impact on this.

RAN2 question on Q8

Reply to Q8:

RAN1 reply to Q8:

- RAN1 does not see any issue for RAN2 on the introduction  a new RNTI (i.e. CB-RNTI) for CB-Msg4 monitoring and CB-Msg3 scrambling. Additionally, from RAN1 perspective, ack-NACK-NumRepetitions-r16 can be supported to configure number of repetitions for HARQ-ACK feedback corresponding to the NPDSCH carrying CB-msg4.

# 2	Actions

To RAN2

ACTION:   RAN1 respectfully asks RAN2 to take into account their response to questions Q1 and Q2, and additionally to Questions Q5 and Q8 not included in the previous RAN1 reply LS (R1-2504905).

# 3	Dates of next TSG RAN WG1 meetings

TSG-RAN1 Meeting #122bis	13 - 17 October 2025	Prague, CZ

TSG-RAN1 Meeting #123	17 - 21 November 2025	Dallas, US