3GPP TSG RAN WG2 #131bis          	                                                             R2-2506787

Prague, Czech Republic, Oct. 13th – 17th, 2025

Source:	CATT

Title:	Considerations on 6GR AS security

Agenda Item:	10.3.3

Document for:	Discussion and Decision

- Introduction

This document regards 5G AS security mechanism as the baseline in 6G, but identifies following issues:

- KgNB* leaking to a third node due to cancelled handover preparation;

- Suboptimal in DL data pre-processing and LTM failure recovery;

- Unprotected MAC CE;

- Unprotected system information.

We propose acknowledging these issues and discussing whether their solution should be studied in RAN2 or SA3.

- Discussion

  - General aspects for AS security

The mechanism of AS security in 5G generally follows the one in LTE. We have not observed any reason to adopt a complete new AS security mechanism in 6G. Therefore, we prefer assuming the 5G AS security mechanism as the baseline of 6G.

Proposal 1: Assume that the AS security mechanism in 5G is the baseline in 6G.

One major change relative to LTE is allowing that the KgNB does not change upon intra-RAN-node handovers, which vastly reduces UP delay/interruption by no longer requiring PDCP reestablishment.

The motivation is persuading, but it seemingly incurred some security risk as well.

The pair {PCI, ARFCN} shall be unique among all cells neighbouring a given cell due to measurement reasons. However, there is no reason to require the pair {PCI, ARFCN} unique among all cells neighbouring at least one served cell of a given RAN node. Considering that one gNB can host up to 16384 cells, but the range of PCI is only 1008, such requirement is neither practical.

Figure 1: Node 1 sends entirely the same KgNB* to Node 2 and Node 3

Assume that there are two cells “Cell 2” and “Cell 3”, hosted by “Node 2” and “Node 3” respectively, sharing the same {PCI, ARFCN} value and neighbouring a large given RAN node, namely “Node 1” at the opposite directions. It is thus possible that:

- Step 0: A UE connects to Node 1 using a KgNB, and is near Cell 2.

- Step 1: Node 1 selects Cell 2 as a candidate target cell for the UE.

- Step 2: Node 1 generates a KgNB* from the current KgNB (or NH) and {PCI, ARFCN} of Cell 2, and sends the KgNB* toward Node 2 in a handover preparation procedure.

- Step 3: Node 1 removes Cell 2 from candidate cell lists and cancels the handover preparation.

- Step 4: The UE moves to the opposite side of Node 1 and is near Cell 3, during which it keeps RRC_CONNECTED and keeps using the original KgNB.

- Step 5: Node 1 selects Cell 3 as a candidate target cell for the UE.

- Step 6: Node 1 generates a KgNB* from the current KgNB (or NH) and {PCI, ARFCN} of Cell 3, and sends the KgNB* toward Node 3 in a handover preparation procedure.

- Step 7: The UE is indeed handed over to Cell 3, turning the derived KgNB* into use.

The KgNB* sent in Step 2 is totally the same as the KgNB* sent in Step 6, due to completely the same derivation input values. As the result, the key used between the UE and Node 3 is known not only in Node 1 but also in Node 2.

Observation 1: Including only KgNB/NH and {PCI, ARFCN} of target cell as inputs to calculate KgNB* while allowing no (master) key update during intra-node handover may incur a situation that the KgNB in use is known not only by the source node of the last inter-node handover but also by a third node.

In later 5G releases, AS mobility enhancements makes the mechanism of AS security more troublesome.

One suboptimal case is on Early Data Forwarding. Early Data Forwarding was introduced for CHO and DAPS, and later reused in (C)LTM. Considering that (C)LTM is widely supported as a Day 1 feature of 6G, Early Data Forwarding will probably be supported from 6G Day 1. In CHO/(C)LTM, it is a common case that multiple cell in one (target) RAN node is configured as candidates, and Early Data Forwarding is performed for all of them.

Figure 2: Difficulty in pre-processing packets received from Early Data Forwarding

However, the target node finds it difficult to pre-process the forwarded DL packets. In LTE and 5G, the PCI and ARFCN of the target cell are inputs in calculating KeNB*/KgNB* in inter-node handovers, making that the KeNB/KgNB used for one candidate cell—and consequently, the KUPenc/KUPint used for that cell—is different from the key used for another candidate cell. The target cell has to make a choice: either to pre-process every packet one copy per candidate cell (using the key for that cell), or to give up pre-processing entirely (i.e. packets are stored only as PDCP SDUs), causing relatively long UP delay/interruption time upon inter-node handovers.

Observation 2: Including {PCI, ARFCN} of target cell as inputs to calculate KeNB*/KgNB* (and consequently KUPenc/KUPint) makes it hard for the target RAN node to pre-process the packets received through Early Data Forwarding.

Another suboptimal case is on intra-node LTM failure recovery through intra-node LTM candidates, which typically requires no key change. The UE sends an RRCReconfigurationComplete message (corresponding to the target cell) over Uu when completing a LTM, and the complete message is encrypted by a “key stream” whose calculation input includes only KRRCenc and PDCP COUNT as variables (other inputs are constant). When it fails, the UE selects another candidate cell and sends another RRCReconfigurationComplete message (corresponding to the recovery cell) over Uu. The two RRC messages shall not use the same “key stream” (otherwise there will be a security risk), so the latter RRC message has to a PDCP COUNT higher than the former one.

Figure 3: Delay in intra-RAN-node LTM failure recovery

However, the RAN node usually did not receive the former RRC message, so that PDCP COUNT is skipped. Due to this gap, the PDCP layer in the RAN node will ordinarily wait a reordering timer length before submitting the RRC message toward the upper layer, causing an unnecessary (and lengthy) delay on LTM failure recovery.

Observation 3: The delay of intra-RAN-node LTM failure recovery in 5G is typically lengthy partly due to the design that KRRCenc does not change upon intra-RAN-node handover.

We hope that 3GPP acknowledge the two suboptimal aspects of 5G, and if possible, prevent them from happening again in the 6G.

Proposal 2: Acknowledge the one risking and two suboptimal aspects of 5G shown in the prior observations, and if possible, to study how to prevent them from happening again in the 6G.

  - Security of MAC CE

In 4G and 5G, RAN adopt AS security for SRB/DRB PDU in PDCP. Security is crucial for UE to prevent eavesdropping and manipulation by attackers, particularly for SRBs. Once AS security is activated, all RRC messages including those containing NAS messages are integrity protected and ciphered by PDCP. When UE in RRC_CONNECTED detects integrity check failure concerning SRB1, SRB2 and SRB3, it will initiate RRC re-establishment procedure. However, MAC CE as another type of Uu signalling, it has no any AS protection. This issue had been mentioned in 5G later release. Restricted by the 3GPP timeline and back-compatible requirement, MAC CE security had not been considered in 5G. In current 6G beginning stage, MAC CE security should be discussed together with the overall MAC layer design.

In the 5G Rel-18 MAC specification [1], there are 56 DL MAC CEs and 59 UL MAC CEs. DL MAC CEs are used to manage Uu resources and UE behaviours dynamically. Their credibility is critical for ensuring reliable data transmission and UE connectivity. For example, LTM Cell Switch Command MAC CE had been concerned in SA3 because it transmits NCC with plaintext and increases the risk of handover failures [2]. The security of other DL MAC CEs such as SCell Activation/Deactivation MAC CE, TCI state indication MAC CEs, has gained attention for its potential impacts on air interface performance. UL MAC CEs are used to indicate UE information such as BSR, PHR, TAR to gNB for efficiet resource allocation and TA control. Credibility requirements for UL MAC CEs appear less stringent than those for DL MAC CEs. However, we should not preclude the security of UL MAC CEs in the too early stage. The security requirements of MAC CEs should be evaluated systematically during RAN2 discussion on MAC CEs.

Observation 4: Security of DL MAC CEs is importat in 6G and should be considered in Day 1.

Observation 5: Security of UL MAC CEs is less essential, but we should not preclude it in current stage.

Proposal 3: RAN2 consider MAC CE security in 6G and the security requirements of MAC CEs should be evaluated systematically.

The introduction of MAC CE security presents several challenges, including resource overhead, implementation complexity and additional latency caused by the complexity. Assuming reusing 5G Uu AS security architecture and algorithms, we analyse the resource overhead and implementation complexity issues introduced by MAC CE security separately.

- Resource Overhead:

Whether for ciphering or integrity protection, COUNT value is required, and the COUNT value must be unique. In 5G MAC PDU, there is no SN field. Referencing 5G PDCP SN which is 12-bit or 18-bit, introducing an SN field for each MAC subPDU containing MAC CE would clearly incur significant overhead. Additionally, the MAC-I field for integrity protection introduces extra overhead, with the MAC-I length at the 5G PDCP being 4 bytes. To mitigate these overheads, we can consider optimized designs, such as a more efficient SN field design or using the L1 time-frequency resource information of MAC CE transmission as COUNT value inputs; furthermore, shorter MAC-I field could be studied. These considerations require joint evaluation with SA3.

- Implementation Complexity:

The transmitter and receiver need to use security algorithms (e.g., ciphering algorithm NEA and integrity algorithm NIA) and known inputs to derive results during ciphering/deciphering and integrity protection/verification procedure. The processing of security algorithms introduces implementation complexity and processing latency. In DL, the base station can pre-process the DL MAC CE including security treatment to reduce additional latency. In UL, UE needs to report the latest uplink information relative to the current uplink transmission occasion. Whether pre-processing of UL MAC CE security is feasible needs to be assessed. Therefore, the complexity and latency introduced by security processing are also critical factors in evaluating the adoption of MAC CE security.

Compared to 5G, 6G demands higher spectral efficiency and more stringent latency performance. We cannot afford excessive resource overhead for MAC CE security, nor tolerate air interface latency caused by increased complexity at base station and UE due to MAC CE security. Therefore, MAC CE security design must strike a balance - ensuring security while developing solutions with minimal overhead and complexity.

Proposal 4: RAN2 consider MAC CE security design with minimal overhead and complexity.

  - Security of system Information

Broadcasting system information (SI) is one of the essential functions of the RRC protocol, defined in 3GPP TS 38.331 [3]. Broadcasted messages are intended for all UEs which are camping on a cell.  In the idle mode or inactive mode, the UE monitors the SI of cells and choose a suitable cell to camp on. The UE typically acquires the SI from the cell and performs initial access to transition to connected state to obtain services. The system information includes information, among others, like cell (re-)selection parameters, neighbouring cell information, frequency priority, blacklisted cell, common channel configuration information, NAS common information, and public warning system (PWS) messages.

SA3 has studied and investigated if and how a new protection mechanism could be introduced against over-the-air attackers who broadcast rogue SI messages or replay previously captured SI messages as-is (without modification). In general, an integrity and replay protected SIs could add security value by at least making it difficult for over-the-air attackers to succeed in using a rogue SI or a previously captures SI at a later time, e.g., to lure UEs using SI messages with incorrect neighbouring cells, and to send self-crafted or old PWS messages. The Key issue details and the related solutions have been included in TR33.809 [4].

However, the security of broadcasting system information has only been studied in 5G but not specified, as the basic and essential security system design for 5G was finished during the initial discussion phase, making core enhancements difficult in subsequent versions during 5G evolution. ‌

Observation 6: Security of broadcasting system information is studied in 5G SA but not be specified in 5G due to the introduction timing.

With current and potential future technological advancements, an increasing number of communication networks and UE applications may be subjected to attacks, such as DoS attacks on network or on UE, replay attacks, privacy compromising attacks, .etc.

Therefore, it is reasonable to continue studying the security of broadcasting system information as a native tech in 6G Day1. Since the MIB, SIB1 and SI message(s) are periodically and frequently broadcasted, encryption imposes a significant burden and large delay on data encoding/decoding in both UE and NW sides, but integrity protection ensures that system information cannot be tampered with. The 6G system should enhance the integrity protection of broadcasting system information.

Proposal 5: Enhance the integrity protection of broadcasting system information in 6G Day 1.

Since the system information is broadcasted by the cell, and all UEs in the cell could receive it, the traditional symmetric algorithm is not appropriate. By using asymmetric algorithm, digital signature could be generated in the NW/OAM based on public security key, and the private security key could be sent to the UE for verification. Since the digital signature should be sent together with each system information message and its data size is slightly larger (maybe more than 500bits),  the proportion of digital signature in the PDCP PDU size limit (about 3000bits in 5G) is relatively large, applying it to all messages seems impractical. We could instead protect only certain portions, such as essential information involving mobility or PWS.

Proposal 6: Apply integrity protection only for essential system Information.

  - Coordination with SA3

All of the abovementioned security aspects require coordination with SA3. There are two typical ways for work split between RAN2 and SA3:

- RAN2 identify only issues and/or requirements observed from RAN2 perspective. All solutions are raised, studied and selected first in SA3.

- RAN2 identify issues and/or requirements observed from RAN2 perspective and study solutions as well. One or more candidate solutions are sent to SA3 in liaisons for confirmation or selection.

Security protection of system information described in Section 2.3 seemingly requires asymmetric algorithm, which is not used in RAN scope before. We thus prefer SA3 to raise and study its solutions. Issues observed 5G AS security mechanism as described in Section 2.1 and security protection of MAC CE described in Section 2.2 seemingly require only optimisation and/or extension on the basis of LTE/5G AS security. We are thus open on whether RAN2 studies potential solutions first. We propose that RAN2 discusses how to split the work between RAN2 and SA3 for the two types of topics.

Proposal 7: On coordination / work split between RAN2 and SA3:

- For security protection for system information, solutions are raised and studies in SA3. RAN2 may provide SA3 only issues and/or requirements observed from RAN2 perspective within liaison(s).

- For issues observed on 5G AS security mechanism and security protection of MAC CE, RAN2 study how to split the work between RAN2 and SA3, e.g. whether RAN2 provide SA3 potential solution(s) in addition to issues and/or requirements observed from RAN2 perspective within liaison(s).

- Conclusion

In this contribution, we provide our views on the 6GR AS security:

Proposal 1: Assume that the AS security mechanism in 5G is the baseline in 6G.

Observation 1: Including only KgNB/NH and {PCI, ARFCN} of target cell as inputs to calculate KgNB* while allowing no (master) key update during intra-node handover may incur a situation that the KgNB in use is known not only by the source node of the last inter-node handover but also by a third node.

Observation 2: Including {PCI, ARFCN} of target cell as inputs to calculate KeNB*/KgNB* (and consequently KUPenc/KUPint) makes it hard for the target RAN node to pre-process the packets received through Early Data Forwarding.

Observation 3: The delay of intra-RAN-node LTM failure recovery in 5G is typically lengthy partly due to the design that KRRCenc does not change upon intra-RAN-node handover.

Proposal 2: Acknowledge the one risking and two suboptimal aspects of 5G shown in the prior observations, and if possible, to study how to prevent them from happening again in the 6G.

Observation 4: Security of DL MAC CEs is importat in 6G and should be considered in Day 1.

Observation 5: Security of UL MAC CEs is less essential, but we should not preclude it in current stage.

Proposal 3: RAN2 consider MAC CE security in 6G and the security requirements of MAC CEs should be evaluated systematically.

Proposal 4: RAN2 consider MAC CE security design with minimal overhead and complexity.

Observation 6: Security of broadcasting system information is studied in 5G SA but not be specified in 5G due to the introduction timing.

Proposal 5: Enhance the integrity protection of broadcasting system information in 6G Day 1.

Proposal 6: Apply integrity protection only for essential system Information.

Proposal 7: On coordination / work split between RAN2 and SA3:

- For security protection for system information, solutions are raised and studies in SA3. RAN2 may provide SA3 only issues and/or requirements observed from RAN2 perspective within liaison(s).

- For issues observed on 5G AS security mechanism and security protection of MAC CE, RAN2 study how to split the work between RAN2 and SA3, e.g. whether RAN2 provide SA3 potential solution(s) in addition to issues and/or requirements observed from RAN2 perspective within liaison(s).

- References

- TS 38.321 NR; Medium Access Control (MAC) protocol specification V18.6.0

- S3-251124 Reply LS on security handling for inter-CU LTM in non-DC cases Feasibility

- TS 38.331 NR; Radio Resource Control (RRC) protocol specification

- TR 33.809 Study on 5G security enhancements against False Base Stations (FBS)