3GPP TSG-RAN WG2 Meeting #131bis	R2-2506799

Prague, Czech Republic, October 13th– 17th, 2025

Source:	vivo

Title:	Considerations on 6GR Control Plane

Agenda Item:	10.3.2

Document for:	Discussion and Decision

- Introduction

In RAN#109, the SID “Study on 6G Radio” was agreed[1]. Main RAN2 leading items regarding control plane are listed in below.

- Radio interface protocol architecture and procedures for 6GR [RAN2, RAN1, RAN4, RAN3] - User Plane architecture and protocol design [RAN2] - Control Plane architecture (including RRC states) and protocol design [RAN2, RAN3] - Access stratum security aspects, in alignment with requirements from SA3 [RAN2] - Radio signalling framework for UE capabilities, aiming at improvements and simplification compared to 5G NR [RAN2, RAN1, RAN4] - Data transfer design to support various types of data (e.g. AI/ML, Sensing, etc) [RAN2, RAN3]

This contribution discusses potential enhancements to control plane design in future 6G systems while taking the existing NR control plane as a baseline. In addition, requirements on CP-related security enhancements are also discussed.

- Discussion

In NR, the RRC layer hosts the control plane functionalities (terminated in gNB on the network side) and has been successfully demonstrated through extensive deployments. And we believe RRC layer should be maintained to host the control plane functions in 6G. In this section, we will discuss some optional enhancements to 6G RRC, considering the lessons learned from NR and emerging requirements from anticipated 6G Day-1 features. This section is structured as follows: fundamental RRC functional improvements are discussed in section 2.1, feature-specific RRC enhancements are analyzed in section 2.2.

  - Fundamental RRC Functional Improvements

    - RRC States

In addition to RRC_IDLE and RRC_CONNECTED states, NR introduced the RRC_INACTIVE state to meet the control plane latency requirement, i.e., the time required to transition from a battery-efficient state to the start of continuous data transfer should be less than 10 ms. When a UE is in the RRC_INACTIVE state, the last serving gNB should keep the UE context and the UE-associated NG connection with the serving AMF and UPF. As a result, the UE can resume to the RRC_CONNECTED state with minimal signaling overhead and latency.

For NR UEs, support for the RRC_INACTIVE state is a mandatory feature. However, the decision to release a UE to either RRC_IDLE or RRC_INACTIVE state is up to gNB implementation. Due to potential additional CAPEX (e.g., expanding storage for maintaining UE contexts) and OPEX (e.g., RNA management), operators may choose not to deploy this feature at NG-RAN. In such cases, the network can only release UEs to RRC_IDLE state rather than RRC_INACTIVE state. Consequently, although UEs are required to support the RRC_INACTIVE state, which increases implementation and testing complexity, operators may choose not to enable this feature.

- In NR, the RRC_INACTIVE state was introduced as a mandatory 5G Day-1 UE feature to meet the control plane latency requirement, while few operators have enabled this feature due to additional CAPEX and OPEX.

In the ongoing SID "Study on 6G Scenarios and Requirements"[2], the control plane latency requirement has been relaxed from 10 ms in NR to 20 ms as follows.

### 5.1.8 Control plane latency Control plane latency refers to the transition time from a most “battery efficient” state (e.g. Idle state) to the start of continuous data transfer (e.g. Active state). The minimum requirement for control plane latency is 20 ms.

From our understanding, the requirement is only necessary for specific scenarios, such as low-latency communication use cases, and the RRC_INACTIVE state can be one alternative to fulfill the requirement.  For other cases, the network can release the UE to RRC_IDLE state for power saving purposes.

Overall, we believe that for 6G, RRC_CONNECTED and RRC_IDLE are still essential and should remain as mandatory RRC states. While for RRC_INACTIVE state, if it is supported in 6G, it should be defined as an optional feature.

- For 6GR, the RRC_IDLE and RRC_CONNECTED should remain as mandatory RRC states. However, the RRC_INACTIVE state, if supported in 6G, should be defined as an optional feature.

    - Cell Model

Cell modelling has evolved several times since the launch of NR. In Rel-15, SUL is defined where two non-continuous carriers can be configured as NUL (Normal Uplink) and SUL (Supplementary Uplink) in one serving cell. The UE is only allowed to send on either SUL or NUL at one time. In Rel-18, cell of less than 5MHz carrier is specified. With this enhancement, scattered spectrums can be re-farmed for 5G even their bandwidths are limited. Then, low-band SDL switching is introduced in R19, where the UE can be configured with Low-Low band CA via semi-statical DL switching pattern.

For 6G, cell modelling needs to address some new issues. First of all, NES (Network Energy Savings) had been intensively studied and specified in NR to resolve the increasing pressure from the operators for better operation cost control. However, the per carrier SSB/SIB transmission for idle mode operation still exists. Secondly, UEs in field nowadays still suffer from great access latency or low throughput in densely populated areas, e.g., stadiums with concerts or sports events, and thus sometimes consumers question the real usage of 5G compared to legacy 4G. Thirdly, operators may have some fragmented spectrums re-farmed from 2G/3G with small bandwidth in sub-1GHz low bands. How to utilize the fragmented spectrums should be considered. Therefore, we think 6G cell model should take the issues into consideration and improve as much as possible.

- For 6GR cell model design, the following issues should be considered on how to: 1) utilize fragmented spectrum; 2) achieve more NES gain; 3) enlarge RACH/Paging capacity in densely populated areas.

In LTE NB-IOT, a serving cell can configure multiple DL/UL carriers on which paging occasions (PO) and PRACH occasions (RO) are available for idle mode operation. This framework is designed due to the fact that massive NB-IOT UEs need to be served in the cell, while the bandwidth of anchor carrier is only 180kHz and the available UL/DL resources on anchor carrier are limited. Note that the anchor carrier is a carrier where the UE assumes that NPSS/NSSS/NPBCH/SIB-NB for FDD or NPSS/NSSS/NPBCH for TDD are transmitted, and a non-anchor carrier is a carrier where the UE does not assume that NPSS/NSSS/NPBCH/SIB-NB for FDD or NPSS/NSSS/NPBCH for TDD are transmitted [3]. All the anchor carrier and non-anchor carriers deployed in a serving cell are within the same band with a bandwidth of 20MHz.

To address the issues raised in Observation 2, we think the legacy NB-IOT multi-carrier cell framework can be referred to for 6G cell design. For discussion reference, we temporarily name such a 6G cell as ‘SCMC’, which is a single cell with multi-carriers of the same or different bands available to be aggregated in it, as Figure 1 shows.

Figure 1: Carriers of SCMC

6G SCMC may have some functional similarity as NB-IOT multi-carrier cell, e.g., SIB is transmitted only on anchor carrier for achieving frequency-domain NES gain, and paging/RACH load can be offloaded to non-anchor carriers, as illustrated in Figure 2. In SCMC, fragmented spectrums can be utilized as a non-anchor carrier without SIB transmission. In this case, the non-anchor carrier may serve like SUL/SDL.

Figure 2: Functions of Carriers in SCMC

As for connected mode operation, in addition to SUL/SDL switching mechanism, we believe that at least simultaneous transmission/reception on multiple carriers within one SCMC cell should also be supported for a single UE, which is like the carrier aggregation (CA) function in LTE/NR. Cross-carrier scheduling mechanism developed in LTE/NR can be studied as a start for SCMC connected mode operation.

In sum, we propose RAN2 to initiate the discussion on how to characterize and define the basic function of 6G SCMC. From our point of view, the basic characteristics for idle mode operation include that the system information is transmitted only on the anchor carrier, and paging/RACH load can be offloaded among multiple carriers. For connected mode operation, simultaneous transmission/reception on multiple carriers should be supported.

- Study single cell with multiple carriers (SCMC) as new cell model in 6G, which is characterized by the following: 1) system information is transmitted only on anchor carrier; 2) Paging/RACH can be offloaded among multiple carriers; 3) supporting simultaneous transmission/reception via multiple UL/DL carriers for UE in CONNECTED state.

    - Access Stratum Security

Access Stratum (AS) security, as the cornerstone of communication technology, will continue to drive the development of 6G radio protocol design. The AS security is expected to comprehensively address security challenges and provide a significantly higher level of security. Based on insights from 5G systems and inputs from SA1 6G study on 6G Use Cases and Service Requirements, we provide an analysis of existing security threats and initial thoughts on potential new security requirements as follows:

AS security for System Information and Paging during RRC_IDLE state

The 5G study on security enhancements against False Base Stations (FBS), as documented in SA3 TR 33.809, identified several threats related to a UE in RRC_IDLE state camping on a rogue cell, including the following:

- UE camping on a fake base station: Attackers can spoof essential SIs (e.g., MIB, SIB1-5) to force UEs onto their malicious networks.

- Public safety threats: Fake SIBs (e.g., SIB 6-8) can be used to transmit false public safety alerts, such as ETWS and CMAS messages, causing social panic.

- Service attacks: SIBs related to services like MBMS SIB20/24, V2X SIB12/13/14, ProSe SIB12, and NTN SIB19/25 etc. can be manipulated to disrupt those services.

Criminals deployed portable fake base stations are real examples that caused significant damage to cellular networks and their users. These security threats highlight the need for security protection for System Information to authenticate a cell. However, implementing security protection for System Information to authenticate a cell would have required significant changes to the fundamental security architecture of NR Release (Rel-15). For this reason, the relevant study was closed without conclusions due to compatibility issues. Notably, this security requirement has been reintroduced in the 6G study. As in SA1 TR 22.870, it is captured that “[PR 5.5.3.2-1] Subject to operator policy and regulatory requirements, the 6G system shall support a means for a UE to be able to distinguish a False Base Station from an authentic Base Station.”. Based on the observations above, now is an opportune time to initiate a thorough study on security protection mechanisms for System Information to address the False Base Station (FBS) issue comprehensively.

In addition to securing System Information, protecting Paging messages is also considered highly effective in preventing tampered/spoofed sensitive information, such as the Paging cause value. This security gap in 5G Paging has hindered the adoption of features that could enhance performance. One example is the EPS fallback feature. “a new EPS fallback indicator included in paging/SIB” was considered for a promising solution, which can accelerate a UE’s fallback from 5G to 4G for voice service. However, according to the LS (S3-221162), this solution was ultimately excluded because these indicators lacked security protection.

Since System Information and Paging messages are both delivered as broadcast signaling for UEs in RRC_IDLE state, their security procedure and fundamental frameworks are similar. Therefore, it would be beneficial to study them together as potential enhancements for AS security in RRC_IDLE state.

AS security for MAC CEs during RRC_CONNECTED state

The potential security threats posed by unprotected MAC CEs are becoming a significant issue. In Rel-19 LTM, the security risk associated to transmitting the NCC in plaintext via the LTM cell switch command MAC CE has been identified, and SA3 has acknowledged that in the LS (S3-251124) response to RAN2, i.e., “SA3 reached a compromise that the NCC can be transmitted to the UE in plaintext via MAC CE message, even though the unprotected MAC leads to increased attack scenarios and probability of handover failure.”. Additionally, some research has indicated that malicious attackers may obtain users’ moving trajectory by monitoring specific MAC CEs, e.g., SCell activation/deactivation MAC CE, see LS from GSMA FSAG in R2-2100003. Based on that, SA3 did some analysis and concluded that “Encrypting MAC CE is not proportionate to the risk”, and sent LS (S3-212305) reply to GSMA FSAG. To mitigate potential security threats, we believe that it is crucial for the 6GR radio protocols to support security protection for at least crucial MAC CEs.

As noted above, securing  System Information, Paging messages, and MAC CEs represents a significant evolution from existing communication technologies, potentially impacting the hardware of both the UE and BS. From a deployment perspective, introducing these security measures as mandatory features in early 6G design would be simpler, as retrofitting enhancements later would be impractical—a lesson learned from the 5G-A experience mentioned above. According to the approved LS in RP-252891 at the RAN#109 meeting, parallel discussions between RAN2 and SA3 are assumed to be underway to advance the AS security topic, with a checkpoint in place to ensure early alignment between WGs. Given this, we recommend prioritizing the analysis of potential security requirements at the AS layer to reach consensus and identify corresponding issues and impacts in RAN2, while deferring solution-level discussions until after the checkpoint. For example, to facilitate RAN2 in conducting a more effective analysis for solution evaluation, as documented in the LS R2-2208985 during the 5G FBS study, RAN2 held discussions and reached conclusions on the identified potential issues/impacts that require SA3 inputs to support security protection of System Information. These are abstracted from the LS as below:

- Size of the security information or feasible ranges for the size

- Latency requirements for the delivery of the security information

- How often and for how long the new information is expected to be sent

- Whether all System Information or some part needs to be protected

- Whether the security information should be updated whenever any of the SIB contents change

Similar methodology as above can be applied to security protection of Paging, as is used for System Information. Additionally, with respect to supporting security protection of MAC CEs, RAN2 may initiate discussions to reach a consensus on potential issues/impacts that require SA3 inputs to support the new security feature. We provide some initial considerations on potential issues/impacts, which may not be exhaustive:

- Whether all UL and DL MAC CEs or only some (e.g., focus on MAC CEs for LTM) need to be protected

- Whether security function for MAC CEs is always activated, or can be activated/de-activated on-demand

- Whether security function for MAC CEs is performed via PDCP-layer or MAC-layer

- RAN2 assumes that security protection for System Information, Paging and MAC-CEs (at least those for LTM) is required as per the requirement confirmed by previous SA3 LS, and considers studying corresponding issues/impacts.

  - Feature-Specific RRC Enhancements

### 2.2.1 eMBB and 6G IoT

As mentioned in our company paper [5], in the era of 6GR, the mobile system will provide its services to a myriad of devices, from low-end IoT devices to high-end advanced eMBB devices, and the design of 6GR should natively support various device types by a single 6G-RAT as described in SID [1], instead of specifying multiple individual RATs for different types of devices, like NB-IoT and LTE.

To achieve a harmonized design for IoT devices and eMBB devices, one issue that needs to be studied is how to transmit configurations/signalling for different device types. In LTE, MIB/SIB transmission is transmitted without continuous repetition, while NB-MIB/NB-SIB with continuous repetition is specified to enable wide coverage for NB-IoT in a later release. However, this independent system information transmission mechanism is inefficient, since some common parameters with the same value are separately configured, e.g., cell selection and cell access related info. As per the discussion in the last RAN1 meeting, the coverage gap for IoT service and eMBB service in 6G is at least 10dB, and there is consensus that coverage enhancement is needed for IoT devices. To achieve this, repetition-based transmission is regarded as the most efficient solution for coverage enhancement will be introduced in 6G. Therefore, system information transmission with different repetition numbers for different device types is needed, which is similar to that in LTE and NB-IoT. However, given that the independent system information transmission mechanism defined in LTE and NB-IoT is inefficient, RAN2 needs to study efficient system information transmission to enable SI repetition and avoid redundant transmission. For example, the reduction of redundant transmission can be realized by avoiding unnecessary repetition and independent transmission for the common parameters/configuration. One example is illustrated in the following figure, the configurations in SIB1 can be classified into two parts and transmitted independently, i.e., the first part named basic SIB1 which includes the common configurations shared by all devices and is transmitted with repetitions, and the second part named additional SIB1 which includes specific configurations for eMBB device only and is transmitted without repetition or less repetitions. From UE perspective, IoT device can read basic SIB1 only, while eMBB device needs to read both basic SIB1 and additional SIB1 and combine them.

Figure 3: Illustration of essential SI transmission for diverse device types

Moreover, congestion control with the consideration of different coverage targets to support different device types also needs to be studied. In NR, besides cell barring indicated by MIB and SIB1,  unified access control, which is based on access identity and access category, is also specified to achieve congestion control. However, neither the UAC nor the cell bar mechanism has considered coverage level. In contrast, UE category-based and CE level-based Cell Barring are introduced to improve access/load control in NB-IoT. As mentioned above, there is a large coverage gap for the eMBB and IoT coverage target, i.e., there are multiple coverage levels, so the coverage level-based congestion control can be considered in 6G system design. Different from NB-IoT, which only considers coverage level-based cell bar, RAN2 also needs to study coverage level-based UAC for congestion control, which is more friendly compared with cell bar. For example, as mentioned in our company paper[5], deep coverage is regarded as one of the common features, and the eMBB device may utilize this feature when it is in poor coverage. If Barred is indicated for the corresponding coverage level of an eMBB UE in poor coverage, the eMBB UE cannot access this cell for a long time. In contrast, there is still a probability to access this cell via UAC mechanism, since the UAC mechanism is based on access probability and access prohibit timer.

- Study a harmonized design for various device types with different target coverage, considering the following aspects:

- Signalling efficient system information transmission to avoid redundant transmission, e.g., common configurations in SIB1 which are shared by different device types shall not be separately transmitted.

- Coverage level-based congestion control, e.g., UAC and/or Cell Bar.

### 2.2.3 UE Power Saving

Efficient UE power saving has been a consistent target in mobile system design. With the transition towards 6G, this issue becomes even more prominent due to wider range of device types (e.g. Smart phone, XR, industrial IoT, metaverse, etc.), and the increasingly diverse set of services (e.g. eMBB, XR/immersive, Massive IoT, Sensing, AI, Voice, Regulatory service, etc.), based on the 6G study as shown in [6], that demand extended device lifetimes, improved energy efficiency, and seamless connectivity.

- In 6G Day-1, UE power saving should be treated as a baseline requirement.

Paging reception is always one of the dominant contributors to UE power consumption in RRC_IDLE state. Therefore, efficient paging power saving mechanisms are essential from Day-1 in 6G.

In 5G, multiple features were introduced to address paging-related power consumption. Paging Early Indication (PEI) is specified in Rel-17, while Low-Power Wake-Up Signal (LP-WUS) was further studied and specified in Rel-18/19. Both were intended to reduce UE power consumption for paging monitoring, but in practice, the introduction of multiple duplicated functionalities created additional complexity. From an implementation and deployment perspective, having several features targeting the same problem has increased the burden on UEs and gNBs, without delivering fully consistent gains across all scenarios.

Besides, between these two functionalities, LP-WUS was shown to provide significant benefits in terms of paging monitoring efficiency, with a dedicated low-power wake-up receiver (WUR). However, LP-WUS was introduced relatively late in 5G (Rel-18/19), which limited its early adoption and integration into the ecosystem. In contrast, PEI provided only limited improvements and became largely overshadowed by LP-WUS. This indicates that a strong, unified, and effective solution is preferable from the beginning of a new generation, rather than incremental and overlapping mechanisms.

- In 5G, multiple duplicated features for paging power saving (e.g., PEI and LP-WUS) increased complexity while providing limited incremental benefit. A single unified solution for efficient paging power saving should be prioritized in 6G Day-1, avoiding the delays seen in 5G.

For LP-WUS, another important aspect is inter-band operation. In 5G discussion in Rel-19, it was acknowledged that LP-WUS may not be supported in all main radio (MR) bands, but standardization of cross-band solutions (e.g., monitoring LP-WUS on one band while completing paging reception and subsequent RACH procedures on another band) was not completed due to limited time. This gap reduced the applicability of LP-WUS in real multi-band deployment scenarios. For 6G, such cases are expected to be even more common, given the increased diversity of spectrum usage (from sub-6 GHz to higher bands). Addressing inter-band paging and RACH procedures is therefore essential.

Finally, paging reception power saving spans both physical-layer signaling (LP-WUS/SS signaling design, receiver type design, detection reliability) and higher-layer procedures (RRC state, paging reception, RACH procedure). This requires close coordination between RAN1 and RAN2, and potentially with RAN4 on RF performance, e.g., sensitivity. Without such coordination, there is a risk of fragmented solutions, as seen in the split introduction of PEI and LP-WUS in 5G. Besides, whether LP-WUR could support all MR bands highly depends on the device type design in RAN1, i.e., OOK OOK-based or OFDM-based LP-WUR. Thus, the discussion on inter-band operation also needs inputs from RAN1.

As discussed above, UE power saving is one of the key design targets for 6G, especially given the expected diversity of device types and service requirements. While features such as LP-WUS/WUR can reduce UE power consumption in paging reception in RRC_IDLE state, another major source of UE power consumption in RRC_IDLE state remains the periodic RRM measurement.

Although the RRM measurement framework for 6G has not yet been discussed/determined, it is highly likely that UEs will still be required to perform periodic measurements (most likely on both serving and neighboring cells) to ensure mobility operation, similar to 5G.

To guarantee mobility, UEs are required to perform periodic measurements on the serving cell and neighboring cell (in case the neighboring cell measurement condition on S(non)IntraSearchP and S(non)IntraSearchQ are fulfilled). This requirement applies even when the UE skips paging monitoring, as the UE still needs to wake up to perform RRM measurements. As a result, even if paging related power saving mechanisms are introduced, the UE still consumes significant power due to periodic RRM measurement, which limits the overall power saving gain.

Similarly, in RRC_CONNECTED state, a UE is required to periodically perform cell-level and beam-level measurements for RRM, RLM, and BFD. Without measurement relaxation or offloading, the benefit of power saving features (e.g., PDCCH monitoring relaxation) is significantly reduced.

- 6G UEs most likely will continue to require periodic RRM measurements for mobility, and without RRM measurement relaxation/offloading mechanisms, the power saving benefits of paging-related features in RRC_IDLE (e.g., LP-WUS/WUR) or PDCCH monitoring in RRC_CONNECTED state will be significantly reduced.

5G introduced RRM measurement relaxation and offloading mechanisms progressively, with different applicability and varying designs across multiple releases:

- Rel-16: Introduced RRM measurement relaxation on neighboring cells for RRC idle/inactive.

- Rel-17: Introduced RRM measurement relaxation RRC_CONNECTED state, and RLM/BFD relaxation.

- Rel-19: With the introduction of LP-WUS and corresponding LP-WUR measurement on LP-SS, further enhancements on RRM measurement relaxation/offloading were made.

The RRM measurement relaxation mechanisms in RRC idle/inactive generally follow this principle: the network configures criteria for RRM measurement relaxation/offloading, the UE evaluates whether the criteria are met, and if so, performs measurements according to relaxed requirements defined by RAN4. The RLM/BFD measurement relaxation mechanism has a similar design, but the relaxation status needs to be reported to network side. While the RRM measurement relaxation in RRC_CONNECTED state is quite different: the network configures criteria for RRM measurement relaxation/offloading, the UE evaluates whether the criteria are met, and if so, reports to network and leaves the network to reconfigure the measurement.

It could be found that different criteria, relaxation factors, and applicability to different UE types (e.g., all UE, or only (e)RedCap/LP-WUS UEs) were standardized across releases, leading to fragmentation.

Besides, each release in 5G only standardized a subset of potential use cases, leading to incomplete use cases and lack of uniformity. For example:

- Some releases addressed only some RRC states or only on serving/neighboring cell measurement cell, but not both.

- Certain mechanisms applied only to specific UE types (e.g., (e)RedCap/LP-WUS UE only), leaving other device categories without equivalent support.

This piecemeal approach meant that a complete and consistent framework for RRM measurement relaxation was never achieved within 5G. Fragmented standardization of relaxation mechanisms across 5G releases highlights the need for a unified approach in 6G.

- In 5G, RRM relaxation was introduced with varying criteria, relaxation factors, and applicable use cases or UE types across releases, resulting in inconsistent designs. This fragmented approach led to inconsistency in applicability and configuration.

Power saving has proven to be a critical feature for UE performance. However, in 5G, the power saving gain from RRM measurement relaxation/offloading and RLM/BFD relaxation only became available in later releases. Without these mechanisms, the power saving functions for paging reception in RRC idle/inactive or for PDCCH monitoring in RRC connected were significantly less effective, as measurement activity dominated UE consumption.

Thus, in 6GR, RRM measurement relaxation/offloading should be considered from Day-1, ensuring that the power saving functionality in all RRC states can achieve its intended benefits.

Finally, designing a unified RRM measurement relaxation/offloading and RLM/BFD relaxation for 6G requires coordinated work across different RAN WGs, e.g.

- RAN1: Study L1 measurement aspects, including timing, measurement filtering, and physical-layer signaling;

- RAN2: Study criteria and procedures for relaxation/offloading, and study the impact on mobility performance;

- RAN4: Study corresponding performance requirements.

- The 6GR should study the UE power saving techniques, including:

- Aim to support LP-WUS/WUR for paging reception from Day-1. Similar design in 5G NR is reused as the baseline. Inter-band/carrier operation between LP-WUS monitoring (and possible paging) and MR operations needs to be studied. Coordination with RAN1 is needed.

- Aim to support RRM measurement relaxation/offloading and RLM/BFD relaxation for power saving purposes on serving and neighboring cells for all RRC states with a unified design, including criteria and mechanism, from Day-1.  Coordination with RAN1 (and possible RAN4) is needed.

    - Network Energy Saving

In Rel-18, the study of Network Energy Saving (NES) is performed in 3GPP to develop a more energy friendly network, to help the operators achieve a better balance between the increasing data traffic and the operation cost.

Various types of NES techniques were raised and discussed in the Rel-18 SI phase, covering frequency, time, power, and spatial domain features, and summarized in [4]. Several NES features were specified in Rel-18 and Rel-19 WI phase, as listed below:

Rel-18 NES:

- inter-band SSB-less SCell;

- cell DTRX for RRC_CONNECTED state (including SIB1 NES barring bit for cell DTRX);

- NES-based CHO;

- Spatial and power domain adaptation with sub-CSI report;

- beam-level restricted paging;

Rel-19 NES:

- OD-SSB on SCell;

- OD-SIB1 by UE request;

- SSB adaptation;

- RACH adaptation;

- Clustered PO configuration;

The above NES features are prioritized to be specified in NR, as the NES gains of them can be observed and further proven by some simulation results, and these features are compatible with NR Day-1 physical layer design. Considering the operators’ consistent interest in controlling the network operation cost with NES, and the need for power saving from the UE perspective too, we think NES should be considered on 6GR Day-1 with most of the NR-specified NES features inherited to 6GR. There may be some further enhancements to these NES features in 6GR due to potential new framework design, and we can adapt accordingly.

Figure 4: Example of standalone OD-SIB1

As for 6GR NES, we think OD-SIB1 with UE request mechanism in NR Rel-19 can be further improved and extended to other scenarios from RAN2 perspective. In NR Rel-19, OD-SIB1 request configuration can only be obtained by broadcasting or dedicated signalling. The disadvantage for Rel-19 OD-SIB1 is that the deployment of OD-SIB1 cell mandates the network to ensure there is at least a cell-A with overlapping coverage with the OD-SIB1 cell. Otherwise, the UEs that have just switched to power on cannot obtain the OD-SIB1 request configuration from an OD-SIB1 cell itself if the SIB1 is not being broadcast and thus cannot trigger OD-SIB1 request procedure. Therefore, only capacity cells may apply Rel-19 OD-SIB1 mechanism for NES purposes. In 6GR, we think the OD-SIB1 mechanism should be extended to the scenario where the UEs can directly obtain the OD-SIB1 request configuration from the SSB of the OD-SIB1 cell, so that the UEs do not need to rely on an underlay cell (cell-A) to obtain the OD-SIB1 request configuration of the OD-SIB1 cell, as Figure 4 shows.

Another shortcoming of Rel-19 OD-SIB1 request operation is that the OD-SIB1 cell still transmits always-on SSB, and thus, when the SSB periodicity is short, the NES gain from OD-SIB1 operation may be limited. Therefore, we think further NES gain may be achieved by the NES cell transmitting SSB in an on-demand manner too. In this case, the NES cell does not transmit always-on SSB. The UE relies on cell A’s SSB for time/frequency tracking and then requests OD-SSB & OD-SIB1 in the NES cell if necessary. We propose RAN2 to study cell A-assisted OD-SSB&OD-SIB1 in 6GR NES, as Figure 5 shows.

Figure 5: Example of OD-SSB & OD-SIB1

In sum, we propose RAN2 to study 6GR NES with inheriting most of the 5G NES features, and extending the NR OD-SIB1 mechanism to more scenarios for further NES gain.

- From RAN2 point of view, to study 6GR NES with:

- NR specified NES features, including cell DTRX for connected mode, NES-based CHO, RACH adaptation, clustered PO configuration, with enhancement in 6GR if necessary;

- Extending NR OD-SIB1 with UE request mechanism to cover: 1) standalone OD-SIB1 (e.g. the UE can obtain OD-SIB1 request configuration from SSB of the OD-SIB1 cell); 2) cell A-assisted OD-SSB&OD-SIB1 (i.e. cell A provides the OD-SSB&OD-SIB1 request configuration of the NES cell).

    - NTN

NTN scenario is defined by extremely large cell coverage ranges, where beams within a single cell serve vastly different geographic areas with divergent link conditions and traffic load. Key challenges in NTN regarding the CP configuration aspect include:

- Link budget discrepancies: there is a link coverage gap between the near and farther beam coverage areas in an NTN cell (e.g., the gap between the nearest and farthest beam can be up to 9dB served by a LEO-600 satellite with a 10° elevation angle). This gap imposes conflicting requirements on coverage configuration parameters. For example, UEs in far-beam areas require a higher PRACH repetition, while UEs in near-beam areas only need a lower repetition. Uniform cell-specific RACH parameter configuration, as in the NR way, would either result in unacceptably low RACH success rates (if only a lower repetition number is configured) for far-beam UEs or lead to inefficient resource usage of resources associated with a higher repetition number in the near-beam area.

- Heterogeneous neighbor cell deployment: different beam areas in an NTN cell may face different neighbor cell layouts (e.g., some beam areas may border multiple adjacent NTN cells on different frequency layers, while others have sparse neighbor coverage). Consequently, different beam areas require distinct measurement configurations for cell re-selection. Cell-specific measurement parameters cannot adapt to these variations.

- Beam hopping needs: NTN satellites operate under strict energy constraints, making it impractical to activate all beams simultaneously. Instead, beams must be activated in a TDM manner (i.e., beam hopping). Therefore, UEs in different beams require explicit knowledge of their serving beam’s activation time patterns to align their transmission and reception with the beam’s active time. A cell-level uniform configuration would fail to capture beam-specific hopping schedules.

Legacy NR systems are designed around cell-specific parameter configuration, which is incompatible with 6G NTN’s beam-heterogeneous requirement. To address the observed challenges, the network should be able to set the specific needs of each beam, meeting 6G’s objectives of reliable, efficient, and harmonized connectivity. Thus, flexible parameter configuration across beam/beam groups should be studied as a potential feature in 6G technical specifications.

- RAN2 to study flexible configuration across beam/beam groups (e.g., support beam-specific SI).

- Conclusion

In this paper, we discuss 6G radio control plane. we have the following observations and proposals:

RRC States

- In NR, the RRC_INACTIVE state was introduced as a mandatory 5G Day-1 UE feature to meet the control plane latency requirement, while few operators have enabled this feature due to additional CAPEX and OPEX.

- For 6GR, the RRC_IDLE and RRC_CONNECTED should remain as mandatory RRC states. However, the RRC_INACTIVE state, if supported in 6G, should be defined as an optional feature.

Cell Model

- For 6GR cell model design, the following issues should be considered on how to: 1) utilize fragmented spectrum; 2) achieve more NES gain; 3) enlarge RACH/Paging capacity in densely populated areas.

- Study single cell with multiple carriers (SCMC) as new cell model in 6G, which is characterized by the following: 1) system information is transmitted only on anchor carrier; 2) Paging/RACH can be offloaded among multiple carriers; 3) supporting simultaneous transmission/reception via multiple UL/DL carriers for UE in CONNECTED state.

Access Stratum Security

- RAN2 assumes that security protection for System Information, Paging and MAC-CEs (at least those for LTM) is required as per the requirement confirmed by previous SA3 LS, and considers studying corresponding issues/impacts.

eMBB and 6G IoT

- Study a harmonized design for various device types with different target coverage, considering the following aspects:

- Signalling efficient system information transmission to avoid redundant transmission, e.g., common configurations in SIB1 which are shared by different device types shall not be separately transmitted.

- Coverage level-based congestion control, e.g., UAC and/or Cell Bar.

UE Power Saving

- In 6G Day-1, UE power saving should be treated as a baseline requirement.

- In 5G, multiple duplicated features for paging power saving (e.g., PEI and LP-WUS) increased complexity while providing limited incremental benefit. A single unified solution for efficient paging power saving should be prioritized in 6G Day-1, avoiding the delays seen in 5G.

- 6G UEs most likely will continue to require periodic RRM measurements for mobility, and without RRM measurement relaxation/offloading mechanisms, the power saving benefits of paging-related features in RRC_IDLE (e.g., LP-WUS/WUR) or PDCCH monitoring in RRC_CONNECTED state will be significantly reduced.

- In 5G, RRM relaxation was introduced with varying criteria, relaxation factors, and applicable use cases or UE types across releases, resulting in inconsistent designs. This fragmented approach led to inconsistency in applicability and configuration.

- The 6GR should study the UE power saving techniques, including:

- Aim to support LP-WUS/WUR for paging reception from Day-1. Similar design in 5G NR is reused as the baseline. Inter-band/carrier operation between LP-WUS monitoring (and possible paging) and MR operations needs to be studied. Coordination with RAN1 is needed.

- Aim to support RRM measurement relaxation/offloading and RLM/BFD relaxation for power saving purposes on serving and neighboring cells for all RRC states with a unified design, including criteria and mechanism, from Day-1.  Coordination with RAN1 (and possible RAN4) is needed.

Network Energy Saving

- From RAN2 point of view, to study 6GR NES with:

- NR specified NES features, including cell DTRX for connected mode, NES-based CHO, RACH adaptation, clustered PO configuration, with enhancement in 6GR if necessary;

- Extending NR OD-SIB1 with UE request mechanism to cover: 1) standalone OD-SIB1 (e.g. the UE can obtain OD-SIB1 request configuration from SSB of the OD-SIB1 cell); 2) cell A-assisted OD-SSB&OD-SIB1 (i.e. cell A provides the OD-SSB&OD-SIB1 request configuration of the NES cell).

NTN

- RAN2 to study flexible configuration across beam/beam groups (e.g., support beam-specific SI).

- Reference

- RP-252912	Revised SID: Study on 6G Radio.

- 3GPP TR 38.914 (V0.2.0)		“Study on 6G Scenarios and Requirements”.

- 3GPP TS 36.331: "Evolved Universal Terrestrial Radio Access (E-UTRA); Radio Resource Control (RRC) protocol specification".

- TR 38.864	 Study on network energy savings for NR.

- R2-2506797 Discussion on 6G General aspects, vivo.

- RP-252960	 pCR for 38.914 on Requirements of new and existing services for 6G, CMCC.