3GPP TSG RAN WG2 Meeting #131bis         	                          R2-2506808

Prague, CZ, Oct 13th – 17th, 2025

Agenda Item:	10.3.1

Source:	Xiaomi

Title:	General considerations on RAN2 6G UP design

Document for:	Discussion and Decision

# Introduction

In RAN#108 meeting, RAN WG study item on “Study on 6G Radio” [1] was approved with the following scope highlighted in yellow regarding to 6G user plane:

- Radio interface protocol architecture and procedures for 6GR [RAN2, RAN1, RAN4, RAN3] - User Plane architecture and protocol design [RAN2] - Control Plane architecture (including RRC states) and protocol design [RAN2, RAN3] - Access stratum security aspects, in alignment with requirements from SA3 [RAN2] - Radio signalling framework for UE capabilities, aiming at improvements and simplification compared to 5G NR [RAN2, RAN1, RAN4] - Data transfer design to support various types of data (e.g. AI/ML, Sensing, etc) [RAN2, RAN3]

In this contribution, we provide our views on 6G UP design.

# Discussion

## SDAP/PDCP/RLC

NR SDAP/PDCP/RLC function redundancy issue:

NR user plane processing flow is captured in TS 38.300 as below:

Figure 1: TS 38.300 Data Flow Example

Following is the list of functionalities in each NR user plane layer:

- SDAP

  - QoS flow to DRB mapping

- PDCP

  - Sequence number, related window management, reordering

  - Ciphering  / integrity protection

  - Header compression (ROHC, EHC)  / UDC

- RLC

  - Sequence number and related window management

  - Segmentation / reassembly

  - ARQ

- MAC

  - Multiplexing (including concatenation), logical channel prioritization, …

It can be seen that there are some duplicated functions (highlighted in cyan) across PDCP and RLC. In addition, the functional split between SDAP and PDCP is artificial to some extent, and there is already layer breaking in 5G, e.g. SDAP header and SDAP control PDU are not ciphered in PDCP, which implies that PDCP needs to be aware of the configuration of SDAP as well as type of SDAP PDUs (detailed RAN2 discussion can be found on R2-1713727 in RAN2#100 meeting). For 6G, there is an opportunity to study how to distribute functionalities into protocol layers and redundant functions can be removed to reduce the user plane complexity. In addition, user plane protocol split option should be aligned with RAN internal function split (e.g. CU-DU split), e.g. real time operations like segmentation and multiplexing should be located in the same network function and therefore one possibility is to move segmentation to MAC layer if CU-DU split is supported.

[Lesson from 5G-1] 5G protocol layer splitting results in duplicated functionalities across layer and cross layer dependency.

HOL blocking issue inside one DRB:

Nowadays, we notice an increasing number of Apps using QUIC protocol, like facebook, google apps( e.g. YouTube), Apple ios, due to its ability to resolve the HOL blocking issue of TCP, as well as its advantage in reducing connection setup delay, and more flexible congestion control algorithm.

However, as the header of QUIC is encrypted, RAN has no idea about the QUIC stream. As a result, different QUIC streams may still be mapped to the same DRB, making the advantage of QUIC in avoiding HOL blocking useless. Besides, in the real commercial scenario, different flows from one App like WeChat, or even different connections from one UE are actually carried in the same DRB, due to the fact that there is no coordination between operator and the app service provider on the QoS profile.

Given this situation, 6G should study how to resolve the HOL blocking issue inside one DRB.

Figure 2: HOL Blocking issue inside one DRB

[Lesson from 5G-2] In real word scenario, different connections/flows of UE are mapped to the same DRB, 5G DRB's single queue operation will lead to HOL blocking issue.

Slow ARQ retransmission issue:

In 5G, ARQ retransmission relies on RLC status report, which is triggered either by t-Reassembly expiry or Polling. t-Reassembly configuration is generally large, wait for its expiry introduces a lot of latency. Polling in theory can reduce the latency of RLC status report, but it comes at the price of significant signalling overhead of frequent RLC status report. In 6G, we should consider how to enable fast ARQ retransmission. One possible way to do this is based on HARQ transmission status.

For uplink transmission, the HARQ ACK/NACK is not provided to UE by gNB. Instead, network will schedule a retransmission if decoding fails. After reaching a maximum retransmission time, network will give up the retransmission and schedule a new transmission. If network can provide this HARQ failure information to UE, UE can use this information to trigger ARQ retransmission.

For downlink transmission, UE sends ACK/NACK feedback to network. However, ACK/NACK may flip. If that happens, network will stop HARQ retransmission and rely on ARQ retransmission to resolve it. In this case, UE will receive a new transmission scheduling following a NACK indication. If UE can tell this situation to network, it will facilitate network fast ARQ retransmission.

[Lesson from 5G-3] 5G ARQ retransmission relies on RLC status report, which is not in a timely manner.

Low transmission efficiency in supporting URLLC:

URLLC is one of the three main scenarios to support in 5G, along with eMBB and mMTC. The essential requirement of URLLC is to satisfy high reliability and low latency simultaneously. To meet such requirement, PDCP duplication is introduced in Rel-15 NR to handle the case that physical layer techniques alone cannot meet the high reliability target when ARQ cannot be used due to low latency requirement. In PDCP duplication, duplicated PDCP PDUs are transmitted in two legs, which were further extended to 4 legs in Rel-16.

In 6G, HRLLC is the evolution of URLLC and one of the 6 scenarios of IMT-2030. Similar requirements of high reliability and low latency is defined for HRLLC in TR 38.914: 10-5 error probability and 1 ms user plane latency. It is expected that PDCP duplication based techniques will be used for 6G HRLLC scenarios. One potential enhancement is to consider network coding as a replacement/generalization for PDCP duplication. As a generalization of PDCP duplication, which is essentially a repetition coding, network coding can provide finer granularity for the trade-off between the resource usage and reliability, therefore is able to improve the spectrum efficiency while satisfying latency and reliability requirements.

Theoretical PER performance of network coding vs. PDCP duplication is shown in Figure 1 below. Denote the BLER after HARQ for each TB is p, then the overall packet error rate after network coding (including PDCP duplication as special case) for (n, k) coding is PER = i=0k-1ni1-pipn-i assuming independent error probability in different paths/legs.

This figure shows the tradeoff that the smaller the value of k / n, the better the reliability performance (PER), the larger the resource usage (due to more redundancy (n – k) / n). It can be seen that network coding can provide finer granularity of radio resource usage compared with PDCP duplication.

Figure 3: Theoretical Performance of Network Coding vs. PDCP duplication

[Lesson from 5G-4] PDCP duplication is not resource efficient if required coding rate is higher than 1/2.

Based the above analysis, we have the following proposal:

Proposal 1: 6G L2 design should consider to study:

- Fast ARQ retransmission;

- Resource efficient transmission in supporting HRLLC;

- HOL blocking issue of flows inside one DRB;

- User plane protocol split options with consideration of removing redundant functions and supporting RAN internal function split.

## MAC

RACH:

1) 4-step RA and 2-step RA

In release 16, 2-step RA was introduced on top of 4-step RA for the following two reasons:

-	To reduce the number of interactions between the UE and network during the connection setup and connection resume, thereby enabling a lower control plane latency for IDLE and INACTIVE UEs.

-	In case of connected mode, a small amount of data can be sent over the 2-step RACH channel thus enabling a lower latency for UL UP data for connected mode UEs.

As the latency benefit of 2-step RA turns out to be very useful for later introduced NTN feature in Rel-17, it is sensible to continue supporting low latency RA in 6G, as NTN is also an important 6G feature.

However, the drawback of 2-step RA is its relatively smaller coverage compared with 4-step RA, as MsgA PUSCH payload does not have TA adjustment as msg3, which results in higher PUSCH interference at cell edge. As 6G needs to provide competent coverage than 5G, we still need to ensure the coverage of RA is no worse than 5G.

[Lesson from 5G-5] 2-step RA has relatively smaller coverage compared to 4-step RA but can significantly reduce RA latency.

2) Fragmented coverage enhancements for RACH

The coverage enhancement for RA procedure in NR spanned multiple releases, leading to fragmented enhancements:

- In Rel-17, msg3 repetition was introduced in CE;

- In Rel-18, msg1 repetition was introduced in R18 CE, msg4 ACK repetition was introduced in R18 NTN coverage enhancement;

- In Rel-19, Msg4 repetition was introduced in R19 NTN coverage enhancement;

- In Rel-20, msg1 repetition with Tx beam switch will be supported.

As these features were introduced in different release, it results the following issues:

- Separate capability indication for msg3 repetition, msg4 repetition, msg4 ack repetition;

- Separate RSRP threshold for msg1 repetition, msg3 repetition, msg4 ack repetition;

  - R19 NR NTN DL coverage enhancement decides to not introduce RSRP threshold for msg4 repetition indication as network can decide on repetition based on msg 1/2/3 reception.

  - For msg3/msg4 ack repetition, RSRP threshold is similar and is only used to reduce the amount of capability indication, one unified RSRP threshold as msg1 repetition 1 or 2 should be sufficient.

- Separate RACH resource for UE w/wo msg1 tx beam switch capability

- Defined as Separate features results in no fallback among these features and segmentated RACH resources.

In 6G, coverage enhancement is envisioned to be pivotal important for IOT and NTN, we should strive to have a unified solution for RACH coverage enhancement.

[Lesson from 5G-6] 5G coverage enhancements for RACH are patched across multiple releases, resulting fragmented design.

3) RACH partitioning

In Rel-17, due to the fact that a lot of features require separate RACH resources for purposes like resource isolation, early indication, resource separation, RACH partitioning was introduced, and the following features are supported:

- Slicing;

- (e)RedCap;

- Msg3 repetition;

- Msg1 repetition;

- SDT.

To support 2-step RA, separate RACH resources are also introduced compared to RACH resources for 4-step RA.

In 6G, most of these features will anyway be supported sooner or later, it would be better we can natively support RACH resource partitioning from day-1. Considering the extensibility of 5G RACH partitioning, it can be reused as baseline.

[Lesson from 5G-7] 5G RACH partitioning feature is very flexible and extensible and turns out to be very useful for many features.

Based on the above analysis, we have the following proposal regarding random access procedure:

Proposal 2: 6G random access design should consider the following:

- RA coverage should be no smaller than 5G RA;

- Support low latency RA;

-Support unified design for coverage enhancement;

-Support RACH resource partitioning.

Power saving:

Power consumption is always a major concern in any generation of mobile communication systems for user equipment (UE) and base station (BS). In 5G, many time domain power saving schemes have been specified to reduce UE’s PDCCH monitoring, e.g. C-DRX and its variant enhancements (e.g. DCP, LP-WUS), PDCCH skipping, SSSG switching, etc., and some scheme even requires additional hardware such as low power receiver. For base station energy saving, time-domain cell DTX/DRX has been specified in NR Rel-18. It is noted that these UE power saving and network energy saving solutions are specified in different releases and the later specified one can just try to co-ordinate with the previous one. For example, cell DTX/DRX pattern cannot be perfectly aligned with UE’s C-DRX pattern. In 6G, UE power saving and network energy saving should be jointly considered in Day 1 when designing time-domain power saving mechanism and solutions should try to avoid too many variations and should be hardware-friendly.

[Lesson from 5G-8] 5G introduces multiple time domain power saving techniques separately for UE or network, resulting in non-universal, complexed and duplicated solution.

Proposal 3: 6G should support a unified time-domain joint UE&NW power saving mechanism from day-1 considering both UEs w/wo additional hardware capability.

MAC CE：

5G MAC has defined a lot of MAC CEs for different purpose and for different cases, which is not in an extensible way, e.g.:

-	For BSR, we introduce 5 different BSR formats to accommodate different number of LCGs, length of LCG ID; We also introduce DSR to extend with delay field.

-	For PHR, we introduce 10 different PHR formats to accommodate different number of entries, extension of fields, etc.;

-	Total 83 type of MAC CEs are defined for different purpose, let alone different MAC CE format for one type of MAC CE.

For each new introduced MAC CE format, RAN2 need to invest a lot of time to design the format, which is not efficient. RAN2 should study an extensible MAC CE format, which allows easy extension. One possible way to do this is to utilize ASN.1 format. ASN.1 Unaligned PER (Packed Encoding Rules) operation as adopted by RRC are optimized for bandwidth efficiency by omitting redundant tags and length fields, reducing payload size and decoding time. Compared to MAC CE, the encoding/decoding complexity of ASN.1 PER mainly comes from:

1. Schema Complexity: Deeply nested structures (e.g., SEQUENCE OF SEQUENCE), CHOICE types (dynamic field selection), and OPTIONAL elements introduce iterative decoding steps. For example, a RRCConnectionSetup message with multiple nested SEQUENCE types requires recursive parsing, increasing latency.

2. Message Size: With message size increase, the complexity of the ASN.1 structure also increases, e.g. a lot of nested structures, optional fields, variable length fields (e.g. OCTET STRING). The required constraint verifications increase accordingly.

By minimizing the nested structure ( e.g. by only allowing 2-nested structure), message size (e.g. limit within hundreds of bits), the number of optional fields, and the number of variable length fields, the encoding/decoding complexity of ASN.1 PER can be largely reduced, comparable to MAC CE processing time.

[Lesson from 5G-9] 5G MAC CE design doesn't allow flexible extension, resulting in a lot of specification work.

Proposal 4: 6G should study extensible MAC CE format.

## Improved QoS

Service Aware QoS handling and finer QoS granularity

In 5G, QoS flow represents the smallest QoS differentiation unit. While 5G introduced "XR-Awareness" for gNB radio resource optimization and defined PDU Set-level QoS metrics (e.g., PDU Set Error Rate, PDU Set Delay Budget), these parameters are provided to gNB on a per-QoS Flow basis in semi-static manner. PDU set and PSI can be identified by CN and UE for DL and UL, respectively. The only QoS differentiation of different PDU sets within the same QoS flow is the PDU set based discard based on PSI, in case of congestion. In normal communication (i.e. other than congestion), gNB does not apply different QoS treatment of different PDU sets within the same QoS flow.. In 6G, immersive communication is supposed to be the mainstream for future wireless communication networks. Different immersive communication services require different composite requirements in terms of different QoS requirement. 6G should enable differentiated QoS fulfilment for distinct PDU Set types within a single QoS Flow, achieving finer-grained QoS control. 6G network also needs to be able to dynamically adjust traffic handling based on service context to enhance QoE and optimize resource utilization.

[Lesson from 5G-10] 5G doesn't allow differentiated QoS fulfilment for distinct PDU set types within a QoS flow.

Multi-modality

5G discussed multi-modal systems integrating inputs/outputs across multiple sources/destinations for richer experiences. For example in immersive XR applications, input can come from multiple sources (e.g. movement of AI glasses, haptic / sensor input from wearables like neuro band), and output can go to multiple destinations (e.g. audio / video of AI glasses, haptic feedback to wearables). Inter-modal synchronization is essential and is already defined in SA1 requirements TS 22.261. In 5G Rel-19, only basic multi-modality awareness is supported via the provisioning of Multi-modal Service ID (MMSID) from 5GC to NG-RAN when establishing and/or updating the corresponding QoS Flows. However, the inter-flow synchronization threshold is not specified. As a result, MMSID solution can only work in the case that inter-flow synchronization threshold is larger/equal to latency KPI of the application, e.g. by ensuring the latency of each flow doesn't exceed the latency KPI. If the inter-flow synchronization threshold is smaller than latency KPI of the application (as showcased in TS 22.261 below), MMSID alone cannot work well.

TS 22.261: For immersive multi-modal VR applications, synchronization between different media components is critical in order to avoid having a negative impact on the user experience (i.e. viewers detecting lack of synchronization), particularly when the synchronization threshold between two or more modalities is less than the latency KPI for the application. Example synchronization thresholds [41] [42] [43] [44] are summarised in table 6.43.1-1. Table 6.43.1-1: Typical synchronization thresholds for immersive multi-modality VR applications Media components; synchronization threshold (note 1). audio-tactile; audio delay: 50 ms; tactile delay: 25 ms. visual-tactile; visual delay: 15 ms; tactile delay: 50 ms. NOTE 1: for each media component, “delay” refers to the case where that media component is delayed compared to the other..

[Lesson from 5G-11] 5G cannot well support the synchronization dependency among QoS flows.

Tethered device support

Nowadays, electric cars are typically equipped with 4G/5G modem. Compared to smartphone, electric car is considered to have much higher capabilities, e.g. transmit power, antenna gain, battery capacity. And the 4G/5G signal received by the car is generally better than the smart phone inside the car due to penetration loss caused by the car body. As a result, it is much desirable for the devices inside the car to utilize the car mobile modem to send/receive data for e.g. internet services, or IMS services. The connection between in-car devices and the car is typically WiFi/Bluetooth, which introduces additional delays and is not taken into account in 5G DSR report and gNB scheduling. 6G should be able to ensure the QoS experience of services of the devices tethered to the car.

Figure 4: Smart phone inside the car tethered to the car 5G module

[Lesson from 5G-12] 5G scheduling doesn't take into account the delay between the UE and its tethered device.

Based on the above discussion, we have the following proposal:

Proposal 5: 6G should consider the service awareness techniques introduced in 5G for XR as a start point, and strive to further improve the QoS experience with the following consideration:

- Support Service Aware QoS handling and finer QoS granularity;

- Support the correlation requirement (e.g. sync requirement) among different QoS flows;

- Support tethered device.

# Conclusion

[Lesson from 5G-1] 5G protocol layer splitting results in duplicated functionalities across layer and cross layer dependency.

[Lesson from 5G-2] In real word scenario, different connections/flows of UE are mapped to the same DRB, 5G DRB's single queue operation will lead to HOL blocking issue.

[Lesson from 5G-3] 5G ARQ retransmission relies on RLC status report, which is not in a timely manner.

[Lesson from 5G-4] PDCP duplication is not resource efficient if required coding rate is higher than 1/2.

Proposal 1: 6G L2 design should consider to study:

- Fast ARQ retransmission;

- Resource efficient transmission in supporting HRLLC;

- HOL blocking issue of flows inside one DRB.

- User plane protocol split options with consideration of removing redundant functions and supporting RAN internal function split.

[Lesson from 5G-5] 2-step RA has relatively smaller coverage compared to 4-step RA but can significantly reduce RA latency.

[Lesson from 5G-6] 5G coverage enhancements for RACH are patched across multiple releases, resulting fragmented design.

[Lesson from 5G-7] 5G RACH partitioning feature is very flexible and extensible and turns out to be very useful for many features.

Proposal 2: 6G random access design should consider the following:

- RA coverage should be no smaller than 5G RA;

- Support low latency RA;

-Support unified design for coverage enhancement;

-Support RACH resource partitioning.

[Lesson from 5G-8] 5G introduces multiple time domain power saving techniques separately for UE or network, resulting in non-universal, complexed and duplicated solution.

Proposal 3: 6G should support a unified time-domain joint UE&NW power saving mechanism from day-1 considering both UEs w/wo additional hardware capability.

[Lesson from 5G-9] 5G MAC CE design doesn't allow flexible extension, resulting in a lot of specification work.

Proposal 4: 6G should support extensible MAC CE format.

[Lesson from 5G-10] 5G doesn't allow differentiated QoS fulfilment for distinct PDU set types within a QoS flow.

[Lesson from 5G-11] 5G cannot well support the synchronization dependency among QoS flows.

[Lesson from 5G-12] 5G scheduling doesn't take into account the delay between the UE and its tethered device.

Proposal 5: 6G should consider the service awareness techniques introduced in 5G for XR as a start point, and strive to further improve the QoS experience with the following consideration:

- Support Service Aware QoS handling and finer QoS granularity;

- Support the correlation requirement (e.g. sync requirement) among different QoS flows;

- Support tethered device.

# Reference

[1] RP-251881 New SID: Study on 6G Radio, NTT DOCOMO