3GPP TSG-RAN WG2 Meeting #131bis	R2-2506809

Prague, Czech, 13-17 October 2025

Agenda item:		10.3.1

Source:	Qualcomm Incorporated

Title:	Discussion on 6G user plane enhancements

WID/SID:	Study on 6G Radio

Document for:	Discussion and decision

# Introduction

In this paper, we discuss which releases should be considered as the baseline for 6G study and which user-plane functions should be considered for enhancements in 6G.

# Discussion

## MAC

### Baseline for 6G MAC

In addition to the legacy services such as voice and eMBB, 6G is expected to support new use cases such as mobile AI, immersive communications, etc. These new services may have new traffic profiles and/or new QoS requirements. However, they are not likely to require major changes to the essential functions/services provided by the MAC entity. This is because what they implement is the minimum set of functionalities needed to maintain an air link and transport data over it. For example,

- uplink scheduling functions such as BSR, LCP and HARQ, PHR are needed by UE to obtain UL grant, multiplex data into a TB and transmit it efficiently across the air;

- link management functions such as UL timing and BFD/R are needed to ensure UE’s uplink stay usable;

- random access is needed to recover or re-establish UE’s connection after its uplink fails;

- bandwidth management functions such as BWP and CA ensure that UE are provided with adequate, but not excessive, amount of bandwidth to send its data;

- power saving functions such as DRX enables UE to have long connection time.

Therefore, our view is that essential user plane functions should continue to be supported in 6G. Such functions include:

- Those related to random access, e.g. CBRA and CFRA;

- Those related to uplink scheduling, e.g. SR, BSR, LCP, HARQ, CG, PHR;

- Those related to link management, e.g. BFD/R, UL timing

- Those related to bandwidth management, e.g. BWP, CA

- Those related to energy savings, e.g. UE DRX and cell DTX/DRX.

Proposal 1. 	The following MAC functions are supported in 6G:

- Functions related to random access, including CBRA and CFRA;

- Functions related to UL scheduling, including SR, BSR, DSR, LCP, HARQ, CG, PHR;

- Functions related to bandwidth management, including BWP, CA;

- Functions related to link management, including BFD/R, UL timing;

- Functions related to energy savings, including UE DRX and cell DTX/DRX.

Many of these functions have had a series of enhancements over the course of 5G. To facilitate the 6G study, we think it is desirable for RAN2 to first agree on a baseline for them. The determination of the baseline (e.g. which release) can be based on at least the following factors.

First, if an enhancement was introduced for a use case that is not going to be an expected day-one feature in 6G, then we do not think it needs to be considered for the baseline. Such examples may include NR-U, sidelink, relay, etc. On the other hand, if a use case is anticipated to be a day-one feature, such as XR or immersive communication, then the related functions and its latest enhancements should be considered for the baseline.

Second, if the design of a function was or is going to be led by another working group, then its baseline should be discussed and agreed by that working group. Examples of such functions may include HARQ, BWP, CA, PHR, etc.

Lastly, the baseline for each function should be discussed and agreed individually. This is because each function was enhanced in different set of releases. For example, RACH was enhanced in every release since R15. On the other hand, LCP did not have any major enhancements between R15 and R19.

Proposal 2.  	Discuss the respective baseline for each MAC function to be supported in 6G except those led by other WGs, which include SR, BSR, DSR, PHR, LCP, CG, BFR, UE DRX, rate control, MAC CE/PDU format.

### Potential enhancements for 6G MAC

Next, we discuss potential enhancements beyond the baseline that RAN2 should study.

# RACH

In NR, RACH partitioning was introduced in R17 to support early indication of feature(s), such as RedCap, SDT, slice, coverage enhancements, etc. Each combination of features requires a separate configuration of RACH resources (aka a RACH partition). When new features that require early indication are introduced in a subsequent release, more RACH partitions need to be added. As the number of such features (N) grow over releases, the total number of required RACH partitions can grow very fast, which is O(N!). This results in poor scalability and very inefficient use of RACH resources. We therefore think this shortcoming needs to be addressed or avoided in the 6G RACH design.

Another inefficiency with the 5G RACH design is that RACH resources are provided via SIB1. Since updates to these resource configurations are based on system information modification, reconfiguring them to handle changes in access loads can be very slow. Based on field data we have collected, there are often temporary, short-term changes in RACH load, e.g. upon occurrence of some event that triggers RACH by many UEs at the same time within the same area. SIB update simply is too slow to handle such changes, resulting in high level of access contention and even failed RACH. In R19, dynamic PRACH via paging DCI was introduced to help network save energy. This enhancement can also help make RACH resources more adaptive to variations in access load. However, it can’t be used by UEs in RRC Connected and those already in RACH procedure.

Resource-efficient RACH configuration and procedure boost system capacity and save network energy. We therefore believe RAN2 should study resource efficient and adaptive design for random access in 6G.

Observation 1.	6G MAC should address the shortcomings in NR’s RACH design (e.g. RACH partitioning) and aim for a more resource efficient and adaptive design.

# UL scheduling

Traffic in 6G is expected to be more diverse. For example, one may expect increasingly more presence of traffic generated by applications such as XR, immersive communication and alike. There may also be traffic generated by mobile AI applications. Their traffic may not have strict delay requirements like XR applications, but their performance depends on the quality of experience (QoE) perceived by their users.

Regardless of the types of traffic 6G may need to support, they all can benefit from lower uplink latency. For eMBB flows, lower uplink latency leads to shorter RTT, which improves the throughput. For delay-critical and QoE-critical flows, lower uplink latency can improve user experience. Within the L2 protocol stack, the MAC and RLC layers offer the most potential for reducing latency. For example,

- BSR triggers may be made more responsive;

- If SR configurations can be more resource efficient, that would incentivize network to configure more SR occasions, which consequently reduces the UL access latency;

- If HARQ can provide status of its retransmissions to upper layers, that helps RLC start retransmissions sooner and thus reduce overall latency of the ARQ procedure.

To more effectively support diverse traffic, the LCP procedure can benefit from more awareness of QoS requirements associated with different logical channels (QoS flows). For example, the LCP procedure may allow UE to use a mix of delay and priority in its scheduling decisions. In addition, its scheduling decisions can be made more adaptive (e.g. dynamic Bj, PBR, etc) considering delay and/or buffer status of the flows.

Similarly, HARQ can apply finer granularities to serve flows with different QoS requirements. For example, a delay critical XR flow can use RLC UM and more HARQ retransmissions to achieve the required reliability while keeping the delay low. Conversely, eMBB flows can benefit from fewer HARQ retransmissions to achieve higher throughput, while relying on RLC retransmission to keep the overall packet error rate low. To implement such behaviors, network needs new ways to direct a HARQ process to a specific set of logical channels.

In summary, we think 6G can benefit from lower uplink latency and better support for increasingly more diverse traffic. These objectives can be achieved by enhancements that are more QoS-aware, more adaptive and better coordinated with upper layers such as RLC.

Observation 2. 	6G can benefit from lower uplink latency and better support for increasingly more diverse traffic. These objectives can be achieved by enhancements that are more QoS-aware, more adaptive and better coordinated with upper layers such as RLC.

# Network/UE energy savings

As traffic is expected to become more diverse in 6G, DRX needs to be more adaptive to keep UE’s power consumption low. An example is that a user may frequently switch between eMBB (e.g. web browsing) and QoE critical applications such as mobile AI. Since these two types of applications have quite different traffic patterns and delay requirements, UE must promptly adjust its DRX configuration according to variations in its traffic. Otherwise, UE suffers from unnecessary delays and higher power consumption (e.g. due to unnecessarily more active time).

Greater diversity in traffic requires improved support for multi-modal traffic. In Rel-18, non-integer DRX cycles were introduced to support video, while configured grants are used to handle other types of periodic flows like voice and control messages. However, this approach may no longer be adequate in 6G. With advances in codec technologies, applications such as XR or immersive communication are expected to support more features and generate more flows and more complex traffic patterns (e.g. more non-periodic bursts). DRX thus needs to enhance its support for multi-modal traffic without substantially increasing its complexity (e.g. each UE still maintains a single DRX state machine).

The same can be said about legacy cell DTX/DRX. In Rel-18, cell DTX/DRX was introduced without much consideration for multi-modal traffic, especially those containing flows with non-integer periodicities. If left unaddressed in 6G, the consequence is that UEs with multi-modal traffic either can’t access a cell with NES enabled or suffers from significant impact on its latency.

In addition, there is a need for better coordination between UE DRX and cell DTX/DRX. In legacy, cell DTX and cell DRX are completely synchronized, i.e. their in-/active time are completely in sync, preventing UE from transmitting during inactive time. When there is delay-sensitive traffic in the cell, this forces the network to either disable the cell DTX/DRX or shorten their cycles. This limits energy savings. Therefore, in 6G, it is important to study enhancements that can effectively improve the coordination between UE DRX and cell DTX/DRX, especially when delay-sensitive traffic is present.

Therefore, the legacy UE DRX and cell DTX/DRX need to be enhanced in 6G to better support the anticipated changes in traffic, as described above.

Observation 3. 	6G MAC can boost network and user energy efficiency across all types of traffic by using improved configuration and more adaptive procedures.

# MAC CE

Over the course of 5G, we have witnessed steady increase in both size and number of MAC CEs, as many features/procedures prefer to use MAC CE as the means to de-/activate radio configurations (e.g. MIMO) or reporting UE status (e.g. LTM). One issue is that all the MAC CEs were designed individually and introduced without any structured way. When their quantity becomes large, this lack of structure is not friendly to UE implementation and can increase their processing time. This is not desirable, because 6G is expected to have higher link rate and tighter timeline.

We expect there will be more use of MAC CEs in 6G. In fact, companies have shown interest in using MAC CE to send a broad range of messages, e.g. PHY-layer messages, measurement reports, etc. It therefore is desirable for MAC CEs in 6G to be flexible, allowing them to be used as a more versatile signaling mechanism for various purposes.

Another aspect is that some of the messages (e.g. measurement reports or configuration commands) to be carried by MAC CEs in 6G are important and do not change fast (e.g. over the span of a typical HARQ process). Then it is worth studying whether it is beneficial to increase the reliability of such MAC CEs, by techniques such as MAC-layer retransmission.

In summary, we think RAN2 should aim for the following design objectives for 6G MAC CE design:

- The MAC CE format should be flexible or extensible;

- MAC CEs should remain easy for UE to process and assemble, even as their quantity increases in future releases.

- MAC CEs can be used as a versatile signaling mechanism for various purposes.

Observation 4.	MAC CEs in 6G can benefit from a flexible/extensible design that makes them a more extensible, secure and versatile signaling mechanism.

Based on the discussion above, we have the following proposal on potential enhancements for 6G MAC:

Proposal 3. 	Study on 6G MAC includes at least the following objectives:

- Higher resource efficiency for random access;

- More adaptive scheduling and lower uplink latency;

- Higher energy efficiency for both network and UE;

- Improved extensibility, versatility and security for MAC CE.

## Upper L2 Sublayers

The first step in designing the L2 of user plane would be to identify the minimum functionality that should be supported as a baseline. Having handled MAC in earlier sections, this section focuses on the functionality and architecture of the other sublayers (SDAP/PDCP/RLC). For those sublayers, a main goal of design would be allocating functionality to different sublayers. However, this task is not straightforward as a lot of the assumption about RAN architecture are still unknown and subject to ongoing discussions.

To this end, RAN2 L2 User Plane discussions should focus on standalone RAN architecture making sure that design does not block progress by RAN2 and other WGs ongoing discussions on the several architecture and migration open issues.

A first step should be defining a minimum functionality that L2 should support. This minimum functionality comprises of the minimum set of User Plane features every UE should support. The exact layering architecture would not be very critical at the beginning but would evolve to accommodate this functionality naturally considering agreements about architecture and migration as they are reached byRAN2 and other WGs.

The minimum functions proposed below are the functions supported in Rel-15. They are a natural starting point as a baseline to build on. The functions below use TS 38.300 (Rel-15, clauses 6.2- 6.5 [1]). The reason we choose Rel-15 as a starting point is that all the features that were developed in (Rel-16 – Rel-19) built on the Rel-15 design and thus were constrained by those design choices. The minimum baseline agreements do not preclude any other functions to be agreed later and added to the minimum L2 functionality. They have been captured without specifying implementation choices occasionally captured in stage 2 spec (e.g. “Header compression and decompression” instead of “Header compression and decompression using the ROHC protocol”). This is to isolate discussions about functions and specific implementation choices which can be discussed separately for every function once there is an agreement to support it.

Proposal 4.  L2 supports the following minimum functionality as a baseline:

| - Transfer of data - Maintenance of Sequence Numbers (FFS if one or more SNs are needed) - Ciphering and deciphering - Integrity protection & verification | - SDU discard - Segmentation & Reassembly - Header Compression & decompression - SDU marking, e.g., QFI | - Reordering and in-order delivery - Out-of-order delivery - Duplicate discarding - Protocol Error detection and recovery |
| --- | --- | --- |

## SDAP

Aside from the functionality, a critical ongoing discussion in L2 UP design would be to specify the L2 data flow (similar to clause 6.6 in 38.300). A first-step in this design would be specifying the traffic granularity incoming into L2. In NR, the QoS flow granularity was used to identify incoming traffic. Since the QoS flow framework is flexible and can accommodate different protocols (e.g., IP/Ethernet), we expect a similar abstraction to be used in 6G-UP as the traffic granularity incoming to UP.

In-order to offer more QoS flexibility 5G-NR introduced two enhancements over LTE [1]:

- Many QoS flows to single DRB mapping

- The ability to dynamically switch flow-to-bearer mapping using in-band SDAP signalling.

While the first enhancement is expected to carry on in 6G, it is worth reflecting on the general SDAP mechanism in light of lessons learned from 5G-NR. SDAP came at a high hardware processing cost since it introduced a mandatory per-packet header check for every DL packet when SDAP is configured, which means a cost that increases proportional with DL data rate. On the other hand, field experience has shown little, if any, practical deployment of the RQoS mechanism. Considering 6G UP design goals of simplification and processing reduction, RAN2 should rethink the mechanism by which NR performed Flow-to-bearer (re)mapping, namely RQoS, and consider available alternatives to perform the same procedure.

Observation 5. 	In 5G-NR, the SDAP RQoS mechanism was not widely deployed. It complicated hardware by mandating a per-packet RQoS check. Additionally, CP may perform flow-to-bearer mapping.

Proposal 5. 	RAN2 to study QoS flow-to-bearer mechanisms including:

- Removing RQoS mechanism and SDAP to reduce HW processing.

- Moving QFI marking to other protocol layers, e.g., PDCP, if needed.

## Enhanced RLC

Both 5G-NR (up to Rel-18) and LTE have had somewhat a similar starting point of allowing DRB traffic to operate in two RLC modes:

- UM mode: Rely on HARQ reliability. RLC only performs segmentation and reassembly. Typically used for voice since this traffic can withstand some L2 loss.

- AM mode: Supplements HARQ reliability with a secondary ARQ mechanism that, as a baseline, can push LCH reliability to 100%. Typically used for data, TCP traffic, etc.

These protocol design choices have implicitly divided traffic into two classes, class handled by AM which operates at high levels of reliability (e.g. 10-5 or 10-6) but may suffer from high latency due to repeated retransmissions, segmentation, HOL blocking, etc. and another traffic class handled by UM which minimizes RLC latency but completely relies on HARQ for error correction and does not track HARQ residual errors or attempt to establish reliability guarantees at the protocol level.

Throughout Rel-18 XR SI, RLC AM was revisited with the aim of improving latency (possibly at the cost of reducing reliability) aiming at establishing some balance between per-packet delay and RLC AM high reliability. Recall the objective and justification in Rel-19 XR WID [2]:

Specify the following user plane enhancements [RAN2]: -RLC re-transmission related enhancements for operation of RLC Acknowledged Mode (AM) with small packet delay budget. RLC-AM is useful to limit data loss, however RLC-AM feedback or retransmission triggering mechanisms are not well adapted for short packet delay budgets applicable to XR traffic. Also, for RLC AM, considerable amounts of data may be in-flight, i.e. in the window, and there is no current way to avoid retransmitting this data, even if the data is old.

It is expected that XR would be an important use case for 6G. Furthermore, early discussions of other types of traffic (i.e. Mobile AI) indicate that traffic and use cases will only become more demanding and diverse in their QoS requirements.

Observation 6.	Up to Rel-18, RLC for DRBs operated in two modes: 1) RLC UM: Minimized latency with no reliability guarantees at RLC level. 2) RLC AM: Operates at a 100% reliability with unbounded RLC latency (except for PDCP discard timer). Rel-19 XR introduced RLC AM latency reduction procedures, often at the cost of RLC reliability.

An important thing to note about such Rel-18/Rel-19 improvements to reduce latency for XR is that they were introduced to an already stable Rel-15 UP design whereby the PDCP/RLC/MAC protocols, state variables, configurable parameters, timers, etc. were already in place, which somewhat limited the design space.

The lesson learned from 5G in that sense was that use cases are becoming more flexible in terms of their granular QoS requirements and L2 is expected to evolve (as it has in XR features) to play a more active role into adapting to diverse QoS requirements, at least in terms of balancing latency and reliability targets. A day 1 design for L2 taking into account those requirements allow for protocols that are better prepared to support diverse use cases without relying on post-hoc feature development in later releases that attempt to extend an existing UP design to cover target QoS requirements.

Proposal 6. 	RAN2 to study how RLC can operate at different reliability and latency targets depending on diverse QoS requirements.

## Intra-bearer RAN Processing

Rel-15 NR User Plane followed LTE in establishing the Data Radio Bearer as the unit of traffic within RAN. The DRB principle earlier on was that any two packets belonging to the same DRB require exactly similar RAN treatment from a QoS standpoint. A Scheduler need only know how much data is pending at the UE/gNB for each bearer to successfully schedule packets. The assumption throughout the previous two Gs was that networks will map diverse traffic into different bearers and provide RAN differentiation at the bearer level with CN supervision.

In practice however, we have observed the proliferation of the “single Data Bearer”, a single bearer that carries almost all traffic, with the exception of IMS voice traffic separately configured in most practical deployments. In a sense, the lesson learned from 5G deployments was that most data tend to end up on a single bearer which looks indistinguishable to network on an individual QoS flow level, at least in uplink.

Aside from practical considerations and field observations, Rel-18 XR has introduced a new UP requirement of identifying and utilizing XR awareness in L2 [3]:

Specify the enhancements for XR Awareness: - Impact of identifying by UE of PDU Sets, Data bursts and PSI, as needed (RAN2);

And to some extent Rel-18 [2]:

-Specify Enhancements for support of UL scheduling to enable high XR capacity while meeting delay requirements/avoiding too late PDUs, as follows [RAN2]: -Specify additional Logical Channel priority handling using delay/deadline information of packets; -Specify enhanced DSR (Delay Status Report) reporting with multiple pairs of remaining time and buffer size for an LCG.

Given the continuation of XR as an important use case and other use new use cases (e.g. Mobile AI) which may come with their own traffic awareness optimization opportunities, it is useful to build on the concept of traffic awareness and tailoring some L2 procedures based that awareness.

Observation 7. 	Most deployments in the field have a single DRB configured for the UE with only voice bearers separately configured. Rel-18/Rel-19 XR has identified concept of traffic awareness in L2, e.g., low importance PSI discard in uplink.

In 6G, we can follow the same logic of those enhancements; That a single DRB may carry diverse traffic, that L2 may have some level of traffic awareness, and that some user plane L2 functions may benefit from more granular operations based on this traffic awareness. This concept need not depend on XR specifics or any other specific application, but RAN2 should study which operations can benefit from traffic awareness within a bearer vs what operations can be performed on the DRB level irrespective of traffic type.

Proposal 7. 	RAN2 to study introduction of intra-QoS flow traffic differentiation. The study can include L2 functions handling intra-QoS traffic differentiation, and the trade-off between DRB level handling vs more granular intra-DRB operations.

# Conclusion

Based on the above analysis, we respectively request RAN2 to discuss and agree to the following proposals:

Proposal 1. 	The following MAC functions are supported in 6G:

- Functions related to random access, including CBRA and CFRA;

- Functions related to UL scheduling, including SR, BSR, DSR, LCP, HARQ, CG, PHR;

- Functions related to bandwidth management, including BWP, CA;

- Functions related to link management, including BFD/R, UL timing;

- Functions related to energy savings, including UE DRX and cell DTX/DRX.

Proposal 2.  	Discuss the respective baseline for each MAC function to be supported in 6G except those led by other WGs, which include SR, BSR, DSR, PHR, LCP, CG, BFR, UE DRX, cell DTX/DRX, rate control, MAC CE/PDU format.

Observation 1.	6G MAC should address the shortcomings in NR’s RACH design (e.g. RACH partitioning) and aim for a more resource efficient and adaptive design.

Observation 2. 	6G can benefit from lower uplink latency and better support for increasingly more diverse traffic. They can be achieved by enhancements that are more QoS-aware, more adaptive and better coordinated with upper layers such as RLC.

Observation 3. 	6G MAC can boost network and user energy efficiency across all types of traffic by using improved configuration and more adaptive procedures.

Observation 4.	MAC CEs in 6G can benefit from a flexible/extensible design that makes them a more extensible, secure and versatile signaling mechanism.

Proposal 3. 	Study on 6G MAC includes at least the following objectives:

- Higher resource efficiency for random access;

- More adaptive scheduling and lower uplink latency;

- Higher energy efficiency for both network and UE;

- Improved extensibility, versatility and security for MAC CE.

Proposal 4. 	L2 supports the following minimum functionality as a baseline:

| - Transfer of data - Maintenance of Sequence Numbers (FFS if one or more SNs are needed) - Ciphering and deciphering - Integrity protection & verification | - SDU discard - Segmentation & Reassembly - Header Compression & decompression - SDU marking, e.g., QFI | - Reordering and in-order delivery - Out-of-order delivery - Duplicate discarding - Protocol Error detection and recovery |
| --- | --- | --- |

Observation 5.  In 5G-NR, the SDAP RQoS mechanism was not widely deployed. It complicated hardware by mandating a per-packet RQoS check. Additionally, CP may perform flow-to-bearer mapping.

Proposal 5. 	RAN2 to study QoS flow-to-bearer mechanisms including: 1) Removing RQoS mechanism and SDAP to reduce HW processing. 2) Moving QFI marking to PDCP, if needed.

Observation 6.  Up to Rel-18, RLC for DRBs operated in two modes: 1) RLC UM: Minimized latency with no reliability guarantees at RLC level. 2) RLC AM: Operates at a 100% reliability with unbounded RLC latency (can be bounded by PDCP discard timer). Rel-19 XR introduced RLC AM latency reduction measures, to support XR traffic, often at the cost of RLC reliability.

Proposal 6. 	RAN2 to study how RLC can operate at different reliability and latency targets depending on diverse QoS requirements.

Observation 7. 	Most deployments in the field have a single DRB configured for the UE with only voice bearers separately configured. Rel-18/Rel-19 XR has identified concept of traffic awareness in L2, e.g., low importance PSI discard in uplink.

Proposal 7. 	RAN2 to study introduction of intra-QoS flow traffic differentiation. The study can include L2 functions handling intra-QoS traffic differentiation, and the trade-off between DRB level handling vs more granular intra-DRB operations.

# References

[1] 3GPP TS 38.300 V15.21.0 Stage 2 (Release 15)

[2] RP-241771: Revised WID on XR (eXtended Reality) for NR Phase 3

[3] RP-232778: Updated WID on XR Enhancements for NR