3GPP TSG-RAN WG2 Meeting #131bis	R2-2506813

Prague, Czech Republic, Oct. 13th-17th

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.331 | CR | 5491 | rev | - | Current version: | 19.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction on the execution of SCG LTM |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | CATT |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R2 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_Mob_enh2-Core |  |  |  |  |  | Date: |  |  | 2025-10-02 |
| Category: | A |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | Fast recovery is not supported by SCG LTM,according to RAN2 agreement, For SCG configured LTM in NR-DC scenario, LTM recovery for SCG is not supported. So upon SCG LTM execution, the case of “for the selected cell in accordance with 5.3.7.3” caused by fast recovery couldn’t occur. Hence the related condition should be removed from 5.3.5.18.6. |  |  |  |  |  |  |  |  |
| Summary of change: |  | In 5.3.5.18.6, delete the case of “for the selected cell in accordance with 5.3.7.3” upon the release of redundant configuration when generating the complete configuration. Impact analysis Impacted 5G architecture options: NR SA, NR-DC Impacted functionality: LTM Inter-operability: If NW implements this CR but UE not, the redundant case is captured when UE determines the target configuration upon SCG LTM to generate the complete configuration; If UE implements this CR but NW not, there is no inter-operatibility issue found. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | - The wrong case for UE to determine the target configuration is involved when UE releases the radio bearer(s) using the secondary key and the SCG logical channel(s) that are not used in the final configuration upon execution of SCG LTM |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.3.5.18.6 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

Start of Change

1>	if the LTM cell switch is triggered on the MCG:

2>	release the radio bearer(s) using the master key and the MCG logical channel(s) that were part of the UE configuration before this LTM cell switch procedure but not part of the LTM candidate configuration either indicated by lower layers or for the selected cell in accordance with 5.3.7.3, or the LTM reference configuration (in case the LTM candidate configuration does not include ltm-ConfigComplete);

1>	else, if the LTM cell switch is triggered on the SCG:

2>	release the radio bearer(s) using the secondary key and the SCG logical channel(s) that were part of the UE configuration before this LTM cell switch procedure but not part of the LTM candidate configuration indicated by lower layers, or the LTM reference configuration (in case the LTM candidate configuration does not include ltm-ConfigComplete);

待协议后更新

End of Change