3GPP TSG RAN WG2 #131bis          	                                                             R2-2506819

Prague, CZ, Oct 13th – 17th, 2025

Source:	CATT

Title:	Overview of 6GR Control Plane

Agenda Item:	10.3.2

Document for:	Discussion and Decision

- Introduction

According to the SID [1], the control plane for 6GR should be discussed in this meeting.

In this contribution, the RRC modelling and connection management, initial and system access (including system information, paging, Cell DTX/DRX, Access control, NTN) are discussed.

- Discussion

  - RRC modelling  and management

NR supports three RRC states, including RRC_IDLE state, RRC_INACTIVE state and RRC_CONNECTED state.

In RRC_INACTIVE, the last serving gNB node keeps the UE context and the UE-associated NG connection with the serving AMF and UPF. Hence, compared with RRC_IDLE state, RRC connection can be resumed in RRC_INACTIVE state with less latency and less signaling overhead. Therefore, we prefer to keep the three RRC states in 6GR.

Observation 1: Compared with RRC_IDLE state, RRC connection can be resumed in RRC_INACTIVE state with less latency and less signaling overhead.

The connection management should take the three states into consideration. Hence the RRC procedure should support the RRC state transition as following which is same as 5G.

Besides the RRC state transition, the RRC reestablishment procedure should also be supported to achieve fast recovery RRC connection. And RRC reconfiguration message should be supported to (re-)configure RB configuration etc.

Due to above procedure are supported in 5G for the same intention, we suggest reusing 5G procedure as baseline for 6GR.

Proposal 1: Three RRC states, i.e., RRC_IDLE, RRC_INACTIVE and RRC_CONNECTED, are defined in 6GR. Generally, the 5G NR procedures and signaling for RRC state transition and RRC Reconfiguration/Reestablishment can be taken as baseline.

Currently, for the access procedures (RRC setup/RRC resume/ RRC reestablishment) which involve CBRA procedure, the UE could be scheduled only after contention resolution, especially for RRC setup procedure, the connection between the RAN node and CN node could be setup only after the RAN node receives the Message 5.

In order to further reduce the latency for access procedure, RAN2 consider introducing unique UE ID mechanism. That unique UE ID could assistant the RAN node to identify the UE when the RAN node is among the applicable area of the unique UE ID.

For a UE which has registered in the CN and is assigned a unique UE ID by the network, when the UE (re)selects to one cell belongs to the configured applicable area and expects to establish the RRC connection, the UE could initiate a fast access procedure which provides the unique UE ID in the Message3.  After the transmission of the Message3, the UE monitors the PDCCH identified by the unique UE ID.

When the RAN node receives the unique UE ID via Message 3, it could identify the UE e.g. could identify its full CN UE ID, and the registered CN. Hence the RAN node could initiate the connection establishment with the registered CN for the UE if needed or perform data transmission. The scheduling information of the subsequent message uses the unique UE ID, which could solve the problem of the contention resolution introduced by CBRA.

When the UE receives the PDCCH addressed by the unique UE ID, the UE considers the fast access procedure is successful.

Proposal 2: RAN2 considers to introduce unique UE ID, which identifies the UE by RAN node among specific cell areas, aiming to optimize UE access procedures..

  - Initial and system access

System information

NR supports on-demand system information. Instead of always periodically transmitting SI, the network can broadcast on-demand SI upon request from UEs in RRC_IDLE, RRC_INACTIVE via either a Msg1-based or a Msg3-based method. And the network can broadcast on-demand SI or send in a dedicated manner upon request from UEs in RRC_CONNECTED. In NR Rel-19, On-demand SIB1 is introduced for UEs in RRC_IDLE, RRC_INACTIVE and RRC_CONNECTED when T311 is running. As system information is provided on-demand, network energy saving can be achieved.

Considering the benefit, on-demand system information and area scope per SIB in NR is reused as a baseline in 6GR.

Observation 2: NR supports on-demand system information in all RRC states, which can achieve network energy efficiency.

In NR, one SIB, i.e., SIB1, is specified for Remaining Minimum SI (RMSI). This can reduce the latency of initial access as all information needed for initial access is combined in SIB1. However, SIB1 contains quite a lot of information, e.g. information required for initial access, the scheduling of other system information blocks and so on. And in every evolution of NR Release, more and more parameters are extended in SIB1. The SIB1 becomes bloated. In order to reduce network energy consumption, On-demand SIB1 was introduced in NR Rel-19. However, it is not energy efficient enough with bloated SIB1. For example: When the network updates the info of SIBx, although the UE only cares about the scheduling info of SIBx in SIB1, the network has to provide the whole SIB1 and the UE has to acquire the complete SIB1. In order to further reduce network and UE energy consumption, it is beneficial to restructure RMSI. Considering the latency of initial access, RMSI in NR (SIB1) can be restructured into sub-RMSI1 and sub-RMSI2. Sub-RMSI1 contains some necessary information for camping and the size of sub-RMSI1 is small, while sub-RMSI2 contains other remaining information for initial access. The details can be studied further.

Observation 3: RMSI (i.e., SIB1) in NR has expanded significantly with network evolution, leading to information overload. The current structure of RMSI results in suboptimal energy efficiency.

Proposal 3: In order to further reduce network and UE energy consumption, RAN2 consider on-demand system information in NR as a baseline and restructure RMSI in 6GR, e.g. RMSI in NR (SIB1) is restructured into sub-RMSI1 and sub-RMSI2.

Paging

eDRX was introduced since NR Rel-17 to reduce UE power consumption as the interval of monitoring paging is increased.

And adaptation of paging in time domain was introduced in NR Rel-19. The value range for parameter N, which is the number of paging frames in one paging cycle, is extended to make it possible to have increased interval between PFs. With this, “sleep time” for the gNB can be increased which can reduce network energy consumption as much as possible.

In NR Rel-17, a PDCCH-based paging early indication (PEI) was introduced to reduce unnecessary PDCCH monitoring in POs. With PEI, the group of UEs monitoring the same PO can be further divided into multiple subgroups. If the subgroup to which the UE belongs is paged as indicated via associated PEI, the UE monitors PDCCH in its PO. Since the false paging probability is reduced with subgrouping, UE reception power consumption in idle/inactive mode can be reduced accordingly.

In NR Rel-19, low-power wake-up signal and receiver (LP-WUS/WUR) was introduced to further reduce the UE power consumption on paging reception in RRC_IDLE/INACTIVE state. LP-WUS carries subgroup indication and up to 31 subgroups can be configured for LP-WUS. If the UE detects LP-WUS and the LP-WUS is associated with the UE, the UE monitors PDCCH in its PO or PEI. Compared with PEI, LP-WUS in RRC_IDLE/INACTIVE in NR can achieve more UE power saving. On the other hand, it is noted that only the UE power saving gain was achieved with LP-WUS in RRC_IDLE/INACTIVE. The power consumption of network is increased as the network has to transmit the additional LP-WUS/LP-SS besides paging message via Main Receiver (MR). It is not consistent with intention of energy saving in 6GR. Therefore, it is beneficial to consider joint network and UE power saving in 6GR. For example: more info could be carried in LP-WUS. Then the cases that the network needs only send LP-WUS without sending PDCCH for PO are increased. Further network energy saving can be achieved.

In 6GR, in order to achieve energy saving, LP-WUS in RRC_IDLE/INACTIVE, eDRX and adaptation of paging in time domain can be considered as starting point of paging. And in order to further reduce network energy saving, joint network and UE power saving in 6GR with low-power wake-up signal and receiver could be considered in 6GR.

Observation 4: eDRX in NR can achieve UE power saving.

Observation 5: Adaptation of paging in time domain in NR can achieve network energy efficiency.

Observation 6: Compared with PEI in NR, LP-WUS in RRC_IDLE/INACTIVE in NR can achieve more UE power saving. However, this gain comes at the cost of increased network energy consumption.

Proposal 4: LP-WUS in RRC_IDLE/INACTIVE, eDRX and adaptation of paging in time domain in NR can be considered as starting point of paging in 6GR. Furthermore, RAN2 consider joint network and UE power saving in 6GR with bidirectional low-power wake-up signal and receiver.

Cell DTX/DRX

In order to increase “sleep time” for gNB to save network energy consumption, Cell Discontinuous Transmission (DTX)/ Cell Discontinuous Reception (DRX) was introduced in Rel-18 NR. When cell DTX is configured and activated for the concerned cell, the UE may not monitor PDCCH in selected cases or does not monitor SPS occasions during cell DTX non-active duration. When cell DRX is configured and activated for the concerned cell, the UE does not transmit on CG resources or does not transmit a SR during cell DRX non-active duration. But this feature is only applicable to UEs in RRC_CONNECTED state and it does not impact Random Access procedure, SSB transmission, paging, and system information broadcasting. To further improve network energy saving, Cell DTX/DRX could be extended to cover UEs IDLE/INACTIVE state. During the non-active period of Cell DTX/DRX, less frequent or sparser SSB and/or common signal transmissions could be jointly considered. Thereby further network energy saving can be achieved via increasing additional sleep opportunities in IDLE/INACTIVE states.

Observation 7: Cell DTX/DRX in NR can reduce network energy consumption. But this feature is only applicable to UEs in RRC_CONNECTED state.

Proposal 5: RAN2 consider Cell DTX/DRX in NR as starting point and extend it to UEs in IDLE/INACTIVE state in 6GR.

Access control

One unified access control framework applied to all UE states (RRC_IDLE, RRC_INACTIVE and RRC_CONNECTED) was introduced NR. NG-RAN broadcasts barring control information associated with Access Categories and Access Identities. The UE determines whether an access attempt is authorized based on the barring information broadcast for the selected PLMN, and the selected Access Category and Access Identity(ies) for the access attempt:

- For NAS triggered requests, NAS determines the Access Category and Access Identity(ies);

- For AS triggered requests, RRC determines the Access Category while NAS determines the Access Identity(ies).

The unified access control framework in NR is forward compatible which can cope with future access attempts/scenarios with less specification impact. In 6GR, the unified access control framework in NR can be reused as baseline.

Observation 8: The unified access control framework in NR is forward compatible which can cope with future access attempts/scenarios with less specification impact.

Proposal 6: The unified access control framework in NR can be reused as baseline in 6GR.

  - CP aspects affected by NTN

Per guidance of the SID [1], 6G targets a harmonized 6G radio design for both TN and NTN. The RRC modeling and management and functions given in section 2.1 and 2.2 are generally applicable to both TN and NTN. However, NTN has distinct characteristics, including long propagation delay, high mobility and broad coverage, which inevitably introduce CP differences compared to TN. In NR, on basic of the TN system, the following NTN-specific enhancements are specified:

- System information: SIB19 is introduced and act as essential system information for NTN access, which includes the serving satellite’s assistance information (e.g., ephemeris and common TA). Considering the satellite assistance information keep changing along with the movement of satellite, a valid time-based system information acquisition mechanism is adopted, i.e., UE re-acquires SIB19 before the end of the duration indicated by ntn-UlSyncValidityDuration and epochTime.

- Access control: an NTN specific barring bit is introduced. The access control of NTN access is realized by this bit.

For 6G day1, it is proposed to study and identify the CP technical aspects affected by NTN characteristics. The NR NTN design can be taken as baseline.

Proposal 7: Study and identify CP technical aspects affected by NTN characteristics. The NR NTN design can be taken as baseline.

- Conclusion

In this contribution, we provide our views on the 6GR control plane:

Observation 1: Compared with RRC_IDLE state, RRC connection can be resumed in RRC_INACTIVE state with less latency and less signaling overhead.

Observation 2: NR supports on-demand system information in all RRC states, which can achieve network energy efficiency.

Observation 3: RMSI (i.e., SIB1) in NR has expanded significantly with network evolution, leading to information overload. The current structure of RMSI results in suboptimal energy efficiency.

Observation 4: eDRX in NR can achieve UE power saving.

Observation 5: Adaptation of paging in time domain in NR can achieve network energy efficiency.

Observation 6: Compared with PEI in NR, LP-WUS in RRC_IDLE/INACTIVE in NR can achieve more UE power saving. However, this gain comes at the cost of increased network energy consumption.

Observation 7: Cell DTX/DRX in NR can reduce network energy consumption. But this feature is only applicable to UEs in RRC_CONNECTED state.

Observation 8: The unified access control framework in NR is forward compatible which can cope with future access attempts/scenarios with less specification impact.

Proposal 1: Three RRC states, i.e., RRC_IDLE, RRC_INACTIVE and RRC_CONNECTED, are defined in 6GR. Generally, the 5G NR procedures and signaling for RRC state transition and RRC Reconfiguration/Reestablishment can be taken as baseline.

Proposal 2: RAN2 considers to introduce unique UE ID, which could identify the UE by RAN node among specific cell areas, aiming to optimize UE access procedures.

Proposal 3: In order to further reduce network and UE energy consumption, RAN2 consider on-demand system information in NR as a baseline and restructure RMSI in 6GR, e.g. RMSI in NR (SIB1) is restructured into sub-RMSI1 and sub-RMSI2.

Proposal 4: LP-WUS in RRC_IDLE/INACTIVE, eDRX and adaptation of paging in time domain in NR can be considered as starting point of paging in 6GR. Furthermore, RAN2 consider joint network and UE power saving in 6GR with bidirectional low-power wake-up signal and receiver.

Proposal 5: RAN2 consider Cell DTX/DRX in NR as starting point and extend it to UEs in IDLE/INACTIVE state in 6GR.

Proposal 6: The unified access control framework in NR can be reused as baseline in 6GR.

Proposal 7: Study and identify the CP technical aspects affected by NTN characteristics. The NR NTN design can be taken as baseline.

- References

- RP-252912 Study on 6G Radio, RAN#109