3GPP TSG-RAN WG2 Meeting #131bis	R2-2506820

Prague, Czech Republic, 13th – 17th October, 2025

Agenda item:	8.11.1

Source:	CATT

Title:	Open issues on Rel-19 Evolution of NR duplex operation (SBFD) Stage 2 CR

Document for:	Discussion and Decision

Introduction

The following email discussion is re-triggered to collect open issues on SBFD Stage 2 CR:

[Post131][219][SBFD] CR for TS 38.300 (CATT)

Intended outcome: Agree the CR for TS 38.300 with RAN3 CR merged

Deadline:  Oct. 1st

The CR from the previous round of the above discussion was agreed in R2-2506604 [1]. In this second round, open issues with the agreed CR are discussed and resolved.

Open issues

There was one comment from IDC during the post email discussion, as following:

| Company | Comments | Proposed Change (if any) | Rapporteur Comments |
| --- | --- | --- | --- |
| IDC | Not related to the changes, but SBFD with multi-carrier (CA) is missed which is already agreed by RAN1. We need to capture the CA operation (in general section or other section) in stage-2 specification with high level description. RAN1#120 agreement Support SBFD operation on one TDD carrier in multi-carrier scenario. - Note: If the TDD carrier for SBFD operation is an Scell, support UE dedicated signaling for cell-specific configuration of time and frequency location of SBFD subbands for the Scell. - FFS whether/how to handle half-duplex CA case |  | RAN1 feature leader confirmed that RAN1 did not provide any TP of stage-2 on this agreement in the LS because they think there is no stage-2 spec impact. Considering only one comment on this change, contribution can be submitted and further discussed for the next meeting. |

Companies are invited to further discuss above issue and describe any other open issues.

| Company | Issue description and potential resolution | Rapporteur comment |
| --- | --- | --- |
| Ericsson | More cases on RO type selection is missing, including 1) CFRA triggered by dedicated RRC signaling 2) CBRA RO selection for initial PRACH transmissions. The motivation to include description on the rest cases are: In stage 2 spec, we need to keep a consistent description on the feature, i.e., RO type selection. We can either remove all texts on RO type selection, or include complete and consistent description for all cases, for better readability perspective. |  |

Summary:

There are two issues raised by companies.

- SBFD with multi-carrier (CA)

It is proposed that we need to capture the CA operation (in general section or other section) in stage-2 specification.

- RO type selection

It is proposed to keep a consistent description on the feature, i.e., RO type selection: 1) CFRA triggered by dedicated RRC signaling 2) CBRA RO selection for initial PRACH transmissions.

From rapporteur’s perspective, more cases on RO type selection are reasonable to keep a consistent description of TP from RAN1. However the CA case will not be captured since RAN1 did not provide any TP of stage-2 on this agreement in the LS. So the issue of SBFD with multi-carrier (CA) will be captured as an open issue for further discussion.

Proposal 1: There is one open issue in TS 38.300: whether SBFD with CA operation is required in clause X.1 General.

Proposal 2: Add more cases of RO type selection to TS 38.300: 1) CFRA triggered by dedicated RRC signalling, 2) CBRA RO selection for initial PRACH transmissions. Take the CR in R2-2506823 as baseline.

Conclusion

In this contribution, we have the following proposals/open issue list:

Proposal 1: There is one open issue in TS 38.300: whether SBFD with CA operation is required in clause X.1 General.

Proposal 2: Add more cases of RO type selection to TS 38.300: 1) CFRA triggered by dedicated RRC signalling, 2) CBRA RO selection for initial PRACH transmissions. Take the CR in R2-2506823 as baseline.

Reference

R2-2506604, Introduction of Rel-19 Evolution of NR duplex operation (SBFD), CATT, RAN2#131, Bangalore, India, August 2025.