3GPP TSG-RAN WG2 Meeting #131bis	R2-2506839

Prague, Czech Republic, 13-17 October, 2025

Agenda Item:	8.2.2

Source:	NEC

Title:	Remaining Issues of Ambient-IoT MAC Layer

Document for:	Discussion, Decision

# Introduction

In this paper, we address remaining issues of ambient IoT MAC layer and propose corresponding text proposals.

# Discussion

## (Issue 5-1) Paging response for permanently disabled AIoT Device

As specified in 23.369 [3], a permanently disabled A-IoT Device does not respond to the Inventory Procedure. However, in the current TS, if the Paging ID Presence Indication field indicates the absence of the Paging ID field, the device may directly determine it is selected and initiate CBRA.

#### 5.2.2.3 Permanent Disable Command An AIoT Device may be permanently disabled. A permanently disabled AIoT Device does not respond to the Inventory Procedure, as described in clause 6.2.2. An AIoT Device is permanently disabled by the Permanent Disable command sent by an AIOTF to the AIoT Device. The Permanent Disable command is sent to an AIoT Device when an authorized AF uses the Permanent Disable command service operations as described in clause 5.2.2.1, or if the network determines to disable the AIoT Device. The Permanent Disable command is sent in the Command Request step and a response is sent in the Command Response step of the Command procedure described in clause 6.2.3. The AIoT Device responds indicating whether the Permanent Disable command was successful. NOTE: The trigger conditions when a network determines to disable the AIoT Device depends on operator and implementation policy. When the AIoT Device has received and verified a Permanent Disable command, it shall no longer respond to the inventory procedure.

To resolve this, we propose that the device should still need the notification from upper layers even when the Paging ID Presence Indication field indicates the absence of the Paging ID field.

- Proposal 2.1 (Issue 5-1):

Upon receiving paging with Paging ID Presence Indication field indicates Paging ID field is absent; device indicate it to the upper layers. The device shall only consider itself selected if the upper layer confirms that it is not permanently disabled.

- Adopt the text proposal #2.1.

### text proposal #2.1

------------------Start of text proposal #2.1------------------

5.2	A-IoT paging

The purpose of this procedure is to transmit A-IoT Paging message to one or more devices. The reader may include the Paging ID field to select a specific device or a group of devices, or may not include Paging ID field to select all devices.

The device always monitors for the A-IoT Paging message, and determines whether the device is selected to initiate the access procedure.

Upon receiving the A-IoT Paging message, the A-IoT MAC entity shall:

1>	if the Access Type field in the A-IoT Paging message indicates CBRA:

2> if the device has no stored Transaction ID; or

2> if the value of the Transaction ID field is different from the stored Transaction ID; or

2> if the value of the Transaction ID field is the same as the stored Transaction ID, and the previous procedure was determined as failed for this Transaction ID as specified in clause 5.5:

3>	release the stored AS ID, if any;

3>	store the received value in Transaction ID field, if the device has no stored Transaction ID, or replace the previously stored Transaction ID with the current received value, if the value of the Transaction ID field is different from the stored Transaction ID;

3>	if the Paging ID Presence Indication field indicates Paging ID field is absent:

4>	consider the device is selected and indicate it to the upper layers;

4> if the upper layers indicate that the device is not permanently disabled:

5> consider the device is selected

3>	else:

4>	forward the content of the Paging ID field to the upper layers;

4>	if the upper layers indicate that the Paging ID is matched:

5>	consider the device is selected;

<text omitted>

------------------End of text proposal #2.1------------------

## Paging ID type indication in paging message

SA3 has sent LS [4] to RAN2 to kindly ask to take into account the details of security aspect of the Filtering Information and the single AIoT Device Identifier in TS 33.369 [1].

In TS 33.369 [1], protection of AIoT device identifier privacy could be done for single device inventory and group device inventory.

### 5.4.1 General This clause describes the mechanisms to protect AIoT device identifier privacy during the inventory procedure. The mechanism is based on the use of a Temporary ID (i.e., T-ID). The T-ID is generated based on the key (i.e., KAIoT_root) shared between AIoT device and ADM. Depending on the situation and deployment scenario, the network operator can choose which paging procedure to use. When privacy protection is not used during the inventory procedure, the AIoT device includes its AIoT device permanent identifier as a device identification information in the procedure specified in clause 5.2.2.

For single device inventory protection, it is called “protection with Temp ID update during Individual inventory”, where Temporary ID (i.e., T-ID) is used and updated in each paging message.

For group device inventory protection, it is called “protection for inventory with filtering information”, where device identification information is not included in the D2R message and Inventory Report message.

In TS 23.003 [2] and 23.369 [3], structure of AIoT Device Permanent Identifier and Filtering Information are specified. Based on the structure, there is no distinguish field to differentiate whether it is AIoT Device Permanent Identifier or Filtering Information in Paging ID field in A-IoT Paging message.

Paging ID type, i.e. choice of Single Device or Group Devices, is specified in RAN3. Protection indication is expected to be specified in RAN3 in the future.

Above all, reader knows the information and it is reader’s duty to indicate the corresponding information to the device by A-IoT Paging message.

Proposal 2.2:

One new paging ID type field is included in CBRA paging message, which is used to indicate the type of paging ID field from either single device ID or group device ID (filter information).

One new paging ID protection field is included in both CBRA and CFA paging message, which is used to indicate whether protection procedure for paging id is applied or not.

- Adopt the text proposal #2.2.

### text proposal #2.2

------------------Start of text proposal #2.2------------------

5.2	A-IoT paging

The purpose of this procedure is to transmit A-IoT Paging message to one or more devices. The reader may include the Paging ID field to select a specific device or a group of devices, or may not include Paging ID field to select all devices.

The device always monitors for the A-IoT Paging message, and determines whether the device is selected to initiate the access procedure.

Upon receiving the A-IoT Paging message, the A-IoT MAC entity shall:

1>	if the Access Type field in the A-IoT Paging message indicates CBRA:

2>	if the device has no stored Transaction ID; or

2>	if the value of the Transaction ID field is different from the stored Transaction ID; or

2>	if the value of the Transaction ID field is the same as the stored Transaction ID, and the previous procedure was determined as failed for this Transaction ID as specified in clause 5.5:

3>	release the stored AS ID, if any;

3>	store the received value in Transaction ID field, if the device has no stored Transaction ID, or replace the previously stored Transaction ID with the current received value, if the value of the Transaction ID field is different from the stored Transaction ID;

3>	if the Paging ID Presence Indication field indicates Paging ID field is absent:

4>	consider the device is selected and indicate it to the upper layers;

3>	else:

4>	forward the content of the Paging ID, Paging ID Type and Paging ID Protection fields to the upper layers;

4>	if the upper layers indicate that the Paging ID is matched:

5>	consider the device is selected;

3>	if the device is selected:

4>	initiate Contention-Based Random Access procedure as specified in clause 5.3.1;

1>	else (i.e., the Access Type field in the A-IoT Paging message indicates CFA):

2>	release the stored AS ID, if any;

2>	release the stored Transaction ID, if any;

2>	forward the content of the Paging ID and Paging ID Protection fields to the upper layers;

2>	if the upper layers indicate that this Paging ID is matched:

3>	consider the device is selected;

3>	initiate Contention-Free Access procedure as specified in clause 5.3.2.

6.2.1.1	A-IoT Paging message

Figure 6.2.1.1-1 and 6.2.1.1-2 show the formats of the A-IoT Paging message.

The fields in this message are defined as follows:

-	R2D Message Type: This field indicates the message type. See the Table 6.1-1. The length of the field is 3 bits.

-	R2D TBS: This field indicates the TBS of this message. The value can be {1, 2, …, 124, 125} byte(s). The length of the field is 7 bits.

-	Access Type (AT): This field indicates CBRA (when set to 1) or CFA (when set to 0). The length of the field is 1 bit.

For CBRA, the following fields are further included:

-	Transaction ID: This field associates an inventory procedure or command procedure as specified in TS 38.300 [3]. The length of the field is 6 bits.

-	Paging ID Presence Indication (PIPI): This field indicates whether Paging ID, Paging ID Type, Paging ID Protection and Length of Paging ID fields are present (when set to 1) or absent (when set to 0). The length of the field is 1 bit.

-	Paging ID Type: If present, this field indicates which is contained in the Paging ID field. Bit value 0 indicates AIoT Device Permanent Identifier is contained, and bit value 1 indicates Filtering Information is contained (as defined in TS 23.369 [4], clause 5 and TS 23.003 [5]). This field is 1 bit.

-	Paging ID Protection: If present, this field indicates whether protection for inventory with filtering information when Filtering Information is contained in Paging ID field or protection with Temp ID update during Individual inventory when AIoT Device Permanent Identifier is contained in Paging ID field is applied (as defined in TS 33.369 [6]). Bit value 0 indicates the protection is not applied, and bit value 1 indicates the protection is applied. This field is 1 bit.

-	Paging ID Length: This field indicates the length of the Paging ID field in unit of bits when Paging ID field is present. If present, the length of the field is 8 bits.

-	Paging ID: If present, this field contains AIoT Identification Information (as defined in TS 23.369 [4], clause 5 and TS 23.003 [5]). This field is of variable length.

-	Number of Access Occasions: This field indicates the number of access occasions. The length of the field is 4 bits. The value 0 (i.e., 0000) indicates the number of access occasions is 20. The value 1 (i.e., 0001) indicates the number of access occasions is 21. The value 2 (i.e., 0010) indicates the number of access occasions is 22. And so on. The maximum number of access occasions is 215 when this field is set to 15 (i.e., 1111).

-	D2R Scheduling Info: This field contains the physical layer parameters used for D2R transmission. The child fields are defined in clause 6.2.1.6. The length of the field is 18 bits.

-	K: This field indicates that the value K is 1 (when set to 0) or 4 (when set to 1) used for determining monitor window for Random ID Response message. The length of the field is 1 bit.

-	Fill Bits: This field is of variable size and is optionally present. It can be used to pad for byte alignment (1-7 bits) and/or contain future extensions. In this release, the MAC entity shall ignore the values of this field.

For CFA, the following fields are further included:

-	Paging ID Protection: This field indicates whether protection with Temp ID update during Individual inventory is applied (as defined in TS 33.369 [6]). Bit value 0 indicates the protection is not applied, and bit value 1 indicates the protection is applied. This field is 1 bit.

-	Paging ID Length: This field indicates the length of the Paging ID field in unit of bit. The length of the field is 8 bits.

-	Paging ID: This field contains AIoT Identification Information (as defined in TS 23.369 [4], clause 5 and TS 23.003 [5]). This field is of variable length.

D2R Scheduling Info: This field contains the physical layer parameters used for D2R transmission. The child fields are defined in clause 6.2.1.6. The length of the field is 24 bits.

-	Fill Bits: This field is of variable size, and can be used to pad for byte alignment (1-7 bits) and/or contain future extensions. In this release, the MAC entity shall ignore the values of this field.

------------------End of text proposal #2.2------------------

## CFA inventory response retransmission

RAN1 has specified the timing (time duration) between CFA paging message and response D2R Upper Layer Data Transfer message. The timing is discussed not considering the time required for authentication procedure. In fact, authentication procedure to calculate RESAIOT using KAIoT_root requires a lot of time. In RFID, in process reply is used for authentication related procedure. Authentication procedure requires more time. However, in current TS, the timing between CFA R2D and responding D2R message is too small that there may be no upper layer data, i.e., inventory report NAS message, generated by upper layer at the time to transmit D2R Upper Layer Data Transfer message after CFA paging message. In that sense, padding will be transmitted in D2R responding message. However, even the inventory report message is generated in the future, information will not be transmitted to reader based on current RAN2 procedure.

In such case, D2R Upper Layer Data Transfer message retransmission due to no upper layer data available for initial transmission should be supported for CFA inventory.

To support retransmission, we have the following text proposal #2.4. By setting the Received Data Size field to 0 when the device has no stored AS ID, and if CFA procedure has been performed in the current procedure, R2D Upper Layer Data Transfer message is used for inventory report message retransmission on the D2R Upper Layer Data Transfer message.

Proposal 2.3:

D2R Upper Layer Data Transfer message retransmission due to no upper layer data available for initial transmission should be supported for CFA inventory procedure. The retransmission is scheduled by R2D Upper Layer Data Transfer message.

- Adopt the text proposal #2.3.

- Note: Authentication calculation may require a lot of time at device side, that device may not generate inventory report at the time to transmit D2R Upper Layer Data Transfer message after CFA paging message.

### text proposal #2.3

------------------Start of text proposal #2.3------------------

5.4.3	R2D message reception

Upon reception of an R2D Upper Layer Data Transfer message, the A-IoT MAC entity shall:

1>	if the device has a stored AS ID and the R2D Upper Layer Data Transfer message is addressed to the device (i.e., the value of AS ID field is identical to the stored AS ID):

2>	if the Choice Indication field indicates that the Data SDU field is included (i.e., CI field set to 1):

3>	forward the upper layer data SDU in the Data SDU field to upper layers;

3>	initiate the following D2R message transmission, as specified in clause 5.4.2;

2>	else if the Choice Indication field indicates that the Received Data Size field is included (i.e., CI field set to 0):

3>	if the Received Data Size field is set to 0:

4>	initiate the D2R message transmission procedure as specified in clause 5.4.2;

3>	else:

4>	initiate the D2R segmentation procedure using this information as specified in clause 5.4.4;

1>	else if the device has no stored AS ID, and if CFA procedure has been performed in the current procedure:

2>	if the Choice Indication field indicates that the Data SDU field is included:

3>	set AS ID to the value indicated by the AS ID field and store the AS ID;

3>	forward the upper layer data SDU in the Data SDU field to upper layers;

3>	initiate the following D2R message transmission, as specified in clause 5.4.2.

2>	else if the Choice Indication field indicates that the Received Data Size field is included and the Received Data Size field is set to 0:

3>	set AS ID to the value indicated by the AS ID field and store the AS ID;

3> perform the D2R message transmission procedure as specified in clause 5.4.2.

------------------End of text proposal #2.3------------------

## (Issue 5-4) Monitor of R2D Upper Layer Data Transfer message

The device behavior for monitoring the R2D Upper Layer Data Transfer message has not yet been defined. Below is an analysis of device behavior under different scenarios:

For devices with a successful CBRA procedure:

- A NACK Feedback message may indicate that the procedure associated with the stored Transaction ID has failed and release of the AS ID. In this case, the device shall stop monitoring the R2D Upper Layer Data Transfer message.

- A CBRA paging message with the same Transaction ID (compared to the stored Transaction ID) may not trigger the release of the AS ID. Consequently, the device shall not stop monitoring the R2D Upper Layer Data Transfer message.

- A CBRA paging message with a different Transaction ID (compared to the stored Transaction ID) may trigger the release of the AS ID. In this case, the device shall stop monitoring the R2D Upper Layer Data Transfer message.

- A CFA paging message may trigger the release of the AS ID. In this case, the device shall stop monitoring the R2D Upper Layer Data Transfer message. In this case, there is no Transaction ID field.

For devices responding to a CFA, any received paging message shall cause the device to stop monitoring the R2D Upper Layer Data Transfer message. In this case, there is no stored Transaction ID.

Based on the above analysis, it can be concluded that: A device shall monitor the R2D Upper Layer Data Transfer message until it receives a NACK Feedback message address to the device, or a paging message—with the only exception being a paging message that carries the same Transaction ID as the stored one.

Proposal 2.4 (Issue 5-4):

A device shall monitor the R2D Upper Layer Data Transfer message until it receives a NACK Feedback message address to the device or a A-IoT Paging message, except for A-IoT Paging messages that contain the same Transaction ID as the stored Transaction ID.

- Adopt the text proposal #2.4.

### text proposal #2.4

------------------Start of text proposal #2.4------------------

5.4.3 	R2D message reception

Once the device transmitted the D2R Upper Layer Data Transfer message, the A-IoT MAC entity monitors for R2D Upper Layer Data Transfer message until the device receives a NACK Feedback message address to the device or A-IoT Paging message except for that the value of the Transaction ID field is the same as the stored Transaction ID. Upon reception of an R2D Upper Layer Data Transfer message, the A-IoT MAC entity shall:

<text omitted>

------------------End of text proposal #2.4------------------

## Confidentiality protection indication

SA3 has specified some security procedure on information protection during command procedure below.

3. The AIOTF shall construct a AIOT NAS Command Request and protect the message based on the KCommand_enc, KCommand_int, the confidentiality and integrity algorithms for the AIoT device. The AIOTF shall send the protected Command Request containing an indication on whether cyphering is activated to NG-RAN. NOTE 1: The whole AIOT NAS Command Request message is integrity protected. If confidentiality algorithm is not null-scheme, the AIOT NAS Command Request message is partly ciphered with the exception that the selected protection algorithms are in clear text. 4. The NG-RAN shall send a R2D message containing the protected AIOT NAS Command Request as specified in as specified in TS 38.300 [3] and TS 38.391 [6].

And it has two ciphering algorithms, which are either NEA0 and 128-NEA2. In that sense, one bit is enough to indicate the ciphering algorithms. There are two reserved bits in R2D Upper Layer Data Transfer message when Choice Indication (CI) indicates Data SDU field is included. Since NAS layer indication may require at least one additional byte while AS layer indication does not require additional bit, i.e., using one of reserved bits, it is benefit to indicate confidentiality algorithm choice by AS layer. Since cyphering activation information is sent to NG-RAN from AIOTF based on TS 33.369 [1], it is not transparent from reader’s perspective. In that sense, reader indicating the corresponding information is valid.

Proposal 2.5:

Since cyphering activation information is sent to NG-RAN from AIOTF, from RAN2 perspective, one reserved bit in R2D Upper Layer Data Transfer message can be used to indicate confidentiality algorithm.

- Send LS to SA3 and CT1 about the information.

## Scheduling two bytes D2R Upper Layer Data Transfer message

D2R TBS in D2R scheduling info field is from {1, 2, …, 125} bytes that indicating 2 bytes for D2R message is valid from signaling perspective. For D2R Upper Layer Data Transfer message, it includes at least two bytes to include D2R Message Type, More Data Indication (MDI) and SDU Length. Data SDU may present if upper layer data is available.

Based on analysis above, upper layer data may be not available for initial transmission. From scheduling perspective, scheduling a large TBS when there is no data available is less efficient. From read’s implementation perspective, reader could poll whether data is available at the device and schedule a large TBS when the data is available. MDI field could be helpful to reader’s implementation. For example, reader could schedule a small TBS used for polling, and if the data is available, MDI will be set to 1. After that, reader may schedule a large TBS for message transmission.

However, for inventory report, there is a problem that reader needs to avoid segmentation for the D2R Upper Layer Data Transfer message for inventory response. We think segmentation means partial data is transmitted in Data SDU field. If Data SDU field is not present, it is not a segmentation. Reader may implement polling mechanism by scheduling a two bytes D2R Upper Layer Data Transfer message. Based on the received MDI field, reader can know whether data is available at device side. It is benefit for authentication calculation where the needed time is unknown and long.

Proposal 2.6:

Clarify that SDU data field is optional. SDU data field in D2R Upper Layer Data Transfer message is absent when upper layer data is not available or when D2R TBS field is indicated as two bytes.

- Adopt the text proposal #2.6.

- Note: Absent Data SDU field is not regarded as segmentation and reader does not need to avoid it for inventory response.

- Note: The two bytes D2R Upper Layer Data Transfer message could be used by reader as polling of whether upper layer data is available at device side.

### text proposal #2.6

------------------Start of text proposal #2.6------------------

5.4.2	D2R message transmission

Upon initiation of the procedure corresponding to the A-IoT access procedure or reception of an R2D Upper Layer Data Transfer message which contains either the Data SDU field or the Received Data Size field set to 0, the A-IoT MAC entity shall:

1>	apply the D2R Scheduling Info, received in the A-IoT Paging message with Access Type set to CFA or in the Random ID Response message or in the R2D Upper Layer Data Transfer message containing either the Data SDU field or the Received Data Size field set to 0, whichever triggered the initiation of the procedure;

1>	if upper layer data is available to be transmitted:

2>	if the size of the resulting MAC PDU including the total upper layer data is smaller than or equal to the resource size given by the D2R TBS in the D2R Scheduling Info:

3>	generate the D2R Upper Layer Data Transfer message, as follows:

4>	set the More Data Indication field to value 0;

4>	include SDU Length field and Data SDU field;

4>	if the size of the resulting MAC PDU including the total upper layer data is smaller than the resource size given by the D2R TBS in the D2R Scheduling Info:

5>	include the MAC Padding field;

3>	instruct the physical layer to transmit the D2R Upper Layer Data Transfer message and indicate the L1 parameters to the physical layer, as specified in clause 6.2.1.6;

2>	else (the size of the resulting MAC PDU including the total upper layer data is larger than the resource size given by the D2R TBS in the D2R Scheduling Info):

3>	initiate the segmentation procedure for the upper layer data SDU as specified in 5.4.4;

1>	else (i.e., upper layer data is not available to be transmitted):

2>	generate the D2R Upper Layer Data Transfer message, as follows:

3>	set the More Data Indication field to 0;

3>	set the SDU Length field to 0;

3>	if the size of the resulting MAC PDU including no upper layer data is smaller than the resource size given by the D2R TBS in the D2R Scheduling Info:

4>	include the MAC Padding field;

2>	instruct the physical layer to transmit the D2R Upper Layer Data Transfer message and indicate the L1 parameters to the physical layer, as specified in clause 6.2.1.6.

NOTE:	It is up to reader’s implementation to avoid segmentation for the D2R Upper Layer Data Transfer message for inventory response. When upper layer data is available to be transmitted and Data SDU field is absent, it is not regarded as segmentation and reader does not need to avoid it.

5.4.4	D2R segmentation

Upon initiation of this D2R segmentation procedure according to clause 5.4.2, or upon reception of R2D Upper Layer Data Transfer message containing the Received Data Size field not set to 0, as specified in clause 5.4.3, after this segmentation procedure is initiated, the A-IoT MAC entity shall:

1>	apply the received D2R Scheduling Info, received in the R2D Upper Layer Data Transfer message containing the Received Data Size field;

1>	generate the D2R Upper Layer Data Transfer message for this segment according to resource size given by the D2R TBS in the D2R Scheduling Info, as follows:

2> if resource size given by the D2R TBS in the D2R Scheduling Info is two bytes:

3> not include the Data SDU field and set the SDU Length field to 0;

3>	set More Data Indication field to value 1;

2> else:

3> include the SDU Length field and set the Data SDU field to include the segment which starts from the (x+1)th byte of the original upper layer data SDU, where x=0 if the Received Data Size field is not included, otherwise x equals to the value indicated by the Received Data Size field;

3>	if the segment is the last segment of the original upper layer data SDU:

4>	set More Data Indication field to value 0;

4>	if the size of the resulting MAC PDU including the segment is smaller than the resource size given by the D2R TBS in the D2R Scheduling Info:

5>	include the MAC Padding field;

3>	else:

4>	set More Data Indication field to value 1;

1>	instruct the physical layer to transmit the D2R Upper Layer Data Transfer message and indicate the L1 parameters to the physical layer as specified in clause 6.2.1.6.

6.2.2.2	D2R Upper Layer Data Transfer message

Figure 6.2.2.2-1 shows the format of the D2R Upper Layer Data Transfer message.

The fields in this message are defined as follows:

-	D2R Message Type: This field indicates the message type. See the Table 6.1-2. The length of the field is 2 bits.

-	R: The 6 bits are set to 0.

-	More Data Indication (MDI): This field indicates whether there are more upper layer data to be sent from the device (when set to 1) or not (when set to 0). This length of this field is 1 bit.

-	SDU Length: This field indicates the length of the Data SDU field in the unit of byte. The length of this field is 7 bits.

-	Data SDU: This field is of variable length and includes the upper layer data. This field is optional.

-	MAC Padding: This field includes padding bits. This field is optional.

------------------End of text proposal #2.6------------------

# Conclusion and Proposal

We have the following observations and proposals:

- Proposal 2.1 (Issue 5-1):

Upon receiving paging with Paging ID Presence Indication field indicates Paging ID field is absent; device indicate it to the upper layers. The device shall only consider itself selected if the upper layer confirms that it is not permanently disabled.

- Adopt the text proposal #2.1.

- Proposal 2.2:

One new paging ID type field is included in CBRA paging message, which is used to indicate the type of paging ID field from either single device ID or group device ID (filter information).

One new paging ID protection field is included in both CBRA and CFA paging message, which is used to indicate whether protection procedure for paging id is applied or not.

- Adopt the text proposal #2.2.

- Proposal 2.3:

D2R Upper Layer Data Transfer message retransmission due to no upper layer data available for initial transmission should be supported for CFA inventory procedure. The retransmission is scheduled by R2D Upper Layer Data Transfer message.

- Adopt the text proposal #2.3.

- Note: Authentication calculation may require a lot of time at device side, that device may not generate inventory report at the time to transmit D2R Upper Layer Data Transfer message after CFA paging message.

- Proposal 2.4 (Issue 5-4):

A device shall monitor the R2D Upper Layer Data Transfer message until it receives a NACK Feedback message address to the device or a paging message, except for paging messages that contain the same Transaction ID as the stored Transaction ID.

- Adopt the text proposal #2.4.

- Proposal 2.5:

Since cyphering activation information is sent to NG-RAN from AIOTF, from RAN2 perspective, one reserved bit in R2D Upper Layer Data Transfer message can be used to indicate confidentiality algorithm.

- Send LS to SA3 and CT1 about the information.

- Proposal 2.6:

Clarify that SDU data field is optional. SDU data field in D2R Upper Layer Data Transfer message is absent when upper layer data is not available or when D2R TBS field is indicated as two bytes.

- Adopt the text proposal #2.6.

- Note: Absent Data SDU field is not regarded as segmentation and reader does not need to avoid it for inventory response.

- Note: The two bytes D2R Upper Layer Data Transfer message could be used by reader as polling of whether upper layer data is available at device side.

# Reference

[1] 3GPP TS33.369-10: "Security aspects of ambient IoT services in 5G".

[2] 3GPP TS23.003-j30: "Numbering, Addressing and Identification".

[3] 3GPP TS23.369-j10: Architecture support for Ambient power-enabled Internet of Things.

[4] S3-252933: Reply LS on paging ID length