3GPP TSG-RAN WG2 Meeting #131bis                                                   R2-2506840

Prague, Czech Republic, Oct. 13th-17th 2025

Source:              CATT, CBN

Title:	             Disucssion on RRC for XR

Agenda Item:	8.7.2

Document for:	Discussion and Decision

Introduction

In this contribution, we will discuss leftover issues of RRC for XR.

Discussion

[V050] Coexistence of remaining time based RLC retransmission and polling

About coexistence of remaining time based RLC retransmission and polling, there’s discussion in RAN2#130 meeting as the following:

Excessive polling and retransmissions due to polling (RLC-8, RLC-9) R2-2503624 Discussion on RLC remaining issues for XR vivo discussion Rel-19 NR_XR_Ph3-Core Proposal 3: (RLC-8) Similar to autonomous retransmission, polling shall only be triggered once per RLC SDU when its remaining time falls below a specified threshold. Proposal 4: (RLC-9) No additional conditions are needed for the polling enhancement. DISCUSSION on P3: - LGE thinks P3 is not needed. It does not matter if multiple polls are triggered and sent. One SR will be sent anyway. Supports P4. - Vivo clarifies that after reviewing specification, they share understanding from LGE. - LGE also thinks P3 causes issues in case of RLC segmentation. - Xiaomi supported original P3 to limit the polling. IDT has similar concerns. - LGE clarifies that the second poll is due to remaining time condition while the first one was due to legacy conditions. Thinks we do not have to do anything about this. - Huawei agrees with LGE and prefers not to agree P3. - Lenovo wonders whether we need to specify that enhanced polling will normally be triggered before autonomous retransmission. - (RLC-8) We just keep the current specifications for polling triggering, i.e. no need to specify that polling shall only be triggered once per RLC SDU when its remaining time falls below a specified threshold, unless an issue is identified with this - (RLC-9) No additional conditions are needed for the polling enhancement.

We agree that when both remaining time based RLC polling and remaining time based RLC retransmission are configured for the same Tx RLC entity, if the threshold for enabling remaining time based RLC retransmission is set to larger than that for enabling remaining time based RLC polling, remaining time based RLC retransmission would have already been triggered before the UE polls the Rx RLC entity to request the STATUS report. However, the polling still makes sense if remaining time based RLC retransmission fails. And also we see no system issues to follow current running CR. Therefore, we propose not to specify that remaining time based RLC polling will normally be triggered before remaining time based RLC retransmission.

Proposal 1: [V050] Do not specify that remaining time based RLC polling will normally be triggered before remaining time based RLC retransmission.

[N091] UAI for measurement gap cancellation preference

In RAN2#130 meeting post email discussion, the agreement about prohibit timer is that when multiple gap configurations are provided, a single timer is maintained for all the gap configurations. This means there’s no need to differentiate the gap configurations when prohibit timer is running. However, if it’s allowed UAI to be sent for a newly configured measurement gap without delay even if UAI has been sent for other MG before, this means the newly configured measurement gap has higher priority than others. This breaks the discussion agreement. Therefore, we propose to keep current running CR unchanged.

Proposal 2: [N091] Keep the current specification unchanged and do not allow UAI to be sent for a newly configured measurement gap without delay.

whether to specify RAN2#129 meeting agreement about dsr- ReportingThresList

In current RRC running CR R2-2506336, dsr-ReportingThresList is specified and listed below:

dsr-ReportingThresList List of remaining time thresholds configured in ascending order for reporting delay status information in the Multiple Entry DSR MAC CE, as specified in TS 38.321 [3]. At least one configured DSR reporting threshold should be no lower than the remainingTimeThreshold. If at least one LCG is configured with dsr-ReportingThresList, any LCG configured with remainingTimeThreshold should be configured with dsr-ReportingThresList. The IE DSR-ReportingThreshold in number of milliseconds.

In RAN2#129 meeting, the agreement about dsr- ReportingThresList is as follows:

If UE is configured to use R19 DSR, then any LCG with a triggering threshold shall be configured with at least one reporting threshold.

The above RAN2#129 meeting agreement about dsr- ReportingThresList has not been captured in the current RRC specificaiton. If this agreement is not specified in RRC running CR and left to network implementation, we find in below Case 2, it would lead to UE malfunction.

For example, based on agreement on DSR UE capabilities, for Rel-19 UE, there’re two cases to report UE capabilities to network. Case 1, UE reports both delayStatusReport and multipleEntryDelayStatusReport capabilities to network. Case 2, UE only reports multipleEntryDelayStatusReport capability to network. If network decides to configure to trigger DSR, for case 1, if network configures to use R19 DSR, network would configure RRC parameter remainingTimeThreshold and dsr-ReportingThresList, if network configures to use R18 DSR, network would configure only RRC parameter remainingTimeThreshold.

And for case 2, network shall configure to use R19 DSR, network shall always configure RRC parameter dsr-ReportingThresList and remainingTimeThreshold based on RAN2#129 meeting agreement, otherwise if network only configures remainingTimeThreshold, it would lead to UE malfunction.

Observation 1: The current RRC specification does not specify RAN2#129 meeting agreement about dsr-ReportingThresList. If it’s left to network implementation, when Rel-19 UE only reports multipleEntryDelayStatusReport to network, and network only configures remainingTimeThreshold, it would lead to UE malfunction and breaks RAN2#129 meeting agreement about dsr- ReportingThresList.

It’s therefore proposed that RRC parameter dsr-ReportingThresList field description should be enhanced.

Proposal 3: RRC parameter dsr-ReportingThresList field description should be enhanced to capture the agreement in RAN2#129 meeting about dsr- ReportingThresList.

Conclusion

According to the analysis in section 2, the following observation and proposals were made:

Proposal 1: [V050] Do not specify that remaining time based RLC polling will normally be triggered before remaining time based RLC retransmission.

Proposal 2: [N091] Keep the current specification unchanged and do not allow UAI to be sent for a newly configured measurement gap without delay.

Observation 1: The current RRC specification does not specify RAN2#129 meeting agreement about dsr-ReportingThresList. If it’s left to network implementation, when Rel-19 UE only reports multipleEntryDelayStatusReport to network, and network only configures remainingTimeThreshold, it would lead to UE malfunction and breaks RAN2#129 meeting agreement about dsr- ReportingThresList.

Proposal 3: RRC parameter dsr-ReportingThresList field description should be enhanced to capture the agreement in RAN2#129 meeting about dsr- ReportingThresList.

The corresponding TP for Proposal 3 are attached in section 4.

Annex

Text Proposal 3

--------------------------------------- Start of the text proposal------------------------------------------------------

- Annex TP to TS 38.331

LCG-DSR-Config field descriptions

lcg-Id Identifier of the Logical Channel Group which the DSR configuration refers to.

remainingTimeThreshold Remaining time threshold used for triggering Single Entry and Multiple Entry DSR for the logical channels belonging to this Logical Channel Group, as specified in TS 38.321 [3]. Value in number of milliseconds.

dsr-ReportingThresList List of remaining time thresholds configured in ascending order for reporting delay status information in the Multiple Entry DSR MAC CE, as specified in TS 38.321 [3]. At least one configured DSR reporting threshold should be no lower than the remainingTimeThreshold. If at least one LCG is configured with dsr-ReportingThresList, any LCG configured with remainingTimeThreshold should be configured with dsr-ReportingThresList. The IE DSR-ReportingThreshold in number of milliseconds. This field is always configured when remainingTimeThreshold is configured in case gNB decides to use Multiple Entry DSR MAC CE.

dsr-ReportNonDelayCriticalData Indicates whether the UE should consider the non-delay reporting data ahead of delay reporting data in the delay status reporting data volume calculation for the Logical Channel Group as specified in TS 38.322 [4] and TS 38.323 [5].

--------------------------------------------- End of the text proposal ------------------------------------------------