3GPP TSG-RAN WG2 #131bis          	                                                             R2-2506845

Prague, Czech Republic, Oct 13th – 17th, 2025

Source:	CATT

Title:	6GR UP Architecture and Functions

Agenda Item:	10.3.1

Document for:	Discussion and Decision

- Introduction

This contribution discusses UP architecture and functions which should be decided in 6G Day1 as a basis of 6G air transmission. We propose unified UP architecture which takes 5G L2 sub-layers as baseline and considers simplification and adaptation. The UP functions are discussed based on the layered architecture. And then we propose considering NTN specific design within the unified framework.

- Discussion

  - UP Architecture

The UP protocol stack has employed PDCP/RLC/MAC sub-layers since 3G. The 5G system inherited this protocol stack division and introduced SDAP sub-layer based on the 5G QoS flow-to-DRB mapping characteristics. When organizing air interface transmission, the transmitter processes each packet through each sub-layer one by one, adding header at each sub-layer, and finally organizes them into TBs for physical layer transmission, as shown in Figure 1. Except for TM mode, which operates top-down without any L2 processing or L2 identifier addition. Due to its well-defined functional partitioning and clear interlayer relationships, this layered processing approach provides advantages in facilitating standardization and rapid industrial development.

Figure 1 5G UP Data Flow Example

Observation 1: The layered UP protocol stack framework facilitates standardized design and industry development.

However, with increasing service types, usage scenarios, and diverse UE types, the rigidity of the fixed protocol stack has become increasingly prominent, which had been mentioned during 5G Rel-18/Rel-19. The issues incurred by the fixed protocol stack mainly include: excessive L2 header overhead with multiple sub-layers performing header processing, resulting in a high ratio of headers to data payload; L2 processing complexity as every sub-layer needs to process each packet.

Considering 5G protocol stack, there are some potential optimizations for UP sub-layers.

- SDAP sub-layer: The DL SDAP header is used for reflective QoS functionality. In practice, after the QoS flow-to-DRB mapping rule is established, if remapping is not required for the QoS flow, the following SDAP headers are unnecessary overhead in downlink. For the UE, parsing the SDAP header of each SDAP PDU to determine whether remapping is needed increases implementation complexity.

- RLC sub-layer: In 5G Rel-15, it was decided that there is no RLC SN for RLC UM PDU without segmentation. However, due to 5G protocol stack design principles, 1-byte RLC header must be added. For the transmitter, this 1-byte RLC header is redundant overhead, and the process of adding RLC headers for each packet increases implementation complexity. For the receiver, even if no RLC function is performed, the RLC header must be parsed and removed, further adding implementation complexity.

- MAC sub-layer: The MAC sub-layer has the function of logical channel multiplexing, thus MAC subheader is generally required in most scenarios. However, for special cases, such as low-capability sensors, or single-purpose devices in IIOT, the function of logical channel multiplexing may be not needed, thus the MAC subheader becomes unnecessary.

Observation 2: The 5G fixed protocol stack architecture has disadvantages of large L2 header overhead and high processing complexity for some scenarios and packets.

To address these issues, some companies have proposed organizing L2 PDUs based on discrete modularization of user plane functions. Note that for a single packet, L2 protocol stack functions are performed serially, and related functions are impossible to be implemented in parallel (e.g., header compression and PDU segmentation cannot be executed simultaneously for one packet). While layered processing offers advantages of clear interlayer relationships and standardization/development convenience, the UP protocol stack framework based on layered structure remains necessary. Therefore, 6G should maintain the layered UP protocol stack framework and pursue methods to overcome the shortages of the fixed protocol stack. The 5G UP sub-layers have clear functional divisions, while 6G services do not require fundamental changes. We can use SDAP/PDCP/RLC/MAC sub-layers as the framework basis for discussion. Further, 6G should support diverse usage scenarios and service requirements without adopting fragmented designs. Customized protocol sub-layers and functions applicable to different scenarios and requirements within a unified framework should be considered.

Observation 3: 6G should efficiently support diverse services (e.g., Immersive Communication, HRLLC) and device types within a unified framework. Thus, customizable protocol stack with variable sub-layers and functions in the unified framework is a reasonable design direction.

Proposal 1: RAN2 consider unified UP protocol stack framework and taking 5G L2 sub-layers as baseline.

Proposal 2: RAN2 further study simplification and/or adaptation protocol stack for certain services or packet level.

  - UP functions

SDAP

SDAP sub-layer was introduced in 5G for performing QoS flow-to-DRB mapping. Its main functions are QoS flow-to-DRB mapping/remapping and reflective QoS. The QoS flow-to-DRB mapping mechanism was decided in CN which can be one-to-one or multiple-to-one, different to the one-to-one mapping between eps-Bearer and DRB in 4G.

The combination of SDAP and PDCP was proposed during pre-6G discussion. Considering for a transmitter, there is only one SDAP entity per PDU session which can be associated to multiple PDCP entities, if SDAP is combined into PDCP, it is not clear how to perform QoS flow mapping and remapping.

Observation 4: The association between SDAP entity and PDCP entity can be one-to-multiple. If SDAP is combined into PDCP, it is not clear how to perform QoS flow mapping and remapping.

Whether to adopt QoS flow mechanism including QoS flow mapping/remapping and reflective QoS flow in 6G will be decided by SA2. RAN2 can keep SDAP in UP protocol stack and assume the basic QoS flow mapping functions in this early stage.

Proposal 3: RAN2 assume at least support QoS flow to DRB mapping function. This assumption can be further revisited based on SA2 progress.

PDCP

In 5G, the PDCP sub-layer supports the following functions:

The PDCP layer supports the following functions: - transfer of data (user plane or control plane); - maintenance of PDCP SNs; - header compression and decompression using the ROHC protocol; - header compression and decompression using the EHC protocol; - uplink data compression and decompression using the UDC protocol; - ciphering and deciphering; - integrity protection and integrity verification; - timer based SDU discard; - PDU Set discard; - PSI based SDU discard; - for split bearers and DAPS bearer, routing; - duplication; - reordering and in-order delivery; - out-of-order delivery; - duplicate discarding.

Some PDCP functions (ROHC, ciphering, integrity protection, duplication, SN maintenance, SDU discard, reordering and in-order delivery, out-of-order delivery, etc.) were supported from Rel-15. EHC and DAPS were introduced in Rel-16. UDC was introduced in Rel-17. PDU set was introduced in Rel-18.

All PDCP functions introduced in 3GPP Release 15 shall be natively supported in 6G networks from Day1. Furthermore, some PDCP features introduced in subsequent releases (Post-15) should also be incorporated into the initial 6G deployment. Additionally, certain PDCP functionalities require performance enhancements to meet evolving service requirements.

Uplink data compression introduced in Rel-17, which enhances SIP signaling transmission while optimizing UL resource utilization and reducing latency. In 6G Day1, IMS is also supposed to be supported. Therefore, UDC should be supported to ensure efficient IMS-SIP signaling handling.

PDU set was introduced in 5G Rel-18 for XR. As defined by ITU's IMT-2030 recommendations, Immersive Communication (encompassing immersive XR, remote multi-sensory telepresence, and holographic communications) represents one of the key usage scenarios. Consequently, PDU set functionality should be natively supported from the initial deployment of 6G networks. Additionally, Release-20's XR Work Item is currently investigating PDU set applications for mobile AI data transmission. Future enhancements may include more comprehensive PDU set descriptors to better characterize PDU/PDU set attributes within QoS flows.

In 5G, PDCP duplication is applied per RB, meaning that once activated, all packets transmitted by the PDCP entity are duplicated. PDCP duplication mechanism significantly enhances reliability. However, its primary drawback lies in reducing resource efficiency. To address this, selective PDCP duplication was proposed in 3GPP Release-16's IIoT work item, where only critical packets within a radio bearer would be duplicated. However, no definitive conclusion was reached due to the lack of QoS parameters finer than QoS flow granularity in Release 16. With the introduction of PDU set in Release-18, selective PDCP duplication can now be implemented based on PDU set information. Moving forward, 6G networks can further study resource efficiency improvement for PDCP duplication by leveraging PDU set-based policies.

Proposal 4: RAN2 support the following basic functions in 6G Day1: ROHC, ciphering, integrity protection, duplication, SN maintenance, SDU discard, reordering and in-order delivery, out-of-order delivery, UDC and PDU set handling and further enhancement should be studied, e.g., PDCP duplication to improve resource efficiency.

RLC

5G RLC layer mainly include 2 functions: segmentation/re-segmentation and ARQ retransmission. 5G removes the concatenation function from the 4G RLC sub-layer and places it in the MAC sub-layer. There are two concerns regarding the RLC sub-layer: 1) The necessity of the RLC ARQ mechanism; 2) L2 overhead caused by the separate RLC header, especially RLC SN.

The necessity of the RLC ARQ mechanism

Reliability is a critical metric in telecommunication. As shown below, ITU tends to set the similar reliability requirement as 5G as below. Meanwhile, SA1 mentions higher target of 1-10-7 [2].

TR38.914 v0.2.0 [1] The minimum requirement for the reliability is 1-10-5 success probability of transmitting a layer 2 PDU (protocol data unit) of 32 bytes within 1 ms in channel quality of coverage edge for the Urban Macro-HRLLC [test environment], assuming small application data (e.g. 20 bytes application data + protocol overhead).

If relying on L1 HARQ feedback and HARQ retransmission, the reliability targets may be achievable within a certain range. However, considering the reliability requirements are general Macro-scenario targets rather than tailored to specific scenarios, and the HARQ feedback indication itself has error probabilities such as NACK-to-ACK error or ACK-to-NACK error, RLC ARQ mechanism as a more robust retransmission mechanism is still needed. Additionally, in ARQ mechanism, RLC PDU/PDU segments are retransmitted, allowing for better adaptation to available UL resources and channel conditions during the reassembly of transport blocks, thereby enabling more efficient retransmissions and resource utilization.

Observation 5: L2 retransmission is still needed for the services with high reliability requirements.

For 5G RLC ARQ, low latency and high reliability are two contradict metrics. ARQ feedback and retransmissions can be triggered only when specific conditions are satisfied. Generally speaking, ARQ retransmission can be triggered after multiple HARQ retransmission failures. The consequence of 5G RLC failure is severe, which leads to RLF and subsequent RRC Re-establishment. These issues have been recognized in 5G XR. 5G Rel-19 XR discussed two issues: timely RLC retransmission(s), and avoid unnecessary retransmissions.

For 6G Day1, protocol stack design could address the above issues fundamentally by tailoring higher-layer retransmissions to service characteristics and packets features. For example, 6G can extend 5G XR awareness feature to broader scope of Service Awareness. When base station and UEs can identify the importance of packets and the correlations among the packets (such as PDU-set) within a QoS flow, they can treat the packets differently. Thus, dynamic ARQ for specified PDUs could be considered.

Observation 6: For 5G RLC AM, low latency and high reliability are two contradict metrics. 6G should improve L2 retransmission mechanism to satisfy reliability within low latency.

L2 overhead due to RLC header

The RLC sub-layer overhead comprises two components: the RLC header in RLC PDU and feedback/retransmission overhead. The latter should be inherently considered during RLC retransmission mechanism design.

The overhead of RLC header includes:

- RLC SN: 5G had discussed only one SN for PDCP and RLC but didn’t adopt it. One reason is that it is not clear how to perform RLC SN maintenance and RLC reordering in DC architecture. However, if we can identify the potential issues and consider it during 6G RLC design (e.g., optimizing RLC window mechanism and the initial values of the related parameters), it may not be a big deal.

- Segment Indicator (SI)/Segment Offset (SO): These fields used to indicate PDU (re)segmentation. Segmentation/re-segmentation is a mandatory function to align transmitted data size with UL grant allocation. Even if segmentation function is put in other L2 sub-layer, the SI/SO overhead would remain unavoidable.
Therefore, to decrease the L2 overhead, reusing PDCP SN and RLC SN can be considered. But SI/SO field may be needed when (re)segmentation is required.

Observation 7: Reusing of PDCP SN and RLC SN can decrease L2 overhead, and SI/SO fields are needed for PDU (re)segmentation.

Proposal 5: RAN2 support following functionalities: (re)segmentation and L2 retransmission and further study: a) reliability and latency of L2 ARQ, and b) the overhead issue of L2 sub-layer headers.

MAC

In 5G, the following functions are supported in MAC:

The MAC sublayer supports the following functions: - mapping between logical channels and transport channels; - multiplexing of MAC SDUs from one or different logical channels onto transport blocks (TB) to be delivered to the physical layer on transport channels; - demultiplexing of MAC SDUs to one or different logical channels from transport blocks (TB) delivered from the physical layer on transport channels; - scheduling information reporting; - error correction through HARQ; - logical channel prioritization; - priority handling between overlapping resources of one UE; - radio resource selection.

These functions are all essential functions for data transmission, hence, the above MAC functions can be inherited from 5G to 6G. In order to support the above functions better, the following MAC procedures can be studied in 6G:

- Random Access;

- UL TA maintenance;

- Scheduling mechanisms (e.g., dynamic scheduling, DL SPS and UL CG);

- UL scheduling information reporting(e.g., SR, BSR, DSR and PHR);

- HARQ;

- LCP;

- Beam management;

- Power saving related mechanism (e.g., DRX and etc.).

Random Access

In 5G Rel-15, 4-step is introduced as the basic Random Access procedure. In Rel-16, 2-step is further introduced for reducing the Random Access latency. Regarding to the two RA types, the lessons from 5G are listed below:

- When Random Access is initiated, UE should first select 2-step RA or 4-step RA based on RSRP. If 4-step is selected, it cannot fallback to 2-step; and if 2-step is selected, it can fallback to 4-step only after a number of RACH attempts. It is inflexible and UE may not able to select the best suitable RA type for each RACH attempt, e.g., when UE initiates the Random Access procedure, if it is in the cell edge, 4-step is selected. But during the RA attempts, it moves to the cell center, it still needs to perform 4-step Random Access procedure. This will increase the RACH latency.

- In 5G Rel-17/18/19, further enhancements (e.g., coverage enhancement, early identification of feature/feature combination and supporting SBFD) were introduced step by step. Since there are two RA types, when discussing enhancements in later release, both the impacts to 2-step RA and to 4-step RA should be considered separately. Hence, defining two RA types is not beneficial for further extension in later release.

Observation 8: In 5G, the design of two RA types (2/4-step RA) results that RA type selection is inflexible for each RA attempt and not easy for further extension in later release.

Based on observation 8, in order to solve the above issues, an unified Random Access procedure can be considered in 6G Day1. In the unified Random procedure, UE can select whether to transmit PUSCH for each RA attempt. One example of the unified solution is as below:

Figure 2 Example of the unified Random Access procedure in 6G Day1

During the Random Access procedure, it is important to identify the Random Access features early, e.g. early identification of Msg1/3 repetition number, UE types or service type. In 5G NR, RACH partitioning was introduced for the early identification, and the corresponding 5G RACH partitioning are summarized in Table 1.

Table 1 RACH partitioning in NR

| Release | RACH partitioning |
| --- | --- |
| Rel-15 | Group A/B |
|  | On demand other SI |
|  | BFR |
| Rel-16 | 2-step RACH (group A/B) |
|  | On demand PosSIB |
| Rel-17 | Redcap, SDT, Slice Msg3 repetition |
|  | Redcap on PosSIB |
| Rel-18 | Msg1 repetition |
|  | eRedcap |
|  | Msg1 repetition for SI request |
|  | Msg1 repetition for PosSIB |
|  | Msg1 repetition for Redcap SI request |
|  | Msg1 repetition for Redcap PosSIB |
| Rel-19 | On demand SIB1 |

It can be seen that various RACH resource configurations bring severe fragmentations on RACH resources which may degrade the transmission efficiency. The evolution of RACH partitioning involves multiple releases and some of the features even inherited from LTE. In light of this, it can be seen that parts of the RACH resource assignment have comparable function, for example, group A/B and SDT which both are required to check the data volume before RA is performed, while some have substitutable procedure, e.g., (e)Redcap, on-demand SIBx, which can be requested via Msg3. Consequently, the number of features for early indication shall be well managed to simplify the system design and reduce RACH partitioning. The solutions can be combination of the RACH features, e.g. Msg1 repetition and Msg3 repetition, e.g., when UE indicating its support of Msg1 repetition, it implicitly indicates UE also supports Msg3 repetition.

Observation 9: RACH partitioning in 5G brings too many RACH resource fragmentations which greatly degrades RACH efficiency.

UL TA maintenance

In 5G, UL TA is maintained based on TAC and TAT for each TAG which is inherited from LTE. In 6G, the same mechanism can be used as baseline.

Scheduling mechanisms

In 5G, both dynamic scheduling and grant-free scheduling (DL SPS and UL CG) were supported. Dynamic scheduling is used for traffic which without fixed traffic pattern; while grant-free scheduling mainly focus on traffic with fixed periodicity and packet size. The benefit of introducing DL SPS/UL CG is to reduce the PDCCH overhead. In 6G, PDCCH overhead is also important, hence both dynamic and grant-free scheduling should also be supported.

UL scheduling information reporting

In 5G, the UL scheduling information includes SR, BSR, DSR and PHR. PHR mainly depends on RAN1, hence, only SR/BSR/DSR are discussed here. Regarding to the SR/BSR/DSR, some lessons can be learnt from 5G:

- Before UE gests UL grant, it should perform first sends SR, and when it gets UL grant, it can further report BSR. This will increase the UL latency;

- For the BSR and DSR, they are both used for buffer status reporting. The difference between them is that only delay-critical data is reported in DSR. These two functions is a bit repetitive.

Observation 10: In 5G, the SR/BSR procedures increase the UL latency and the functions of BSR and DSR are a bit repetitive.

From the above 5G lessons, in 6G, it is suggested to consider unified design for BSR and DSR. Furthermore, it can also consider using SR to carry more information, but whether it is feasible depends on RAN1.

LCP

In 5G Rel-15, LCP is based on token bucket. The main motivation of using token bucket is to avoid starvation. Furthermore, in 5G Rel-18 XR, DSR and latency based LCP (e.g., additional priority for delay-critical LCH) were introduced to meet the delay-critical service requirements. The DSR and latency based LCP can also avoid starvation in a certain degree, which seem a bit repetitive with token bucket mechanism from the function perspective. Hence, in 6G, it can further evaluate whether token bucket should be reused or new LCP procedure can be considered.

Beam management

In 5G, beam management is introduced for FR2. In 6G, there are three frequency ranges, e.g., FR1/2/3, will be used, hence, it should also support beam management. But how to select the beam and perform beam failure detection mainly depends on RAN1, hence, RAN2 can wait for more RAN1 progress before discussing this topic.

Power saving related mechanisms

In NR, for the RRC_CONNECTED state, UE power saving related to DRX mainly focuses on reducing unnecessary PDCCH monitoring. For RRC connected mode, various power saving techniques were supported in 5G NR including DRX, DCP, dual DRX group and LP-WUS.

The design target of the PDCCH monitoring adaptation in 6GR should be perfectly match the different types of traffic patterns, such as immersive traffic, IM, web browsing, video streaming, gaming etc.  That means the PDCCH monitoring behavior could be adaptively adjusted based on the characteristics of the traffic patterns. DRX with/without DCP is more suitable for periodic traffic, but may lead to power saving gain decrease or the latency increase for the aperiodic traffic. While Option 1-2 of LP-WUS can well adapt to the uncertainty of the traffic arrival with guarantee of acceptable power saving gain. Therefore, LP-WUS which enables both lower transmission latency and acceptable power saving gain can be supported in 6G Day1. However, considering details on LP-WUS design and frequency range is still under RAN1 discussion, RAN2 can study power saving taking RAN1 progress into account.

Proposal 6: RAN2 support the main MAC functions in 5G and further study streamline and simplification of the corresponding procedures considering, e.g.: a) unified RA procedure and simplified RACH partitioning, b) unified BSR/DSR procedure, c) improved LCP procedure, and d) improved energy efficiency.

  - UP affected by NTN

Per guidance of the SID [3], 6G aim at a harmonized 6G radio design for TN and NTN. The UP architecture and functions given in section 2.1 and 2.2 are generally applicable to both TN and NTN. However, compared to the TN system, the NTN has a significantly longer propagation delay, which unavoidably brings differences in UP aspect. In 5G, on basic of the TN system, the following NTN specific enhancements to UP functions are specified:

- PDCP: discardTimer is extended up to 2000ms; t-Reordering is extended up to 4400ms.

- RLC: t-Reassembly timer is extended up to 2200ms.

- MAC:

  - The start of ra-ResponseWindow and ra-ContentionResolutionTimer is delayed with the UE-gNB RTT;

  - UE autonomously pre-compensate the TTA for the RTT between the UE and the RP based on the GNSS position, the ephemeris, and the Common TA parameters.

  - The UE may be configured to report Timing Advance to assist the network for scheduling.

  - Support HARQ-disabled mechanism.

  - UL scheduling aspects, including BSR enhancement and sr-ProhibitTimer extension.

In 6G Day1, it is proposed to study and identify the UP technical aspects affected by NTN characteristics. The 5G NTN design can be taken as baseline.

Proposal 7: Study and identify the UP technical aspects affected by NTN characteristics. The 5G NTN design can be taken as baseline.

- Conclusion

In this contribution, we provide below observations and proposals:

Observations:

Observation 1: The layered UP protocol stack framework facilitates standardized design and industry development.

Observation 2: The 5G fixed protocol stack architecture has disadvantages of large L2 header overhead and high processing complexity for some scenarios and packets.

Observation 3: 6G should efficiently support diverse services (e.g., Immersive Communication, HRLLC) and device types within a unified framework. Thus, customizable protocol stack with variable sub-layers and functions in the unified framework is a reasonable design direction.

Observation 4: The association between SDAP entity and PDCP entity can be one-to-multiple. If SDAP is combined into PDCP, it is not clear how to perform QoS flow mapping and remapping.

Observation 5: L2 retransmission is still needed for the services with high reliability requirements.

Observation 6: For 5G RLC AM, low latency and high reliability are two contradict metrics. 6G should improve L2 retransmission mechanism to satisfy reliability within low latency.

Observation 7: Reusing of PDCP SN and RLC SN can decrease L2 overhead, and SI/SO fields are needed for PDU (re)segmentation.

Observation 8: In 5G, the design of two RA types (2/4-step RA) results that RA type selection is inflexible for each RA attempt and not easy for further extension in later release.

Observation 9: RACH partitioning in 5G brings too many RACH resource fragmentations which greatly degrades RACH efficiency.

Observation 10: In 5G, the SR/BSR procedures increase the UL latency and the functions of BSR and DSR are a bit repetitive.

Proposals:

Proposal 1: RAN2 consider unified UP protocol stack framework and taking 5G L2 sub-layers as baseline.

Proposal 2: RAN2 further study simplification and/or adaptation protocol stack for certain services or packet level.

Proposal 3: RAN2 assume at least support QoS flow to DRB mapping function. This assumption can be further revisited based on SA2 progress.

Proposal 4: RAN2 support the following basic functions in 6G Day1: ROHC, ciphering, integrity protection, duplication, SN maintenance, SDU discard, reordering and in-order delivery, out-of-order delivery, UDC and PDU set handling and further enhancement should be studied, e.g., PDCP duplication to improve resource efficiency.

Proposal 5: RAN2 support following functionalities: (re)segmentation and L2 retransmission and further study: a) reliability and latency of L2 ARQ, and b) the overhead issue of L2 sub-layer headers.

Proposal 6: RAN2 support the main MAC functions in 5G and further study streamline and simplification of the corresponding procedures considering, e.g.: a) unified RA procedure and simplified RACH partitioning, b) unified BSR/DSR procedure, c) improved LCP procedure, and d) improved energy efficiency.

Proposal 7: Study and identify the UP technical aspects affected by NTN characteristics. The 5G NTN design can be taken as baseline.

- References

- RP-252882	TR 38.914 v0.2.0 for Study on 6G Scenarios and requirements

- 3GPP TR22.870 v0.4.0 Study on 6G Use Cases and Service Requirements

- RP-252912 Study on 6G Radio, RAN#109