3GPP TSG-RAN WG2 Meeting #131bis	R2-2506850

Prague, Czech Republic, Oct. 13th - 17th, 2025

Agenda Item:	10.3.1

Source:	OPPO

Title:	Discussion on the 6G user plane features

Document for:	Discussion, Decision

# Introduction

According to the 6G SID [1] for RAN WGs as quoted below, the user plane design is one major objective for RAN2 6G study.

RAN WG 6G SID in: Objective - Radio interface protocol architecture and procedures for 6GR [RAN2, RAN1, RAN4, RAN3] - User Plane architecture and protocol design [RAN2] Interim Milestone Interim results shall be delivered as per the milestones below, in coordination with the Rel-20 RAN Plenary 6G Study (FS_6G_RAN_Scen_Req). TSG#112 (June/2026): … RAN3 to provide interim study results to allow TSGs to make a decision on: - RAN-CN interface: P2P vs SBI - RAN internal interfaces: CU-DU split, CP-UP split. NOTE: It is planned to decide on Release-21 timeline in June/2026. TSG#113 (September/2026): RAN plenary to make a decision on additional migration option(s) (other than standalone, MRSS, and inter-RAT mobility between NR-6G). This includes decision on additional 6G-6G aggregation beyond 6G CA: 6G-6G DC. RAN plenary will task relevant RAN WGs for any specific technical analysis, as needed.

As the “RAN internal interfaces: CU-DU split, CP-UP split” and “additional migration option(s) (other than standalone, MRSS, and inter-RAT mobility between NR-6G)” are to be decided by the RAN plenary in TSG#112 (June/2026) and TSG#113 (Sep/2026) respectively, this contribution focuses on the user plane discussions on the L2 protocol stacks of standalone 6G, the support of both TN and TN and how to make the 6G user plane protocols more efficient and more agile to various services.

# Discussion

## RAN2 6G UP study and cross-WG coordination

According to the RAN2-led TR 38.804 for the Rel-14 5G study as quoted below, the user plane discussion of the air interface in RAN2 needs to consider the following aspects (with the coordination with other WGs) (Note: The SA indicated below for the user plane discussion mainly refers to SA2).

- 3GPP TR 38.804 [3], “Study on New Radio Access Technology; Radio Interface Protocol Aspects”: UP protocols functions and bearer types. (RAN2-led)

  - Overviews of L2 functions and L2 data flow (impacting RAN1/RAN3/SA)

  - Services and functions provided by each UP layer (including MAC/RLC/PDCP/SDAP) (impacting RAN1/RAN3/SA)

  - Description of bearer types (e.g. split/SCG bearer) (impacting RAN3)

  - Numerologies and scheduling durations (e.g. SCS and symbol/slot duration according to the inputs from RAN1)

  - ARQ and HARQ (impacting RAN1)

  - Scheduling schemes (e.g. DL/UL dynamic/configured scheduling) (impacting RAN1)

  - QoS control (e.g. QoS flow to DRB mapping) (impacting SA)

  - Random access procedure (impacting RAN1/RAN4)

  - UE power saving (e.g. DRX) (impacting RAN1/RAN4)

We consider that the UP aspects as listed should be considered and discussed for the 6G UP discussion. Additionally, we consider that the 6G UP discussion also needs to consider the extra 6G requirements, including network energy saving and the support of various device types (including satellite and IoT devices).

Observation x: The 6G UP discussion in RAN2 includes the following aspects:

- Overviews of L2 functions and L2 data flow (e.g. differentiation of UP layers)

- Services and functions provided by each UP layer (e.g. MAC/RLC/PDCP/SDAP)

- Differentiation of DC architectures, including the differentiation of different bearer types (e.g. split/SCG bearer) (TBD by RAN plenary)

- Numerologies and scheduling duration (e.g. timing/scheduling unit reflected in MAC)

- ARQ and HARQ

- Scheduling schemes (e.g. DL/UL dynamic/configured scheduling and BSR/SR)

- QoS control (e.g. QoS flow to DRB mapping)

- UE power saving (e.g. DRX)

- Network energy saving (e.g. cell DRX/DTX)

In the following sections, our discussions focus on the function grouping of the L2 protocols (especially for RLC and PDCP), the QoS handling, scheduling schemes (especially for the uplink), the power saving from both the UE and the network and the support of NTN.

## L2 protocol stacks

Figure 1: Protocol stack evolution from 4G to 6G

3GPP TS 38.300 [11]: The main services and functions of the MAC sublayer include: - Mapping between logical channels and transport channels; - Multiplexing/demultiplexing of MAC SDUs belonging to one or different logical channels into/from transport blocks (TB) delivered to/from the physical layer on transport channels; - Scheduling information reporting; - Error correction through HARQ (one HARQ entity per cell in case of CA); - Priority handling between UEs by means of dynamic scheduling; - Priority handling between logical channels of one UE by means of logical channel prioritisation; - Priority handling between overlapping resources of one UE; - Padding. The main services and functions of the RLC sublayer depend on the transmission mode and include: - Transfer of upper layer PDUs; - Sequence numbering independent of the one in PDCP (UM and AM); - Error Correction through ARQ (AM only); - Segmentation (AM and UM) and re-segmentation (AM only) of RLC SDUs; - Reassembly of SDU (AM and UM); - Duplicate Detection (AM only); - RLC SDU discard (AM and UM); - RLC re-establishment; - Protocol error detection (AM only). The main services and functions of the PDCP sublayer include: - Transfer of data (user plane or control plane); - Maintenance of PDCP SNs; - Header compression and decompression using the ROHC protocol; - Header compression and decompression using EHC protocol; - Compression and decompression of uplink PDCP SDUs: DEFLATE based UDC only; - Ciphering and deciphering; - Integrity protection and integrity verification; - Timer based SDU discard; - For split bearers, routing; - Duplication; - Reordering and in-order delivery; - Out-of-order delivery; - Duplicate discarding. The main services and functions of SDAP include: - Mapping between a QoS flow and a data radio bearer; - Marking QoS flow ID (QFI) in both DL and UL packets.

According to the protocol stack evolution as shown above and the function grouping for each protocol layer of 5G L2, compared with 4G protocols stacks, the 5G protocol stacks of user plane add one extra protocols layer of SDAP (which is used to map QoS flow to DRB according to the requirements from SA), and reuse other 4G protocol layers including PDCP/RLC/MAC. In general, we consider that the main functions for each layer of 5G can still be reused as the inputs for 6G, as the services provided by 5G should still be applicable for 6G. In 6G, new services (e.g. XR/AI/Sensing) with the requirements of higher data rate and more secure transmission would require the regrouping of the L2 function. By a proper regrouping of L2 functions, we can make the L2 protocol stack more robust and more efficient than 5G, and more adaptive to 6G new services.

Observation 1: The current 5G protocol stacks are adaptive to the 5G services (e.g. eMBB/ mMTC) which are still applicable for 6G. New 6G services (e.g. AI/Sensing) with the requirements of higher data rate and more secure transmission require the regrouping of the L2 functions.

Proposal 1: RAN2 studies the function regrouping in each L2 protocol layer, by taking into account the 5G L2 protocol stacks as the inputs.

In 6G, we think that the need of SDAP can be based on the inputs from SA (e.g. whether reflective QoS is needed), and the function grouping of other features in different protocol layers can be discussed in RAN2 by comparing the performance between 5G protocol stack and 6G protocol stack. As illustrated below, if PDCP and RLC are merged to a single layer, 6G protocol stack can achieve more efficient L2 processing, by removing duplicated ARQ function in both layers, reducing signalling overhead of two-layer SN(s) and reducing the cross-layer interactions.

Figure 2: Single ARQ with single SN in 6G

According to the 5G specifications of TS 38.323 [5] for PDCP and TS 38.322 [6] for RLC, both PDCP and AM RLC support the packet retransmission with corresponding status report, and this requires two separate SNs (Sequence Number) allocated in both PDCP and RLC. The ARQ in PDCP is triggered only for specific use cases (e.g. handover and bearer type change), and the ARQ in RLC is used for the AM RLC. The two ARQ functions in two separate protocol layers cause extra signalling overheads, implementation complexity and standard efforts.

3GPP TS 38.323 for PDCP: ### 4.2.1 PDCP structure … Each RB (except for SRB0 for Uu interface) is associated with one PDCP entity. Each PDCP entity is associated with one, two, three, four, six, or eight RLC entities depending on the RB characteristic (e.g. uni-directional/bi-directional or split/non-split) or RLC mode: - For split bearers, each PDCP entity is associated with two UM RLC entities (for same direction), four UM RLC entities (two for each direction), or two AM RLC entities; - For RBs configured with PDCP duplication, each PDCP entity is associated with N UM RLC entities (for same direction), 2 × N UM RLC entities (N for each direction), or N AM RLC entities, where 2 <= N <= 4; - For DAPS bearers, each PDCP entity is associated with two UM RLC entities (for same direction, one for source and one for target cell), four UM RLC entities (two for each direction on source cell and target cell), or two AM RLC entities (one for source cell and one for target cell); - For UM MRBs, each PDCP entity is associated with one UM RLC entity (for MTCH or for downlink DTCH), two UM RLC entities (one for MTCH and one for downlink DTCH, or one for downlink DTCH and one for uplink DTCH), or three UM RLC entities (one for MTCH, one for downlink DTCH, and one for uplink DTCH); - For AM MRBs, each PDCP entity is associated with one AM RLC entity (for downlink DTCH and uplink DTCH), or one UM RLC entity (for MTCH) and one AM RLC entity (for downlink DTCH and uplink DTCH); - For MP split bearers with SL indirect path, each PDCP entity is associated with one or more Uu RLC entities and one SRAP entity; - For MP split bearers with N3C indirect path, each PDCP entity is associated with one or more Uu RLC entities and the N3C; - Otherwise, each PDCP entity is associated with one UM RLC entity, two UM RLC entities (one for each direction), or one AM RLC entity.

According to the 5G protocols stacks, the tight cross-layer coordination is needed between PDCP and RLC. According to the PDCP specification of TS 38.323 [5] as quoted above, the PDCP behaves differently when the PDCP entity is associated with different RLC-mode entity and/or when the number of RLC entity associated with PDCP entity changes.

Figure 3: Tight coordination between PDCP and RLC

According to the Rel-18/19 XR discussion in 5G (as given in the agreed CR [7] for Rel-19 XR item), in order to make the XR traffic transmission properly, the PDCP as illustrated above needs to provide different indications (i.e. SDU discard indication, remaining-time-based RLC retransmission indication and remaining-time-based RLC polling indication) to RLC entity so as to allow the differentiated RLC discard/retransmission/polling behaviours (i.e. SDU discard, remaining-time-based RLC retransmission, RLC-only retransmission, remaining-time-based RLC polling and RLC-only polling).

Observation 2: In 5G, the duplicated ARQ functions in both PDCP and RLC cause extra signalling overheads (e.g. SN and status report), implementation complexity and standard efforts. The tight cross-layer coordination between PDCP and RLC (e.g. differentiated PDCP behaviours depending on different RLC modes and differentiated RLC discard/retransmission/polling behaviours depending on different PDCP indications) causes extra implementation complexity and standard efforts.

In conclusion, considering the duplicated functions and the tight coordination between PDCP and RLC in 5G, we think that the two sublayers of PDCP and RLC can be merged to a single layer, so that we can achieve more efficient L2 processing.

Proposal 2: PDCP and RLC is merged to a single layer, i.e. single SN and single ARQ.

3GPP TS 24.501: #### 9.11.4.7 Integrity protection maximum data rate .. 8; 7; 6; 5; 4; 3; 2; 1. Integrity protection maximum data rate IEI; octet 1. Maximum data rate per UE for user-plane integrity protection for uplink; octet 2. Maximum data rate per UE for user-plane integrity protection for downlink; octet 3. Figure 9.11.4.7.1: Integrity protection maximum data rate information element

According to the 3GPP TS 38.323 [5], when DRB integrity protection is configured, each PDCP SDU with a separate SN creates a separate MAC-I, which causes heavy processing load at the UE and lots of signalling overhead. According to the 3GPP TS 24.501 [8], an extra UE capability indication (i.e. “Integrity protection maximum data rate”) is introduced to differentiate the UE processing capability for DRB integrity protection at different data rate (i.e. one for 64 kbps and one for full data rate). The capability as quoted above means that when the DRB integrity protection is configured, some low-cost UE implementation may only be able to support up-to 64 kbps data rate. In 6G, if multiple PDCP SDUs are concatenated to a single PDCP PDU with single SN and single MAC-I, the UE processing load for calculating MAC-I can be reduced, and the signalling overhead of separate SN and MAC-I for each PDCP SDU is also reduced.

Observation 3: In 5G, the MAC-I generated for each PDCP SDU causes extra signalling overheads and UE processing capability (i.e. impacting the UE’s transmission bit rate).

Proposal 3: Multiple PDCP SDUs can be concatenated to a single PDCP PDU.

## QoS handling

Multi-modal service

According to 3GPP TS 22.261 [9], new service requirements to support coordinated transmission for multi-modality flows are defined. In some cases, these multiple types of flows, e.g. video/audio stream, and haptic or sensor data for a more immersive experience are linked with each other and located at a single UE. In other cases, such multiple types of flows which need synchronized transmission are associated with multiple UEs.

Figure 4: Multi-modal interactive system (in TS 22.261)

As described in TS 22.261 [9], immersive multi-modal VR applications have critical synchronization requirements between different media components, in order to avoid having a negative impact on the user experience. The table below shows an example of synchronization requirements.

Table 1: Typical synchronization thresholds for immersive multi-modality VR applications (in TS 22.261)

| Media components | synchronization threshold (note 1) |  |
| --- | --- | --- |
| audio-tactile | audio delay: 50 ms | tactile delay: 25 ms |
| visual-tactile | visual delay: 15 ms | tactile delay: 50 ms |
| NOTE 1: for each media component, “delay” refers to the case where that media component is delayed compared to the other. |  |  |

In 5G, the concept of Multi-modal Service ID is introduced to indicate the correlated QoS flows (i.e. “Data flows belonging to the same multi-modal service share the same Multi-modal Service ID” [20]), and the 5G system uses the MMSID (which is transparent to both the UE and the gNB) via the network implementation to ensure the synchronization transmission in the unit of QoS flow (e.g. cross multiple QoS flows). According to the control signalling procedure as defined in 3GPP TS 23.502 [18] in 5G, via the coordination between AF (Application Function), PCF (Policy Control Function) and SMF (Session Management Function), the QoS parameters for multiple flows of a multi-model service are generated, and the gNB provides a proper scheduling and transmission scheme by follows the QoS parameters (as provided by the core network) for multiple flows of a multi-model service. From our understanding, as the correlation of multiple QoS flows is transparent to RAN, the 5G QoS-low level mechanism for multi-modal service introduces lots of restrictions on the scheduling at the gNB and cannot guarantee the tight synchronization or coordination of 6G multi-modality service requirements, e.g. the uplink packet arrival time and the uplink packet differentiation/correlation within a single QoS flow or cross multiple flows within a single DRB are unknown to the gNB. In 6G, we think that RAN2 should consider packet-level handling (e.g. the correlation between different packets within a QoS flow or multiple flows within a single DRB) beyond QoS flow-level handling for multi-modality services and make the cross-flow coordination visible to the RAN.

XR service

3GPP TS 38.413 [21]: #### 9.3.1.264 PDU Set QoS Information This IE defines the PDU Set QoS Information to be applied to a QoS flow. IE/Group Name; Presence; Range; IE type and reference; Semantics description. PDU Set Packet Delay Budget; O; Extended Packet Delay Budget 9.3.1.135; PDU Set Delay Budget as specified in TS 23.501 [9].. PDU Set Error Rate; O; Packet Error Rate 9.3.1.81; PDU Set Error Rate as specified in TS 23.501 [9].. PDU Set Integrated Handling Information; O; ENUMERATED (true, false, …); PDU Set Integrated Handling Information as specified in TS 23.501 [9]..

According to 3GPP TR 23.700-60 [10], the packets belonging to a single QoS flow of XR services may have individual QoS requirement. For this purpose, according to the “PDU Set QoS” [21] provided from the CN to RAN, differentiated QoS handling for different packet sets is introduced in NR Rel-18. We understand that such a spirit of finer QoS handling for different packets within a single flow should be inherited from 5G to 6G.

AI token-based data transmission for 6G

Regarding the 6G new services, there is a significant trend for the fast growth of token consumption [13]. AI traffic has some special characteristics and token is the most popularly used “data” for application AI traffic. As reported recently, in some countries, daily Token consumption has exceeded 30 trillion and continues to grow rapidly [14]. In 6G, with the proliferation of mobile AI, token-based data transmission is destined to become a typical requirement and a new service modality that 6G needs to support.

Figure 5: An example for Token-based data transmission

The above figure illustrates a typical token-based data transmission flow:

- UE Tokenizes the source information (e.g., an image) and then transmits the Token sequence to the NW side.

- The receiver de-Tokenizes the Tokens to recover the information, which is then fed into an AI service (e.g., a classification task).

For AI services, the essential requirement is service performance (e.g., classification accuracy), not 100% correct data transmission. Note that: 100% data correctness ≠ High classification accuracy, and also Non-100% data correctness ≠ Failure to meet service requirements. Thus, the AI token is a typical scenario of error tolerant communication.

The following description shows some simulations on token-based data transmission, in which the ImageNet dataset with 3000 images belonging to 1000 categories (https://www.image-net.org) and a classification task [15] is used. Each image is converted into 256 Tokens (each Token is 12 bits), and the impact of token-based transmission error rate on the classification accuracy (Top-1/Top-5) is evaluated. As a certain TER (Token Error Rate), Top-5 shows better classification accuracy than Top-1. The classification accuracy shows significant tolerance to token errors. When errors are randomly distributed across all 256 Tokens, different TER significantly impact classification accuracy. For instance, if the requirement is only to ensure Top-5 accuracy > 90%, the system can tolerate a TER as high as 20% (TER=0.2).

Some advanced tokenizer can generate Token representations differentiated by “importance”. For example, the 256 Tokens can be numbered from 0 to 255 in descending order of importance; a lower number indicates higher criticality for information expression, while a higher number corresponds to more detailed information. In this case, as shown in the following figures, there is a content recovery in two situations:

a) only transmit a number of most important tokens without error while abandoning the rest of tokens, and

b) transmit a number of most important tokens without error and transmit rest of tokens with error.

Figure 6: Exmple-1 of token-based data transmission

Figure 7: Exmple-2 of token-based data transmission

The two figures below show the roles played by token importance. Compared to transmitting all 256 tokens with randomly distributed token error rate in Figure-8, the performance can be obviously better in Figure-9, i.e., selecting and guaranteeing lossless transmission for certain number of important Tokens, while applying lossy transmission (allowing a certain error rate) to the rest of Tokens. This approach both ensures basic performance through the critical Tokens and allows flexibly enhancing accuracy through supplementary transmission, adapting to constraints like channel conditions and limited transmission resources.

Figure 8: 256 tokens are transmitted with randomly distributed token error rate

Figure 9: 256 tokens are transmitted with the 32 most important tokens transmitted losslessly

According to the above evaluations, some KPIs of token are provided in the table-1 below with considering relevant references.

Table-1 example token size

| Token type | Payload Size | Error rate | Scenario |
| --- | --- | --- | --- |
| Video | 400 bits per token, 500 tokens per video frame [17] | 0.01% for important tokens. [20%] for other tokens | Realtime video processing |
| Image | 12 bit per token, 256 tokens per image [15] |  | The picture in ImageNet data set |
| Text | 8-24 bits per token [16] | 0.01% for all tokens | Human-chatbot communication |
| NOTE: The token size in the table is for example. the token size may vary from specific scenarios. |  |  |  |

Based on the simulation given above, for some scenarios, guaranteeing the most important Tokens (e.g. top 25%) can achieve a high-quality recovery. This active selection based on Token importance, which involves discarding or downgrading QoS for non-important Tokens as needed, also helps build efficient transmission schemes, reducing the amount of data transmitted and improving efficiency. Thus, more adaptive/dynamic QoS control for AI token-based data transmission should be considered in 6G user plane design.

In addition, 3GPP SA1 has begun discussing the service requirements for different services in 3GPP TR 22.870 [22], e.g., for error tolerant communication. In TR 22.870, some use cases (e.g., in clause 6.7, 6.28, 9.11, 9.14) have been agreed on how to reduce QoS (e.g., reliability) while maintaining deterministic QoE for some real time service, e.g., potentially falling under the 99.99% to 50% reliability range proposed in SA1 TR 22.870 clause 9.11. Furthermore, SA2 is discussing whether and how the adaptive QoS mechanisms for AI characteristics traffic e.g. under work task 3.2. The corresponding impacts in RAN2 also need to be studied.

According to the above observations, in 5G, as different QoS requirements/enhancements introduced in RAN are provided by different 5G releases and by different work items, this causes many isolated functions in the user plane protocols and the complexity of the coexistence between different user plane functions due to the differentiated QoS requirements. In 6G, we think that the 6G user protocol stacks should take all the QoS requirements into account, and have more agile and simplified design of the user plane functions.

Observation 4: In 5G, the QoS flow-level coordination for multi-modality services which is invisible to the RAN cannot guarantee accurate synchronization or coordination for multi-modality services in RAN2 L2 data transmission. The differentiated QoS handling for packets belonging to a single QoS flow should be inherited from 5G to 6G. The adaptive/dynamic QoS change of AI token-based data transmission in 6G should be considered in the RAN2 L2 protocol design.

Proposal 4: RAN2 user plane design should comply with the 6G QoS requirements, including the synchronization transmission cross QoS flows, differentiated QoS handling for packets within a single QoS flow and adaptive/dynamic QoS parameter change, by taking into account new 6G services (e.g. AI token-based communication).

## UL scheduling

In 5G, the acquisition of UL grant for uplink data transmission primarily relies on the following scheduling mechanisms:

- Dynamic grant (DG) which is dynamically scheduled via PDCCH.

- Configured grant (CG) which is configured semi-persistently by RRC.

- UL grant in RAR or associated with the PUSCH resources of MsgA.

Their characteristics are summarized as in the table below.

Table 3: UL scheduling solutions

|  | DG | CG | UL grant via RACH |
| --- | --- | --- | --- |
| Pros | Flexibility for providing real-time UL resources based on latest radio link conditions and data arrival status. | Reduction of UL transmission latency, e.g., SR/BSR/DCI reception steps can be saved. | Capable of providing UL resources for UE in all RRC states and scenarios lacking network control, e.g., initial access. |
| Cons | Additional latency caused by SR/BSR/scheduling DCI reception steps required before performing UL transmission. | Potential inefficient utilization of UL resources caused by traffic variation. | Non-skippable preamble transmission even when TA is valid. |

In 6G, the UL scheduling schemes defined in 5G should be supported as the starting point since their advantages under different cases can be still valid in 6G. While as summarized, there is still room for further optimization on UL scheduling scheme in terms of latency, resource efficiency and capacity, i.e., 6G can further study the UL scheduling schemes to achieve a more efficient UE access procedure. As a potential trade-off, contention-based UL grant can be investigated for faster access by skipping Msg1/2 when TA acquisition has been done, which can be considered as RACH enhancement and aim to be applicable to various RACH trigger events (e.g. initial access and BSR) with valid TA.

Figure 10: The relation between contention-based CG access, SR and BSR

According to the figure illustrated above, the contention-based CG resource can be used for both initial access procedure and the (SR)/BSR procedure, to reduce the latency and the overhead caused by the RACH procedure for initial access and UL scheduling.

Observation 5: In 5G, UL transmission via DG provides flexible scheduling without the waste of reserved uplink resources of PUSCH, but with more UL scheduling latency due to the SR/BSR/DCI transmission/reception. The UL transmission via CG is of low latency but may cause resource waste due to pre-configuration. The UL transmission via RACH MsgA/Msg3 supports diverse scenarios and RRC state but has unnecessary overhead (e.g., preamble transmission) and latency for initial access and UL scheduling.

Proposal 5: For UL scheduling schemes in 6G, RAN2 studies to: 1) support 5G UL scheduling schemes as the baseline, i.e., DG and type-1/type-2 CG. 2) further investigate UL scheduling schemes for contention-based CG resources for fast access.

## Power saving

UE power saving

In order to save UE power consumption, a variety of power saving features have been introduced in NR sine Rel-15, as summarized in the following tables.

Table 4: UE power saving in time-domain

| UE PS feature | Pros | Cons |
| --- | --- | --- |
| DRX/dual DRX group | Enable UE to monitor the PDCCH discontinuously. Support dynamic switching between long DRX cycle and short DRX cycle, to guarantee the latency requirement for different services | UE needs to wake up in every DRX cycle, even there is no scheduling for the UE |
| DCP | Used to determine whether or not to start the on-duration timer, so that the power consumption caused by unnecessary PDCCH monitoring can be avoided | The transmission latency remains constrained by the DRX cycle. |
| LP-WUS | UE power saving gain is expected to be more competitive to DCP since LP-WUS monitoring consumes less power than PDCCH monitoring. Transmission latency could be reduced if UE is configured with a shorter LP-WUS duty cycle compared to DRX cycle. | Due to the poor coverage of LP-WUS compared to legacy channel, e.g., PDCCH, UEs at the cell edge cannot benefit from LP-WUS. |

As a foundational feature inherited from LTE, DRX is supported in NR Rel-15. For a UE configured with DRX, the UE monitors the PDCCH discontinuously, resulting in a trade-off between data transmission latency and UE power consumption. However, it requires the UE to wake up in every DRX cycle, even there is no scheduling for the UE. Therefore, DCP and LP-WUS are introduced in Rel-16 and Rel-19, respectively, both of which can be used to reduce unnecessary PDCCH monitoring on top of DRX.

In 6G, the time-domain power saving features can be simplified. For example, PEI and LP-WUS have duplicated design, there seems no need to support both PEI and LP-WUS simultaneously in 6G. Instead, a single wake-up signal is sufficient. In our view, LP-WUS is an effectively mechanism for UE power saving, which can reduce UE power consumption and latency by being able to monitor the WUS with a LR such that the UE’s MR can be kept in a sleep state. The main drawback of LP-WUS in NR is that it may only support partial coverage of the cell due to the degraded link budget of LP-WUS/LP-SS. How to enhance the coverage performance will be studied in RAN1. Form RAN2 perspective, we can study further enhancement on LP-WUS procedure for RRC CONNECTED UEs in 6G, e.g., LP-WUS operation without DRX.

Table 5: UE power saving in frequency-domain

| UE PS feature | Pros | Cons |
| --- | --- | --- |
| BWP switching | DCI based BWP switching enables fast bandwidth adaptation within a carrier. | UE's RF circuitry cannot be shut off, so that the power saving gain is limited. |
| SCell (de-)activation | For a deactivated SCell, almost all circuitry related to this SCell (e.g.., RF, ADC) can be powered down, with high power saving gain. | Prolonged SCell activation time |
| Dormant BWP for SCell | A balance between UE power saving gain and latency. | Slightly higher UE power consumption compared to SCell deactivation because the RF chain remains partially “on” for measurements. |

Besides PDCCH monitoring adaption in time-domain, UE power saving can also be achieved by reducing the UE working bandwidth when the traffic load is low. In NR, bandwidth adaptation can be achieved through SCell (de-)activation and BWP switching. SCell (de-)activation is carrier-level bandwidth adaptation, while BWP switching enables fast bandwidth adaptation within a carrier. In 6G, bandwidth adaptation is also needed. As this may depend on how to support aggregate spectrum and/or the BWP design in RAN1, we think RAN2 can prioritize the UE-side power saving discussion on the time-domain power saving feature.

Network power saving

To facilitate reducing gNB transmission/reception activity time, cell DTX/DRX is introduced in Rel-18 NR. A UE can be configured with a periodic cell DTX/DRX pattern. During non-active periods of cell DTX/DRX, certain channels/signals (i.e., PDCCH in selected cases, SPS and LP-WUS for DL, CG and SR for UL) are not received/transmitted on the cell by the UE. While this can effectively achieve network energy saving, it presents a challenge for delay-sensitive service. For example, if UE triggers a SR due to UL data arrival of a logical channel with low latency requirement, or if gNB receives DL data of a delay-sensitive traffic, since either SR or PDCCH for new transmission are prohibited during the Cell DRX/DTX non-active period and shall be postponed at least to the next Cell DRX/DTX active period, the data transmission may suffer significant latency. In 6G, we should study to optimize Cell DTX/DRX mechanism for energy efficiency while guaranteeing transmission latency.

In 5G, UE power saving and network energy saving were studied independently, and many features are introduced in later releases. Due to the backward compatibility issue, the power saving on one side often comes at the cost of increased burden on the other side. For example, the support of LP-WUS requires gNB to additionally transmit LP-SS and LP-WUS. In 6G, we should study native power saving mechanism, target for joint power saving for UE and network.

Observation 6: In 5G UE power saving, multiple time domain solutions (i.e. DRX/ DCP/ LP-WUS) cause more standard works and extra implementation complexities at both the UE and the network. Whether the 5G bandwidth adaptation is reused in 6G depends on how to support the spectrum aggregation and/or the BWP design in 6G. In 5G network power saving, cell DTX/DRX has challenges for latency-sensitive services. In 5G, the energy saving on one side often comes at the cost of increased burden on the other side.

Proposal 6: In 6G, RAN2 studies native power saving mechanism, including: 1）the joint power saving for UE and network; 2) for UE power saving, RAN2 studies the simplified UE power saving features, e.g. design a single wake-up signal and support wake-up signal operation without DRX; 3) for network power saving, RAN2 studies to enhance cell DTX/DRX mechanism to reduce the impact on latency, especially for latency-sensitive services.

## NTN

In 5G, the support of NR was introduced in Rel-17 on top of the TN features, and NTN specific features of user plane are summarized in the table below.

Table 6: NTN specific UP features in 5G

| NTN features | UP impact in 5G |
| --- | --- |
| UL synchronization | Support TA pre-compensation based on UE GNSS position. Support TA reporting during random access procedure and/or in connected mode. |
| HARQ enhancements | Larger HARQ process number Support enabling/disabling HARQ feedback for DL and HARQ mode A/B for UL |
| L2 timer enhancements | RA timer and DRX timer adaptation Extension of RLC/PDCP timer length |

In NR, it is assumed that all UEs supporting NTN are GNSS-capable. With this assumption, during random access procedure, UE shall be able to autonomously pre-compensate the TA and frequency doppler shift based on its GNSS position and the ephemeris, etc, for uplink transmission. And in connected mode, the UE could continuously update the TA and frequency pre-compensation. The UE may be configured to report TA during random access procedures or in connected mode. In Rel-20, GNSS resilience NTN operation will be studied, and this is also considered as a critical scenario in 6G NTN. How to perform UL synchronization for non-GNSS capable UEs is RAN1’s scope. From RAN2 perspective, for both GNSS capable and non-GNSS capable UE, the function of TA reporting should be supported in 6G NTN, as this would be helpful for uplink scheduling adaptation at network side, e.g. in GEO scene, it could assist network to indicate the UE with a proper UE-specific K or K_offset value to reduce the UL latency. Same as NR NTN, TA reporting can be defined as an optional feature, which can be configurable by network.

In NR NTN, HARQ has been enhanced to solve the HARQ stalling issue due to a much larger propagation delay in NTN compared to TN. In our view, this should be an essential feature in 6G NTN. To achieve a unified design for TN and NTN, both HARQ process number and HARQ modes can be configurable as the same way in NR.

Another enhancement in 5G NTN is to adapt the start time of some timers for which UE-gNB RTT should be taken into account. In addition, for some timers (e.g. t-Reassembly and discardTimer), the value range has been extended to accommodate large propagation delay in NTN. In 6G, this should be natively supported.

Observation 7: In 5G NTN, both TN features and NTN-specific features of user plane are used together to support the NTN deployment, and NTN-specific UP features (i.e. enhancements of UL synchronization, HARQ and L2 timer) are designed independently via separate UE capabilities.

Proposal 7: In 6G, RAN2 studies a unified user plane design for both TN and NTN, and also takes into account NTN-specific features (e.g. enhancements of UL synchronization, HARQ and L2 timer).

# Conclusion

According to the 6G discussion in both RAN and SA, 6G would require more efficient and more agile design of the user plane protocol stacks, by complying with various services to be supported by 6G. On the other hand, RAN2 protocol stack also needs to consider the power saving requirements from both the UE and the network, and support of the satellite deployment.

L2 protocol stacks

Observation 1: The current 5G protocol stacks are adaptive to the 5G services (e.g. eMBB/ mMTC) which are still applicable for 6G. New 6G services (e.g. AI/Sensing) with the requirements of higher data rate and more secure transmission require the regrouping of the L2 functions.

Proposal 1: RAN2 studies the function regroup in each L2 protocol layer, by taking into account the 5G L2 protocol stacks as the inputs.

Observation 2: In 5G, the duplicated ARQ functions in both PDCP and RLC cause extra signalling overheads (e.g. SN and status report), implementation complexity and standard efforts. The tight cross-layer coordination between PDCP and RLC (e.g. differentiated PDCP behaviours depending on different RLC modes and differentiated RLC discard/retransmission/polling behaviours depending on different PDCP indications) causes extra implementation complexity and standard efforts.

Proposal 2: PDCP and RLC is merged to a single layer, i.e. single SN and single ARQ.

Observation 3: In 5G, the MAC-I generated for each PDCP SDU causes extra signalling overheads and UE processing (i.e. impacting the UE’s transmission bit rate).

Proposal 3: Multiple PDCP SDUs can be concatenated to a single PDCP PDU.

QoS handling

Observation 4: In 5G, the QoS flow-level coordination for multi-modality services which is invisible to the RAN cannot guarantee accurate synchronization or coordination for multi-modality services in RAN2 L2 data transmission. The differentiated QoS handling for packets belonging to a single QoS flow should be inherited from 5G to 6G. The adaptive/dynamic QoS change of AI token-based data transmission in 6G should be considered in the RAN2 L2 protocol design.

Proposal 4: RAN2 user plane design should comply with the 6G QoS requirements, including the synchronization transmission cross QoS flows, differentiated QoS handling for packets within a single QoS flow and adaptive/dynamic QoS change, especially by taking into account new 6G services (e.g. AI token-based data transmission).

UL scheduling

Observation 5: In 5G, UL transmission via DG provides flexible scheduling without the waste of reserved uplink resources of PUSCH, but with more UL scheduling latency due to the SR/BSR/DCI transmission/reception. The UL transmission via CG is of low latency but may cause resource waste due to pre-configuration. The UL transmission via RACH MsgA/Msg3 supports diverse scenarios and RRC state but has unnecessary overhead (e.g., preamble transmission) and latency for initial access and UL scheduling.

Proposal 5: For UL scheduling schemes in 6G, RAN2 studies to: 1) support 5G UL scheduling schemes as the baseline, i.e., DG and type-1/type-2 CG. 2) further investigate UL scheduling schemes for contention-based CG resources for fast access.

Power saving

Observation 6: In 5G UE power saving, multiple time domain solutions (i.e. DRX/ DCP/ LP-WUS) cause more standard works and extra implementation complexities at both the UE and the network. Whether the 5G bandwidth adaptation is reused in 6G depends on how to support the spectrum aggregation and/or the BWP design in 6G. In 5G network power saving, cell DTX/DRX has challenges for latency-sensitive services. In 5G, the energy saving on one side often comes at the cost of increased burden on the other side.

Proposal 6: In 6G, RAN2 studies native power saving mechanism, including: 1）the joint power saving for UE and network; 2) for UE power saving, RAN2 studies the simplified UE power saving features, e.g. design a single wake-up signal and support wake-up signal operation without DRX; 3) for network power saving, RAN2 studies to enhance cell DTX/DRX mechanism to reduce the impact on latency, especially for latency-sensitive services.

NTN

Observation 7: In 5G NTN, both TN features and NTN-specific features of user plane are used together to support the NTN deployment, and NTN-specific UP features (i.e. enhancements of UL synchronization, HARQ and L2 timer) are designed independently via separate UE capabilities.

Proposal 7: In 6G, RAN2 studies a unified user plane design for both TN and NTN, and also takes into account NTN-specific features (e.g. enhancements of UL synchronization, HARQ and L2 timer).

# Reference

[1] RP-252912, “New SID: Study on 6G Radio”

[2] 3GPP TR 38.801, “Study on new radio access technology: Radio access architecture and interfaces”

[3] 3GPP TR 38.804, “Study on New Radio Access Technology; Radio Interface Protocol Aspects”

[4] 3GPP TR 38.912, “Study on New Radio (NR) Access Technology”

[5] 3GPP TS 38.323, “NR; Packet Data Convergence Protocol (PDCP) specification”

[6] 3GPP TS 38.322, “NR; Radio Link Control (RLC) protocol specification”

[7] R2-2506337, “Introduction of R19 XR enhancements for PDCP spec.”

[8] 3GPP TS 24.501, “Non-Access-Stratum (NAS) protocol for 5G System (5GS); Stage 3”

[9] 3GPP TS 22.261, “Service requirements for the 5G system; Stage 1”

[10] 3GPP TS 23.700-60, “Study on XR (Extended Reality) and media services”

[11] 3GPP TS 38.300, “NR; NR and NG-RAN Overall Description; Stage 2”

[13] Explaining Tokens — the Language and Currency of AI (https://blogs.nvidia.com/blog/ai-tokens-explained/)

[14] China Reports Surge in Token Consumption as AI Applications Expand (https://technode.com/2025/08/19/china-reports-surge-in-token-consumption-as-ai-applications-expand/)[3] 	K. Miwa, et al. One-D-Piece: Image Tokenizer Meets Quality-Controllable Compression, ICML Tokenization Workshop, 2025.

[15] K. Miwa, et al. One-D-Piece: Image Tokenizer Meets Quality-Controllable Compression, ICML Tokenization Workshop, 2025.

[16] Wu, et al, DeepSeek-VL2: Mixture-of-Experts Vision-Language Models for Advanced Multimodal Understanding, arXiv, Dec. 2024.

[17] Cheng, Yihua, et al. "{GRACE}:{Loss-Resilient}{Real-Time} video through neural codecs." 21st USENIX Symposium on Networked Systems Design and Implementation (NSDI 24). 2024.

[18] 3GPP TS 23.501, “System architecture for the 5G System (5GS); Stage 2”

[19] 3GPP TS 23.502, “Procedures for the 5G System (5GS); Stage 2”

[20] 3GPP TS 23.503, “Policy and charging control framework for the 5G System (5GS); Stage 2”

[21] 3GPP TS 38.413, “NG-RAN; NG Application Protocol (NGAP)”

[22] 3GPP TR 22.870, “Study on 6G Use Cases and Service Requirements; Stage 1”