3GPP TSG-RAN WG2 Meeting #131bis	R2-2506854

Prague, Czechia, 13th – 17th October 2025

Agenda Item:	10.3.1

Source:	MediaTek Inc.

Title:	On 6G user plane architecture considerations and user plane functions

Document for:	Discussion and decision

# Introduction

This document discusses design considerations for user plane data handling in 6G and provides some directions that RAN2 could pursue to meet those considerations.

# Discussion

## Design considerations for 6G UP

Figure 1: Spot the difference! Layer 2 structure from (left) LTE [3] and (right) NR [2]

Latency improvement for eMBB data

NR’s design for eMBB data transfer has been largely inherited from LTE, as LTE’s L2 design was considered as the baseline for NR during the NR study [1]. Over the course of the NR study, the LTE baseline was improved to handle higher eMBB throughput requirements with the focus on a hardware friendly design. However, the latency associated with this eMBB data transfer in L2 was not a priority and therefore eMBB latency performance remained largely the same as in LTE. We list below some of the key contributors to latency experienced during eMBB data transfer in an NR (and LTE) UE in a connected state.

- Fundamental round trip time (RTT): This is the basic downlink (DL) to uplink (UL) turnaround delay in the system. For both DL and UL, key contributor to this delay is the TDD pattern and to a lesser degree, the processing limits of the UE and the NW scheduler.

- Data scheduling initiation delay: This is the delay experienced to initiate data transfer. Largely seen on the UL, where the key contributor to this delay is SR availability (typically ~40slots).

- HARQ retransmission delay: This is the delay experienced due to HARQ retransmissions in case of transmission failures. Typically, this delay is ‘number of HARQ retransmission attempts X fundamental RTT’.

- ARQ retransmission delay: This is the delay experienced due to ARQ retransmissions in cases where HARQ cannot recover lost data. The delay is at least duration of the reassembly timer (typically 40ms).

- Reordering delay: This is the delay caused by head-of-line blocking, where a delayed or missing packet also delays the delivery of subsequent packets to the application. The delay is driven by the reordering timer (typically 100 ms or larger).

At the same time, NR was also designed to enable extremely low end-to-end (E2E) latency of 1ms as part of its ability to handle URLLC traffic. This tight E2E latency requirement was achieved at the expense of spectral efficiency with the use of techniques [2] such as packet duplication, CQI/MCS tables with lower spectral efficiency etc. Such a trade-off was suitable to serve low data rate URLLC in bespoke deployments such as industrial automation. However, later in the 5G-advanced era when we started to explore latency sensitive eMBB services such as immersive communication and cloud gaming, we found that this trade-off against spectral efficiency could not scale. While E2E latency requirements for immersive communications were not as stringent (10-30ms) as URLLC, higher eMBB data rates coupled with practical deployment considerations resulted in capacity constraints [3].

This led to user plane discussions focussing on improving the latency performance of such eMBB data transfer in NR over the last few releases (viz. FS_NR_XR_enh, NR_XR_enh-Core, NR_XR_Ph3-Core). While these discussions have yielded capacity benefits [3], the scope of the discussions have been limited by practical considerations such as backwards compatibility and hardware design impact. As highlighted in the 6G SID [4] and elaborated in the 6G use-cases study [5], latency associated with eMBB data transfer will continue to remain an increasingly important metric during 6G’s lifetime, to handle use-cases such as immersive communication, AI and edge computing. At the same time, free from backwards compatibility constraints, we have a once-in-a-generation opportunity to design a communication system that isn’t focussed on a niche such as URLLC, but rather on improving latency performance associated with general eMBB data transfer for all.

Observation 1: Latency associated with eMBB data transfer will be an increasingly important metric to serve emerging use-cases during 6G’s lifetime.

Observation 2: Free from backwards compatibility constraints, this 6G study gives us a once-in-a-generation opportunity to design a communication system that improves latency performance associated with eMBB data transfer to benefit all 6G users.

Proposal 1: 6G user plane is designed to reduce the latency associated with eMBB data transfer compared to NR.

### Hardware-friendly design

Figure 2: The user plane production line - fast and efficient

An important improvement in the L2 stack that NR provided over LTE was that NR was designed from the ground up with a focus on a hardware friendly design. This was necessary to meet NR’s significantly higher data rate requirements, while keeping power consumption and implementation costs within reason. With the pressure of even higher 6G data rate requirements coupled with a need to lower power consumption, we need to ensure that we continue to focus on a hardware friendly design for 6G. We list some key considerations below that are essential to meet 6G performance requirements while keeping power and cost in check.

- Hardware friendly headers: Simple header structures which are of a fixed size and with fixed bit locations to enable preprocessing at the transmitter and hardware-driven packet parsing at the receiver. This is  necessary to handle peak 6G data rate requirements.

- Lean stack functionality: Standardising just the bare essential user-plane functionality to meet 6G performance requirements is essential to keep processing overheads within reason.

- Memory-conscious choices: At high throughputs, any ‘traffic jams’ due to delays such as reassembly and reordering delays can lead to significant memory requirements for buffering purposes. Data transfer through the stack needs to be streamlined to minimise specification-induced memory bloat.

Proposal 2: 6G user plane is designed to be hardware-processing friendly while keeping memory requirements low.

## Essential 6G user plane functionality

### A comprehensive overview

Figure 3: An overview of salient 6G user plane functionality

We list below an overview of the functionality that we expect the 6G user plane stack to provide.

- Uplink resource acquisition: This is the functionality related to initiation of UL data transfer. In LTE and NR, this was achieved by BSR, SR and RACH which experience significant delays as outlined in section 2.1.1. For 6G, this functionality needs to be redesigned to minimise its latency. We expand on how to achieve this latency reduction in section 2.2.2.

- Reliable data transfer: This is the functionality to ensure that data reliably reaches the receiver over a lossy physical channel. In LTE and NR, this was achieved by HARQ, with ARQ running on top. ARQ is rather complex functionality that suffers from significant delays and, for 6G, needs to be redesigned to minimise its latency and processing impact. We expand further on how to do so in section 2.2.3.

- Data recovery at mobility: Reliable data transfer should also work as the UE moves across the network and switches between RAN nodes. In LTE and NR, this was achieved by lossless handover functionality. For 6G, we propose to ensure that such functionality is retained.

- Security of data and control information: Security is a basic requirement of 6G data transfer. The security functions to use need to follow SA3 conclusions to ensure that they meet 6G requirements such as quantum safe operation. RAN2 needs to design the placement of security functionality in the data pipeline. In addition to what we have for NR today, there is a need to secure L2 control information such as that carried in MAC CEs in NR. This additional security consideration needs to be carefully designed given the time-critical nature of some of this information.

- Data handling differentiation in the RAN: This is the functionality that provides differentiated data handling in the RAN to meet QoS requirements. In NR this was provided at a radio bearer level which is of a coarser granularity than QoS flows (16 DRBs vs 64 QoS flows) to limit RAN processing requirements. In the field we never see more than 4 DRBs used in NR. For 6G, we propose to continue using such a coarse data differentiation concept as radio bearers from NR to keep processing requirements within reason, and do not see a need to expand the number of radio bearers.

- Multiplexing of radio bearers into a physical layer transmission: This is functionality that multiplexes data with different QoS requirements in the RAN into a physical layer transmission. In NR, this was achieved using the Logical Channel Prioritisation (LCP) function in MAC. Given that we require differentiated data handling, we also need such functionality to map this data to a transmission. Care needs to be taken during the design of this functionality in 6G, to keep their processing requirements in check.

- Segmentation and reassembly of data: When whole IP data cannot fit into a physical layer transmission resource, segmentation of IP data at the transmitter and reassembly of segmented information at the receiver is needed to maximise the use of physical layer resources. While at low data rates (e.g. due to congestion, or UE being at cell-edge) maximising the use of scarce physical resources is of paramount importance, the same is not true at high data rates. At high data rates, segmentation and reassembly can lead to increased memory requirements and processing overheads due to the need to handle an excessive number of segments. Care needs to be taken when designing this functionality for 6G to ensure that it is only used when absolutely necessary.

- In order delivery: Some transport layer protocols such as TCP react poorly to out of order data by triggering congestion control. This would lead to a sub-par end-user experience despite a good underlying communications link. There is therefore a need to ensure in-order delivery of data in 6G. In NR, reordering in NR is done at a DRB granularity where typically a single DRB is configured to handle all Internet traffic in a UE. This leads to significant head-of-line blocking delays for all traffic of a UE. For 6G, consequentially there is a need to redesign reordering functionality to keep head-of-line blocking to a minimum. We expand further on how to do so in section 2.2.4.

- Compression for low bitrate radio bearers: Header compression mechanisms such as Robust Header compression (RoHC) and Ethernet Header compression (EHC) as well as data compression is present in NR to transfer low bitrate applications such as voice efficiently. For 6G, we expect similar services need to be supported and therefore propose to retain compression functionality from NR for low bitrate radio bearers.

- Data discard: Dropping data at the transmitter end is necessary for active queue management (AQM) purposes and to tackle buffer bloat due to stale data in case of latency sensitive applications. As part of 6G design, we need to ensure that the transmitter can discard data as needed, and the receiver is able to cope with discarded data.

Proposal 3: 6G user plane is designed with the following functionality:

- Early uplink resource acquisition

- Simplified reliable data transfer

- Data recovery at mobility

- Security of data, RRC and some L2 control information

- Data handling differentiation in the RAN using radio bearers

- Multiplexing of radio bearers into a physical layer transmission

- Segmentation and reassembly of data, while avoiding excessive memory requirements

- In-order delivery for applications

- Compression for low bitrate radio bearers

- Data discard

### Early uplink resource acquisition

Figure 4: Typical UL resource acquisition delays in NR

When an NR UE has UL data and no UL resources available to transmit the data, it transmits a scheduling request (SR) on pre-allocated dedicated SR resources to indicate its need to acquire UL resources to the base station. At this moment, the base station only knows that this UE has ‘some’ data to transmit but does not know how ‘much’ data is to be transmitted. To not waste radio resources, the base station usually allocates a small grant to the UE for it to indicate the amount of data in its UL buffers using a Buffer Status Report (BSR). On reception of the BSR, the base station can allocate an appropriately sized grant to the UE to transmit its data as illustrated in Figure 4. The current mechanism to acquire UL resources introduces significant latency to the start of an UL transmission. Table 1 provides a typical NR configuration in the field related to UL resource acquisition delays.

| Steps | Values |
| --- | --- |
| 0. UL data arrival | - |
| 1. Average SR delay | 10ms (20 slots) |
| 2. UL DCI | 0.5ms (1 slot) |
| 3. Scheduling delay (K2) | 1ms (2 slots) |
| 4. BSR transmission | 0.5ms (1 slot) |

Table 1: Typical UL scheduling configuration in NR

To counter this delay, NR introduced the configured grant (CG) feature with which the base station pre-allocates periodic UL resources to a UE. While useful for services with periodic traffic of deterministic size such as URLLC or voice calls, it is not spectrally efficient to use such a mechanism to serve aperiodic eMBB traffic, as resources need to be periodically reserved for specific UEs on the off chance that it may be used. Therefore, we seldom observe CG deployed in NR. Instead, what is often seen is that base stations randomly schedules small UL grants to the UE. Such a mechanism targets the SR delay in step 1 of Table 1. Avoiding step 1 delays would only be possible if the base station can perfectly predict UL data arrival timing. However, if base station cannot achieve ‘perfect’ prediction, step 1 delay will still exist while radio resources are wasted. Furthermore, if the UE is not configured to skip UL transmissions when there is no data, this mechanism also results in increased power consumption.

Observation 3: Initiating uplink transmissions in NR faces significant delay due to SR unavailability.

Figure 5: Proposed mechanism for early resource acquisition in 6G

Ideally, the base station should know of the UL data as soon as it arrives at the UE i.e. steps 1-3 in Table 1 are skipped and step 4 is initiated on data arrival. This would require the presence of UL resources that are always available for the UE to use for BSR transmissions while remaining spectrally efficient (i.e. avoiding the pitfalls of CG). We propose the use of a pool of always-available UL resources that are shared by several UEs. When UL data arrives at the UE, it immediately sends a BSR on this shared resource to acquire UL resources early as illustrated in Figure 5. Such a shared contention-based resource pool needs to be carefully designed to achieve a balance between low collision probability and keeping the resource overhead per UE to a minimum; and would require liaising with RAN1 early.

Proposal 4: Always-available contention-based uplink resources are introduced in 6G to reduce the latency associated with uplink data transmission initiation.

### Reliable data transfer

We have had two mechanisms to achieve data reliability for the past few generations viz. HARQ at MAC/PHY and ARQ at RLC. HARQ offers baseline reliability for all data transmissions while ARQ runs on top to additionally ensure no data loss for specific bearers. Fundamentally HARQ operates on units of transport blocks. If the receiver is unable to decode the transport block, feedback is sent to the transmitter to trigger retransmission of the transport block. If decoding of the transport block fails despite several retransmission attempts, HARQ gives up on this transport block. ARQ then kicks in to ensure that data from bearers that require a higher level of reliability than what HARQ offers are not lost.

Figure 6: Typical retransmission delays experienced in NR

As discussed in section 2.1.1, the HARQ retransmission delays are driven by the fundamental RTT, and the latency experienced due to data retransmission is as low as possible. ARQ relies on detection of packet losses at the receiver and a subsequent timer expiry before feedback is sent to the transmitter to trigger retransmissions. The typical time value seen in the field is 40ms, which means that while most of the data gets through to the receiver as quickly as possible due to HARQ, those packets that fail HARQ will experience a minimum latency of 40ms before retransmissions can be triggered.

ARQ uses a timer to trigger retransmissions because it has been designed independent of HARQ and is unaware when and if data got through HARQ successfully. This timer models the time it takes for the underlying HARQ mechanism to complete, to prevent ARQ retransmissions when HARQ retransmissions are ongoing. As such, it models the worst-case delays HARQ would experience leading to the case where all ARQ based retransmissions experience the worst-case HARQ delay as illustrated in Figure 6.

Observation 4: The consequence of designing ARQ independently of HARQ is that all ARQ retransmissions experience significant delays.

Another consequence of ARQ’s design based on such long delays is that it can lead to significant processing and memory overheads. A receiver is required to track the reception status of every byte of every SDU within the AM window which can go up to 131072 SDUs. Such a large window is a consequence of combining high data rates with large retransmission delays. The need to support high data rates remains for 6G, so we should look to designing a mechanism that can provide similar reliability for data transmission as today while reducing its latency, processing and memory overheads when compared to NR.

Observation 5: Existing ARQ design has significant processing and memory overheads.

Figure 7: Proposed simplified mechanism to reduce retransmission delays for 6G

Given that ARQ retransmissions are triggered only for data lost due to the HARQ procedure, ARQ and HARQ functionality can be combined into a single procedure that we refer to as SHARQ, as illustrated in Figure 7. HARQ like retransmissions can take place just as in NR. Before a transport block is to be dropped from the HARQ process, the transmitter would check the feedback status for the HARQ process. If the HARQ process has not been yet acknowledged, data from the transport block would be returned to higher layers to consider for retransmission. With such a joint design, we trigger retransmissions as soon as they are needed and avoid timer-based worst-case delays as seen with ARQ. It also opens up the opportunity to drop the AM window altogether and its associated processing and memory overheads. Like with the previous proposal, this is an area that needs to be designed together with RAN1 and would require liaising with them early.

Proposal 5: Combine ARQ and HARQ functionality into a single 6G procedure to improve latency while simplifying the stack.

### In-order delivery

Figure 8: Head of line blocking typically seen in the field with NR

An end user of mobile phone typically uses several applications in parallel. For example, it is common when playing a cooperative game to have a call ongoing with other players, while other applications run in the background. While the radio bearer concept has been part of 3GPP design for a very long time, the predominant scenario in the field is that all services for a user are transferred over the same radio bearer. This bearer is nearly always configured with in-order delivery, as several services (especially those that rely on TCP for transport) are sensitive to out of order data arrival. The consequence of transmitting data from multiple services over this common data pipe as widely seen in deployments today is head-of-line blocking, where the loss of a single packet affects all services as illustrated in Figure 8.

Observation 6: The use of a single radio bearer to transport all services for a UE as is typical in the field, leads to head-of-line blocking of all services at the receiver.

Figure 9: Proposed solution to limit head of line blocking with 6G

The use of a single bearer for several services is likely to be the norm in the 6G timeframe for several reasons such as net neutrality considerations or that these services share the same QoS enforcement. The reason for head-of-line blocking at the NR receiver is because all services in the bearer share the same sequence number (SN) space. Instead, if these services are assigned independent SN spaces, they can be handled separately at the receiver preventing one service from blocking another as illustrated in Figure 9. By doing so, we limit the impact of data loss only to the affected service(s) improving the latency experienced by other services, and we reduce the amount of data needed to be buffered at the receiver. To enable such operation, RAN needs to be service-aware, which needs further discussion over the course of the 6G study.

Proposal 6: 6G data radio bearers are designed such that one service does not cause head-of-line blocking of other services carried in the same bearer.

# Conclusion

In this contribution we have the following observations and corresponding proposals.

Design considerations and essential functionality:

Observation 1: Latency associated with eMBB data transfer will be an increasingly important metric to serve emerging use-cases during 6G’s lifetime.

Observation 2: Free from backwards compatibility constraints, this 6G study gives us a once-in-a-generation opportunity to design a communication system that improves latency performance associated with eMBB data transfer to benefit all 6G users.

Proposal 1: 6G user plane is designed to reduce the latency associated with eMBB data transfer compared to NR.

Proposal 2: 6G user plane is designed to be hardware-processing friendly while keeping memory requirements low.

Proposal 3: 6G user plane is designed with the following functionality:

- Early uplink resource acquisition

- Simplified reliable data transfer

- Data recovery at mobility

- Security of data, RRC and some L2 control information

- Data handling differentiation in the RAN using radio bearers

- Multiplexing of radio bearers into a physical layer transmission

- Segmentation and reassembly of data, while avoiding excessive memory requirements

- In-order delivery for applications

- Compression for low bitrate radio bearers

- Data discard

Early uplink resource acquisition:

Observation 3: Initiating uplink transmissions in NR faces significant delay due to SR unavailability.

Proposal 4: Always-available contention-based uplink resources are introduced in 6G to reduce the latency associated with uplink data transmission initiation.

Reliable data transfer:

Observation 4: The consequence of designing ARQ independently of HARQ is that all ARQ retransmissions experience significant delays.

Observation 5: Existing ARQ design has significant processing and memory overheads.

Proposal 5: Combine ARQ and HARQ functionality into a single 6G procedure to improve latency while simplifying the stack.

In-order delivery:

Observation 6: The use of a single radio bearer to transport all services for a UE as is typical in the field, leads to head-of-line blocking of all services at the receiver.

Proposal 6: 6G data radio bearers are designed such that one service does not cause head-of-line blocking of other services carried in the same bearer.

# References

- R2-164670 - Report of 3GPP TSG RAN WG2 meeting #94 held in Nanjing, China May 23 - 27, 2016

- 3GPP TS 38.300 V18.5.0 - NR and NG-RAN Overall Description; Stage 2

- 3GPP TS 36.300 V15.14.0 - E-UTRA and E-UTRAN Overall description; Stage 2

- 3GPP TR 38.838 V17.0.0 - Study on XR (Extended Reality) Evaluations for NR

- https://www.3gpp.org/technologies/xr-nr

- RP-251881 - New SID: Study on 6G Radio

- 3GPP TR 22.870 V0.3.1 - Study on 6G Use Cases and Service Requirements; Stage 1